3GPP TSG-RAN WG4 Meeting #114	R4-2501418

# Athens, Greece, 17 Feb. – 21 Feb., 2025

Source: 	vivo

Title: 	Further considerations on RRM for LP-WUR

Agenda Item:	7.26.4.2

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2] and RAN #105 [3].

The objectives of WI are duplicated as below:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

The initial discussion for this topic was initially discussed at RAN4 110bis meeting. The WF on this WI of RAN4 113 meeting can be found at [4]. The topic summary of RAN4 113 can be found at [6].

The following agreements were achieved at previous RAN4 meeting for RRM core part.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements

Agreement:

If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements

- based on LO periodicity

  - FFS whether to define an upper bound

  - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement.

Agreement:

When LP-SS is configured, apply the same agreement as for when LP-SS is not configured

- The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3

Agreement:

Scaling factor(s) is/are >= 4. Further down-select from

- Option 1:  1 fixed scaling factor

- Option 2: multiple values configured by NW

- Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo)

| DRX cycle length [s] | Scaling factor |
| --- | --- |
| 0.32 | TBD |
| 0.64 | TBD |
| 1.28 | TBD |
| 2.56 | TBD |

Issue 1-3-2: On Neighbour cell and serving cell relaxation factor

Agreement:

Same relaxation factor(s) applies to serving and neighbour cell measurements in idle/inactivate state.

In this contribution we provide our further consideration on the following topic.

Discussion

General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

- Proposals

Regarding case 2:

P1: Support (LG CMCC vivo);

P2: De-prioritize (xiaomi oppo);

P3: Not consider (ZTE Apple Ericsson Huawei MTK);

The case to be considered for LP-WUR has been discussed, case 1 and case 3 have already been agreed.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

The remaining listed cases at RAN4 111 meeting are case 2 and case 4 where case 4 is covered by higher priority frequency search hence only case 2 is considered here.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |

As we indicated a few times based on figure 1, case 2 is a natural solution if the threshold between case 3 and case 1 is not the legacy for triggering neighbour cell measurement, i.e., whether a new threshold will be defined for entry/exit for case 1 or the legacy threshold to trigger neighbour cell measurement (intra or inter) is reused as the threshold for entry/exit of case 1.

Having more thresholds will provide extra NW implementation flexibility, of course, with the cost of increasing implementation complexity. In practice the thresholds used for triggering neighbour cell measurements could be configured close the edge of the cell coverage. Under this scenario, if case 1 is triggered by using the legacy threshold for neighbour cell measurement, the performance of LP-WUR maybe not at ideal since it is close to LP-SS coverage.

Figure 1 Illustration on fully offloading and relaxed cases of LP-WUR

If a new threshold for case 1 is introduced, as illustrated in figure 1, case 2 is a natural extension of case 3 when neighbour cell measurement is not needed, as mentioned before. Compared with case 1, case 2 create a different trade-off point between power consumption and mobility performance, since the measurement of MR on the serving cell may also help mobility performance. Hence, for the RRM relaxation for MR and serving cell RRM measurement offloaded to LP-WUR, besides case 1 and case 3, it is preferred to consider case 2.

Alternatively whether have case 2 or not can be up to RAN2’s decision.

- For the “Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements” in the WID, besides agreed case 1 and case 3, case 2 is preferred to be considered. Alternatively whether have case 2 or not can be up to RAN2’s decision.

Issue 1-1-2: Threshold for switch between different cases

- Proposals

  - P1: Up to RAN2 decision (CMCC)

  - P2: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1). LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). (Apple)

  - P3: (Huawei)

  - For switch from Case #1 to Case #3 based on LR measurement.

    - Option 1: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ

    - Option 2: Use existing thresholds for SS-RSRP and SS-RSRQ, and define offsets for LP-RSRP and LP-RSRQ

  - For switch from Case #3 to Case #1 based on LR or both LR and MR measurements.

    - Option 1: LR measurement only

    - Option 2: both LR and MR measurements, and UE stops neighbor cell measurement when both measurements are below their corresponding thresholds

  - P4: Need to discuss whether case 2 will be considered or not first (LG)

  - P5: New LP-based threshold(s) based on LP-RSRP or LP-RSRQ used for switch into case 1 from other cases or exit from case 1 to other cases are needed (ZTE vivo)

For issue 1-1-2, to transfer from case 1 to case 3 or case 2 (to be determined), a LR-based threshold should to be defined since MR measurement is not available anyway. The alternative method proposed for switching from case 1 to other cases by using existing thresholds for SS-RSRP and SS-RSRQ plus offset(s) is not preferred since the offset should also be configured by NW and the total efforts is not reduced compared with introducing a new LP-based threshold. Introducing new thresholds based on LP-RSRP and LP-RSRQ is also consistent with the following RAN2 #127 meeting agreement:

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

- Introducing new LP-based threshold(s) based on LP-RSRP and LP-RSRQ used for switch/exit from case 1 to other cases is necessary and details should be up to RAN2.

Issue 1-1-3: Measurement requirements to be specified for LP-WUR

- Proposals

  - P1: Do not define requirements for OFDM-based LR serving cell measurement based on LP-SS; RAN4 defines measurement requirements for the following: (xiaomi CMCC ZTE Apple oppo)

    - Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state;

      - Same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS with and without overlaid OFDM sequence at Idle/Inactive state. (Apple)

    - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

  - P2: The measurement requirements of OFDM based LP-WUR based on LP-SS with or without overlaid OFDM sequence(s) will be the same as that of OOK based LP-WUR based on LP-SS. (Huawei vivo)

At RAN1 118bis meeting the following agreements are available:

Agreement

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

- Based on RAN1 agreement, overlaid OFDM sequences(s) for LP-SS is supported however there is no intention to introduce any new requirements. Based on discussion, there is no question to define the following requirements:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state.

- Considering the OFDM based LP-WUR, when it detects LP-SS signal, the LP-SS could be with and without overlaid OFDM sequence hence the corresponding requirements, if to be defined, can be classified as the following:

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state with overlaid OFDM sequence

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state without overlaid OFDM sequence

- Following RAN1’s agreement “no intention to introduce any new requirements”, the former requirements a, b can both reuse “requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state”, if to be defined.

- At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state.

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

For “measurement requirements for OFDM-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state with/without overlaid OFDM sequence”,  the requirement could reuse “requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state”.

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offlaoding/MR RRM measurement relaxation and LP-WUS monitoring

- Proposals

  - P1-1: Define same entry/exit conditions for LP-WUS monitoring and RRM measurement relaxed cases (case 3). (xiaomi LG Apple vivo).

  - P2: The LR-based LP-WUR measurement threshold can be less than to the LR-based LP-WUS monitoring threshold. (LG)

  - P3: RAN4 to suggest RAN2 to define same entry/exit condition for LP-WUS monitoring, serving cell RRM measurement offloading and MR RRM measurement relaxation. (Huawei)

- For this topic, the related RAN2 128 meeting agreement are copied below:

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

- Since RAN2 has already achieved noticeable progress on entry/exit conditions for these cases. It is suggested RAN4 postpone the discussion on this issue unless triggered by RAN2.

- For issue 1-1-7, on jointly consideration on Criteria (entry/exit conditions) for fully offlaoding/MR RRM measurement relaxation and LP-WUS monitoring, suggest to postpone the discussion unless triggered by RAN2.

Issue 1-1-8: LP-WUR operating carrier frequency

At RAN 106 meeting, the following agreement was achieved [7]:

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656).  For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered.

The solution 1, 2, 3 in [RP-242656] are copied below for reference:

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band)

- Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria)

- Solution 3:  MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation)

From RAN4 RRM requirement perspective, there is no direct impact before RAN2’s final decision on which intra-frequency case/solution will be used. To our understanding if solution 2 in [RP-242656] is adopted by RAN2, a cell selection is needed between paging and random access when the MR is in the RRM offloading case hence RAN4 may need define an interruption.

- For issue 1-1-8, LP-WUR operating carrier frequency, postpone the discussion until RAN2’s solution is stable.

- If solution 2 in [RP-242656] is adopted by RAN2, a cell reselection is needed between paging and random access when the UE is in the RRM offloading stage hence RAN4 may need define corresponding wake up delay requirement.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

- Proposals

  - P1: LR status is ON (xiaomi)

  - P2: LP-WUR can be ON or OFF for serving cell measurement. If the threshold for LP-WUS monitoring and LP-WUR RRM measurement are same the LP-WUR can be OFF, else if the threshold for LP-WUS monitoring is higher than for the LP-WUR RRM measurement the LP-WUR can be ON. (LG)

  - P3-1: LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation. (Apple)

  - P3-2: When a LP-based threshold is configured, at legacy case LP-WUR needs be “ON” at least some duration early before comparing with a LP-based threshold. (vivo Huawei Ericsson)

    - No need define any LR measurement requirements when a UE is at legacy case (vivo Huawei)

- For the status of LP-WUR after exiting LP-WUR monitoring, RAN2 has the following working assumption at RAN2 #127 and #127bis meeting:

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements

- The thresholds are summarized in the following table 1:

Table 1 Summary on entry/exit conditions for LP-WUS monitoring, case 1 and case 3

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | For serving cell RRM relaxation, at least MR threshold, FFS if LR measurement is needed. FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation | FFS |

- Since the entry conditions may include LP-based threshold, the LP-WUR should be ON to evaluate the LP-based threshold. In practice if the UE enters MR RRM relaxation or fully offloading cases early than enter into LP-WUS monitoring, there is no issue since the LP-WUR is already ON.

- However, one issue is when before starting LP-WUS monitoring, the UE is in the legacy scenario performing legacy serving cell or serving and neighbour cell measurement (either legacy Rel-15 scenario or legacy Rel-16 UE power saving scenario), then the LP-WUR still need “ON”, or at least to be “ON” some time before the entry conditions, following RAN2’s agreement. Since the LP-WUR only needs “ON” some time to perform measurement used to compare with LP-based threshold and when and how long the “ON” duration is up to UE implementation, it is suggested no requirement is defined for this scenario.

- At legacy state, when LP-WUR is ON for measurement and LR threshold evaluation is up to UE implementation.

- At legacy state, when LP-WUR measurement is needed to evaluate a LR threshold, LP-WUR needs be “ON” at least some duration early before the comparison, no requirement will be defined for this duration, i.e., no LP-WUR measurement and LP-WUR entry threshold evaluation requirements for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3).

Issue 1-1-10: Considerations on higher priority frequency layer

- Proposals

  - P1: Support relaxed higher priority frequency layer search (xiaomi CMCC CT ZTE vivo Ericsson)

  - P1-1 (CMCC)

    - When fulling offloading case happens (Case#1), the relaxed higher priority layer measurement requirement can be applied, i.e., per higher priority layer within 1h

    - When partial offloading case happens (Case#3), open to further discuss which higher priority layer measurement requirement should be applied

  - P1-2: For case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. (vivo Ericsson)

  - P2: Higher priority layer neighbor cell measurement shall also be OFF for case #1. (Apple)

  - P3: UE should perform measurement for higher priority frequency layers, if configured (Huawei)

    - If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements

    - If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed.

For higher priority frequency layer search, it could be stopped in case 1 since the MR is off in case 1. However since the frequency of higher priority frequency layer search is not high, especially considering the further relaxation defined in Rel-16, it is fine that the higher priority frequency layer search is performed in relaxed mode similar to what has been defined in Rel-16 UE power saving WI. For relaxed cases 3, higher priority frequency layer search could be performed in relaxed mode when relaxed conditions are satisfied.

In Rel-16, when the serving cell quality is higher than the threshold for inter-frequency neighbour cell measurement, relaxation on higher priority frequency layer search is performed as below and similar approach can be reused.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60.

- For case 1, higher priority frequency layer search could be OFF. Alternatively, for case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: LP-WUR Accuracy requirements

- Proposals

  - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (xiaomi LG Apple CT Huawei Ericsson MTK)

  - P2: Define LP-WUR measurement accuracy in dedicated section. Whether/how to reflect it in the core requirements can be further discussed after the simulation by taking the results into account (CATT)

Regarding the accuracy requirements, no dedicated accuracy requirement could be fine since it follows traditional RAN4’s methodology. However currently in RAN4 specs these margins are only defined in the sections for intra/inter/inter-RAT measurement requirements and there is no margin defined for the serving cell measurement. For LP-WUR, it is possible to introduce a margin to reflect accuracy performance when LP-WUR serving cell measurement results is used to compare with a threshold since it is a new introduced receiver.

- It is fine to use P1, “introduce a margin to reflect accuracy performance of LP-WUR in core specs”, when LP-WUR serving cell measurement results is used to compare with a threshold.

Issue 1-2-2: Lower bound for periodicity for LP-WUR measurement delay requirements

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

- Proposals

  - P1: RAN4 to define the lower limit for measurement delay requirements in case the periodicity of LP-SS are 80ms or 160ms. (xiaomi)

For this issue we have the following agreements in RAN4 112bis and RAN4 113 respectively.

Agreement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

For periodicity for SSB based LP-WUR measurement delay requirements:

Agreement:

If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements

- based on LO periodicity

  - FFS whether to define an upper bound

  - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement.

When LP-SS is configured, apply the same agreement as for when LP-SS is not configured

- The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

The LP-SS periodicity was still under discussion in RAN1. One issue is the 80ms or 160ms may be the candidate for LP-SS periodicity as well. There is one proposal to define a lower bound for LP-SS periodicity when LP-SS periodicity is either 80ms or 160ms with the intention to avoid too frequently measurement of LP-WUR.

In addition, whether an upper bound should be defined, the concern is the separation between two consecutive measurements may be too long when the LO periodicity is too largely, for example 2.56s.

Both the above suggestions are reasonable and it is preferred that the periodicity is still 320ms when the LP-SS periodicity is either 80ms or 160ms and suggest to discuss whether an upper bound is needed for the case when the LO periodicity is 2.56s.

- Suggest to consider the following lower bound for LP-SS and upper bound for SSB:

- Periodicity for measurement and evaluation delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode is 320ms if LP-SS periodicity 80ms and 160ms are determined to be included.

- Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s.

Issue 1-2-3-1: On LP-WUR/MR evaluation requirement when both MR and LR threshold are configured

- Proposals

  - P1: For the evaluation period for evaluating an entry threshold when both MR and LR threshold are configured, defined MR and LR evaluation requirements separately (vivo Huawei MTK)

  - P2: RAN4 need to further how to specify evaluation period for LP-WUS monitoring entrance if both LR and MR threshold configured. (Samsung)

  - P3: (LGE)

  - For the evaluation period for evaluating an entry threshold for LP-WUS monitoring or LP-WUR measurement

    - LP-SS based LP-WUR measurement period

  - For the evaluation period for evaluating exit threshold for fully offloading

    - The MR serving cell measurement period

This issue was discussed at previous RAN4 meeting however no consensus has been achieved. The entry/exit conditions for LP-WUS monitoring, case 1 and case 3 are summarized in table 1 which is duplicated below. Obviously for some conditions, for example entry conditions for fully offloading, both MR and LR are considered whereas for other conditions, like exit conditions for fully offloading, only LR is considered.

Summary on entry/exit conditions for LP-WUS monitoring, case 1 and case 3

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | For serving cell RRM relaxation, at least MR threshold, FFS if LR measurement is needed. FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation | FFS |

During the online discussion some concerns are whether need to consider the requirement for MR and LR jointly. Since we have already had the agreement that the measurement samples from different receivers will not be combined. Hence requirements for entry/exit threshold evaluation for MR and LR should be discussed separately.

- MR or LR evaluates whether its own entry/exit threshold is satisfied separately. MR and LR evaluation requirements should be discussed/defined separately.

Issue 1-2-3-2: On LR measurement requirement

For the LR measurement requirement, the following two options can be identified:

Option 1: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed.

Option 2: Define LR measurement requirement as the minimum duration to meet LR accuracy requirement. Assuming [x] samples and [y] samples are number of samples to meet LP-SS based LR and SSB based LR accuracy requirement, the measurement period is defined as [x] * LP-SS periodicity for LP-SS based LR and [y] * LO periodicity for SSB based LR.

Option 1 is similar to what we have in the serving cell RRM measurement and neighbor cell measurement. For example, for intra-frequency measurement, the current specs has “The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see Table 4.2.2.3-1, Table 4.2.2.3-2, Table 4.2.2.3-3, Table 4.2.2.3-4 or Table 4.2.2.3-5) for intra-frequency cells that are identified and measured according to the measurement rules.”

Following the former spec the detailed measurement requirement using option 1 could be: The UE shall use LP-WUR measure the LP-RSRP and/or LP-RSRQ of the serving cell at least once every Tmeasure, NR_LP-SS ( Tmeasure,NR_LP-SS = LP-SS periodicity) or measure the SSB of the serving cell at least every Tmeasure ,NR_LP-SSB (Tmeasure ,NR_LP-SSB = LO periodicity).

- For LR measurement requirements, the following options are available and option 1 is preferred:

- Option 1: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed.

- Option 2: Define LR measurement requirement as the minimum duration to meet accuracy requirement as [x] * LP-SS periodicity for LP-SS based LR or [y] * LO periodicity for SSB based LR, assuming [x] samples and [y] samples are number of samples to meet LP-SS based LR or SSB based LR accuracy requirement.

In addition, filtering related requirements should be considered similar to what we have for the MR.

- The UE shall filter the LP-RSRP/LP-RSRQ and SSB based measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either LP-SS periodicity/2 for LP-SS based LR or LO periodicity /2 for SSB based LR.

Issue 1-2-3: On requirements for LR for entry/exit threshold evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3)

  - P1: Do not need a dedicated evaluation delay requirement. Use the MR measurement period and/or LR measurement period to check the criteria for WUS paging monitoring/ Fully Offloading /MR RRM relaxation (CATT CMCC CT vivo Samsung)

  - P2: (Apple)

    - For WUS paging monitoring/MR RRM relaxation: use the MR measurement period and/or LR measurement period to check the criteria;

    - For MR fully offloading; use the MR measurement period and/or LR measurement period for measurement filtering and a dedicated evaluation delay requirement

  - P3: Define evaluation requirements for the exit condition based on single measurement period; Define evaluation requirements for the entry condition based on evaluation period which includes multiple measurement period; Each measurement should be based on filtering of at least 2 samples. (Huawei)

  - P4: (MTK)

    - RAN4 to define same requirements to evaluate the exit condition for each: 1) exiting from Case#1 to legacy UE and 2) exiting from Case#3 to legacy UE based on LR.

    - RAN4 to use the same evaluation requirements for entry condition when switching from Case#1 to Case#3 and from Case#3 to Case#1, where

      - entry conditions from Case#1 to Case#3 can be based on LR only.

      - entry conditions from Case#3 to Case#1 can be based on LR or LR+MR.

    - RAN4 to consider LR measurements only to define same requirements to evaluate the entry condition for each: 1) switching from Case#1 to Case#3 and 2) switching from Case#3 to Case#1; RAN4 to define requirements to evaluate the entry condition from legacy UE to Case#1 or Case#3 based on MR measurements only.

    - Discuss how to avoid the ping pong behaviour when transitioning between different cases based on RRM measurements.

  - P5: (LG)

    - RAN4 to define evaluation period for evaluating entry threshold for LP-WUS monitoring or LP-WUR measurement.

One issue is for the requirements for LP-WUR for entry threshold evaluation from legacy state to LP-WUS monitoring or RRM measurement fully offloading, whether to define any requirement or not under this scenario is still discussed by issue 1-1-9, in addition the entry conditions from legacy state to case 3 is not finished yet hence any agreements achieved in this issue will not apply for the following entry cases, which is also discussed in issue 1-1-9:

entry to LP-WUR monitoring from legacy case

entry to RRM measurement fully offloading (case 1) from legacy case

entry to RRM measurement relaxation (case 3) from legacy case

Except for the former scenarios, the LP-WUR evaluation requirement will apply for the following scenarios:

- The LR evaluation requirements will apply for the following scenario:

- exit to legacy case from case 1

- entry to case 1(RRM measurement fully offloading) from case 3

- entry to case 3(RRM measurement relaxation) from case 1

- [exit from LP-WUR monitoring case to legacy case]

- [exit to legacy case from case 3]

On single or multiple LR evaluation requirements

Based on available proposals so far, single or different LR evaluation requirements are discussed. Checking available proposals a few options can be identified:

Option A: Use same LP evaluation requirement for both LR entry and exit threshold evaluation for all related scenarios (do not have differentiation on entry or exit; or on different cases);

Option B: Consider multiple LR evaluation requirements:

- Option B-1: LR evaluation requirements is differentiated for entry threshold evaluation and exit threshold evaluation – for example LR evaluation requirement 1 for the entry threshold evaluation, LR evaluation requirement 2 for the exit threshold evaluation (LR evaluation requirement 1 is longer/tighter than LR evaluation requirement 2)

- Option B-2: LR evaluation requirements is differentiated on different cases - for example LR evaluation requirement 1 for the entry and exit threshold evaluation for case 1, LR evaluation requirement 2 for the entry and exit threshold evaluation for LP-WUS monitoring and case 3 (LR evaluation requirement 1 is longer/tighter than LR evaluation requirement 2)

For option B-2, since the entry condition to case 1 from case 3 could be the exit condition from case 3, or the exit condition to case 3 from case 1 could be the entry condition to case 3, there will be ambiguity and requirements may contradict each other. Hence option B-2 should not be considered.

For option B-1, for some cases, some considerations are the entry condition evaluation could be more cautious than exit condition evaluation. The motivation of these conditions is understandable however the necessity is low. Considering Table 1 the entry conditions for case 1 (RRM fully offloading) will be satisfied only when both MR and LR entry condition for case 1 are satisfied whereas the exit condition of case 1 is satisfied when only LR exit condition for case 1 is satisfied. Hence from the criteria design the above considerations has already been addressed and there is no necessity to have different evaluation requirement for entry/exit condition evaluation.

- Define a single LP evaluation requirement for both LR entry and exit threshold evaluation for all related scenarios, i.e., LP evaluation requirement is not differentiated on entry or exit; or on different cases.

- If LR evaluation requirements is differentiated on different cases, there will be ambiguity and requirements may contradict each other. And there is no necessity to have different LR evaluation requirement for entry/exit condition evaluation.

On how to define LR evaluation requirements

Based on previous discussion the following options are available:

- Option 1: Evaluation requirements for entry/exit evaluation is the same as that of LP-WUR measurement period.

- Option 2: Evaluation requirements for entry/exit conditions is longer than that of LP-WUR measurement period.

- Option 3: Evaluation requirements for entry evaluation is longer than that of LP-WUR measurement period; evaluation requirements for exit evaluation is the same as that of the LP-WUR measurement period.

Option 3 is regarding single or multiple evaluation requirements should be defined and has covered by the above analysis.

Both option 1 and option 2 are based on the assumptions that at Issue 1-2-3-2, the LR measurement period is defined by using option 2. Assuming [x] sample is used by a LR to satisfy accuracy requirement, either option 1 (sample number in the evaluation requirement is larger than x) or option 2 (sample number in the evaluation requirement is the same as x) is ok.

However one thing needs be further clarified is the format of the LR evaluation requirements. Based on the current specs, the evaluation requirement can either defined as a time window or as number of counting and the following options are available:

- Option i: LR evaluation requirement is defined as a time window - similar to idle state evaluation requirement for neighbor cell measurement; evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 >=x and y1>=y

- Option ii: LR evaluation requirements is defined as the counting duration where a threshold is fulfilled (entry) or not fulfilled(exit) for N consecutive LP_SS periodicity or LO periodicity; that threshold is evaluated at least once per LP_SS periodicity or LO periodicity

To our understanding the two options have little difference. The LR evaluation and measurement requirements is further illustrated in the following figure. Assuming 3 samples will satisfy the accuracy requirement, the evaluation can be defined as a duration corresponding to the duration (option i) to collect these 3 samples (option 1) or collect more samples (option 2). Example on evaluation requirement by using 4 samples is used in the figure. If evaluation requirement is defined as the counting duration, similar effects could be achieved. Actually by using a sliding window, no matter which format of the evaluation requirement, one evaluation result can be output when a new sample is available. Between option i and ii, option i is preferred due to its simplicity for specification.

Figure 2 LR Evaluation period – Option A when LR evaluation period is the same as its measurement period

- LR evaluation requirement is defined as a time window, i.e., evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 >=x and y1>=y

For illustration purpose, the measurement and evaluation requirements for LP-SS based and SSB based LR is illustrated below:

Table 2 LP-SS based LR measurement and evaluation requirement

| LP-SS periodicity [s] | Scaling Factor (N1) |  |  | Evaluation requirement Tevaluate, LR_LP-SS (number of LP-SS cycles) | Measurement requirement Tmeasure, LR_LP-SS [s] (number of LP-SS cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1 | FR2-2 |  |  |
| 0.32 | 1 |  |  | Using option i, then Option A [x1] x N1, (X1>=x) | Option 1: N1 Option 2: [x]*N1 |

Table 3 SSB based LR measurement and evaluation requirement

| LO periodicity [s] | Scaling Factor (N1) |  |  | Evaluation requirement Tevaluate,LR_SSB (number of LO cycles) | Measurement requirement Tmeasure, LR_SSB [s] (number of LO cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 | FR2-2 Note2 |  |  |
| 0.32 | 1 |  |  | Using option i, then option A: [y1] x N1, (y1>=y) | Option 1: N1 Option 2: [y]*N1 |
| 0.64 |  |  |  |  | … |
| 1.28 |  |  |  |  | … |
| 2.56 |  |  |  |  | …. |

Issue 1-2-4: On requirements for MR for entry/exit threshold evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3)

The MR measurement and evaluation should also be defined for LP-WUR monitoring, case 1 and case 3. Based on table 1, the application of the MR evaluation requirements is described in the following propose.

- The MR evaluation requirements will apply for the following scenario:

- Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case

- [exit to legacy case from case 3]

- entry to case 1(RRM measurement fully offloading) from case 3

In the legacy specification, the MR measurement and evaluation for the serving cell is quite mature and it is suggested to reuse as much as possible unless some numbers.

For the cell selection evaluation, the current specs are:

“…the UE has evaluated according to Table 4.2.2.2-1 or Table 4.2.2.2-4 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.”

To our understanding, the former specs should be reused for MR evaluation requirement for LP-WUR monitoring, case 1 or case 3 and only the value of Nserv needs further investigation. Regarding detail requirements, firstly whether M1 needs kept or not needs be discussed and current M1 is kept. Regarding Nserv_WUR, 3 options are provided. Option 1 is simply reuse the current Nserv value. Option 2 reduce current Nserv value by half considering the threshold for LP-WUR monitoring, case 1 and case 3 should be much higher than the cell selection threshold hence less samples could be used for evaluating purpose.

At Rel-16 UE power saving WI, the fulfilment of the not at cell edge criteria is defined as below,

The evaluation duration is not explicitly specified however it was agreed in [8] that:

- The evaluation rate for measurement relaxation mode triggering shall be the same as current serving cell evaluation rate defined in 38.133.

And the serving cell evaluation rate defined in 38.133 is copied below:

4.2.2.2	Measurement and evaluation of serving cell

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle;

Hence one evaluation can be performed in M1*N1 DRX according to Rel-16 UE power saving WI, which is the option 3 in the table. 4.

Table 4 Nserv_WUR for MR evaluation requirement for LP-WUR monitoring, case 1 and case 3

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Option 1: Nserv_WUR [number of DRX cycles] | Option 2: Nserv_WUR [number of DRX cycles] | Option 3: Nserv_WUR [number of DRX cycles] |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1 | FR2-2 |  |  |  |
| 0.32 | 1 |  |  | M1*N1*4 | M1*N1*2 | M1*N1 |
| 0.64 | 1 |  |  | M1*N1*4 | M1*N1*2 | M1*N1 |
| 1.28 | 1 |  |  | N1*2 | N1*1 | N1 |
| 2.56 | 1 |  |  | N1*2 | N1*1 | N1 |

- For MR evaluation requirement for LP-WUR monitoring, case 1 and case 3, consider the following options: option 1: reuse Nserv ; option 2 reduce Nserv value by half; Option 3: without Nserv ,i.e., Nserv =1.

Issue 1-2-5: On MR wake up delay related requirements

- Proposals

  - P1: Wait for RAN1 further progress or based on RAN1’s conclusion. (CATT CMCC oppo Huawei)

  - P2: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. (Apple)

  - P3: Suggest to use the term “MR activation” time to replace “interruption” time; Defining interruption requirement for the normal case and worst case where a cell reselection is included in the worst case, ramping duration is based on RAN1 conclusion, detailed conditions are FFS. (vivo)

    - “MR activation” duration = Ramping duration + (2 + 2) samples * SSB_ periodicity

    - “MR activation” duration = Ramping duration + (2 + 2 + 23) samples * SSB_ periodicity + TSI-NR + 2*Ttarget_cell_SMTC_period

  - P4: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement; MR sync time, need to be further discuss with different side conditions (Samsung)

    - Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g.  1~2 SSB sufficient

    - Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required

- For issue related to MR wake up delay, the following alternatives will be considered by RAN1:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

- Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}.

- Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time.

  - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]}

  - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively.

- Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

At previous RAN4 meeting, it is also indicated in P1 that RAN4 could wait for RAN1’s progress on this issue. The Alt 4 in the above alternatives in RAN1’s conclusion also suggests the wake-up delay is up to RAN4’s decision. To our understanding it is better for RAN4 to determine or at least provide reference value from RAN’4 perspective on the wake-up delay for MR.

Regarding the components to be considered in this “wake up delay”, one component should be definitely included is ramping time which has been discussed in RAN1. Based on the RAN1 118bis meeting agreements as below, multiple values for ultra-deep sleep based on RAN1 agreements should be considered in RAN4 requirements. To our understanding, this ramping time has already included any uncertainty from LP start to trigger till MR get this message.

Agreement

Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state.

The three candidate value could be {[20ms, 400ms, 800ms]} or {[80ms], [500ms] and [900ms]} from the above alternatives in RAN1 119 meeting and it is up to RNA1’s decision.

Another component to be considered is the time for a MR to get synchronous after triggered from ultra-deep mode, this component will consist of at least the time for AGC, raw timing sync and other related procedure if necessary. Typically 1-2 SSBs (1-2 samples) are needed for AGC and raw timing sync respectively. When the MR is wake up by LP-WUS, the normal case is that its serving cell measurement should be in good status and a few samples (1-2 samples) is sufficient for MR measurement purpose even when the MR is in offloading or RRM relaxation mode otherwise it will exit these modes. And if the worst case is considered, where the MR may immediately need a cell reselection procedure after being waking up. Under this scenario, two extra components should be considered. The first one is the component for cell detection requirements, i.e., how many samples a UE needs to detect a newly cell. The other component is the maximum paging interruption due to SI reading defined in the current spec which is TSI-NR + 2*Ttarget_cell_SMTC_period can be reused.

To our understanding, no matter the normal case or the worst case, the MR should perform measurement as soon as possible after being waken up hence the measurement should follow SSB periodicity instead of DRX periodicity. Since the MR wake up is only a transient period, the impact on UE power consumption is negligible.

- After MR is triggered to be waken up and during the waking up duration, MR should follow SSB periodicity instead of DRX periodicity to perform measurement.

Following the above principle, the wake up delay for normal case is:

“wake up delay” = Ramping duration + [1(AGC) + 1 (raw timing) + 2 (measurement)] samples * SSB periodicity

For the worst case where a cell reselection procedure is needed, although these cases may be rare, it may need be covered. The components to be considered for this case has been provided above. Using the number of samples corresponding to detection requirement when DRX is 1.28s as an example, i.e., 25 samples, the “wake up delay” duration is:

“wake up delay” = Ramping duration + (1 + 1 + 25) samples * SSB periodicity + TSI-NR + 2*Ttarget_cell_SMTC_period

- Defining MR wake up requirement for the normal case and the “wake up delay” duration for the normal is:

- “wake up delay” duration = Ramping duration + [5] samples * SSB periodicity

- Whether to further consider a worst case where a cell reselection is needed after ME is wake up needs FFS.

Issue 1-2-6: On LP-WUR cell selection evaluation requirements for case 1

- Proposals

  - P1: No need to define the LR-WUR based cell selection criterion evaluation in case 1. (Apple vivo Huawei MTK)

  - P2: Define LP-WUR based serving cell selection criterion S evaluation requirement in case 1. (CATT Ericsson)

  - P2-1：The existing MR based evaluation requirements can be the baseline (CATT)

For the cell selection evaluation requirement for case 1, the cell selection for LP-WUR is used to trigger MR to perform neighbor cell measurement, as indicated in P3-1, however the exit conditions of case 1 serve the same purpose and may even with a higher exit threshold hence the necessity to have a cell selection evaluation requirements for LP-WUR is low.

- For issue on LP-WUR cell selection evaluation requirements for case 1, support P2, i.e., no need to define the LR based cell selection criterion evaluation in case#1.

Issue 1-2-7: LR measurement metrics used for entry/exit threshold evaluation for LP-WUS monitoring/RRM relaxation

- Proposals

  - P1:  Following RAN2’s conclusion, For LP-WUR, LR measurement metrics have (LP-)RSRP and optional (LP-)RSRQ. (LG Apple)

  - P2: For LP-WUS monitoring, RAN4 to discuss whether RSRP and RSRQ measurement from LR can reflect the impact of the inter-carrier interference. (Huawei)

Recommendations:

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ agreed at RAN1 118 meeting are copied below:

Agreement

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

- LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS.

- LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

However, the reference point of the above measurement metrics has not yet been discussed and usually the reference point of measurement metrics should be discussed within RAN4.

The reference point of SS-RSRP and SS-RSRQ from TS38.215 are copied below:

5.1.1	SS reference signal received power (SS-RSRP)

| Definition | … For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches. |
| --- | --- |

5.1.3	SS reference signal received quality (SS-RSRQ)

| Definition | … For frequency range 1, the reference point for the SS-RSRQ shall be the antenna connector of the UE. For frequency range 2, NR Carrier RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch, where the combining for NR Carrier RSSI shall be the same as the one used for SS-RSRP measurements. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRQ value shall not be lower than the corresponding SS-RSRQ of any of the individual receiver branches. |
| --- | --- |

During the NR reference point discussion, the reference point for FR1 is relative straightforward and it took few meetings to achieve consensus for FR2 reference point. For the LP-WUR, for FR1 the reference point for SS-RSRP can be reused. For FR2, the reference point definition is up to further discussion.

- For frequency range 1, the reference point for the LP-RSRP and LP-RSRQ shall be the antenna connector of the UE.

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

For accuracy for MR, margins, which reflect accuracy requirements, are directly defined for intra-frequency/inter-frequency and inter-RAT measurements. However margins are not defined for serving cell measurement when serving cell measurement is used to compared with a threshold, either for cell selection or for triggering intra/inter frequency measurement or for triggering Rel-16 RRM relaxation.

Since the margin has never been defined for the MR serving cell measurement before, following the same logic, it is not necessary to introduce any new margin when the MR serving cell measurement is used to compare with thresholds for LP-WUR operation.

Option 1: For MR, margin is also introduced when MR serving cell measurement result comparing with a threshold, this margin will be based on legacy MR accuracy requirements.

Option 2: For MR, margin is not also introduced when MR serving cell measurement result comparing with a threshold as the legacy specs.

Alternatively P2 is used for issue 1-2-1.

- No new accuracy requirements will be introduced for MR either normal or relaxed serving cell measurement, i.e., no margin will be introduced for MR serving cell measurement.

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

- Proposals

  - P1: The legacy accuracy for relaxed MR measurement should be reused if Case#2 and/or Case#3 introduced. (CMCC vivo Huawei)

For the accuracy requirements for relaxed MR measurement, the legacy requirements are used during Rel-16 UE power saving WI. It is straightforward to the use same principle, i.e., use the legacy accuracy requirements for relaxed MR measurement.

- For relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

Issue 1-2-10: LP-WUS monitoring requirements while UE is not required to monitor paging

- Proposals

  - P1: Define requirements for LP-WUS monitoring while UE is not required to monitor paging

  - UE may stop paging monitoring once the LP-WUS monitoring conditions are fulfilled. During this time, LP-WUS monitoring requirements shall apply.  (Nokia)

Issue 1-2-11: LP-WUS monitoring and MR relaxation requirements

- Proposals

  - P1: Define LP-WUS monitoring requirements independently from the MR measurement relaxation and offloading requirements (Nokia)

  - MR relaxation requirements while UE is monitoring LP-WUS are defined independently from legacy requirements.

For this issue, it is not clear which kind of LP-WUS monitoring requirements to be defined. Considering the legacy MR idle/inactive requirements where there is no paging monitoring requirements except for maximum interruption in paging reception, it is suggested that RAN4 does not define LP-WUS monitoring requirements.

- RAN4 does not define LP-WUS monitoring requirements

MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

At RAN4 113 meeting, the following agreements are available:

Scaling factor(s) is/are >= 4. Further down-select from

- Option 1: 1 fixed scaling factor

- Option 2: multiple values configured by NW

- Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo)

| DRX cycle length [s] | Scaling factor |
| --- | --- |
| 0.32 | TBD |
| 0.64 | TBD |
| 1.28 | TBD |
| 2.56 | TBD |

Issue 1-3-2: On Neighbour cell and serving cell relaxation factor

Agreement:

Same relaxation factor(s) applies to serving and neighbour cell measurements in idle/inactivate state.

For the 3 options in issue 1-3-1 for down-select, the pros and cons for each options are summarized below:

Table 1 Pros and cons of options for MR RRM relaxation

|  | Pros | Cons |
| --- | --- | --- |
| Option 1: 1 fixed scaling factor | Simple and consistent with Rel-16 design | Less flexibility |
| Option 2: multiple values configured by NW | Flexibility is good, provide multiple trading points on power saving gain and RRM performance | May have extra effort on SIB reading, NW configuration may be conservative. Or relative long measurement period when both DRX value and scaling factor is large |
| Option 3: multiple values, scaling factor value depends on DRX cycle length | Flexibility is good, provide multiple trading points on power saving gain and RRM performance. | N/A |

Following former analysis, option 3 and option 1 is preferred. If option 3 is agreed to be considered, it is suggested the following scaling factors to be considered:

| DRX cycle length [s] | Scaling factor |
| --- | --- |
| 0.32 | 16 |
| 0.64 | 16 |
| 1.28 | [16] |
| 2.56 | [8] |

The reason for selecting the former scaling factor is based on investigation of the power saving gain when MR enters RRM relaxation mode compared with I-DRX with or without PEI has been carried out at SI phase for LP-WUS duty-cycled monitoring and continuously monitoring, the results are captured in the section 8.1.1.5 and 8.1.1.6 in [9] and summarized in the following table:

Table 1 Power saving gain against MR relaxation times

| MR RRM relaxation times | mean power saving gain average (LP-WUR duty-cycled mode) | mean power saving gain average (LP-WUR continuously mode) |
| --- | --- | --- |
| MR relaxed < 8 times | average: 4%, | average: 22% |
| 8 times<= MR relaxed <=16 times | average: 40%, | average: 32% |
| MR relaxed > 16 times | average: 60%, | average: 57% |
| MR offload RRM to LR | average: 83%, | average: 74% |

From the above outcome apparently the fully offloading mode where MR does not perform any RRM measurement achieves the maximum power saving gain when the serving cell RRM measurement is offloaded to LP-WUR. It is also observed a significant power saving gain increase happens when the relaxation times of MR changes from less than 8 to between 8 and 16 for the LP-WUR duty-cycled mode. In general, 8 relaxation times is needed to achieve a reasonable 40% power saving gain base on the former table.

- For MR RRM relaxation for serving cell/neighbour cell, option 3 and option 1 is preferred. For option 1, the value could be down-select from [8 16]; for option 3, it is suggested the following scaling factors to be considered:

| DRX cycle length [s] | Scaling factor |
| --- | --- |
| 0.32 | 16 |
| 0.64 | 16 |
| 1.28 | [16] |
| 2.56 | [8] |

LP-WUR connected mode

Issue 1-4-1: LP-WUR at CONNECTED mode

- Proposals

  - P1: No RRM objectives is expected for connected mode in this WI. (CATT xiaomi vivo LG ZTE Apple MTK)

  - P2: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress. Paging reception requirement impact can be discussed after further input from RAN2 and RAN1 (Samsung)

  - P3: Postponed until more progress in other groups (Ericsson Huawei)

  - P4: RAN4 to discuss interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. (Qualcomm)

    - Postpone the discussion on this issue (vivo)

Recommendations:

The majority view is there is no RAN4 actions on CONNECTED state. However it is also possible for issues being identified depending on RAN1/2 progress. Hence the discussion can be postponed until more progress.

Regarding P4, the assumption is that MR and LR works at different time, when the WUS monitoring occasion collides with a MG, one possible solution is the MR uses the MG to perform measurement and LR monitors LP-WUR after MG. However the detailed design could be up to RAN1 design and since there is no RAN4 scope on CONNECTED mode, it is suggested to postpone the discussion on this issue.

- For issue 1-4-1, postpone the discussion.

Other issues

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

- Proposals

  - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple)

  - P2: The principle of P1 is reasonable. The issue can be discussed later (vivo)

  - P3: The methodology of EMR requirement definition under each Cases can refer to the methodology of high priority layer measurement requirement definition. (CMCC)

  - P4: Discuss the UE behaviour while MR relaxation is applied to idle-mode measurements. (Nokia)

    - If T331 timer is configured UE shall not enter LP-WUS mode (MR relaxation / offloading) (e.g. for the first 300 seconds in idle-mode)

    - RAN4 to also discuss if the rel-18 idle-mode behavior follows the same principle where UE shall keep the MR ON.

Regarding EMR related issue, to our understanding the principle P1 is reasonable. The issue itself is not crucial for Rel-19 requirements point of view and can be discussed later.

- For issue 1-5-1, the principle of P1 is reasonable. The issue can be discussed later.

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

- If solution 2 in [RP-242656] is adopted by RAN2, a cell reselection is needed between paging and random access when the UE is in the RRM offloading stage hence RAN4 may need define corresponding wake up delay requirement.

- If LR evaluation requirements is differentiated on different cases, there will be ambiguity and requirements may contradict each other. And there is no necessity to have different LR evaluation requirement for entry/exit condition evaluation.

- For the “Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements” in the WID, besides agreed case 1 and case 3, case 2 is preferred to be considered. Alternatively whether have case 2 or not can be up to RAN2’s decision.

- Introducing new LP-based threshold(s) based on LP-RSRP and LP-RSRQ used for switch/exit from case 1 to other cases is necessary and details should be up to RAN2.

- At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state.

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

For “measurement requirements for OFDM-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state with/without overlaid OFDM sequence”,  the requirement could reuse “requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state”.

- For issue 1-1-7, on jointly consideration on Criteria (entry/exit conditions) for fully offlaoding/MR RRM measurement relaxation and LP-WUS monitoring, suggest to postpone the discussion unless triggered by RAN2.

- For issue 1-1-8, LP-WUR operating carrier frequency, postpone the discussion until RAN2’s solution is stable.

- At legacy state, when LP-WUR is ON for measurement and LR threshold evaluation is up to UE implementation.

- At legacy state, when LP-WUR measurement is needed to evaluate a LR threshold, LP-WUR needs be “ON” at least some duration early before the comparison, no requirement will be defined for this duration, i.e., no LP-WUR measurement and LP-WUR entry threshold evaluation requirements for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3).

- For case 1, higher priority frequency layer search could be OFF. Alternatively, for case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search.

- It is fine to use P1, “introduce a margin to reflect accuracy performance of LP-WUR in core specs”, when LP-WUR serving cell measurement results is used to compare with a threshold.

- Suggest to consider the following lower bound for LP-SS and upper bound for SSB:

- Periodicity for measurement and evaluation delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode is 320ms if LP-SS periodicity 80ms and 160ms are determined to be included.

- Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s.

- MR or LR evaluates whether its own entry/exit threshold is satisfied separately. MR and LR evaluation requirements should be discussed/defined separately.

- For LR measurement requirements, the following options are available and option 1 is preferred:

- Option 1: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed.

- Option 2: Define LR measurement requirement as the minimum duration to meet accuracy requirement as [x] * LP-SS periodicity for LP-SS based LR or [y] * LO periodicity for SSB based LR, assuming [x] samples and [y] samples are number of samples to meet LP-SS based LR or SSB based LR accuracy requirement.

- The UE shall filter the LP-RSRP/LP-RSRQ and SSB based measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either LP-SS periodicity/2 for LP-SS based LR or LO periodicity /2 for SSB based LR.

- The LR evaluation requirements will apply for the following scenario:

- exit to legacy case from case 1

- entry to case 1(RRM measurement fully offloading) from case 3

- entry to case 3(RRM measurement relaxation) from case 1

- [exit from LP-WUR monitoring case to legacy case]

- [exit to legacy case from case 3]

- Define a single LP evaluation requirement for both LR entry and exit threshold evaluation for all related scenarios, i.e., LP evaluation requirement is not differentiated on entry or exit; or on different cases.

- LR evaluation requirement is defined as a time window, i.e., evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 >=x and y1>=y

- The MR evaluation requirements will apply for the following scenario:

- Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case

- [exit to legacy case from case 3]

- entry to case 1(RRM measurement fully offloading) from case 3

- For MR evaluation requirement for LP-WUR monitoring, case 1 and case 3, consider the following options: option 1: reuse Nserv ; option 2 reduce Nserv value by half; Option 3: without Nserv ,i.e., Nserv =1.

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Option 1: Nserv_WUR [number of DRX cycles] | Option 2: Nserv_WUR [number of DRX cycles] | Option 3: Nserv_WUR [number of DRX cycles] |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1 | FR2-2 |  |  |  |
| 0.32 | 1 |  |  | M1*N1*4 | M1*N1*2 | M1*N1 |
| 0.64 | 1 |  |  | M1*N1*4 | M1*N1*2 | M1*N1 |
| 1.28 | 1 |  |  | N1*2 | N1*1 | N1 |
| 2.56 | 1 |  |  | N1*2 | N1*1 | N1 |

- After MR is triggered to be waken up and during the waking up duration, MR should follow SSB periodicity instead of DRX periodicity to perform measurement.

- Defining MR wake up requirement for the normal case and the “wake up delay” duration for the normal is:

- “wake up delay” duration = Ramping duration + [5] samples * SSB periodicity

- Whether to further consider a worst case where a cell reselection is needed after ME is wake up needs FFS.

- For issue on LP-WUR cell selection evaluation requirements for case 1, support P2, i.e., no need to define the LR based cell selection criterion evaluation in case#1.

- For frequency range 1, the reference point for the LP-RSRP and LP-RSRQ shall be the antenna connector of the UE.

- No new accuracy requirements will be introduced for MR either normal or relaxed serving cell measurement, i.e., no margin will be introduced for MR serving cell measurement.

- For relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

- RAN4 does not define LP-WUS monitoring requirements

- For MR RRM relaxation for serving cell/neighbour cell, option 3 and option 1 is preferred. For option 1, the value could be down-select from [8 16]; for option 3, it is suggested the following scaling factors to be considered:

| DRX cycle length [s] | Scaling factor |
| --- | --- |
| 0.32 | 16 |
| 0.64 | 16 |
| 1.28 | [16] |
| 2.56 | [8] |

- For issue 1-4-1, postpone the discussion.

- For issue 1-5-1, the principle of P1 is reasonable. The issue can be discussed later.

- References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

[2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

[3] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 105, vivo, NTT DOCOMO

[4] R4-2420100, WF on RRM requirements for NR_LPWUS, RAN4 113, vivo

[5] TS38.133, V18.4.0

[6] R4-2418284,	Topic summary for [113][226] NR_LPWUS, RAN4 113,vivo

[7] RP-243266, Way forward on Rel-19 LP-WUS separate band issues, RAN # 106

[8] R4-2009265, WF on RRM measurement relaxation for Power Saving, RAN4 96e, CATT