3GPP TSG-RAN WG4 Meeting #114		R4-2501460

Athens, GR, 17th – 21st February, 2025

Agenda item:		7.10.3.4

Source:		Samsung

Title:	Discussion on RRM requirements of Rel-19 NR-NTN in less than 5MHz

Document for:		Discussion

- Introduction

In previous RAN4 meetings, we have already discussed all the RRM requirements and latest achieved agreements have been approved in WF [1]. There is only one open issue left. In this contribution, we provide our view on this open issue in RRM core requirements for less than 5MHz for NR-NTN.

- Discussion

In RAN4#113 meeting, RAN4 achieved agreements in [1]. There is only one open issue listed as below:

### Issue 1-1-2: Measurement(SSB index detection requirements) Wayforward: - Option 1: - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Option 2: Two sets of requirements - Set 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Set 2: smaller delay with higher SNR side condition. - FFS on the exact number of delay to be extended base on the simulation results under the same simulation assumptions for TN less than 5MHz [R4-2306400] - FFS on the separate UE capability to support set 2 requirements

In current TN less than 5MHz scenario, for SSB based L3 measurement, time period for time index detection of both intra-frequency measurement without MG or with MG, 4 additional samples are needed. For inter-frequency measurement without MG or with MG, 3 additional samples are added.

In TN less than 5MHz system, the simulation results were captured in [2].

Compare the difference between NTN 3MHz system and TN 3MHz system in simulation, the only difference is the channel model, e.g. AWGN/TDL-A/B/C/AWGN+500km/h were used in TN simulation while NTN-TDLC (LOS), NTN-TDLA can be used in NTN simulation.

As mentioned in last meeting, we think the system parameters in physical layer are the same in NTN and TN systems. In addition, the side condition in NTN and TN are also the same. If the side conditions in NTN and TN systems are the same, there is no further reason to use different values of number of samples in NTN and TN systems. Therefore, we propose to use option 1 above to reuse 4/3 additional samples for intra-frequency measurement and inter-frequency measurement, respectively.

In last RAN4#113 meeting, the proponent of above option 2 proposed two sets requirements for NTN 3MHz system. We understand the motivation to have a better side condition to shorten the delay requirements. However, our concern is the side condition is the general factor in the system.

The Es/Iot condition is only known at UE side. From network perspective, NW doesn’t know what is the condition of SNR levels for UEs.

The potential solution in Option 2 is by using different side conditions based on UE capability. Some UEs with this advanced UE capability can measure with faster duration in better side condition which other UEs still have the same behavior like additional 4/3 samples with the same side condition as that in TN system.

Firstly, this UE capability can be indicated after RRC connected. However, side condition is general such as for SS-RSRP is also used for RRC_Idle mode.

Even if just say it in L3 measurement, when there are different UE types in the same network, they have different behaviors like one UE can meet the condition of event triggered reporting in the measObjects while the other UE cannot. We cannot observe benefit for network optimization by this different UE capabilities.

In addition, although NTN system doesn’t support CA until this release, if go in the direction as option 2, it will bring further potential issue as some different side conditions exist in different cells with different configurations.

Unless further benefits for option 2 can be provided, we still support option 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively.

Proposal 1: For time index detection requirement, reuse same 4/3 additional samples for intra-frequency measurement and inter-frequency measurement, respectively.

Proposal 2: For Intra-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(7 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

For Intra-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

For Inter-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320ms | Ceil(6 x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

For Inter-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, Ceil(6 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter K_satellite |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(6 1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter K_satellite |
| DRX cycle > 320ms | Ceil(6 x Kgap) DRX cycle CSSFinter K_satellite |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

- Conclusion

In this contribution, we provide our proposals for RRM core requirements for less than 5MHz for NR-NTN:

Proposal 1: For time index detection requirement, reuse same 4/3 additional samples for intra-frequency measurement and inter-frequency measurement, respectively.

Proposal 2: For Intra-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(7 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

For Intra-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

For Inter-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320ms | Ceil(6 x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

For Inter-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, Ceil(6 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter K_satellite |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(6 1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter K_satellite |
| DRX cycle > 320ms | Ceil(6 x Kgap) DRX cycle CSSFinter K_satellite |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

- Reference

[1] R4-2420091, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi

[2] R4-2321344, Summary of simulation results for RRM requirements for less than 5MHz, Summary of simulation results for RRM requirements for less than 5MHz, Nokia, Nokia Shanghai Bell