3GPP TSG-RAN WG4 Meeting #114	R4-2501475
Athens, Greece, 17th – 21nd February, 2025

Agenda item:	7.10.2.3.1

Source: 	OPPO

Title: 	Discussion on Tx requirements for IoT-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Tx RF requirements with IoT NTN operating in IoT-NTN bands were discussed and some agreements are approved in WF [1]:

Issue 1-1-1: For NB-Iot Based Iot-NTN, whether and how to define GSM_ACLR. - Proposals - Option 1: Remove this requirement, [at least for HPUE] - Option 2: Keep this requirement and reusing the PC3 requirement for HPUE. - Option 3: Others - Agreement: - Remove GSM_ACLR for NB-IoT based NTN HPUE. Issue 1-1-2: PC1/2 ACLR should not below PC3 requirements, based on the assumption that PC3 requirement can be relaxed if necessary. - Proposals - Option 1: Yes. - Option 2: No - Option 3: Others - Agreement: - HPUE ACLR should not be relaxed than that for PC3 for NB-IoT NTN. Issue 1-1-3: For NB-Iot based NTN, considering to extend the SEM for PC2 and PC1 as following - Agreement: For NB-Iot based NTN, consider PC3 SEM as a starting point. More evaluation can be considered. Issue 1-1-4: SEM relaxation for the case UE CBW is much narrower compared to BS CBW - Proposals - When UE PC2 ACLR is decided, regarding SEM for IoT-NTN HPUE, if no specific concern, consider relaxing UE SEM requirements within SAN CBW but still meet ITU SEM regulatory requirements. - WF - FFS whether MPR reduction technique currently discussed in Rel-19 can be considered here Issue 1-1-5: NB-Iot based Iot-NTN, MPR=0 reference point - Proposals - Proposal 1. Considering center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation. - Option 2: Others - Agreement: - As the starting point, consider the center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation. Issue 1-1-6: NB-Iot based Iot NTN, P-MPR has not been defined. - Proposals - Revise the configured transmitted Power section for NB-Iot Based Iot-NTN in 36.102 to incorporate P-MPR. Details FFS. - Agreement: - Revise the configured transmitted Power section for NB-Iot Based Iot-NTN in 36.102 to incorporate P-MPR. - Details are FFS. - Other solutions for SAR are not precluded. Issue 1-1-7: General guideline for eMTC based Iot-NTN, - Proposals - As a general guideline, for eMTC based Iot-NTN, the HPUE requirements can using legacy LTE requirements as starting point [unless otherwise stated.] - WF - FFS Issue 1-1-8: The nominal MOP tolerance] - Background: - HD-FDD FE IL is obviously lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP. - From EC Decision 2016/687, at least a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread. - As for IoT-NTN eMTC and NB1/NB2, the actual NTN-IoT PC2 HD-FDD TX output power upper bound could be 28.8dBm (i.e., 26.8dBm+2dB) but exceeds conventional PC2 TX upper power bound of 28dBm. - From ETSI EN 301 908, PC3 TX output power upper bound can be specified higher (i.e., 23dBm +2.7dB or 23dBm +3dB). - Proposals As for IoT-NTN eMTC and NB1/NB2, regarding solution for PC2 HD-FDD TX MOP upper bound higher than FD-FDD TX MOP upper bound, consider a specific or general solution for all IoT-NTN bands. The solution is applicable for solving the same issue in PC3 and PC5. The options are as follows. - Option1: Add additional note to increase the upper tolerance for NTN HD-FDD operation in applicable regions. EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). 256; 26; +22; 23; +22, -2; 20; +22, -2. 255; 26; +22; 23; +22, -2; 20; +22, -2. 254; 26; +22; 23; +22, -2; 20; +22, -2. 253; 26; +22; 23; +22, -2; 20; +22, -2. 252; 26; +22; 23; +22, -2; 20; +22, -2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When UEs recognizes and supports NTN HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD].. - Option 2: Add additional UE capability and NW signal to control NTN HD-FDD power increment. - Option 2-1: EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). 256; 26; +22; 23; +22, -2; 20; +22, -2. 255; 26; +22; 23; +22, -2; 20; +22, -2. 254; 26; +22; 23; +22, -2; 20; +22, -2. 253; 26; +22; 23; +22, -2; 20; +22, -2. 252; 26; +22; 23; +22, -2; 20; +22, -2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB.. - Option 2-2: EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). 256; 262; +2; 232; +2, -2; 202; +2, -2. 255; 262; +2; 232; +2, -2; 202; +2, -2. 254; 262; +2; 232; +2, -2; 202; +2, -2. 253; 262; +2; 232; +2, -2; 202; +2, -2. 252; 262; +2; 232; +2, -2; 202; +2, -2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The reference power is increased by ΔPNTNHdFdd.. - Option 3: Add new power classes (e.g., PC4.5_NTN-HD-FDD, PC2.5_NTN-HD-FDD, PC1.75_NTN-HD-FDD). - Option 4: - Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance can be considered as starting point (same as for band n14) for PC2 for IoT-NTN HPUE. - WF - FFS Issue 1-2-1: Discuss the unsettled requirements applicability for Iot-NTN: - Agreement (Ad Hoc): - No 2Tx PC1/PC2 requirement will be specified for IoT NTN in Rel-19; - Only consider non-handheld for PC1 1Tx eMTC based with full duplex Iot-NTN - FFS whether requirements should be defined for PC1 1Tx handheld UE for NB-Iot based Iot-NTN - Updated Status: (For Information) Table: Current status for requirements consideration and feasibility for Iot-NTN Power Class; Antenna Numbers; Handheld and/or non-handheld. NB-Iot based; PC1; 1Tx; FFS. PC2; 1Tx; Both. eMTC based with half duplex; PC1; 1Tx; FFS. PC2; 1Tx; Both. eMTC based with full duplex; PC1; 1Tx; Non-handheld. PC2; 1Tx; Both.

This contribution further discussed the UE Tx RF requirements for IoT NTN HPUE.

- Discussion

Issue 1-1-8: The nominal MOP tolerance

About the limitation of PC3 TX output power upper bound in ETSI EN 301 908, the maximum transmit power limit of the B20/28/40 has been changed from 25.7dBm to 25.0dBm in the latest version EN 301 908-13 V13.3.1, that is, the output power upper bound for B20/28/40 has been changed to 23dBm+2dB, as shown in Table 2-1. The purpose is to reduce the interference to other wireless devices.

Table 2-1 the output power upper limit in ETSI EN 301 908

Hence, option 1 that adding additional note to increase the upper tolerance for NTN HD-FDD operation in applicable regions is unfeasible for IoT NTN.

Actually, the tolerance of MOP is to guarantee the UE can meet the MOP requirements due to the limitation of state-of-art technology or under extreme environment. The upper limit shouldn’t increase due to the insertion loss is reduced.

Considering the coexistence of different wireless device, it’s better to re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance for PC2 IoT NTN and +2/-3 tolerance for PC1 IoT NTN.

Proposal 1: Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance for PC2 IoT NTN.

Proposal 2: Re-use LTE TN TS 36.101-1 nominal MOP and +2/-3 tolerance for PC1 IoT NTN.

Issue 1-2-1: Discuss the unsettled requirements applicability for Iot-NTN:

In the R-19, the purpose of IoT NTN HPUE is to introduce PC1 and PC2 for IoT NTN bands and

- PC2 will apply to both handheld and non-handheld UE

- PC1 are prioritized for non-handheld UE

In previous meeting, some companies proposed that it can reuse the GSM PA for PC1 handheld IoT NTN UE. Although the insertion loss of the duplexer in half-duplex mode does not need to be considered, the front-end filter still needs to be considered, it’s hard to get the 31dBm for PC1 handheld UE with 1Tx. In additional, it also needs consider GSM delisting issue. So, reusing GSM PA for PC1 handheld IoT NTN UE is not an optimal choice in the long term. Until a suitable PA for PC1 handheld NTN UE is found, PC1 with 1Tx is best for non-handheld IoT NTN UE only.

Proposal 3: PC1 with 1Tx only apply to non-handheld IoT NTN UE.

- Conclusion

In this paper we give further discussion on the TX requirement for IoT NTN HPUE. The proposals are as following:

Proposal 1: Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance for PC2 IoT NTN.

Proposal 2: Re-use LTE TN TS 36.101-1 nominal MOP and +2/-3 tolerance for PC1 IoT NTN.

Proposal 3: PC1 with 1Tx only apply to non-handheld IoT NTN UE.

- Reference

[1] R4-2420404, WF on HPUE for IoT NTN, vivo, RAN4#113 2024-11.