# 3GPP TSG-RAN WG4 Meeting # 114                                   R4-2501501

# Athens, GR, 17th – 21st February, 2025

# Title: 	Discussion on R18 ATG k1 value

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	5.6.4

# Document for:	Approval

# 1 Introduction

In last meeting, RAN4 discussed how to address the k1 value in the ATG scenario. Ultimately, RAN4 agreed to introduce the outOfOrderOperationDL-r16 as a solution to the ATG HARQ timing. However, during the last meeting, some companies proposed that has a potential issues for outOfOrderOperationDL-r16 capability.

In this contribution, we would like to discuss how to address the legacy issues.

# 2 Discussion

In previous meeting, RAN4 discussed how to address the k1 value for new TDD pattern. Out of order processing, as an optional solution is captured into the specification [1].

However, in last meeting, some companies mentioned that the capability of outOfOrderOperationDL-r16 is bundled with other features, as shown in following.

Regarding the k1 value issue, RAN4 is making every effort to find solutions under the premise of ensuring no disruption to the RAN1 specification. Despite the increased complexity and latency on the BS side due to out of order scheduling, this scheme ensure effective scheduling of new pattern (30D4S6U) in practical network, thereby significantly enhancing downlink throughput.

To solve the above issue, we propose to introduce a new capability outOfOrderOperationDLATG-r18 to support Rel-18 ATG feature. And also needs to send LS to RAN2 to introduce new capability of outOfOrderOperationDLATG-r18. The draft LS can be found in Annex.

Proposal 1. Propose to introduce a new capability capability outOfOrderOperationDLATG-r18 to support Rel-18 ATG feature.

Proposal 2. Propose to send a LS to RAN2 to introduce new capability of outOfOrderOperationDLATG-r18.

# 3 Conclusion

In this contribution, we give some discussions on the HARQ k1 value for ATG, The conclusion are:

Proposal 1. Propose to introduce a new capability capability outOfOrderOperationDLATG-r18 to support Rel-18 ATG feature.

Proposal 2. Propose to send a LS to RAN2 to introduce new capability of outOfOrderOperationDLATG-r18.

# 4 References

- [1] 3GPP TS 38.101-4 V18.6.0

# 5 Annex

Title:	LS on introduce a new capability for the R18 ATG feature

Release:	Rel-18

Study Item:	NR_ATG

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:	TSG RAN WG1

Contact Person:

Name:

E-mail Address:

Attachments: None

1. Overall Description:

R18 ATG reused out-of-order operation to support new TDD pattern (30D4S6U) scheduling, which is basically same as R16 UE capability outOfOrderOperationDL-r16 in TS 38.306.

| outOfOrderOperationDL-r16 Indicates whether the UE supports out of order operation for DL. The UE that indicates support of this feature shall support multiDCI-MultiTRP-r16. The capability signalling comprises the following parameters: - supportPDCCH-ToPDSCH-r16 indicates support out-of-order operation for PDCCH to PDSCH; - supportPDSCH-ToHARQ-ACK-r16 indicates support out-of-order operation for PDSCH to HARQ-ACK. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

However, RAN4 found that if UE indicates support outOfOrderOperationDL-r16 feature, it also shall support multiDCI-MultiTRP-r16. Currently, R18 ATG feature can’t support multiDCI-MultiTRP. Therefore, RAN4 would like to introduce a new UE capability outOfOrderOperationDLATG-r18 only support out-of-order operation for PDSCH to HARQ-ACK on R18 ATG.

2. Actions:

To RAN2

ACTION: 	RAN4 kindly requests RAN2 to take above information into consideration, and introduce a new UE capability to support out-of-order operation for R18 ATG feature.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #114bis 	 4 Apr - 11 Apr 2025	       China, CN

TSG-RAN WG4 Meeting #115 	19 May - 23 May 2025	   Malta, MT