# 3GPP TSG-RAN WG4 Meeting #114                                  R4-2501510

# Athens, GR, 17th – 21st February, 2025

# Title: 	Discussion on BS demodulation performance requirements MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.18.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the revised WID of NR demodulation performance requirements for NR BS with MMSE-IRC receiver was approved in [1], the objectives related to NR BS MMSE-IRC receiver are listed :

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we give some initial discussion on the NR BS MMSE-IRC receiver.

- Discussion

Based on the conclusion in RAN4 #113 meeting[2], some issues are still undecided for BS demodulation performance requirements MMSE-IRC.

In last meeting, it is proposed to check the performance gain of 1 or 2 interferers for 2Rx.

Number of interferers for 2Rx - Option 1: Define BS MMSE-IRC requirements for 2Rx - Option 1A: 1 interferer - Option 1B: 2 interferers - Option 1C: Consider 2 interferers for HomNet and 1 interfere for HetNet - Option 2: Do not need to specify requirements for 2Rx - Companies are encouraged to provide simulation results for 1 or 2 interferers for 2Rx, to check the performance gain.

From our simulation results provided last meeting [3], we can observe that for 2 interfers 2Rx, the gain of MMSE-IRC over MMSE is from 1.1 to 2.2 dB. TDLA30-10 is selected for both serving cell and interfering cell. While the gain for 1 interfering cell and 2Rx is from 2.2 to 5.1 dB. From the simulation results, we think the gain is obvious for some scenario.

Observation 1. In some scenarios, obvious performance gain , such as 2 dB or above, can be observed for 2 interfers 2Rx.

With regard to the number for interferers for 2Rx, it is better align with LTE TS36104. From Table 8.2.6.1-4 in 36104 we can find that the requirements for 2/4/8 Rx are verified.

Table 8.2.6.1-4: Enhanced performance requirement type A for PUSCH, 10 MHz Channel Bandwidth Number of TX antennas (Note 1); Number of RX antennas (Note 1); Propagation conditions and correlation matrix (Annex B) (Note 2); DIP set; FRC (Annex A); Fraction of maximum throughput; SINR [dB] (Note 3). Tested signal; Interferer 1; Interferer 2. 1; 2; EPA 5 Low; ETU 5 Low; N/A; Set 2; A12-4; 70%; -5.4. EVA 70 Low; ETU 70 Low; N/A; Set 1*; A12-4; 70%; -2.7. 4; EPA 5 Low; ETU 5 Low; ETU 5 Low; Set 2; A13-4; 70%; -4.2. EVA 70 Low; ETU 70 Low; ETU 70 Low; Set 1*; A13-4; 70%; -0.1. 8; EPA 5 Low; ETU 5 Low; ETU 5 Low; Set 2; A4-6; 70%; -4.5. EVA 70 Low; ETU 70 Low; ETU 70 Low; Set 1*; A4-6; 70%; -0.2. Note*: Not applicable for Local Area BS and Home BS.. Note 1: Antenna configuration applies for each of the tested signal, interferer 1 and interferer 2. Note 2: The propagation conditions for the tested signal, interferer 1 and interferer 2 are statistically independent. Note 3: SINR corresponds to of the tested signal as defined in clause 8.1..

Option 1 is preferred between the two options to align with LTE BS MMSE-IRC. Among the 3 options from option 1, it is better to see whether the MMSE-IRC gain over MMSE is obvious. If the gain is not obvious, option 1A is preferred

Proposal 1. To consider option 1 for BS MMSE-IRC requirements and option 1B is preferred if the MMSE-IRC gain is observed from companies’ simulation results.

With regard to the channel bandwidth, the following options are reached last meeting.

Channel bandwidth - Proposals on the CHBW for requirement definition for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: - For 15kHz SCS, cover 5MHz, 10MHz - For 30kHz SCS, cover 10MHz, 40MHz - Option 3: minimum CBW - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS - Proposals for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz

We provided our simulation results comparing performance between different channel bandwidth[5] from which we can find that the performance between different channel bandwidth is similar.

Proposal 2. To consider option 3 for CHBW for requirement definition for CP-OFDM .

With regard to MCS, 3 options are identified last meeting to reuse the FRC defined in TS 38104.

- Consider the following options for performance evaluation and as candidates for further down-selection: o For 2Rx:  Option 1: MCS 2  Option 2: MCS 16 o For 4Rx:  Option 1: MCS 16 o For 8Rx:  Option 1: MCS 20  Option 2: MCS 16 - Note: Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 for MCS 2, MCS 16 and MCS 20 respectively. - Check whether the existing test applicability rule can be reused if different MCS is configured for different Rx configuration.

The MCSs (MCS2, MCS16, and MCS20) used to define indicators in TS38104 are candidate MCSs for BS MMSE-IRC. From TS 36.104[4] we can find that different MCS is selected for 2Rx/4Rx/8Rx separately. To align with LTE, different Rx choose different MCS, MCS for 2Rx, 4Rx, and 8Rx are MCS 2, MCS16, MCS20.

Proposal 3. To consider the following MCSs for performance evaluation:

For 2Rx:

Option 1: MCS 2

For 4Rx:

Option 1: MCS 16

For 8Rx:

Option 1: MCS 20

The following is copied from TS38.141-2 8.1.2:

“Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the lowest and highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.”

If different MCS is applied for different Rx and if the lowest and highest numbers of supported connectors are to be tested, maybe the 4Rx with MCS 16 will not be tested. As pointed out by other companies, the requirement definition of BS MMSE-IRC is to verify whether the receiver has the processing capability of MMSE-IRC. If the receiver requirements of the maximum and minimum Rx configurations pass the test, it is reasonable to believe that MMSE-IRC with other antenna configurations can also work well even if different MCSs are configured.

Proposal 4. To consider applicability rule when different Rx with different MCS requirement are involved.

With regard to propagation condition, the following options are reached last meeting.

Propagation condition – For serving cell, consider the following options for performance evaluation and as candidates for further down-selection: o Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet o Option 2: Consider different channel for different MCS  Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10 – For the interference cell, use TDLC300-100.

If multiple MCSs are determined for performance requirement, it is preferred to select different channel model for different MCS. From LTE 36104 we can find that different channels are selected for HetNet and HomNet, one of table is copied below.

[36104] Table 8.2.6.1-4: Enhanced performance requirement type A for PUSCH, 10 MHz Channel Bandwidth Number of TX antennas (Note 1); Number of RX antennas (Note 1); Propagation conditions and correlation matrix (Annex B) (Note 2); DIP set; FRC (Annex A); Fraction of maximum throughput; SINR [dB] (Note 3). Tested signal; Interferer 1; Interferer 2. 1; 2; EPA 5 Low; ETU 5 Low; N/A; Set 2; A12-4; 70%; -5.4. EVA 70 Low; ETU 70 Low; N/A; Set 1*; A12-4; 70%; -2.7. 4; EPA 5 Low; ETU 5 Low; ETU 5 Low; Set 2; A13-4; 70%; -4.2. EVA 70 Low; ETU 70 Low; ETU 70 Low; Set 1*; A13-4; 70%; -0.1. 8; EPA 5 Low; ETU 5 Low; ETU 5 Low; Set 2; A4-6; 70%; -4.5. EVA 70 Low; ETU 70 Low; ETU 70 Low; Set 1*; A4-6; 70%; -0.2. Note*: Not applicable for Local Area BS and Home BS.. Note 1: Antenna configuration applies for each of the tested signal, interferer 1 and interferer 2. Note 2: The propagation conditions for the tested signal, interferer 1 and interferer 2 are statistically independent. Note 3: SINR corresponds to of the tested signal as defined in clause 8.1..

From the table above, we can see that the channel model for different Rx configuration under the same scenario(HomNet or HetNet) are the same. So option 1 is preferred for channel model of serving cell.

Proposal 5. To consider option 1 for channel model of serving cell.

With regard to waveform, it is pointed by company that DFT-s-OFDM is rare in the real deployment. If DFT-s-OFDM is rarely used in real deployment we can only define performance requirements for CP-OFDM.

Waveform – whether to cover DFT-s-OFDM - Candidate options: - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform - Option 2: Cover CP-OFDM only

Proposal 6. Option 2 is preferred for waveform.

With regard to DMRS, the DMRS type and DMRS sequence need further discussion.

DMRS configuration - Cover DMRS configuration type 1. FFS whether to include DMRS configuration type 2. - Use DMRS port #0 for both serving and interfering PUSCH - On DMRS sequence generation, use different DMRS sequence for each gNB: - For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively - FFS DMRS sequence generation configuration for DFT-s-DFDM

It is pointed by company that type 2 DMRS needs to be included to ensure the covariance matrix estimation accuracy. From the initial simulation results provided last meeting, MMSE-IRC shows obvious gain compared to MMSE. Maybe type 2 DMRS can provide better accuracy for covariance matrix estimation, but from the perspective of performance requirement, it is preferred to consider only type 1.

Proposal 7. To only consider DMRS configuration type 1.

With regard to test requirement definition, two options are recognized.

Test requirement definition - Candidate options: - Option 1: FFS whether SINR based definition is needed - Option 2: Use the SNR based requirement definition

Given a SNR and DIP set, the SINR can be derived. As for the requirement definition, SNR is preferred because it does not require the step of converting SNR to SINR.

Proposal 8. Option 2 is preferred for test requirement definition.

With regard to manufacturer declaration, the following are proposed.

Applicability rule for different test scenarios - Proposals on applicability for Homnet and Hetnet scenarios: - Proposal 1: HomNet requirements are not applicable for Local Area BS - Proposal 2: HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS - Proposals on additional applicability rule: - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped - Proposal 2: Introduce the applicability rules that only one of HomNet and HetNet scenarios is tested

From LTE specification, we can find that set 1( HomNet) not applicable for Local Area BS and Home BS. Proposal 2 says that the HetNet requirements are only applied for Home BS and local BS which means that HetNet requirements are not applied for Wide Area BS while LTE does not have this restriction. From the perspective of test effort, it can be considered to test one of the scenarios(HomNet and HetNet).

Proposal 9. Proposal 1 is preferred when considering applicability for Homnet and Hetnet scenarios.

Proposal 10. Proposal 1 is preferred when considering additional applicability rule.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements of MMSE-IRC receiver for NR BS, The conclusions are:

Observation 1. In some scenarios, obvious performance gain , such as 2 dB or above, can be observed for 2 interfers 2Rx.

Proposal 1. To consider option 1 for BS MMSE-IRC requirements and option 1B is preferred if the MMSE-IRC gain is observed from companies’ simulation results.

Proposal 2. To consider option 3 for CHBW for requirement definition for CP-OFDM .

Proposal 3. To consider the following MCSs for performance evaluation:

For 2Rx:

Option 1: MCS 2

For 4Rx:

Option 1: MCS 16

For 8Rx:

Option 1: MCS 20

Proposal 4. To consider applicability rule when different Rx with different MCS requirement are involved.

Proposal 5. To consider option 1 for channel model of serving cell.

Proposal 6. Option 2 is preferred for waveform.

Proposal 7. To only consider DMRS configuration type 1.

Proposal 8. Option 2 is preferred for test requirement definition.

Proposal 9. Proposal 1 is preferred when considering applicability for Homnet and Hetnet scenarios.

Proposal 10. Proposal 1 is preferred when considering additional applicability rule.

# 4	Reference

- RP-241297, Revised WID on NR demodulation performance: Phase 5, .

- R4-2419780, Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS

- R4-2419170, Performance comparison with 1 or 2 interferers for 2Rx, ZTE Corporation, Sanechips

- 3GPP TS 36.104 V18.5.0