# 3GPP TSG-RAN WG4 Meeting #114                                  R4-2501511

# Athens, GR, 17th – 21st February, 2025

# Title: 	Discussion on Discussion on NTN_testing_NGSO_channel_model

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.10.4

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the revised WID of Enhanced requirements and conductive test methodology for NR NTN and IoT NTN was approved in [1], wherein the objectives related to channel model and demodulation requirements are copied below:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model Less than 5MHz for NTN - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

In this contribution, we provide some of our viewpoint on the NGSO channel model.

- Discussion

Based on the conclusion in RAN4 #113 meeting, the general procedure for satellite trajectory is reached. The following are subtracted from the agreement with some content omitted.

Agreement: - WF - For Keplerian definition motion base method, consider the following general procedure to generate the time varying Doppler shift and propagation delay. The detail formulars for each step can be considered as reference in Appendix 1.4. - Step 1: Set the initial ephemeris information parameters with format of position/velocity state vector in Earth-centered earth-fixed frame (ECEF) format according to the existing RAN5 test and initial UE location parameters with the following tables - Step 1.1: covert the initial ephemeris information in ECEF format in table 1 into the format of orbital parameters (Keplerian orbital elements) in ECI (Earth-centered inertial) format in table 2 for orbit generation based on two-body Keplerian model at epoch time T0 - Step 1.2: covert the configured UE location in a format of latitude, longitude, and altitude in table 3 into the ECEF format in table 4 to align with the satellite position/velocity - FFS on the value in table 1, 3 for initial ephemeris information and UE location information - Step 2: Generate the estimated satellite position/velocity vector based on Keplerian model at the time T1 with the initial ephemeris information provided in step 1 - Step 2.1: Calculate the Mean Anomaly value - Step 2.2: Derive the Eccentric Anomaly at time T1 by solving Kepler’s equation - Step 2.3: Derive the True Anomaly at time T1 - Step 2.4: Covert the orbital elements in ECI format into the satellite position/velocity state vector in ECEF format with considering the earth rotation - Note: The detail numerical methods to solve the Kepler’s equation in step 2.2, including number of iterations, accuracy of requirement, can be up to implementation, for example - Numerical method - Newton-Raphson method - Number of iterations - Option 1: 4 - Accuracy requirement E−E0>ϵ - Option 1: ϵ=10-8 - Option 2: ϵ=10-6 - Note: Satellite position/velocity estimation period, T1-T0, is less than or equal to the satellite ephemeris information update period in Issue 1-1-3-4 - Step 3: Obtain the propagation delay and Doppler shift trajectory based on the interpolation operation at the anytime between T0 and time T1. - Step 3.1: Obtain the satellite position/velocity at time T1 - Step 3.2: Obtain the propagation delay and Doppler shift trajectory between time T0 and time T1, (T0 <= T1) - Note: The update granularity of propagation delay and Doppler shift should be [1] slot - Note: The details interpolation method can be up to implementation, with meets some accuracy requirement, for example - Numerical method - Option 1: cubic spline interpolation - Option 2: linear interpolation - Option 3: calculate propagation delay and Doppler shift at any time instant - Other options are not precluded - Accuracy requirement - Option 1: - Max¦delay-error¦ ~ 3e-13 s - Max¦delay-rate error¦ ~ 3.28e-12 - Other options not precluded - Note: RAN4 will conduct the calibration work for each step in post-meeting. Companies are encouraged to provide the calibration and comparison results with GAMT before RAN4#114 meeting. Based on calibration and comparison results, further discuss how to capture the time varying Doppler shift and propagation delay derivation procedure and formulars into specification in next meeting. - The proponents to use the equation of motion to derive the satellite position/velocity in Steps 1/2, as an alternative method as the Keplerian model, is encouraged to provide more detail procedure and the comparison results with the Keplerian model in the next meeting

With regard to the interpolation method, 3 options are identified:

Option 1: cubic spline interpolation

Option 2: linear interpolation

Option 3: calculate propagation delay and Doppler shift at any time instant

The above options can be divided into two categories: interpolation and calculating per TTI, the following 2 figures compare the linear interpolation, spline interpolation and the data with 1s granularity.

Figure 1, Comparison of 2 interpolation methods and the per 1ms calculate values for Doppler shift

Figure 2, Comparison of 2 interpolation methods and the per 1ms calculate values for Delay

From the above 2 figures we can find that the Doppler and Delay are very close for linear and spline interpolation.

Furthermore, figure 3 and figure 4 compare the differences between different interpolation method and the per TTI calculated values.

Figure 3, Difference of 2 interpolation methods for Doppler shift

Figure 4, Difference of 2 interpolation methods for Delay

From figure 3 and figure 4 we can see that spline interpolation method can get smaller errors compared to the linear interpolation method.

Observation 1. Spline interpolation method can get smaller errors compared to linear interpolation method.

Proposal 1. To consider spline interpolation method for Doppler shift interpolation and delay interpolation.

With regard to the initial ephemeris, two options below are proposed. The first option is from TS 38.521-5 and TS 38.508-1 for NGSO LEO-600 and NGSO LEO-1200 and option 2 states the value is from GMAT.

- Option 1 - Consider the NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point to specify the TE-emulated Doppler shift and propagation delay model omitted... - Option 2 - GMAT Other options are not precluded

The selection of initial ephemeris needs to consider the following aspects:

- Elevation angle range to be tested;

- UE location

- The UE location is agreed in Issue 1-1-3-8. To determine the initial ephemeris, the elevation angle range also needs to be determined. Different elevation angle ranges demand different initial satellite positions. Taking 30~150 elevation range as an example, the initial position of the satellite needs to be selected to ensure that the initial elevation angle of the UE is 30 degree.

- Figure 5. Elevation trajectory for a UE in a fixed location

- From Table 6.4.1_2.4.3-1 of 38.521-5, we can find a fixed ephemeris. If different elevation angle ranges need to be considered, the appropriate ephemeris can be selected from GMAT.

- The table below shows candidate elevation angle ranges for different test.

- Set more satellite position/velocity depending on test purpose and pre-existing test configuration - For demodulation test - Option 1: starting from [30°] for both NR and IoT NTN - Option 2: - starting from [ 30°] for NR NTN - starting from [10°] for IoT NTN - For RF test - starting from [10°] for both NR and IoT NTN with considering maximum positive/negative doppler - starting from [60°] for both NR and IoT NTN with considering the half maximum positive/negative doppler - For RRM test - Option 1: starting from [30°] - Option 2: starting from [90°]

Observation 2. Different initial ephemeris may be needed as different elevation angle ranges need to be considered.

Proposal 2. To select appropriate initial ephemeris which is specified in the test specification to ensure the initial elevation is within the appropriate range.

With regard to starting elevation angle for different test, it was proposed to select 30 degrees as the elevation angle as Rel-17 use this value for NTN channel model.

Proposal 3. Option 1 is preferred for demodulation test.

# 3 Conclusion

In this contribution, we give some discussions on NTN_testing_NGSO_channel_model, The conclusions are:

Observation 1. Spline interpolation method can get smaller errors compared to linear interpolation method.

Proposal 1. To consider spline interpolation method for Doppler shift interpolation and delay interpolation.

Observation 2. Different initial ephemeris may be needed as different elevation angle ranges need to be considered.

Proposal 2. To select appropriate initial ephemeris which is specified in the test specification to ensure the initial elevation is within the appropriate range.

Proposal 3. Option 1 is preferred for demodulation test.

# 4	Reference

- RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

- R5-237216 - NGSO satellite Ephemeris file generation methodology for NTN NR UE testing, THALES

- 3GPP TS 38.133 V18.3.0