3GPP TSG-RAN WG4 Meeting #114	  R4-2501527

Athens, Greece, February 17 – 21, 2025

Agenda Item:	7.19.2

Source:	Huawei, HiSilicon

Title:	Discussion on CSI reporting requirement framework for AIML CSI prediction

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #113 [1], we discuss the remaining issues related to the RRM core requirement for AIML positioning.

# Channel model for CSI prediction tests

RAN4 #113 WF [1]: Channel model for CSI prediction tests For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test.

It is observed that in RAN4 legacy, TDL channel models are used for PMI reporting testing. Spatial channel model for demodulation performance requirements is still under study in RAN4. The discussion of whether introducing CDL channel model for AI CSI prediction testing could start if CDL model is used to define requirement for non-AI PMI reporting.

Proposal 1: RAN4 to discuss whether introducing CDL channel model for AI CSI prediction testing, if CDL model is used to define requirement for non-AI PMI reporting.

For generalization test, there are two options to verify the performance of AI CSI prediction. One is to define several requirements under different test configurations/conditions. If the DUT can pass all the tests, then it means that the performance of CSI prediction can be guaranteed in a generalized scenario consisting of these configurations/conditions. This method is what RAN4 legacy uses.

RAN4 #113 WF [1]: Testing conditions Use static conditions as baseline. - FFS on whether to have non-static condition

Another is that the test configurations/conditions vary over time during testing. This method has never been used in RAN4 before. Firstly, the intention is not clear to introduce more complicated testing compared legacy for generalized performance verification. Secondly, when model switching is involved under non-static condition to ensure the performance, LCM procedure testing and generalized performance testing are coupled together in one testing.

Proposal 2: For generalization testing, there is no need to use non-static condition.

For LCM procedure, if model/performance monitoring and management are performed by NW, there is no need to introduce non-static condition for UE testing either. When to activate/deactivate AI is up to NW implementation. For UE-side monitoring, two types are on the table in RAN1. One is to report performance metric, another is to report performance monitoring output. The former one needs to define the performance metric and corresponding report mechanism. The latter one needs to define the performance metric, threshold criterion and corresponding report mechanism. The performance metric and threshold definition may be subject to configurations/conditions, which has not been discussed yet in other WGs.

Proposal 3: RAN4 will start to discuss whether/how to define non-static condition during testing for UE-side monitoring after other WGs achieve sufficient progress.

# Testing configurations for CSI prediction tests

RAN1 #118bis Agreement (#118bis) For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used R19 WID on AIML [2]: - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

We understand that the performance is related to the match degree between training dataset and testing dataset. According to the agreement of 118bis, two aspects are to be optionally evaluated for studying the training/inference consistency issue: tilt angle and TXRU mapping. However, according to the revised WID [2], other WGs will start to study the necessity of specifying consistency of training/inference from 2025 Q1, RAN4 can wait RAN1 progress for defining the test configurations.

Observation 1: Whether/How to ensure that the testing dataset aligns well with training dataset is still an open issue.

Proposal 4: RAN4 to discuss training/inference consistency related testing configurations after other WGs achieve sufficient progress.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: RAN4 to discuss whether introducing CDL channel model for AI CSI prediction testing, if CDL model is used to define requirement for non-AI PMI reporting.

Proposal 2: For generalization testing, there is no need to use non-static condition.

Proposal 3: RAN4 will start to discuss whether/how to define non-static condition during testing for UE-side monitoring after other WGs achieve sufficient progress.

Observation 1: Whether/How to ensure that the testing dataset aligns well with training dataset is still an open issue.

Proposal 4: RAN4 to discuss training/inference consistency related testing configurations after after other WGs achieve sufficient progress.

# References

- R4-2420513, “WF on AI/ML air interface part 1”, 3GPP RAN4 #113, Orlando, USA 18th ‒ 22nd November, 2024.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #105, September 9-12, 2024.