3GPP TSG-RAN WG4#114	R4-2501577

Athens, Greece, February 17th – 21st, 2025

Agenda item:	6.3.3

Source: 	Qualcomm France

Title: 	CA_n40A-n41C and CA_n41C-n79A MSD

Document for:	Approval

# Introduction

Considerations on CA_n40A-n41C and CA_n41C-n79A MSD are provided in this contribution.

- Background

WF for CA_n40A-n41C and CA_n41C-n79A was agreed in RAN4#113 [1]:

CA_n40A-n41C Agreement:

- For the first and second test point align to the highest band n40 5MHz victim channel at 2397.5MHz.

  - For the first test point average of 41.3dB and 42.5dB = 41.7dB is used

  - For the second test point, proponent of the 50MHz case could provide a scaled 5MHz MSD for averaging of values.

- For the CA_n41C carrier frequencies use the CA channel separation with Fc_low = 2565MHz and Fc_high = 2644.8 and rework 1RB+1RB positions to hit 2397.5MHz 5MHz victim channel.

- For the second test point use 270RB3 and 162RB0 as the allocation which correspond to fully allocated contiguous RB allocation

- The Table format and two test point place in the TS is discussed in topic 3.

CA_n41C-n79A Agreement:

- For the first test point align to the highest band n79 10MHz victim channel at 4995MHz.

  - For the first test point average of 12.4dB and 8.4dB = 10.4dB is used

- For the second test point align to the highest band n79 10MHz victim channel at 4995MHz or 40MHz victim channel at 4980MHz.

- For the second test point, proponent of the 40MHz or 10MHz case could provide a scaled MSD for averaging of values.

- For the CA_n41C carrier frequencies use the CA channel separation with Fc_low = 2565MHz and Fc_high = 2644.8 and rework 1RB+1RB positions to hit the 4995MHz 10MHz victim channel.

- For the second test point use 270RB3 and 162RB0 as the allocation which correspond to fully allocated contiguous RB allocation

- The Table format and two test point place in the TS is discussed in topic 3.

Capturing in TS Agreement:

- Applicable to 1UL band case with contiguous intra-band ULCA unless otherwise stated.

- Companies are encouraged to propose how to capture the two test points in the same table so that it is they are not confused as two separate issues.

- The 1RB+1RB is the mandatory baseline test point.

- For H2 and H3 related interference, the victim channel is chosen the closest to the related harmonic frequency.

- The second optional test point uses contiguous fully allocated intra-band contiguous ULCA case.

- FFS if a new table is needed but if so, the legacy 1UL 2CC test points should be moved to that table for both contiguous and non-contiguous intra-band UL CA

- FFS any further restrictions on the second test point.

- Discussion

We did measurements and analysis with the following outcome.

For CA_n40A-n41C 1+1 RB case, aligned with the WF of uppermost 5MHz channel being victim we reworked the RBstart positions with the following proposal.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 41.7 | TDD | IMD3 |
|  | n41 | 2565 | 100 | 1 (RBSTART= 43) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=139) | 2644.8 |  |  |  |

For CA_n40A-n41C full RB case, we did evaluate both 5MHz victim centered at 2397.5MHz and 50MHz victim centered at 2345MHz with the following results. According to WF, 5MHz centered at 2397.5MHz is proposed.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 38.1 | TDD | IMD3 |
|  | n41 | 2565 | 100 | 270 (RBSTART= 3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART= 0) | 2644.8 |  |  |  |

For CA_n41C-n79A 1+1 RB case, aligned with the WF of uppermost 10MHz channel being victim we reworked the RBstart positions with the following proposal.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n79 | N/A | 10 | N/A | 4995 | 10.4 | TDD | IMD4 |
|  | n41 | 2565 | 100 | 1 (RBSTART= 89) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART= 93) | 2644.8 |  |  |  |

For CA_n41C-n79A with full RB allocation, we did evaluate both 10MHz victim centered at 4995MHz and 40MHz victim centered at 4980MHz with the following results. In our view, it would be better to use 10MHz victim.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n79 | N/A | 10 | N/A | 4874 | 9.7 | TDD | IMD4 |
|  | n41 | 2565 | 100 | 270 (RBSTART= 3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART= 0) | 2644.8 |  |  |  |

Current Table 7.3A.5-1 suits well for capturing these cases, see an example below. The only new thing is IMD specification using full RB allocations. There is absolutely no need for a new table as the additional test point was like one-off kind of situation, hence we are not assuming there would be other similar cases for other band combinations.

| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 41.7 | TDD | IMD3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2565 | 100 | 1 (RBSTART=43) | 2545 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=139) | 2640 |  |  |  |
|  | n40 | N/A | 5 | N/A | 2397.5 | 38.1 | TDD | IMD3 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2545 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2640 |  |  |  |

- Conclusion

Considerations on CA_n40A-n41C and CA_n41C-n79A were provided with the following proposals.

Proposal 1: Consider the following test point with already agreed 41.7dB MSD for CA_n40A-n41C 1+1 RB case

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 41.7 | TDD | IMD3 |
|  | n41 | 2565 | 100 | 1 (RBSTART= 43) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=139) | 2644.8 |  |  |  |

Proposal 2: Consider the following test point for CA_n40A-n41C additional test case with full RB’s

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 38.1 | TDD | IMD3 |
|  | n41 | 2565 | 100 | 270 (RBSTART= 3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART= 0) | 2644.8 |  |  |  |

Proposal 3: Consider the following test point with already agreed 10.4dB MSD for CA_n41C-n79A 1+1 RB case

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n79 | N/A | 10 | N/A | 4995 | 10.4 | TDD | IMD4 |
|  | n41 | 2565 | 100 | 1 (RBSTART= 89) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART= 93) | 2644.8 |  |  |  |

Proposal 4: Consider the following test point for CA_n41C-n79 additional test case with full RB’s

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n79 | N/A | 10 | N/A | 4995 | 9.7 | TDD | IMD4 |
|  | n41 | 2565 | 100 | 270 (RBSTART= 3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART= 0) | 2644.8 |  |  |  |

Proposal 5: Use table 7.3A.5-1 to capture these requirements.

- Reference

[1] R4-2420471, “WF on UL CA_n41C MSD test point and its capture in the TS”, Skyworks Solution Inc., Huawei, Qualcomm