3GPP TSG-RAN WG4#114	R4-2501580

Athens, Greece, February 17th – 21st, 2025

Agenda item:	5.2

Source: 	Qualcomm France

Title: 	On UE BW’s in MSD tables

Document for:	Approval

# Introduction

SRS IL imbalance indication considered in this contribution.

- Discussion

We noticed that for some MSD requirements, problematic UE BW’s are used for NR CA and EN-DC. For instance, n40 supports 5MHz only when carrier is configured as an SCell part of DC or CA configuration. n41 supports 5MHz optionally.

Below are just few examples of the current MSD specification which cannot be tested with Pcell (n40) or can be tested only if UE supports optional BW (n41).

RAN4 should discuss how to resolve this issue. Our initial proposal is to update the tables by replacing 5MHz for TDD bands by 10MHz and modifying MSD accordingly.

Proposal 1: RAN4 should consider how to modify MSD requirements which currently use either optional BW or BW with Scell restriction.

Observation 1: Easiest way could be to replace 5MHz BW for TDD bands by 10MHz BW, and to modify MSD’s accordingly. Most of the problematic cases are in Intermodulation tables, but all tables should be scrutinized for this.

- Conclusion

Considerations on problematic UE BW’s in MSD tables are provided with the following proposal.

Proposal 1: RAN4 should consider how to modify MSD requirements which currently use either optional BW or BW with Scell restriction.

Observation 1: Easiest way could be to replace 5MHz BW for TDD bands by 10MHz BW, and to modify MSD’s accordingly. Most of the problematic cases are in Intermodulation tables, but all tables should be scrutinized for this.

- Reference