3GPP TSG-RAN WG4 Meeting #114	R4-2501614

Athens, Greece, 17-21 February, 2024

Source:	Ericsson

Title:	(NR_newRAT-core) On gradual timing adjustment for CA and DC

Agenda item:	5.1.2

Document for:	Approval

# 1	Introduction

In recent meetings, including RAN4#113 Apple has proposed Update to gradual timing adjustment [1,2] from NR release-15.

… #### 7.1.2.1 Gradual timing adjustment When the transmission timing error between the UE and the reference timing exceeds ±Te then the UE shall adjust the timing such that timing error is within ±Te. The reference timing shall be before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules: 1) The maximum amount of the magnitude of the timing change in one adjustment shall be Tq. 2) The minimum aggregate adjustment rate shall be Tp per second. 3) The maximum aggregate adjustment rate shall be Tq per 200 ms. where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2.1-1. In any of the configurations below the gradual timing adjustment is not applied if such adjustment results in transmit timing difference larger than the maximum timing difference in section 7.6. - Inter-band CA - Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation - Intra-band EN-DC if UE doesn’t support asynchronous operation - Inter-band NE-DC if UE doesn’t support asynchronous operation - Inter-band NR-DC if UE doesn’t support asynchronous operation …

A similar issue is brought up for gradual timing adjustment in case of multi-DCI mTRP transmission with 2 TA in [3]. In this contribution we discuss the issue and forward our own proposals.

# 2	Discussion

## 2.1	Baseline specifications

In LTE TS 36.133 we have a similar gradual adjustment rule as proposed for NR in [2]:

… ## 7.1 UE transmit timing ### … ### 7.1.2 Requirements The UE initial transmission timing error shall be less than or equal to ±Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies when it is the first transmission in a DRX, eDRX_CONN cycle for PUCCH, SPUCCH, PUSCH of subframe, slot or subslot duration and SRS, or it is the first transmission after RACH-less handover, or it is the PRACH transmission. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA_Ref for PRACH is defined as 0. (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA_Ref for other channels is not changed until next timing advance is received. Table 7.1.2-1: Te Timing Error Limit Downlink Bandwidth (MHz); Te_. 1.4; 24*TS. ≥3; 12*TS. Note: TS is the basic timing unit defined in TS 36.211. When it is not the first transmission in a DRX or eDRX_CONN cycle or there is no DRX or no eDRX_CONN cycle, and when it is the transmission for PUCCH, SPUCCH, PUSCH of subframe, slot or subslot duration and SRS transmission or it is not the first transmission after RACH-less handover, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in clause 7.3 is applied. When in a TAG the transmission timing error between the UE and the reference timing exceeds ±Te, or in a sTAG the UE changes the downlink SCell for deriving the UE transmit timing for cells in the sTAG configured with one or two uplinks, the UE is required to adjust its timing to within ±Te in that TAG, as long as, - the UE is configured with a pTAG and one or two sTAG, the transmission timing difference between TAGs does not exceed the maximum transmission timing difference (i.e., 32.47us) after such adjustment, or - the UE is configured with synchronous dual connectivity, the transmission timing difference between pTAG and psTAG does not exceed the maximum transmission timing difference (i.e., 35.21us) after such adjustment. If the transmission timing difference after such adjustment is bigger than the maximum transmission timing difference UE may stop adjustment in this TAG. For a UE configured with more than one serving cell and with ShortTTI -r15 or with ShortProcessingTime=TRUE, UE may stop the transmit timing adjustment if the conditions specified in the subclause B.7.1 cannot be fulfilled after such adjustment. The reference timing shall be before the downlink timing of the reference cell. All adjustments made to the UE uplink timing under the above mentioned scenarios shall follow these rules: 1) The maximum amount of the magnitude of the timing change in one adjustment shall be Tq seconds. 2) The minimum aggregate adjustment rate shall be 7*TS per second. 3) The maximum aggregate adjustment rate shall be Tq per 200ms. …

However, in same LTE, TS 36.133 we also have an SCell may stop transmitting rule:

… ### 7.9.2 Minimum Requirements for Interband Carrier Aggregation The UE shall be capable of handling at least a relative received timing difference between the subframe timing boundaries of the signals received from the PCell and the SCell at the UE receiver of up to 30.26 µs when one SCell is configured. When two, three, or four SCells are configured, the UE shall be capable of handling at least a relative propagation delay difference between the subframe timing boundaries of the signals received from any pair of the serving cells (PCell and the SCells) at the UE receiver of up to 30.26 µs. The UE shall be capable of handling a maximum uplink transmission timing difference between the pTAG and the sTAG of at least 32.47µs provided that the UE is: - configured with inter-band CA and - configured with the pTAG and the sTAG, A UE configured with pTAG and sTAG may stop transmitting on the SCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and SCell exceeds the maximum value the UE can handle as specified above. The UE shall be capable of handling a maximum uplink transmission timing difference between the pTAG and any of the two sTAGs or between the two sTAGs of at least 32.47µs provided that the UE is: - configured with inter-band CA and - configured with the two sTAGs, A UE configured with two sTAGs may stop transmitting on the SCell if after timing adjusting due to received TA command the uplink transmission timing difference between SCell in one sTAG and SCell in other sTAG exceeds the maximum value the UE can handle as specified above. … ### 7.9.3 Minimum Requirements for Intraband non-contiguous Carrier Aggregation The UE shall be capable of handling at least a relative received timing difference between the subframe timing boundaries of the signals received from the PCell and the SCell at the UE receiver of up to 30.26 µs. The UE shall be capable of handling a maximum uplink transmission timing difference between the pTAG and the sTAG of at least 32.47µs provided that the UE is: - configured with intra-band non-contiguous CA and - configured with the pTAG and the sTAG, A UE configured with pTAG and sTAG may stop transmitting on the SCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and SCell exceeds the maximum value the UE can handle as specified above. …

In NR TS 38.133 we have neither a gradual adjustment rule nor a rule about SCell may stop transmitting. We can summarize this in observations:

Observation 1: 	In LTE TS 36.133 we have a gradual adjustment rule stating UE may stop adjustment in this TAG if the transmission timing difference after such adjustment is bigger than the maximum transmission timing difference.

Observation 2: 	In LTE TS 36.133 we have a rule that A UE configured with pTAG and sTAG may stop transmitting on the SCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and SCell exceeds the maximum value the UE can handle.

Observation 3: 	In NR TS 38.133 we have no statements regarding gradual adjustment or SCell transmissions when transmission timing difference between PCell and SCell exceeds the maximum value the UE can handle.

## 2.2	Discussion

Gradual adjustment is a map from the DL time reference and the current TA-command value the UE has to the nominal UL TX point in time relative to DL time reference. The DL time reference can change due to UE mobility of the UE moves closer to, or further away from the cell. The gradual adjustment mechanism in LTE and NR is a fundamental system function, active per TAG and not related to any particular feature like CA or DC.

Observation 4: 	The gradual adjustment mechanism in LTE and NR is a fundamental system function, active per TAG and not related to any particular feature like CA or DC.

If there is an issue with MTTD being exceeded for a pair of carriers, for a particular CA or DC feature, we prefer to resolve any issues in specific specification text related to the feature, instead of changing the fundamental gradual adjustment mechanism.

Proposal 1: 	Resolve any issues in specific text related to the CA and DC features, section 7.5 Maximum Transmission Timing Difference, instead of changing the fundamental gradual adjustment mechanism.

A proposal can be found in [4] and below:

… ## 7.5 Maximum Transmission Timing Difference … #### 7.5.2.1 Minimum Requirements for inter-band synchronous EN-DC A UE configured with E-UTRA PCell and PSCell shall stop transmitting on the PSCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and PSCell exceeds the specified minimum required MTTD and the UE cannot continue to regulate its transmit timing. … ### 7.5.3 Minimum Requirements for intra-band EN-DC A UE configured with E-UTRA PCell and PSCell shall stop transmitting on the PSCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and PSCell exceeds specified minimum required MTTD and the UE cannot continue to regulate its transmit timing. … ### 7.5.4 Minimum Requirements for NR Carrier Aggregation A UE configured with pTAG and sTAG shall stop transmitting on the SCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and SCell exceeds specified minimum required MTTD and the UE cannot continue to regulate its transmit timing. … #### 7.5.5.1 Minimum Requirements for inter-band synchronous NE-DC A UE configured with PCell and E-UTRA PSCell shall stop transmitting on the PSCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and PSCell exceeds specified minimum required MTTD and the UE cannot continue to regulate its transmit timing. … ### 7.5.6 Minimum Requirements for inter-band NR DC A UE configured with PCell and PSCell shall stop transmitting on the PSCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and PSCell exceeds specified minimum required MTTD and the UE cannot continue to regulate its transmit timing.

Proposal 2: 	Approve CR R4-2501615, CR, (NR_newRAT-core) SCell transmission based on MTTD

# 3 	Summary

Observation 1: 	In LTE TS 36.133 we have a gradual adjustment rule stating UE may stop adjustment in this TAG if the transmission timing difference after such adjustment is bigger than the maximum transmission timing difference.

Observation 2: 	In LTE TS 36.133 we have a rule that A UE configured with pTAG and sTAG may stop transmitting on the SCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and SCell exceeds the maximum value the UE can handle.

Observation 3: 	In NR TS 38.133 we have no statements regarding gradual adjustment or SCell transmissions when transmission timing difference between PCell and SCell exceeds the maximum value the UE can handle.

Observation 4: 	The gradual adjustment mechanism in LTE and NR is a fundamental system function, active per TAG and not related to any particular feature like CA or DC.

Proposal 1: 	Resolve any issues in specific text related to the CA and DC features, section 7.5 Maximum Transmission Timing Difference, instead of changing the fundamental gradual adjustment mechanism.

Proposal 2: 	Approve CR R4-2501615, CR, (NR_newRAT-core) SCell transmission based on MTTD

# References

[1] 		R4-2418540, 	Discussion, On gradual timing adjustment for CA and DC, Apple

[2]		R4-2418542, CR, (NR_newRAT-core) Update to gradual timing adjustment-R15, Apple

[3]		R4-2418559, CR, CR on gradual timing adjustment for 2TA, Apple

[4]		R4-2501615, CR, (NR_newRAT-core) SCell transmission based on MTTD