3GPP TSG-RAN WG4 Meeting #114	R4-2501616

Athens, Greece, 17-21 February, 2024

Source:	Ericsson

Title:	Remaining requirements for type 4 UEs in non-colocated scenario

Agenda item:	7.5.3

Document for:	Approval

# 1	Introduction

In RAN4#113 a WF was agreed [1]:

| Index | Company | Companies |
| --- | --- | --- |
| 1 | (7.6 Maximum Receive Timing Difference) 7.6.4 Minimum Requirements for NR Carrier Aggregation | Nokia |
| 2 | (8.1.7 Scheduling availability of UE during radio link monitoring) 8.1.7.2 Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1 | Ericsson |
| 3 | (8.2 Interruption) 8.2.1.2.3 Interruptions at SCell addition/release 8.2.1.2.4 Interruptions at SCell activation/deactivation 8.2.2.2.1 Interruptions at SCell addition/release 8.2.2.2.2 Interruptions at SCell activation/deactivation 8.2.2.2.3 Interruptions during measurements on deactivated SCC | Samsung |
| 4 | (8.3 SCell Activation and Deactivation Delay) 8.3.2 SCell Activation Delay Requirement for Deactivated SCell | ZTE |
| 5 | (8.5 Link Recovery Procedures) 8.5.2 Requirements for SSB based beam failure detection 8.5.3 Requirements for CSI-RS based beam failure detection 8.5.5 Requirements for SSB based candidate beam detection 8.5.6 Requirements for CSI-RS based candidate beam detection | Huawei |
| 6 | 8.5.7.2 Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1 8.5.8.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 | Ericsson |
| 7 | (9.2 NR intra-frequency measurements) 9.2.5.3.1 Scheduling availability of UE performing measurements in TDD bands on FR1 9.2.5.3.2 Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 (9.3 NR inter-frequency measurements) 9.3.9.3.1 Scheduling availability of UE performing measurements in TDD bands on FR1 9.3.9.3.2 Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 (9.5 L1-RSRP measurements for Reporting) 9.5.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 (9.7 Cross Link Interference measurements) 9.7.4.1 Scheduling availability of UE performing measurement on FR1 (9.8 L1-SINR measurements for Reporting) 9.8.6.2 Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 (9.10 CSI-RS based L3 measurements) 9.10.2.6 Scheduling availability of UE during CSI-RS based intra-frequency measurements (9.13 L1-RSRP measurements for a cell with different PCI from serving cell) 9.13.6 Scheduling availability of UE during L1-RSRP measurement | Apple |

In this contribution we present our part of the CR work split, items 2 and 6,  Scheduling availability with a different subcarrier spacing than PDSCH/PDCCH on FR1.

# 2	Discussion

## 2.1	TS 38.133 section 8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier

We have adapted the CR to the existing structure. That is, we are using the same logic and structure as existing release-18 text, with the addition of signaling element [nonCollocatedTypeNR-CA-r19]. The final signaling for release-19 is not yet agreed and the CR will have to be updated accordingly when we reach agreement.

Observation 1: We have assumed same logic and structure as existing release-18 text, with the addition of signaling element [nonCollocatedTypeNR-CA-r19] in brackets. The final signaling for release-19 is not yet agreed and the CRs will have to be updated accordingly when we reach agreement.

Draft CR [2]:

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring based on SSB as RLM-RS. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to radio link monitoring based on SSB as RLM -RS.

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for radio link monitoring.

When intra-band carrier aggregation in FR1 is performed, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band if nonCollocatedTypeNR-CA-r18 is not provided. Otherwise, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating [intraBandNR-CA-non-collocated-r19], there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band if [nonCollocatedTypeNR-CA-r19 with value 0, Type 4] is provided. Otherwise, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols if [nonCollocatedTypeNR-CA-r19 with value 1, Type 0] is provided.

When inter-band FR1-FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to radio link monitoring performed on FR1 serving PCell or PSCell in different bands.

## 2.2	TS 38.133 section 8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1 and 8.5.8.2 L1-RSRP measurement

The detailed structure of the rel-19 capabilities and signals has not yet been agreed, hence the brackets, [•], around that part of the CR. The general idea of designing the CR is to keep to rel-18 structure and organisation of text. Draft CR [3]:

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to beam failure detection when SSB is configured as BFD. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to beam failure detection when SSB is configured as BFD.

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for beam failure detection.

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on FR1 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating [intraBandNR-CA-non-collocated-r19] and if [nonCollocatedTypeNR-CA-r19 with value 0, Type 4] is provided, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on FR1 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols if [nonCollocatedTypeNR-CA-r19 with value 1, Type 0] is provided.

When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell or PSCell is configured.

When inter-band FR1-FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in the bands due to beam failure detection performed on FR1 serving PCell or PSCell in different bands.

=== ===

The detailed structure of the rel-19 capabilities and signals has not yet been agreed, hence the brackets, [•], around that part of the CR. The general idea of designing the CR is to keep to rel-18 structure and organisation of text.

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as link recovery detection resource. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured as link recovery detection resource.

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH, TRS, CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for L1-RSRP.

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on one serving cell apply to all other serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on one serving cell apply to all other serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating [intraBandNR-CA-non-collocated-r19] and if [nonCollocatedTypeNR-CA-r19 with value 0, Type 4] is not provided, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on one serving cell apply to all other serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols if [nonCollocatedTypeNR-CA-r19 with value 1, Type 1] is provided.

When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands.

When inter-band FR1-FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) in one band due to candidate beam detection performed on FR1 serving cell(s) in other bands.

# 3 	Summary

Draft CR as per work split have been presented above and in [2,3].

Observation 1: We have assumed same logic and structure as existing release-18 text, with the addition of signaling element [nonCollocatedTypeNR-CA-r19] in brackets. The final signaling for release-19 is not yet agreed and the CRs will have to be updated accordingly when we reach agreement.

Proposal 1: Approve CRs in [2] and [3] according to work split.

# References

[1] 		R4-2420090, WF on RRM requirements for NonCol_intraB_ENDC_NR_CA_Ph2, Huawei, HiSilicon

[2]		R4-2501617, Draft CR: Scheduling availability of UE during radio link monitoring, Ericsson

[3]		R4-2501618, Draft CR: Scheduling availability of UE performing beam failure detection + L1-RSRP, Ericsson