3GPP TSG-RAN WG4 Meeting #114	R4-2501727

# Athens, GR, 17th – 21st Feburary, 2025

Agenda Item:	7.29.2.1

Source:	MediaTek Inc.

Title:	Discussion on Rel-19 NR-NTN RedCap UE RF requirements

Document for:	Approval

- Introduction

In RAN#102 plenary meeting, the Rel-19 WID of Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in [1]. One of the main objectives is to support Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN band.

In RAN4#113 meeting, the WF [2] for guiding further discussion on residual issues was proposed. In this contribution, our views on the residual issues are provided.

- Discussion

2.1		Guidance for Rel-19 phase 3 NR-NTN UE RF requirements

The Rel-19 WID [1] objectives indicate that NTN for NR Phase 3 supports Redcap and eRedcap with NR NTN operating in FR1-NTN band. The objectives are shown below.

4.1 Objective of SI or Core part WI or Testing part WI Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

The Rel-19 NR_NTN_Ph3_UERF WF [2] in RAN4#113 provided the guidance as follows.

Issue 1-1: PC3 output power for (e)RedCap Agreement: - FFS whether and how a potential new scheme to increase output power is addressed in specifications - Alt 1:Further confirm whether rel-18 power boost is already applicable for NR NTN including (e)RedCap - If confirmed, no further work is needed. - Alt 2: Power increase by network controlled optional new signalling (e.g capability) - Alt 3: Change in power tolerances - E.g. Upper tolerance exception to UE in specific regions via table note. - FFS whether solution is common for HD-FDD and FD-FDD - If no solution agreed, PC3 tolerance will remain unchanged to +/- 2 dB.

Issue 1-2: HD-FDD refsens for 2 Rx Agreement: Further discussion on potential improvement beyond TN HD-FDD 2 RX REFSENS level is limited to maximum 0.2 dB improvement based on average of inputs to RAN4.

2.2		Maximum output power

It is observed that the HD-FDD RF front-end insertion loss (FE IL) is lower compared to FD-FDD FE IL and can improve the RX REFSENS and TX maximum output power (MOP) simultaneously.

Observation 1: HD-FDD FE IL is around 0.8dB lower compared to FD-FDD IL, which can improve NTN HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP when the same PA is used.

Observation 2: The actual NTN-RedCap PC3 HD-FDD Tx upper bound can be 25.8dBm but exceeds conventional PC3 FD-FDD TX upper power bound of 25dBm.

Observation 3: Regarding relaxing TX output power upper limit for NTN UE HD-FDD operation, in RAN4#113 WF [2], it was agreed that FFS whether and how a potential new scheme to increase output power is addressed in specifications.

Proposal 1: Select one of possible options below for relaxing the nominal MOP or MOP tolerance upper limitation for NTN HD-FDD operation.

- Option 1: NOTE X: The TX upper power tolerance limit relaxation and related requirements (e.g., relaxed value, specific regions) in HD-FDD operation are specified in sub-clause [xxx].

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26 | +2x, -2 | 23 | +2x, -2 |
| n255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 2: NOTE X: The TX power limit relaxation and related requirements (e.g., relaxed value, NW signaling) in HD-FDD operation are specified in sub-clause [xxx].

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26x | +2, -2 | 23x | +2, -2 |
| n255 | 26x | +2, -2 | 23x | +2, -2 |
| NOTE X: The power limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

Observation 4: In RAN4#113 WF [2], FFS whether solution of increasing output power is common for HD-FDD and FD-FDD.

Proposal 2: Whether increasing output power could be also applicable for FD-FDD, confirm with FE vendors for further discussion.

Proposal 3: If it is confirmed by FE vendors that increasing output power could be also applicable for FD-FDD, select one of possible options below for relaxing the nominal MOP or MOP tolerance upper limitation for NTN FD-FDD operation.

- Option 1: NOTE Y: The TX upper power tolerance limit relaxation and related requirements (e.g., relaxed value, specific regions) in FD-FDD operation are specified in sub-clause [xxx].

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26 | +2x, -2 | 23 | +2x, -2 |
| n255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE Y: The upper power tolerance limit relaxation and related requirements in FD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 2: NOTE Y: The TX power limit relaxation and related requirements (e.g., relaxed value, NW signaling) in FD-FDD operation are specified in sub-clause [xxx].

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26x | +2, -2 | 23x | +2, -2 |
| n255 | 26x | +2, -2 | 23x | +2, -2 |
| NOTE Y: The power limit relaxation and related requirements in FD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

2.3		HD-FDD refsens for 2 Rx

Proposal 4: In RAN4#113 WF [2], it was agreed that further discussion on potential improvement beyond TN HD-FDD 2 RX REFSENS level is limited to maximum 0.2 dB improvement based on average of inputs to RAN4. Therefore, consider -100.2dBm/5MHz as REFSENS for n255 2RX HD-FDD.

Proposal 5: In RAN4#113 WF [2], it was agreed that further discussion on potential improvement beyond TN HD-FDD 2 RX REFSENS level is limited to maximum 0.2 dB improvement based on average of inputs to RAN4. Therefore, consider -100.1dBm/5MHz as REFSENS for n254 2RX HD-FDD.

3 Conclusion

In this paper, our views on NTN-RedCap UE RF requirements in HD-FDD mode are provided as follows:

Observation 1: HD-FDD FE IL is around 0.8dB lower compared to FD-FDD IL, which can improve NTN HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP when the same PA is used.

Observation 2: The actual NTN-RedCap PC3 HD-FDD Tx upper bound can be 25.8dBm but exceeds conventional PC3 FD-FDD TX upper power bound of 25dBm.

Observation 3: Regarding relaxing TX output power upper limit for NTN UE HD-FDD operation, in RAN4#113 WF [2], it was agreed that FFS whether and how a potential new scheme to increase output power is addressed in specifications.

Proposal 1: Select one of possible options below for relaxing the nominal MOP or MOP tolerance upper limitation for NTN HD-FDD operation.

- Option 1: NOTE X: The TX upper power tolerance limit relaxation and related requirements (e.g., relaxed value, specific regions) in HD-FDD operation are specified in sub-clause [xxx].

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26 | +2x, -2 | 23 | +2x, -2 |
| n255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 2: NOTE X: The TX power limit relaxation and related requirements (e.g., relaxed value, NW signaling) in HD-FDD operation are specified in sub-clause [xxx].

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26x | +2, -2 | 23x | +2, -2 |
| n255 | 26x | +2, -2 | 23x | +2, -2 |
| NOTE X: The power limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

Observation 4: In RAN4#113 WF [2], FFS whether solution of increasing output power is common for HD-FDD and FD-FDD.

Proposal 2: Whether increasing output power could be also applicable for FD-FDD, confirm with FE vendors for further discussion.

Proposal 3: If it is confirmed by FE vendors that increasing output power could be also applicable for FD-FDD, select one of possible options below for relaxing the nominal MOP or MOP tolerance upper limitation for NTN FD-FDD operation.

- Option 1: NOTE Y: The TX upper power tolerance limit relaxation and related requirements (e.g., relaxed value, specific regions) in FD-FDD operation are specified in sub-clause [xxx].

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26 | +2x, -2 | 23 | +2x, -2 |
| n255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE Y: The upper power tolerance limit relaxation and related requirements in FD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 2: NOTE Y: The TX power limit relaxation and related requirements (e.g., relaxed value, NW signaling) in FD-FDD operation are specified in sub-clause [xxx].

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26x | +2, -2 | 23x | +2, -2 |
| n255 | 26x | +2, -2 | 23x | +2, -2 |
| NOTE Y: The power limit relaxation and related requirements in FD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

Proposal 4: In RAN4#113 WF [2], it was agreed that further discussion on potential improvement beyond TN HD-FDD 2 RX REFSENS level is limited to maximum 0.2 dB improvement based on average of inputs to RAN4. Therefore, consider -100.2dBm/5MHz as REFSENS for n255 2RX HD-FDD.

Proposal 5: In RAN4#113 WF [2], it was agreed that further discussion on potential improvement beyond TN HD-FDD 2 RX REFSENS level is limited to maximum 0.2 dB improvement based on average of inputs to RAN4. Therefore, consider -100.1dBm/5MHz as REFSENS for n254 2RX HD-FDD.

5 Reference

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”

- R4-2419897, “Way Forward for [113][310] NR_NTN_Ph3_UE_RF”, Qualcomm