3GPP TSG-RAN WG4 Meeting #114	R4-2501740

Athens, Greece, USA, 17th – 21st February 2025

Source:	Qualcomm Incorporated

Title:	3 Tx NR specifications

Agenda item:	7.22.2

Document for:	Approval

# 1.	Introduction

In RAN4#113 there were several agreements on 3 TX that were captured in the WF [1]. In this paper we present further views on the remaining unresolved issues.

# 2. 	Discussion

In RAN4#113 it was agreed that the 3Tx architecture cannot support PC1.5 using full power mode0 in Rel-19 as RAN4 assumes that the highest output power from a single PA is restricted to PC2 due to technology constraints.

Observation 1: Not possible to configure a UE for PC1.5 in mode 0 using a single PA based on current technology.

Another way of achieving higher output power is by using TxD where multi-Tx transmission is allowed. However, for 3Tx TxD is precluded as captured in the main session report [2] illustrated below.

Issue 1-1: PC for 3Tx requirement

Agreement:

- PC for 3Tx requirement

  - PC3

  - PC2

  - FFS on PC1.5

    - FFS on UE type, i.e., FWA and/or handheld UE

- RAN4 understanding is that TxD for 3Tx is not in the scope of this WI

- Full power mode 0 apply to PC2 and PC3 only.

  - FFS on for which UE types it should apply.

Observation 2: Not possible for a UE to support higher power using TxD as TxD is precluded for 3 Tx.

In RAN4#113 the following was agreed:

<Agreement>: PC2 for 3Tx

- Use the same MPR as MPR for PC2 single carrier.

  - FFS on whether PC2 MPR with 1Tx or 2Tx will be applied

<Way forward>: FFS on PC1.5 MPR to use the same MPR as PC1.5 MPR with 2Tx. Subject to  Tx Diversity support

As TxD is precluded for 3Tx the 1Tx PC2 MPR should be used.for attaining PC2 when using the 3X26dBm architecture.

Proposal 1: As TxD is precluded for 3 Tx the 1 Tx MPR should be used for achieving PC2 when using the 3X26dBm architecture

For a UE to achieve PC1.5 it must be able to support TxD. An examination of table 6.2.1-1 [3] indicates that support of PC1.5 is only possible using TxD with a multi-Tx (for example dual-Tx) architecture.

Table 1: Excerpt of table 6.2.1.1 from [3]

Proposal 2: As TxD is precluded in 3Tx the support of PC1.5 is not possible

# Conclusion

In this paper we further present our views on power class support for 3 Tx and MPR table creation and make the following observations and proposals:

Observation 1: Not possible to configure a UE for PC1.5 in mode 0 using a single PA based on current technology.

Observation 2: Not possible for a UE to support higher power using TxD as TxD is precluded for 3 Tx.

Proposal 1: As TxD is precluded for 3 Tx the 1 Tx MPR should be used for achieving PC2 when using the 3X26dBm architecture

Proposal 2: As TxD is precluded in 3Tx the support of PC1.5 is not possible.

# References

[1] R4-2420400, WF on UE RF requirements for MIMO enhancement, November 18th ‒ 22nd November, 2024

[2] RAN4_112bis Main Session report, October 14th – October 18th, 2024

[3] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V18.7.0