3GPP TSG-RAN WG4 Meeting #114	R4-2501745

Athens, Greece, 17th – 21st February 2025

Source:	BT, Ericsson

Title:	Measurement results for MIMO channels in band n78

Agenda item:	7.16.1

Document for:	Information

Introduction

The Study on Spatial Channel Modelling for Demodulation Performance Requirements was approved at RAN#104 [1], and RAN4#112, RAN4#112b and RAN4#113 approved way forward agreements [2], [3] and [4] respectively.

In this contribution we provide observations of spatial characteristics of measured MIMO channels from typical deployments, focusing on the evolution of spatial preferences of channels in time. The purpose of this contribution is to support the group with identification of important properties of typical propagation environments to be reflected as close as reasonably possible in a spatial channel model. In this contribution we extend the set of measurements to n78 which is an important band for many commercial networks.  Other observations of basic properties of MIMO channels in sample typical deployments are provided in [5, 6, 7] focusing on 2100 and 2600 MHz bands.

Field measurements from typical deployments

Measurement setup and scope

Uplink sounding reference signals (SRS) were used for evaluation of respective MIMO channel between a typical commercial MIMO base station and a UE.  Measurements were conducted using a 40MHz channel in n78 band (3680 – 3720 MHz) in four locations A to D shown in Figure 1. Reported samples were collected in static positions during office hours, with some movement of people and vehicles in the coverage sector, yet not in the immediate vicinity of the measurement setup. The output of the measurements was reconstructed into channel gains per transmitter and receiver port pair (the H matrices), including all system gains and losses. The H matrices were post-processed, and in this contribution we report angles of arrival determined using MUSIC algorithm for four locations A to D.

Locations A and C can be characterised as having a dominant line-of-sight component, yet with several reflections from surrounding buildings. Locations B and D can be characterised as non-line-of-sight due to obstructions from surrounding buildings. However, the LoS or NLoS determination is indicative. The determination is based on analysis of power delay profiles from previous test campaigns utilising same locations. Sample typical PDP for downlink direction for Location A is provided in Figure 3 of [6].

This contribution presents an empirical analysis of angular spread distributions and eigenmode behaviour based on measurement data. The analysis provides insights into spatial dispersion characteristics and the evolution of channel rank in a practical deployment scenario. The SCM should have sufficient flexibility to accurately capture the observed behaviors, ensuring a realistic representation of multipath propagation and MIMO channel characteristics.

Figure 1. Measurement point locations for the sample MIMO channels.

| Figure 2. Empirical CDF of Eigen Modes for Locations A and C. |  |
| --- | --- |
| Figure 3. Empirical CDF of Eigen Modes for Locations B and D. |  |

Empirical CDF of Eigenmode Gains

Figures 2 and 3 illustrate the empirical cumulative distribution function (CDF) corresponding to the 4 locations. In this analysis, the focus was on exploring the eigenmode structure of sample MIMO channel from deployment. To analyze this variability, the eigenvalues of the measured MIMO channel matrices were calculated and converted to dB scale. The empirical CDF was then computed by sorting these values and determining their cumulative probability.

The results show that layers 1 & 2 have eigenmode gains concentrated in the −30 dB to −20 dB range, leading to a steep CDF, suggests strong eigenmodes with consistent gains, which are beneficial for low-rank transmission. Furthermore, the broader distribution of layers 3 & 4 introduces performance fluctuations that impact higher-rank transmission. These observations further confirm that when multiple (e.g. two) codewords are used, the codewords mapped to stronger layers (layers 1 & 2) is likely to experience a better performance, whereas the codeword mapped to layers 3 & 4 is likely to benefit from more robust MCS compared to codeword 1. This reinforces the simulation conclusion that the CW performance obtained using CDL models, as illustrated in Fig. 12 of [8], is more reflective of real-world deployments.

In typical MIMO deployments, dominant eigenmodes remain within a narrow high-gain range, while weaker eigenmodes are more spread out, leading to performance disparities among codewords.

Angular Spread in MIMO channels

| Figure 4. Empirical CDF of Angular Spread for Locations A and C |  |
| --- | --- |
| Figure 5. Empirical CDF of Angular Spread for Locations B and D. The empirical CDF of angular spread has been computed for 4 different locations, as shown in Figures 4 and 5, to analyze spatial clustering characteristics. The Angles of Arrival (AoAs) were estimated using the MUSIC algorithm, identifying three dominant AoAs per location. The empirical CDF of angular spread is then computed separately around each of these AoAs. The angular spread calculations are based on antenna correlation estimates, as described in [9]. The analysis is conducted in the azimuth domain, where correlations are derived from the horizontal spatial relationships between antenna elements. To enhance statistical robustness, each estimate combines 25 consecutive SRS receptions, representing an observation period of 500 ms. This approach ensures a reliable representation of angular spread variations across different locations. The analysis of angular spread CDFs around primary, secondary and tertiary AoAs suggests the presence of different types of spatial clusters in the MIMO channel environement. Primary clusters, characterized by a narrow angular spread, are crucial for high-SNR transmissions, while diffuse clusters with a broader angular spread enable higher-rank MIMO transmission. This presence of distinct spatial clusters indicates a spatial preference in MIMO channels, which is more acurately captured by CDL models. Observation 2: MIMO channels exhibit distinct spatial clusters indicating a spatial preference. |  |

Conclusion

Observations and proposals for the SCM are summarised below.

Observation 1: In typical MIMO deployments, dominant eigenmodes remain within a narrow high-gain range, while weaker eigenmodes are more spread out, leading to performance disparities among codewords.

Observation 2: MIMO channels exhibit distinct spatial clusters indicating a spatial preference.

- Acknowledgements

The measurements and data processing for this contribution have been conducted by Ericsson Research UK and BT Research teams under UK DSIT REASON project and UKRI Future Leaders Fellowship (MR/Y020065/1).

References

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, RAN4#112, Aug 2024.

- R4-2416591, Way Forward for [112b][328] FS_NR_demod_SCM, RAN4#112b, Oct 2024.

- R4-2419782, Way Forward for [113][320] NR_SCM, RAN4#113, Nov 2024.

- R4-2411557, “Scenarios and requirements for spatial channel modelling”, BT, RAN4#112, Aug 2024.

- R4-2415673, “Further observations from deployment and MU-MIMO use cases for SCM evaluation”, BT, Orange, RAN4#112b, Oct 2024.

- R4-2419338, “Further observations on the properties of MIMO channels in typical deployments”, BT, Ericsson, RAN4#113, Nov 2024.

- R4-2418043, “Discussion on Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#113, Nov 2024

- Buehrer, R. Michael. "Generalized equations for spatial correlation for low to moderate angle spread." Wireless Personal Communications: Bluetooth and Other Technologies. Boston, MA: Springer US, 2002. 101-108.