3GPP TSG-RAN4 Meeting #114	R4-2501810

# Athens, GR, 17th – 21st February, 2025

Title:	Discussion on IoT-NTN HPUE Tx requirements and handheld SAR

Source:	Samsung

Agenda item:	7.10.2.3.1

Document for:	Approval

1	Introduction

In previous several RAN4 meetings, the WF[2] has captured the agreements and open issues for IoT-NTN Tx requirements, and also the potential SAR solutions.

In this paper, we propose to further discuss the ACLR requirements given the initial conclusion from co-existudy is given in [1], and also to share our view on a duty cycle based SAR solution.

2	Tx requirements

From the last meeting WF [1] and also we proposed it in a separate paper, it can be concluded from co-ex study point of view, that the ACLR for IoT-NTN for PC2 and PC1 are much less than the PC3 IoT-NTN ACLR (UTRA_ACLR as 37dB). The required ACLR are below 25 dBc for up to PC1 and regardless of the isolation distance.

Observation 1: The required ACLR from co-ex study are below 25 dB for up to PC1 for IoT-NTN regardless of isolation distance.

Considering the agreement reached in last meeting in [2], which is not to have more relaxed ACLR compared to PC3 for NB-IoT NTN. It is proposed to re-use the PC3 NB-IoT NTN ACLR for PC2 and PC1, if specified.

Proposal 1: To re-use the PC3 NB-IoT NTN ACLR for IoT-NTN PC2 and PC1, if specified.

3	SAR solution

In [2], the SAR solutions were proposed by different companies, and initially discussed. From the discussion, we list several findings below:

- The common view is to maintain the HPUE at its desired nominal power as much as possible;

- The TN dutycycle solution is a good reference;

- A ’TDD duty cycle’ is needed for NTN FDD band if duty cycle like solution is to be used.

So based on above, and considering P-MPR will always be a baseline solution, we try to establish several working assumptions before we continue exploring the potential SAR solutions. And these working assumptions are:

- UE to transmit in its declared power class nominal power, without applying P-MPR, as much as it can. (Otherwise, it’s P-MPR baseline solution)

- UE to take responsibility of its own SAR monitoring, but to fall back as less as possible. (Not possible for NW to take reponsibility of UE SAR compliance)

- There is a NW indication about the uplink duty ratio for a beam or a cell. (NTN_Phase3 is working on beam hopping, and each beam will have different level of service chance in TDM hopping pattern.)

- UE to report to NW about it’s decision, and this decision needs to be specified (otherwise it’s non 3GPP method)

Proposal 2: To consider following working assumptions for discussing a SAR solution other than baseline P-MPR or non-3GPP tech:

- UE to transmit in its declared power class nominal power, without applying P-MPR, as much as it can. (Otherwise, it’s P-MPR baseline solution)

- UE to take responsibility of its own SAR monitoring, but to fall back as less as possible. (Not possible for NW to take reponsibility of UE SAR compliance)

- There is a NW indication about the uplink duty ratio for a beam or a cell. (NTN_Phase3 is working on beam hopping, and each beam will have different level of service chance in TDM hopping pattern.)

- UE to report to NW about it’s decision, and this decision needs to be specified (otherwise it’s non 3GPP method)

With the above working assumption, if we go through the process from UE side:

- UE acknowledged a TDD-like uplink ratio that current cell or beam utilized (from TDM beam hopping information);

- UE report its power class and associated maxUplinkDutyCycle;

- UE to not consider P-MPR in power configuration;

- Option 1: UE to fall back power classes based on information above ( legacy duty cycle solution, not quite aligned with purpose to transmit as much and as long as possible)

Option 2: UE to fall back several dB but not entire power class, based on information above ( technically UE can reach more power than Option 1)

Option 3: UE to not fall back any power, but to monitor SAR by itself. ( In this case, UE may fall into the situation that it reached the bar for SAR compliance.)

- Option 3-1: UE to enter something like a DTx pattern, or UL Gap pattern for a certain duration.

- Option 3-2: UE to give up a certain ratio of uplink opportunities for a certain duration to recover its margin for SAR compliance.

- Option 3-3: UE to fall back to lower power class for a certain duration to recover its margin for SAR compliance.

From above, we believe a TDD-like uplink ratio information from NW side seems critical in defining a duty cycle like SAR solution in 3GPP, otherwise the UE can monitor and maintain its own SAR performance with existing outside 3GPP technologies, and P-MPR can be simply used as baseline.

And among the current NTN and FDD WI, the beam hopping information in NTN Phase 3 for coverage enhancement and energy saving seems the only TDM information which has the direct impact to the UE UL duty in time domain that probably will be spcified.

With this, we propose to use this beam hopping information as the UE uplink duty ratio in time domain if duty cycle-based SAR solution is to be specified.

Proposal 3: To use beam hopping information as the UE uplink duty ratio in time domain if duty cycle-based SAR solution is to be specified.

And the above analysis for duty cycle solution is provided for further discussion:

Proposal 4: To consider following options in our analysis:

With the above working assumption, if we go through the process from UE side:

- UE acknowledged a TDD-like uplink ratio that current cell or beam utilized (from TDM beam hopping information);

- UE report its power class and associated maxUplinkDutyCycle;

- UE to not consider P-MPR in power configuration;

- Option 1: UE to fall back power classes based on information above ( legacy duty cycle solution, not quite aligned with purpose to transmit as much and as long as possible)

Option 2: UE to fall back several dB but not entire power class, based on information above ( technically UE can reach more power than Option 1)

Option 3: UE to not fall back any power, but to monitor SAR by itself. ( In this case, UE may fall into the situation that it reached the bar for SAR compliance.)

- Option 3-1: UE to enter something like a DTx pattern, or UL Gap pattern for a certain duration.

- Option 3-2: UE to give up a certain ratio of uplink opportunities for a certain duration to recover its margin for SAR compliance.

- Option 3-3: UE to fall back to lower power class for a certain duration to recover its margin for SAR compliance.

4	Conclusion

For IoT-NTN HPUE Tx requirements and handheld SAR, we propose the following:

Observation 1: The required ACLR from co-ex study are below 25 dB for up to PC1 for IoT-NTN regardless of isolation distance.

Proposal 1: To re-use the PC3 NB-IoT NTN ACLR for IoT-NTN PC2 and PC1, if specified.

Proposal 2: To consider following working assumptions for discussing a SAR solution other than baseline P-MPR or non-3GPP tech:

- UE to transmit in its declared power class nominal power, without applying P-MPR, as much as it can. (Otherwise, it’s P-MPR baseline solution)

- UE to take responsibility of its own SAR monitoring, but to fall back as less as possible. (Not possible for NW to take reponsibility of UE SAR compliance)

- There is a NW indication about the uplink duty ratio for a beam or a cell. (NTN_Phase3 is working on beam hopping, and each beam will have different level of service chance in TDM hopping pattern.)

- UE to report to NW about it’s decision, and this decision needs to be specified (otherwise it’s non 3GPP method)

Proposal 3: To use beam hopping information as the UE uplink duty ratio in time domain if duty cycle-based SAR solution is to be specified.

Proposal 4: To consider following options in our analysis:

With the above working assumption, if we go through the process from UE side:

- UE acknowledged a TDD-like uplink ratio that current cell or beam utilized (from TDM beam hopping information);

- UE report its power class and associated maxUplinkDutyCycle;

- UE to not consider P-MPR in power configuration;

- Option 1: UE to fall back power classes based on information above ( legacy duty cycle solution, not quite aligned with purpose to transmit as much and as long as possible)

Option 2: UE to fall back several dB but not entire power class, based on information above ( technically UE can reach more power than Option 1)

Option 3: UE to not fall back any power, but to monitor SAR by itself. ( In this case, UE may fall into the situation that it reached the bar for SAR compliance.)

- Option 3-1: UE to enter something like a DTx pattern, or UL Gap pattern for a certain duration.

- Option 3-2: UE to give up a certain ratio of uplink opportunities for a certain duration to recover its margin for SAR compliance.

- Option 3-3: UE to fall back to lower power class for a certain duration to recover its margin for SAR compliance.

5	Reference

[1] R4-2420403, WF on NR NTN HPUE, Samsung

[2] R4-2420404, WF on HPUE for IoT NTN, vivo