3GPP TSG-RAN4 Meeting #114	R4-2501811

# Athens, GR, 17th – 21st February, 2025

Title:	Discussion on IoT-NTN HPUE Rx requirements

Source:	Samsung

Agenda item:	7.10.2.3.2

Document for:	Approval

1	Introduction

In previous several RAN4 meetings, the Rx requirements, especially the RSD requirements for different power classes were discussed and analyzed.

In this paper, we continue to discuss the remaining issues and to progress the work.

2	Rx requirements

In WF[1], the current RSD discussions are as follows:

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based) - Proposals - N256: - 0 dB (vivo, oppo, Ericsson) - [1] dB (Qualcomm) - N255: - 0 dB (oppo) - Using n3 RSD as reference (Samsung, last meeting) - N254: - [0] dB (vivo, Samsung, Ericsson) - WF - N254: [0] dB - Other bands FFS; Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based) - Proposals - Option 1: 0dB for n254/n255/n256 (vivo, oppo) - Option 2: others - WF - [0] dB for n254/n255/n256 Issue 3-1-x: General for RSD Way forward: - Encourage companies to investigate and discuss the impact of Rx non-linearity in RSD evaluations.

For PC1, at least for the non-handheld PC1 eMTC based IoT-NTN, the RSD can be agreed to be 0, considering the size of the non-handheld UE. And this can applies to the examplary bands, including n256.

Proposal 1: For n256, non-handheld PC1 eMTC based full duplex IoT NTN HPUE, the RSD to be 0dB.

With regards to the eMTC based IoT NTN PC2 handheld HPUE, especially for n256 and n254, we tends to support the options of 0dB RSD. The same rationale is given in NR-NTN HPUE PC2 RSD discussion.

Proposal 2: For n256, n254, the RSD can be 0 dB for PC2 eMTC based IoT-NTN full duplex handheld UE.

3	Conclusion

For NR-NTN HPUE Rx requirements, we propose the following:

Proposal 1: For n256, non-handheld PC1 eMTC based full duplex IoT NTN HPUE, the RSD to be 0dB.

Proposal 2: For n256, n254, the RSD can be 0 dB for PC2 eMTC based IoT-NTN full duplex handheld UE.

4	Reference

[1] R4-2420403, WF on HPUE for IoT NTN, vivo