3GPP TSG-RAN WG4 Meeting #114	R4-2501826

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	draft LS reply on UE Capabilities for FR2-NTN

Agenda item:	5.21.1

Document for:	Approval

Introduction

RAN2#128 discussed UE capabilities for FR2-NTN bands and sent LS to RAN4 to ask whether UE capabilities for 200 MHz channel bandwidth and 2 MIMO layers are mandatory for FR2-NTN bands [1].

Discussion

For TN FR2 bands, 50, 100, and 200 MHz channel bandwidths are mandatory, as is described in UE capability per FSPC (feature set per CC), supportedBandwidthDL and supportedBandwidthUL in TS 38.306. UE supported channel bandwidth per band may be also signalled in channelBWs-DL and channelBWs-UL, where a bit for 200 MHz shall be set 1.

For TN, it may be possible that multiple channel bandwidths are used in the same operator network or other operator network in roaming, so it is strongly preferred that all the channel bandwidths are supported by UE. For NTN operation, it may not be required as strongly as TN for UE to support all the possible channel bandwidth configurations to support the roaming. Thus, it may not be necessary to mandate all the channel bandwidths for UE to allow various network operation demands.

Since the link budget is very tight for NTN and the correlation between antennas is expected to be high due to the line-of-sight conditions, we expect the applicability of multi-layer MIMO operation of any kind (apart from beam forming) is quite limited. Thus, we see the 2 MIMO layer does not need to be mandatory for all UEs.

200 MHz channel bandwidth (or any other) and 2 MIMO layers can be optional for UE.

- References

- R2-2411195/R4-2500014 LS on UE Capabilities for FR2-NTN, RAN2#128

- Annex: draft LS reply

Title:	draft LS reply on UE Capabilities for FR2-NTN

Response to:

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:			Hisashi Onozawa

E-mail Address:	hisashi.onozawa@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

# 1. Overall Description:

At RAN4#114, RAN4 has discussed LS on UE Capabilities for FR2-NTN (R2-2411195/R4-2500014). RAN4 would like to inform that UE capabilities for 200 MHz (and any other) channel bandwidth and 2 MIMO layers are optional for FR2-NTN bands.

# 2. Actions:

To RAN2 group:

RAN4 respectfully asks for RAN2 to take the above conclusion into account.

# 3. Date of Next RAN4 Meetings:

RAN4#114-bis	from 2025-04-07	to 2025-04-11		China, CN

RAN4#115	from 2025-05-19	to 2025-05-23		Malta, MT