3GPP TSG-RAN WG4 Meeting #114                                         R4-2501832

Athens, Greece, February 17 - 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On power domain enhancements for single carrier

Agenda Item:	7.1.1.2.1

Document for:	Approval

Introduction

During RAN4#113, power domain enhancement for single carrier has been extensively discussed. Conclusions and FFS points can be found in [1].

- Issue 1-1-1: Approaches of converting outer RB allocation to inner RB allocation Agreement in main session: - Extended CBW based approach - where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE - Both scenario 1 and 2 should be covered - The network should ensure the co-existence be met when signaling the extended CBW.

In this contribution, we would like to share our views on this topic concentrating on the unified solution.

Discussion

Undetermined details of the general solution

One issue needs further discussion is what exactly should be conveyed towards the UE about the extended CBW. To our understanding, the UE should acquire following information to enable MPR reduction:

- Where the applied frequency range of OOB requirements should be relocated, and

- Taking the extended CBW into account, what is the actual NRB that should be used to determine the Inner/Outer RB allocations

We think the RB/MHz level indication leads to any unspecified value of CBW should not be pursued, because the start of applied frequency range of OOB requirements is only defined as the edge of one of those specified CBW in TS 38.101-1. With arbitrary RB or MHz number indicated by network, UE has to check whether it can deliver reduced MPR for each outer RB allocation but the network cannot know the result in advance unless additional feedback from the UE can be expected, which must significantly increase implementation complexity.

Observation 1: For the length of the extended CBW, any unspecified RB number or CBW value should not be pursued. Because the network still needs UE feedback on whether reduced MPR can be expected for a concerned RB allocation.

As it was recognized by RAN4 in last meeting, any extended CBW larger than 1/2 of the assigned UE CBW won’t help the UE in terms of MPR reduction anymore, so it is necessary to hardcode such upper limit for the sake of avoiding invalid information exchange between BS and UE. But in last meeting, there was a proposal about only allowing the extended CBW to be configured as 1/2 of the assigned UE CBW in exchange of less implementation complexity. In our view, technical reason behind such restriction is not clear. Looking at the UE CBW configuration itself, there is no doubt that the UE can be configured with a CBW smaller than the maximum one it supports. What the two compared things in common is that the UE can handle a shifted start of OOB requirements as long as its frequency won’t exceed the one it has advertised & calibrated. Thus we don't think such restriction is necessary.

Observation 2: For a UE supports MPR reduction on full set of Outer RB allocations, it can automatically support the MPR reduction on a subset of Outer RB allocations when the extended CBW is configured as a specified CBW smaller than 1/2 of the assigned UE CBW.

The above limitation is also applicable when the assigned UE CBW is not even. For instance, if the assigned UE CBW is 35MHz, then the configurable extended CBW would be one of 15MHz, 10MHz, 5MHz (not for 60kHz SCS) and 3MHz (only for 15kHz SCS).Last but not the least, since RAN4 already agreed that both Scenario 1 and 2 should be considered, it means that the asymmetric extension should be considered in order to handle all possible spectrum holdings. In conclusion, we have the following proposal.

Proposal 1: Adopt following bullet points aiming for completing the definition of single carrier MPR reduction:

- The UE should still be single carrier operation when enabling this feature, the applicable assigned UE CBW should be up to 100MHz.

- The extended CBW can be configured at one side or both sides starting from the edge of the assigned UE CBW. In other words, asymmetric extension should be allowed.

- The length of each extended CBW can be independently configured as one of the CBW in MHz that have been specified in Table 5.3.2-1 in TS 38.101-1.

- The configured value should not be larger than 1/2 of the assigned UE CBW.

  - For instance, if the assigned UE CBW is 35MHz, then the configurable extended CBW would be one of 15MHz, 10MHz, 5MHz (not for 60kHz SCS) and 3MHz (only for 15kHz SCS).

Regarding how to get the actual NRB which is the input for new Inner/Outer allocations determination, a straightforward solution would be referring to the same table, i.e. Table 5.3.2-1 in TS 38.101-1, as we proposed for extended CBW. Hence the function for deriving total NRB can be replaced to the following one:

N'RB=Nextended_low+NRB+Nextended_high,

where Nextended is the maximum transmission bandwidth configuration of the extended CBW corresponding to the length of the extended CBW, expresses in MHz, configured by network and the subcarrier spacing of this component carrier. The rest of the suffix is to distinguish the extended CBW at the lower frequency side from the higher one.

Proposal 2: Adopt following equation for deriving the total maximum transmission bandwidth configuration, where Nextend_low/high is the maximum transmission bandwidth configuration corresponding to the subcarrier spacing of the component carrier and the extended CBW, expresses in MHz, located at lower or higher frequency side, respectively.

N'RB=Nextended_low+NRB+Nextended_high

For better illustration, we can provide an example as:

- The assigned UE CBW is 20MHz with 30kHz SCS

- Both lower and higher extended CBW are configured as 10MHz

Then N'RB would be 24+51+24=99 (which is not the NRB=106 defined for 40MHz).

Figure 1. Example on how to determine the new Inner RB allocations with configured N'RB

Applicable requirements

- How to relax the out-of-band requirements

For the out-of-band (OOB) and spurious domain definition, 3GPP adopted a more strict requirement than the one defined by ITU-R which is introduced as 250% of the necessary bandwidth from the center of transmission as the boundary according to SM.329. In this sense, we think from regulatory point of view there would be no issue to consider shifting the applicable range of OOB and spurious emision together to half of the UE CBW away. But if the applicable range and/or shifting distance will be based on BS CBW, then there could be a risk on regulatory violation. Thus we have the following preferences with the intention of maximizing the gain:

- ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW, since this is way more general than only consider the applicable range from the edge of BS CBW.

- The application range of SE should be altered with the shifting of the edge of the UE CBW, which has no violation to the regulatory.

- The applicable range of ACLR, SEM and SE requirements should still be based on UE CBW, because the UE CBW is where UL transmission will be scheduled but not the extended UE CBW.

Proposal 3: The OOBE requirements should be based on UE CBW rather than extended UE CBW.

Proposal 4: The spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the extended channel bandwidth.

- How to define the IBE within the extended CBW

RAN4 has agreed that IBE should be used between edges of UE CBW and the extended UE CBW, but how to derive the extended IBE would require further study. For the partial RB allocation scenario, the first RB adjacent to the UE CBW can be counted as the next RB index to the one used by the very last RB within the scheduled block at the same side.

Figure 2. Illustration on the partial RB allocation scenario where IBE applies to the extended UE CBW

Proposal 5: For the partial RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs at one side is numbering right after the adjacent usable RB within UE CBW at that side.

For the full RB allocation scenario, the RB index for the first RB (pair) adjacent to the UE CBW becomes +1(-1) and it is applied for all extended RBs, so the guard band within the UE CBW is not counted. The equation derived from the general part of IBE for this case would be:

max-25, 10log10EVM-3, -57dBm+10log10SCS15kHz-PRB

Proposal 6: For the full RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs is set to +1(-1).

Figure 3. Illustration on the full RB allocation scenario where IBE applies to the extended UE CBW

Another tricky part of this issue is whether the UE can get real relaxation in adjacent channel from IBE. As a result, we compare the emission limits between the legacy SEM mask at the first 1MHz interval and IBE, as the PSD within this interval may not roll down fast enough. Numbers are summarized in table below.

Table. Comparison of emission level @ the first 1MHz adjacent to the UE CBW

|  | UE CBW (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| SEM (dBm/30kHz) | -18.2 | -20.0 | -21.2 | -22.2 | -23.0 | -23.6 | -24.2 | -24.8 | -24.0 |  |  |  |  |  |
| IBE QPSK 26dBm (dBm/30kHz) | -16.7 | -18.7 | -20.0 | -21.0 | -21.8 | -22.5 | -23.2 | -23.7 | -24.1 | -25.0 | -25.7 | -26.3 | -26.8 | -27.2 |
| IBE 16QAM 26dBm (dBm/30kHz) | -19.6 | -21.6 | -22.9 | -24.0 | -24.8 | -25.5 | -26.1 | -26.6 | -27.1 | -28.0 | -28.6 | -29.2 | -29.7 | -30.2 |
| IBE BPSK 26dBm (dBm/30kHz) | -12.0 | -14.0 | -15.3 | -16.4 | -17.2 | -17.8 | -18.5 | -19.0 | -19.5 | -20.3 | -21.0 | -21.6 | -22.1 | -22.6 |

Note that those IBE requirements are derived from the general equation assuming full bandwidth allocation. It can be observed that IBE requirements are more stringent than SEM when the EVM of concerned modulation order applies, which must be against the intention for MPR reduction. Instead, using the BPSK EVM can achieve more relaxed emission limits in the first 1MHz in adjacent channel for each of every UE CBW.

Observation 3: To achieve OOB emission limits more relaxed than SEM in extended CBW, one should use the EVM for BPSK in IBE equation instead of the EVM for QPSK/16QAM.

So we propose:

Proposal 7: For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK.

Our initial thinking on how to implement this feature into TS 38.101-1 can be found in our draft CR [2].

Conclusion

In this contribution we discussed on the Rel-19 MPR reduction for single CC. We have the following observations and proposals:

Observation 1: For the length of the extended CBW, any unspecified RB number or CBW value should not be pursued. Because the network still needs UE feedback on whether reduced MPR can be expected for a concerned RB allocation.

Observation 2: For a UE supports MPR reduction on full set of Outer RB allocations, it can automatically support the MPR reduction on a subset of Outer RB allocations when the extended CBW is configured as a specified CBW smaller than 1/2 of the assigned UE CBW.

Observation 3: To achieve OOB emission limits more relaxed than SEM in extended CBW, one should use the EVM for BPSK in IBE equation instead of the EVM for QPSK/16QAM.

Proposal 1: Adopt following bullet points aiming for completing the definition of single carrier MPR reduction:

- The UE should still be single carrier operation when enabling this feature, the applicable assigned UE CBW should be up to 100MHz.

- The extended CBW can be configured at one side or both sides starting from the edge of the assigned UE CBW. In other words, asymmetric extension should be allowed.

- The length of each extended CBW can be independently configured as one of the CBW in MHz that have been specified in Table 5.3.2-1 in TS 38.101-1.

- The configured value should not be larger than 1/2 of the assigned UE CBW.

  - For instance, if the assigned UE CBW is 35MHz, then the configurable extended CBW would be one of 15MHz, 10MHz, 5MHz (not for 60kHz SCS) and 3MHz (only for 15kHz SCS).

Proposal 2: Adopt following equation for deriving the total maximum transmission bandwidth configuration, where Nextend_low/high is the maximum transmission bandwidth configuration corresponding to the subcarrier spacing of the component carrier and the extended CBW, expresses in MHz, located at lower or higher frequency side, respectively.

N'RB=Nextended_low+NRB+Nextended_high

Proposal 3: The OOBE requirements should be based on UE CBW rather than extended UE CBW.

Proposal 4: The spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the extended channel bandwidth.

Proposal 5: For the partial RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs at one side is numbering right after the adjacent usable RB within UE CBW at that side.

Proposal 6: For the full RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs is set to +1(-1).

Proposal 7: For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK.

- References

- R4-2420527, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #113, Orlando, US, November 18-22, 2024.

- R4-2501833, “Draft CR for TS 38.101-1 to introduce MPR reduction for single carrier”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting#114, Athens, Greece, February, 17-21, 2025.

- Annex

- Scenario classification based on the worldwide spectrum allocation

As RAN4 agreed to study below three scenarios, it would be beneficial to look closely at each one and take the worldwide spectrum allocation and deployment into account, so that the corresponding solution can be more targeted.

In the following part, we list all the suitable examples under each scenario. What needs to be noted is that the spectrum allocation/deployment may not be static by the time passage where spectrum policy and business decision could weight in, so we only explicitly mark the region but waive the operator.

The distinction between Scenario 1-1 and Scenario 1-2

For these two scenarios, we understand the key criteria to distinguish Scenario 1-1 from Scenario 1-2 is whether there is any other operator(s) close to the holding spectrum. Specifically, the Scenario 1-1 stands for the case where a sufficient gap (at each side) exists between two allocations of spectrum (not shorter than the half of the indicated maximum UE channel bandwidth or the indicated OOB boundary shifting offset for UE to apply inner MPR to out RB allocations). Otherwise, it should be Scenario 1-2. Note that the case where the holding spectrum is larger than 100MHz should be included as well, because besides the gap there are operator’s own frequencies that can be used as offset for shifting the OOB boundary.

Observation: The key criteria to distinguish Scenario 1-1 from Scenario 1-2 is whether there is sufficient gap between operator holding spectrums.

- The gap (at each side) is sufficient if it is larger than the half of the indicated maximum UE channel bandwidth or the indicated OOB boundary shifting offset for UE to apply inner MPR to out RB allocations.

- The case where the concerned holding spectrum is larger than 100MHz should be included as well.

Accordingly, we can screen out the matched ones based on the current spectrum allocation and list them in Table 2. One interesting finding is that for a same band e.g. n78, it can belong to different scenario due to certain spectrum allocation and deployment.

Table 1. On how the scenario belonging would change for band n78 in different region

| Region | Spectrum | Matched Scenario |
| --- | --- | --- |
| China |  | 1-1 (300MHz) |
| Romania |  | 1-2 (115MHz, at least 15MHz at each side can be used as offset) |

Table 2. On the operator holding spectrum that fits Scenario 1-1 or Scenario 1-2

| Region | NR band | Holding spectrum for UL | Matched Scenario | Note |
| --- | --- | --- | --- | --- |
| Australia | n7 | 2510MHz ~ 2550MHz 40MHz | 1-2 | Insufficient gap adjacent to this CBW (>2550MHz is occupied by adjacent operator) |
| Britain | n40 | 2350MHz ~ 2390MHz 40MHz | 1-1 | Single operator |
| China | n28 | 703MHz ~ 733MHz 30MHz | 1-1 | Co-constructed and shared by operators |
|  | n5 | 825MHz ~ 835MHz 10MHz | 1-1 | Single operator |
|  | n39 | 1885MHz ~ 1915MHz 30MHz | 1-2 | Insufficient gap (5MHz) adjacent to this CBW |
|  | n1 | 1920MHz ~ 1965MHz 45MHz | 1-2 | Co-constructed and shared by operators Insufficient gap (15MHz) adjacent to this CBW |
|  | n34 | 2010MHz ~ 2025MHz 15MHz | 1-1 | Single operator |
|  | n40 | 2300MHz ~ 2370MHz 70MHz | 1-1 | Single operator |
|  | n41 | 2515~2675MHz 160MHz | 1-1 | Single operator |
|  | n78 | 3300~3600MHz 300MHz | 1-1 | Co-constructed and shared by operators |
|  | n79 | 4800~4960MHz 160MHz | 1-1 | Co-constructed and shared by operators |
| Croatia | n78 | 3480MHz ~ 3600MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (>3600MHz is occupied by adjacent operator) |
| Denmark | n1 | 1920MHz ~ 1940MHz 20MHz | 1-2 | Insufficient gap adjacent to this CBW (>1940MHz is occupied by adjacent operator) |
|  | n40 | 2300MHz ~ 2400MHz 100MHz | 1-1 | Single operator |
|  | n78 | 3410MHz ~ 3540MHz 130MHz | 1-2 | Insufficient gap adjacent to this CBW (>3540MHz is occupied by adjacent operator) |
|  | n78 | 3540MHz ~ 3660MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (<3540MHz is occupied by adjacent operator) |
| Finland | n78 | 3410MHz ~ 3540MHz 130MHz | 1-2 | Insufficient gap adjacent to this CBW (>3540MHz is occupied by adjacent operator) |
| Greek | n78 | 3410MHz ~ 3450MHz 40MHz | 1-2 | Insufficient gap adjacent to this CBW (>3450MHz is occupied by adjacent operator) |
| Hungary | n78 | 3410MHz ~ 3470MHz 60MHz | 1-2 | Insufficient gap adjacent to this CBW (>3470MHz is occupied by adjacent operator) |
| Ireland | n40 | 2300MHz ~ 2360MHz 60MHz | 1-1 | Single operator |
| Lithuania | n40 | 2310MHz ~ 2390MHz 80MHz | 1-1 | Single operator |
| Luxembourg | n78 | 3420MHz ~ 3530MHz 110MHz | 1-2 | Insufficient gap adjacent to this CBW (>3540MHz is occupied by adjacent operator) |
|  | n78 | 3540MHz ~ 3650MHz 110MHz | 1-2 | Insufficient gap adjacent to this CBW (<3530MHz and >3650MHz are occupied by adjacent operator, respectively) |
|  | n78 | 3650MHz ~ 3750MHz 100MHz | 1-2 | Insufficient gap adjacent to this CBW (<3650MHz is occupied by adjacent operator and 3800MHz is the edge of n78) |
| Netherland | n34 | 2010MHz ~ 2025MHz 15MHz | 1-1 | Single operator |
| Norway | n78 | 3400MHz ~ 3480MHz 80MHz | 1-1 | Sufficient gap (100MHz) adjacent to this CBW |
|  | n78 | 3580MHz ~ 3700MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (>3700MHz is occupied by adjacent operator) |
| Romania | n78 | 3700MHz ~ 3750MHz 50MHz | 1-1 | Sufficient gap (>=50MHz) adjacent to this CBW |
|  | n78 | 3490MHz ~ 3530MHz 40MHz | 1-2 | Insufficient gap adjacent to this CBW (>3530MHz is occupied by adjacent operator) |
|  | n78 | 3530MHz ~ 3645MHz 115MHz | 1-2 | Insufficient gap adjacent to this CBW (<3530MHz and >3700MHz are occupied by adjacent operator, respectively) |
| Slovenia | n34 | 2010MHz ~ 2025MHz 15MHz | 1-1 | Single operator |
| Spain | n28 | 703MHz ~ 733MHz 30MHz | 1-1 | Co-constructed and shared by operators |
|  | n78 | 3420MHz ~ 3500MHz 80MHz | 1-2 | Insufficient gap adjacent to this CBW (<3420MHz is occupied by adjacent operator) |
| Sweden | n78 | 3400MHz ~ 3500MHz 100MHz | 1-2 | Insufficient gap adjacent to this CBW (>3500MHz is occupied by adjacent operator) |
|  | n78 | 3500MHz ~ 3620MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (<3500MHz is occupied by adjacent operator) |
| Switzerland | n78 | 3500MHz ~ 3580MHz 80MHz | 1-2 | Insufficient gap adjacent to this CBW (>3580MHz is occupied by adjacent operator) |

On Scenario 2

For this scenario, there is a previous agreement that the shifted boundary of where ACLR and SEM applied cannot exceed the BS CBW in order to avoid additional interference to adjacent operators. Hence, any spectrum allocation with no gap to other operator in adjacent channels can be incorporated to Scenario 2. But one should focus on the case where the operator holding spectrum is larger than 100MHz instead and it doesn’t need to confine to single carrier operation at BS side because:

- The multi-carriers deployment could have a disposition to configure 100MHz to one CC as it could be a practical decision after considering achievable peak throughputs and overall coverage performance along with the maximum CBW capability normally indicated by an eMBB UE i.e. 100MHz.

- But if it is arbitrarily confined to single carrier, to accommodate the required offset e.g. 1/2 UE CBW into holding spectrum, the gNB may have to configure the UE with a smaller CBW if it wants to enable this feature.

- The reduced MPR is still the same as the original one for inner region, which can be achieved under inner RB allocations scheduling.

- Since a smaller UE CBW is configured, the original outer RB allocation cannot be scheduled anymore.

Figure 1. Illustration on the drawbacks when confining to single carrier at BS side

Observation: As depicted in Figure 1, for Scenario 2, one should focus on the case where the operator holding spectrum is larger than 100MHz and it is multi-carriers operation at BS side. Because:

- For >100MHz contiguous spectrum, it could have a disposition to configure 100MHz to one CC as it could be a practical decision after considering achievable peak throughputs and overall coverage performance along with the maximum CBW capability normally indicated by an eMBB UE i.e. 100MHz.

- But if it is arbitrarily confined to single carrier, to accommodate the required offset e.g. 1/2 UE CBW into holding spectrum, the gNB may have to configure the UE with a smaller CBW rather than the supported maximum UE CBW, if it wants to enable this feature.

- The reduced MPR is still the same as the one that can be achieved on the very same RB allocations, which are identical to the inner RB allocations within the original maximum UE CBW.

- Since a smaller UE CBW is configured, the original outer RB allocation cannot be scheduled anymore.

- Last but not the least, the motivation of this feature is to unleash the UE MPR potential but not power saving.

Then we can list the qualified cases in below table.

Table 3. On the operator holding spectrum that fits Scenario 2

| Region | NR band | Holding spectrum for UL | Note |
| --- | --- | --- | --- |
| Any kind of spectrum allocation that the BS sees fit for extended UE CBW |  |  | Under a certain spectrum allocation, the BS may need to reconfigure the UE CBW in order to accommodate the extended UE CBW. |
| Canada | n77 | >100MHz (depending on specific province) | Co-constructed and shared by operators |
| Finland | n78 | 3540MHz ~ 3670MHz 130MHz | No gap adjacent to this CBW (<3540MHz and >3670MHz are occupied by adjacent operator, respectively) |
| Greek | n78 | 3450MHz ~ 3600MHz 150MHz | No gap adjacent to this CBW (<3450MHz and >3600MHz are occupied by adjacent operator, respectively) |
| Hungary | n78 | 3540MHz ~ 3660MHz 120MHz | No gap adjacent to this CBW (<3540MHz and >3660MHz are occupied by adjacent operator, respectively) |
| Slovenia | n78 | 3560MHz ~ 3700MHz 140MHz | No gap adjacent to this CBW (<3560MHz and >3700MHz are occupied by adjacent operator, respectively) |
| Spain | n78 | 3600MHz ~ 3710MHz 110MHz | No gap adjacent to this CBW (<3600MHz and >3710MHz are occupied by adjacent operator, respectively) |
| Switzerland | n78 | 3580MHz ~ 3700MHz 120MHz | No gap adjacent to this CBW (<3580MHz and >3700MHz are occupied by adjacent operator, respectively) |

System level simulation on including BS side multi-carriers operation for MPR reduction

In TR 36.942, the assumption for determining co-existence requirements i.e. UE ACLR who dominates the ACIR is captured as a 3%~5% throughput loss tolerance. By taking this experience value as metric, we evaluate the co-existence performance on applying MPR reduction to BS side multi-carriers operation with following considerations:

- For the BS side multi-carriers operation, it is two contiguous CCs (same RB length) in the same band.

- For a victim PC3 UE on CC1, the aggressor is another PC3 UE on CC2 that will apply MPR reduction by shifting the ACLR boundary overlapping with CC1.

- As the ACLR boundary shifting away, the interference in the first adjacent channel (the overlapping part in CC1) is increased. Following figure can better explain it.

Figure 2. Illustration on why the interference in the 1st adjacent channel is increased

- As depicted in Figure 2, since the PSD would be lower in the “new” 1st adjacent channel and the same ACLR value applies in that integral region, there must be a room for the aggressor to deliver higher output power by utilizing reduced MPR. Thus the interference landed in CC1 will increase.

- By observing the throughput loss on CC1 with different shifted ACLR boundary, we can identify the offset when the loss hits e.g. 3%.

|  | Throughput loss. ACLR boundary; @50% CDF; @90% CDF. Shift to 1/5 UE CBW; 0.88%; 1.43%. Shift with 1/2 UE CBW; 2.26%; 2.78%. |
| --- | --- |

Figure 3. Comparison on the CDF curve of throughput when different ACLR boundary is applied

From the above results, we can find that the throughput loss due to reduced MPR is acceptable when the ACLR integral region is overlapping with the adjacent CC.

Observation: Taking 3% throughput loss as the metric, the interference to the 1st adjacent channel (overlapping with another CC right next to the concerned CC) caused by a UE enables MPR reduction is acceptable.