3GPP TSG-RAN WG4 Meeting #114                                         R4-2501834

Athens, Greece, February 17 - 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On MPR applicability for FR1 intra-band UL CA

Agenda Item:	7.1.1.2.2

Document for:	Approval

Introduction

During RAN4#113, MPR applicability for FR1 intra-band contiguous/non-contiguous UL CA has been discussed. Following conclusions can be found in [1].

- Intra-band contiguous UL CA Sub-topic description Open issues and candidate options before meeting: - Issue 2-1-1: Applicable emission requirements for PC2/PC3 intra-band contiguous CA with single activated cell Agreement in main session: - ULCA emission requirements (SEM/ACLR/SE) based on the aggregated BW should apply, independent of the dualPA IE indication. - Issue 2-1-2: Applicable emission requirements for PC1.5 intra-band contiguous CA with single activated cell Agreement - Drop the discussion, as it was already discussed and agreed in last meeting that to further discuss handling of PC1.5 once the UL CA requirements are available - Intra-band non-contiguous CA - Issue 2-2-1: Applicable MPR and corresponding spectrum requirements for PC2/PC3 intra-band NC CA Agreement in main session: - No change of applicable MPR requirement in current spec but with clarification that applicable emission requirements are composite based regardless of dualPA IE indication. - FFS on whether update MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support depending on UE capability. - Note: Composite emissions means combination of individual sub-block emissions.

In this contribution, we would like to share our views regarding this topic.

Discussion

One remaining issue is whether update the MPR applicability for single CC fallback from non-contiguous ULCA w/o dualPA supporting. The proposal in [2] is to further improve the MPR granularity for small bandwidth by using the evaluation results in [3]:

reproduced from [2] MPC3 = 5.5; 0 ≤ B < 1.08 MPC2 = 6.5; 0 ≤ B < 1.08 3.5; 1.08 ≤ B < 3.24 5; 1.08 ≤ B < 3.24 2; 3.24 ≤ B < 4.5 3; 3.24 ≤ B < 4.5 1.5; 4.5 ≤ B < 9 2; 4.5 ≤ B < 11.52 0; 9 ≤ B 0; 11.52 ≤ B

Since in last meeting RAN4 has agreed that no change of applicable MPR requirement in current spec for FR1 intra-band non-contiguous ULCA in general, we prefer not to reflect the aforementioned 4dB gap for this specific case especially at the late stage of this release.

Proposal 1: No need to consider update MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support.

Another thing that needs to be determined is the granularity of the UE capability for Rel-19 FR1 intra-band ULCA. As the previous discussion related to the feasibility of MPR applicability would depend on whether the UE supports dualPA-architecture, which is a per BC capability, we think it is more appropriate to introduce the UE capability for Rel-19 FR1 intra-band UL CA as per BC as well, whether to consider separate capability for contiguous and non-contiguous case can be FFS.

Proposal 2: The granularity of Rel-19 UE capability on FR1 intra-band UL CA MPR improvement should be per BC. Whether separate capabilities for contiguous and non-contiguous case can be FFS.

Conclusion

In this contribution we discussed on the remaining issue for Rel-19 MPR applicability for FR1 intra-band UL CA. We have the following proposals:

Proposal 1: No need to consider update MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support.

Proposal 2: The granularity of Rel-19 UE capability on FR1 intra-band UL CA MPR improvement should be per BC. Whether separate capabilities for contiguous and non-contiguous case can be FFS.

- References

- R4-2420527, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #113, Orlando, US, November 18-22, 2024.

- R4-2419463, “NC-ULCA Single CC Fallback MPR”, Murata, 3GPP TSG-RAN4 Meeting #113, Orlando, US, November 18-22, 2024.

- R4-2204225, “1CC Fallback MPR for NC ULCA with 1LO Architecture”, Qualcomm, TSG-RAN4#102e, e-meeting, February 21st - March 3rd, 2022.