3GPP TSG-RAN WG4 Meeting #114	R4-2501843

Athens, Greece, February 17 – 21, 2025

Source:	Nokia

Title:	Simulation Results – NR BS Demodulation Performance Requirements for MMSE-IRC

Agenda item:	7.18.3

Document for:	Information

Introduction

The way forwards (WFs) from the discussions on Rel-19 NR demodulation performance Phase 5 BS Demodulation in previous RAN4#112, RAN4#112bis and RAN4#113 meetings are documented in [1], [2] and [3], respectively. The WFs captured the agreements concerning the required link-level simulation parameters and, in [3], the remaining open parameters with few options to choose from.

In this contribution, we provide our simulation results based on the agreements documented in the WFs. Our views on Rel-19 BS Demodulation MMSE-IRC can be found in [4] with some observations and proposals made based on the simulation results presented here.

Receiver Algorithms, Interference Modeling and Key Parameters

Receiver Algorithms

As described in [1], the agreements on the receiver algorithms for the MMSE-IRC and MMSE are as follows:

MMSE-IRC receiver assumption - Reuse the same receiver model for MMSE-IRC in TR36.884 [5] as below: ## MMSE-IRC receiver The MMSE-IRC receiver weight matrix is usually defined as where and denote the estimated channel matrix and covariance matrix, respectively. To obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. The covariance matrix is estimated at DM-RS REs by following equations: , , where the DM-RS symbols are used to estimate the covariance matrix, and Nsp is the number of sampling DM-RS REs. The estimation of interference covariance matrix is performed at per PRB and per TTI basis.. Baseline receiver assumption (for performance gain study only) - MMSE receiver as defined in TR36.884 [5]: ## Baseline receiver: MMSE receiver The MMSE receiver weight matrix is expressed as follow: , where and denote the estimated channel matrix and noise power, respectively. P1 is the transmission power of UE scheduled by the serving cell and is equal to ..

Interference Modeling

As stated in [2], the INR (Interference-to-Noise Ratio) is now used instead of the DIP (Dominant Interference Proportion) as in LTE. Nonetheless, the values of the INR themselves are derived from their corresponding DIP values from LTE in [5]. They are tabulated in [2] as below:

- Cases for link level evaluations based on the legacy LTE profile Scenario; (DIP1, DIP2) dB; Transferred (INR1, INR2) dB. HetNet with 1 interferer; (-0,43, N/A); (9.82, N/A). HetNet with 2 interferers; (-0,43, -13,78); (12.38, -0.97). HomNet with 1 interferer; (-1,11, N/A); (5.35, N/A). HomNet with 2 interferers; (-1,11, -10,91); (7.28, -2.52)..

Other Parameters

The following parameters have been agreed in [2] and [3]:

DMRS configuration [2] - Cover DMRS configuration type 1. FFS whether to include DMRS configuration type 2. - Use DMRS port #0 for both serving and interfering PUSCH - On DMRS sequence generation, use different DMRS sequence for each gNB: - For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively - FFS DMRS sequence generation configuration for DFT-s-DFDM PUSCH mapping type [2] - Agreement - Cover both PUSCH mapping Type A and Type B - PUSCH time domain allocation S=0, L=14 Test metric [2] - Agreement: - Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation Channel bandwidth [2] - Agreement: - For CP-OFDM: - For performance evaluation purpose, Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz. - Further discuss the CHBW for requirement definition Antenna configuration [2] - Agreement - Cover 2/4/8 Rx with 1Tx for simulation evaluation purpose Propagation condition [3] – For serving cell, consider the following options for performance evaluation and as candidates for further down-selection: o Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet o Option 2: Consider different channel for different MCS ü Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10 – For the interference cell, use TDLC300-100.

Simulation Results

In this section, the simulation results based on the agreed simulation parameters, cf.,  [1], [2], and [3], are presented.

We note here that the following parameters concerning the Interfering Cells’ (ICs) signals are fixed, namely,

- Modulation: 16QAM [2]

- Propagation condition: TDLC300-100 [3]

Furthermore, the following note is added in [3]:

- Note: Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 [6] for MCS 2, MCS 16 and MCS 20, respectively.

In [4], we have stated the reasoning of having the propagation condition according to the MCS, namely, MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10. Our simulation results are presented in Section 3.1 below.

Propagation Conditions according to the MCS

The option of propagation conditions based on MCS is as follows [3]:

- MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10.

The simulation results based on such propagation conditions are tabulated in Table 1 and Table 2 for (SCS, BW) of (15KHz, 5MHz) and (30KHz and 10MHz), respectively.

Table 1. SCS 15 KHz, BW 5 MHz

| Number of |  | Serving Cell |  | (INR1, INR2) dB | Mapping Type | SNR dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx | ICs | MCS | Propagation Condition |  |  | MMSE-IRC | MMSE | Gain |
| 2 | 1 | 2 | TDLB100-400 | (5.35, N/A) / [HomNet] | A | -0,6 | 0,7 | 1,3 |
|  |  |  |  |  | B | -0,4 | 0,8 | 1,2 |
|  |  |  |  | (9.82, N/A) / [HetNet] | A | 1,8 | 4,2 | 2,4 |
|  |  |  |  |  | B | 2,2 | 4,3 | 2,2 |
|  |  | 16 | TDLC300-100 | (5.35, N/A) / [HomNet] | A | 11,3 | 13,2 | 1,8 |
|  |  |  |  |  | B | 11,4 | 13,2 | 1,8 |
|  |  |  |  | (9.82, N/A) / [HetNet] | A | 13,0 | 16,6 | 3,7 |
|  |  |  |  |  | B | 13,1 | 16,6 | 3,5 |
|  | 2 | 2 | TDLB100-400 | (7.28, -2.52) / [HomNet] | A | 1,3 | 2,6 | 1,2 |
|  |  |  |  |  | B | 1,5 | 2,7 | 1,3 |
|  |  |  |  | (12.38, -0.97) / [HetNet] | A | 4,4 | 6,8 | 2,4 |
|  |  |  |  |  | B | 4,7 | 6,9 | 2,3 |
|  |  | 16 | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 13,2 | 15,1 | 1,9 |
|  |  |  |  |  | B | 13,2 | 15,1 | 1,9 |
|  |  |  |  | (12.38, -0.97) / [HetNet] | A | 15,4 | 19,2 | 3,8 |
|  |  |  |  |  | B | 15,5 | 19,2 | 3,7 |
| 4 | 2 | 16 | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 7,4 | 11,0 | 3,5 |
|  |  |  |  |  | B | 7,5 | 11,0 | 3,5 |
|  |  |  |  | (12.38, -0.97) / [HetNet] | A | 8,6 | 14,9 | 6,3 |
|  |  |  |  |  | B | 8,8 | 14,9 | 6,1 |
| 8 | 2 | 16 | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 3,2 | 7,8 | 4,6 |
|  |  |  |  |  | B | 3,2 | 7,8 | 4,6 |
|  |  |  |  | (12.38, -0.97) / [HetNet] | A | 4,1 | 11,7 | 7,6 |
|  |  |  |  |  | B | 4,1 | 11,6 | 7,5 |
|  |  | 20 | TDLA30-10 | (7.28, -2.52) / [HomNet] | A | 5,3 | 9,7 | 4,4 |
|  |  |  |  |  | B | 5,3 | 9,6 | 4,3 |
|  |  |  |  | (12.38, -0.97) / [HetNet] | A | 5,8 | 13,2 | 7,4 |
|  |  |  |  |  | B | 5,8 | 13,1 | 7,2 |

Table 2 SCS 30 KHz, BW 10 MHz

| Number of |  | Serving Cell |  | (INR1, INR2) dB | Mapping Type | SNR dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx | ICs | MCS | Propagation Condition |  |  | MMSE-IRC | MMSE | Gain |
| 2 | 1 | 2 | TDLB100-400 | (5.35, N/A) / [HomNet] | A | -1,5 | 0,2 | 1,7 |
|  |  |  |  |  | B | -1,4 | 0,1 | 1,6 |
|  |  |  |  | (9.82, N/A) / [HetNet] | A | 0,4 | 3,6 | 3,1 |
|  |  |  |  |  | B | 0,5 | 3,5 | 3,0 |
|  |  | 16 | TDLC300-100 | (5.35, N/A) / [HomNet] | A | 11,9 | 13,5 | 1,7 |
|  |  |  |  |  | B | 11,9 | 13,5 | 1,6 |
|  |  |  |  | (9.82, N/A) / [HetNet] | A | 13,7 | 16,9 | 3,2 |
|  |  |  |  |  | B | 13,8 | 16,9 | 3,2 |
|  | 2 | 2 | TDLB100-400 | (7.28, -2.52) / [HomNet] | A | 0,4 | 2,1 | 1,7 |
|  |  |  |  |  | B | 0,4 | 2,1 | 1,7 |
|  |  |  |  | (12.38, -0.97) / [HetNet] | A | 2,9 | 6,1 | 3,2 |
|  |  |  |  |  | B | 3,0 | 6,1 | 3,1 |
|  |  | 16 | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 13,6 | 15,4 | 1,7 |
|  |  |  |  |  | B | 13,7 | 15,4 | 1,7 |
|  |  |  |  | (12.38, -0.97) / [HetNet] | A | 16,1 | 19,5 | 3,3 |
|  |  |  |  |  | B | 16,2 | 19,5 | 3,3 |
| 4 | 2 | 16 | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 8,2 | 11,4 | 3,2 |
|  |  |  |  |  | B | 8,2 | 11,3 | 3,2 |
|  |  |  |  | (12.38, -0.97) / [HetNet] | A | 9,7 | 15,3 | 5,7 |
|  |  |  |  |  | B | 9,7 | 15,3 | 5,6 |
| 8 | 2 | 16 | TDLC300-100 | (7.28, -2.52) / [HomNet] | A | 3,9 | 8,3 | 4,4 |
|  |  |  |  |  | B | 3,9 | 8,2 | 4,3 |
|  |  |  |  | (12.38, -0.97) / [HetNet] | A | 4,9 | 12,3 | 7,4 |
|  |  |  |  |  | B | 4,9 | 12,2 | 7,2 |
|  |  | 20 | TDLA30-10 | (7.28, -2.52) / [HomNet] | A | 5,5 | 9,9 | 4,3 |
|  |  |  |  |  | B | 5,5 | 9,8 | 4,3 |
|  |  |  |  | (12.38, -0.97) / [HetNet] | A | 6,2 | 13,4 | 7,2 |
|  |  |  |  |  | B | 6,2 | 13,4 | 7,2 |

Summary

In this contribution, we provide our simulation results on Rel-19 NR Demodulation Performance Phase 5: BS demodulation performance for MMSE-IRC. More on our observations and the corresponding proposals can be found in [4].

- References

- R4-2413570, WF on NR demodulation performance: Phase 5, 3GPP RAN4#112, China Telcom, Maastricht, Netherlands, August 19-23, 2024

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#112bis, China Telcom, Hefei, China, October 14-18, 2024

- R4-2419780, Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#113, China Telcom, MediaTek, Orlando, USA, November 18-22, 2024

- R4-2501842, NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC, 3GPP RAN4#114, Nokia, Athens, Greece, February 17-21, 2025

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, v13.1.0, 09-2016

- 3GPP TS 38.104, NR - Base Station (BS) radio transmission and reception, V18.6.0, 06-2024