3GPP TSG-RAN WG4 Meeting #114	R4-2501849

# Athens, GR, 17th – 21st Feburary, 2025

Agenda Item:	7.30.2

Source:	MediaTek Inc.

Title:	Discussion on RF requirement impact for IoT NTN phase 3

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the WID of “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” was approved in [1]. One of the main objectives is to enable multiplexing of multiple UEs via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH and to update RF requirements accordingly if needed.

In this contribution, our views on the RF requirements impact are provided for discussion.

- Discussion

2.1		RF requirement impact for IoT-NTN phase 3

Some of the main Rel-19 WID [1] objectives for IoT NTN are shown below for reference.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account

In RAN4#113 meeting, the agreed WF [2] provides the following works for considering RF requirement impact. The contents are shown as follows.

RF core requirements NOTE1: Enhancements to enable multiplexing of multiple UEs in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH. NOTE2: For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs. Issue 1-4: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2 - Agreement: - Pending on RAN1 progress of slot-level OCC2 resource mapping. Issue 1-5: Whether there would be UE RF requirement impact when OCC schemes would cross gap in UL transmission. - Agreement: - Whether to study the case that gap between OCC slots may affect RF requirement pending on RAN1. Issue 1-6: Whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs. - Agreement: - Further discuss whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

Regarding Tdocs in RAN1 meetings, the agreed proposals [3-6] and useful evaluation [7] are provided below for reference.

RAN1#116bis “Session notes for 9.11.4 Non-Terrestrial Networks (NTN) for NR Phase 3” [3] Agreement For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs.

RAN1#118 R1-2407297 “FL Summary #2 for IoT-NTN” [4] contents below for reference: Agreement The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 2: OCC2, Symbol-level, CDM DMRS with new pattern - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 6: OCC4, Symbol-level, CDM DMRS with new pattern The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 5: OCC4, Symbol-level, TDM DMRS - Option 7: OCC4, Slot -level, TDM DMRS - Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS. Note 2: Companies to report DMRS sequence applied. Agreement For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

RAN1#118bis R1-2409192 “FL Summary #1 for IoT-NTN” [5] contents below for reference: Agreement At least the following schemes are supported for single-tone: - For 3.75kHz SCS OCC for NPUSCH format 1: - OCC length 2, Symbol-level - FFS: DMRS pattern(s) - For 15kHz SCS OCC for NPUSCH format 1: - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g. with or without spreading RAN1#119 R1-2410765 “Final FL Summary for IoT-NTN” [6] contents below for reference: The impact of UL gaps needs to be resolved. Maybe we could agree on which UL gaps are problematic for OCC. We could also discuss whether OCC codewords are dropped or postponed around UL gaps.

Proposal 1: For slot-level OCC2 of 15 kHz single-tone NPUSCH format 1, consider no UE RF requirement impact.

| OCC schemes | Max CFO ±0.1 ppm Max ΔΘ [deg] ST 15 kHz |
| --- | --- |
| Slot-level OCC 2 | 36 |

RAN1#119 R1-2410555 “Further discussion on uplink capacity enhancements for NR over NTN” [8] Carrier frequency offset discussion According to the discussion in previous meetings, some companies have raised concerns about PUSCH with OCC performance impairment due to unpredictable CFO fluctuation. In Figure 8, the instantaneous measurement result of CFO for PUSCH transmissions of four UEs with RNTI of 44018, 55631, 56359 and 60034 are shown during the period of about 25 seconds. The variance of CFO fluctuation is about 50Hz except some spikes that is caused by measurement error and are easy to filter out. Figure 8 Estimates of CFO at gNB side from a live terrestrial network. Observation 7: From CFO measurement results of UEs in live network, the variance of CFO fluctuation is within tolerable range for performing UE grouping and OCC across slots. To assess the impacts of some UEs potentially not having the same CFO, we have assumed a simulation scenario with four UEs, where two UEs are offset in terms of CFO (one at +50 Hz, and one at -50 Hz) in alignment with the measurements in Figure 8. The simulation results for this setup are shown in Figure 9, where it is seen that the SINR loss is less than 0.5 dB. Figure 9 Plot of BLER performance for a scenario where UEs are observing offsets to the CFO of +50 Hz and -50 Hz respectively. Observation 8: Four UE employing OCC across 4 repetitions, the average BLER degradation is very marginal, even for worst UE, the degradation is about 0.5dB at target BLER of 0.02. Additionally, and considering that CFO seems to not excessively fluctuate over time, mechanisms for allowing for FO control at the gNB could be studied in RAN1, so to enable the gNB to further align the FO properties of the multiplexed UEs supporting the OCC feature. It is to be noted that mechanisms for FO post-compensation at gNB are not applicable for the OCC case, since each RE would contain information symbols of different UEs affected by different FO values. Proposal 14: RAN1 to discuss mechanisms for UE frequency offset control by the gNB. Based on the above analysis, the CFO impairments to OCC length 4 can be controlled by either UE grouping or CFO dynamic compensation, and therefore inter-slot OCC with length 4 should be supported.

Observation 1: In R1-2410555[8], from CFO measurement results of UEs in live network, the variance of CFO fluctuation is within tolerable range for performing UE grouping and OCC across slots.

Proposal 2: Discuss whether UEs would not need to re-synchronize the TX frequency periodically to avoid frequency inaccuracy between OCC UEs because SAN/BS could perform UE CFO grouping.

3 Conclusion

In this paper, our views on RF requirements impact for NPUSCH with OCC feature are provided as follows:

Proposal 1: For slot-level OCC2 of 15 kHz single-tone NPUSCH format 1, consider no UE RF requirement impact.

| OCC schemes | Max CFO ±0.1 ppm Max ΔΘ [deg] ST 15 kHz |
| --- | --- |
| Slot-level OCC 2 | 36 |

Observation 1: In R1-2410555[8], from CFO measurement results of UEs in live network, the variance of CFO fluctuation is within tolerable range for performing UE grouping and OCC across slots.

Proposal 2: Discuss whether UEs would not need to re-synchronize the TX frequency periodically to avoid frequency inaccuracy between OCC UEs because SAN/BS could perform UE CFO grouping.

5 Reference

- RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc.

- R4-2419899, “Way Forward for [113][315] IoT_NTN_Ph3”, MediaTek Inc.

- Session notes for 9.11.4 Non-Terrestrial Networks (NTN) for NR Phase 3, Agenda Item 9.11.4, Changsha, Hunan Province, China, April 15th – 19th, 2024.

- R1-2407297, “FL Summary #2 for IoT-NTN”, Sony

- R1-2409192, “FL Summary #1 for IoT-NTN”, Sony

- R1-2410765 “Final FL Summary for IoT-NTN”, Sony

- R1-2408719, “IoT-NTN uplink capacity and throughput enhancement”, MediaTek Inc.

- R1-2410555 “Further discussion on uplink capacity enhancements for NR over NTN”, Nokia