3GPP TSG-RAN WG4 Meeting #114 	R4-2501851

Athens, Greece, Feb 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	Expected EIRP conformance declarations

Agenda Item:	7.13.4.1

Document for:	Approval

1 Introduction

The Expected EIRP core requirement has been agreed and some draft CR’s [1,2] have been agreed for the conformance work. The declarations required to identify the worst case for the conformance test and the methodology to align the BS co-ordinate system and the horizon need some clarifications.

In this paper we identify a declaration methodology which will resolve these issues.

1 Discussion

1.4	Declarations

So far in [2] we have agreed:

- to add a symbol reference to each of the test directions defined in D.10 (OTA peak directions set maximum steering direction(s)).

- An additional declaration for mechanical tilt (D64)

- An additional note (note 21) indicating that D10 and D64 may need a transformation – an editors note is also still present questioning how this can be done.

There a number of issues which require clarification.

    - Beam identifiers

The declaration for the both the OTA peak directions set (D9) and the OTA peak directions set maximum steering directions (D10) which describe the “steering range” are made for every beam in D3 (Beam identifier).

The original intention for this being that the BS may have the capability to generate different type of beam of different beam width (high gain user beams and lower gain high coverage cell beams being the usual ones considered). These beams may have different steering ranges (or OTA peak direction sets). Whilst the vendor is free to declare as many beam identifiers as they wish (allowing flexibility for the vendor, different beams could potentially have different output power for example – which in retrospect is something that could be useful for EEIRP) there is a minimum set of beams which it is mandatory to declare

1)	A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used.

2)	A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used.

3)	A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used.

4)	A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used.

5)	A beam which provides the highest intended EIRP of all possible beams.

It was envisaged that an AAS may have limited resources which might be distributed between azimuth and elevation and as such it may not be possible to achieve the narrowest (or widest) beam in azimuth and elevation at the same time. It seems that in most cases azimuth and elevation performance are not dependent in each other and as 1,2 and 5 would in most cases be the same beam, as would 3 and 4. This requirement then is effectively 2 beams the narrowest and the widest.

For EEIRP the requirement is based around user beams distributed over the steering range, it is possible that for EEIRP part of the band steering restrictions and/or output power restrictions may be applied. As such these will be different beams and will require different beam identifiers. This can be achieved by either adding additional EEIRP specific cases to D3 or an EEIRP specific beam identifier declaration.

Proposal 1: EEIRP specific beam identifier need to be added to the declarations list.

In [3] we proposed the worst case conditions which the EEIRP should be tested as follows:

The minimum mechanical down tilt and the maximum electrical steering range shall be declared.

If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt

This is not yet agreed, but will be used as an example of the required declarations.

The new beam identifiers have been added as declaration 3a so they can be placed next to the existing beam identifier declaration. This makes it easier to understand than if it were to be added at the bottom of the list. An alternative would be to add more EEIRP specific beams to existing D3 but this might over complicate how the declaration is referred to.

Proposal 2: Add EEIRP specific beam identifier declaration as follows:

| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 3) |
| --- | --- | --- |
| D3a | EEIRP beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. For BS required to meet EEIRP requirements in sub-clause 6.9. the following beams shall also be declared 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ intended to be operated with the minimum mechanical tilt (D64) If the maximum steering range and the minimum mechanical tilt (D64) are not simultaneously supported, the manufacturer shall declare and test the following two instances: 2a) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ intended to be operated with the minimum mechanical tilt (D64) and the reduced electrical steering range 2b) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ intended to be operated with the maximum electrical steering range and increased mechanical tilt (D64) (Note 22) |

With the new note

NOTE 1: Manufacturer declarations applicable per BS requirement set were marked as "x". Manufacturer declarations not applicable per BS requirement set were marked as "n/a". NOTE 2: For BS type 1-H, the only radiated declarations are related to EIRP and EIS requirements. For BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3]. For declarations marked as 'c', related conducted declarations in TS 38.141-1 [3] apply. When separately declared, they shall still use the same declaration identifier. NOTE 3: Depending on the capability of the system some of these beams may be the same. For those same beams, testing is not repeated. NOTE 4: These operating bands are related to their respective single-band RIBs. NOTE 5: As each identified OSDD has a declared minimum EIS value (D.27), multiple operating band can only be declared if they have the same minimum EIS declaration. NOTE 6: If the BS type 1-H or BS type 1-O is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. NOTE 7: Although EISREFSENS_50M level is based on a reference measurement channel with BWChannel = 50 MHz, it does not imply that BS has to support 50 MHz channel bandwidth. NOTE 8: Not applicable for BS type 2-O. NOTE 9: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. NOTE 10: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error, TAE or EVM. NOTE 11: The OTA coverage reference direction may be the same as the Reference beam direction pair (D.8) but does not have to be. NOTE 12: If a BS type 2-O is capable of 64QAM DL operation but not capable of 256QAM DL operation, then up to two rated output power declarations may be made. One declaration is applicable when configured for 64QAM transmissions and the other declaration is applicable when not configured for 64QAM transmissions. NOTE 13: If D.57 and D.58 are declared for certain frequency range (D.56), there shall be no "Rated beam EIRP" declaration (D.11) for the operating band containing that particular frequency range. NOTE 14: If a BS is capable of 1024QAM DL operation then up to three rated output power declarations may be made. One declaration is applicable when configured for 1024QAM transmissions, a different declaration is applicable when configured 256QAM transmissions and the other declaration is applicable when configured neither for 256QAM nor 1024QAM transmissions. NOTE 15: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. NOTE 16: If BS is declared to support Band n20 (D.4), the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 17: In case of BS type 1-H, this declaration applies per TAB connector. NOTE 18: If a BS type 2-O is capable of 256QAM DL operation, then up to three rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions, a different declaration is applicable when configured for 64QAM transmissions and the other declaration is applicable when not configured neither for 256QAM nor 64QAM transmissions. NOTE 19: If BS is declared to support Band n24 (D.4), the manufacturer shall declare if the BS may operate in geographical areas where FCC regulations apply. Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 20: If a BS is capable of 256QAM DL operation but not capable of 1024QAM DL operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions, and the other declaration is applicable when not configured for 256QAM transmissions NOTE 22: Applicable to band n104 only

The mechanical down tilt has been declared in the new declaration D64

| D.64 | The mechanical tilt | The declaration of mechanical tilt together with D.10 |
| --- | --- | --- |

The referenced declaration D10 does not describe the required mechanical tilt values, these are described in the beam identifier, this should be modified to clarify this point.

Proposal 3: Modify the mechanical tilt declaration as follows:

| D.64 | The mechanical tilt | The declaration of mechanical tilt. Declared for every beam in (D3a) |
| --- | --- | --- |

Finally there are many places where declarations are made for every beam identifier, D10 is an example of this. Some of these will be required for the new beam identifiers in D3a and some may not.

Looking at each in turn:

| D.4 | Operating bands and frequency ranges | List of NR operating band(s) supported by the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Supported bands declared for every beam (D.3). (Note 4) | Declaration D3a is specifically for band supporting EEIRP, currently this is only 1 band (or part of) so this is perhaps not strictly necessary. But for future proofing there is no harm in list bands for the beams in D3a Of course the bands in D3a will be a subset of the bands in D3 |
| --- | --- | --- | --- |
| D.7 | BS channel band width and SCS support | BS supported SCS and channel bandwidth per supported SCS. Declared for each beam (D.3) and each operating band (D.4). | The conformance test states testing for highest supported PSD and corresponding BW. If there are limitations for bands in D3a then it may be useful to distinguish |
| D.8 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3) and (D.3a). | Needed for the test directions definitions |
| D.9 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3) and (D.3a). | Needed for the test directions definitions |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax:The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3) and (D.3a). | Needed for the test directions definitions |
| D.11 | Rated beam EIRP | The rated EIRP level per carrier (Prated,c,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared for every beam (D.3) and (D.3a). (Note 12, 14, 18, 20) | As EEIRP compliant conditions may need to have reduced power then power needs to be declared separately for these cases |
| D.12 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3) and (D.3a). | Needed for the test directions definitions |
| D.15 | Number of carriers at maximum TRP | The number of carriers per operating band the BS is capable of generating at maximum TRP declared for every beam (D.3). | As we only are concerned with single carrier this is not needed for EEIRP testing |
| D.20 | CA-only operation | Declared of CA-only (with equal power spectral density among carriers) but not multiple carriers operation, declared per operating band (D.4) and per beam (D.3). | As we only are concerned with single carrier this is not needed for EEIRP testing |

Proposal 4: Modify declaration D8, D9, D10, D11, 12 to include EEIRP case beam identifiers D.3a

    - Aligning coordinate systems

Currently the coordinate system is defined with respect to the BS enclosure we have the symbol definitions:

The angle in the reference coordinate system between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the y/z plane. The angle is aligned with the down-tilt angle

The angle in the reference coordinate system between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°

Note. The theta definition here (38.141-2) is incorrect, the zero direction is wrong, the version in 37145-2 is correct

The angle in the reference coordinate system between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the direction perpendicular to the y/z plane. The angle is aligned with the down-tilt angle.

This should be corrected irrespective of this work, we have submitted a CR in the BS maintenance agenda.

These do not mention the reference so can be somewhat generic.

We have the declarations which allow the orientation of the coordinated reference with the BS enclosure:

| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the BS enclosure. |
| --- | --- | --- |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the BS enclosure. |

And we have an entire sub-clause describing the coordinate system:

4.14	Reference coordinate system

Radiated requirements are stated in terms of electromagnetic characteristics (e.g. EIRP and EIS) at certain angles with respect to the base station. To be able to declare radiated characteristics part of radiated requirements a reference coordinate system is required. The reference coordinate system is should be associated to an identifiable physical feature on the base station enclosure. The location of the origin and the orientation of the reference coordinate system are for the base station manufacturer to declare.

The reference coordinate system is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () as showed in figure 4.14-1.

Figure 4.14-1: Reference coordinate system

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°, inclusive. Note that  is defined as positive along the down-tilt angle.

There are a few points to note:

- The horizon is not currently considered

- The existing definitions refer to down-tilt angle – this is not explicitly defined but is the same as (or is related to) mechanical tilt we have described in the new declaration although down-tilt could also refer to a pre-tilt ? This may need further clarification.

- The theta angle is current orientated along with the convention of a positive down-tilt i.e. below the horizon is positive this is different to the EEIRP definition where above the horizon is positive.

The last point is unfortunate and has a risk of causing confusion, as such we must be careful when describing angles.

For the purpose of clarity in this document alone consider the 2 coordinate systems elevation plane with different variable names. Theta is used for the BS coordinate system and is defined with respect to the BS enclosure, and A is used for the coordinate system with respect to the horizon.

The 2 systems are related to each other by the mechanical tilt (MT)

So

A=-(MT+θ)

In the core we have already agreed to use the term θH (theta wrt horizon) instead of A which makes things a little more confusing but as long as we are consistent should be manageable. Hence:

θH=-θ+MT

As we already have a sub-clause called “reference coordinate system” this seems like the obvious location to document how the 2 coordinate systems are aligned rather than some distributed notes. This is the obvious location to look for information of coordinate systems. Notes can be used to point to this clarification sub-clauses if necessary.

Observation 1: We should agree if down-tilt refers to mechanical tilt (if so merge the terms) or mechanical tilt and electrical pre-tilt (if so we should clarify this).

Proposal 5: Use sub-clause 4.14 to document the alignment of the coordinate systems

Ideally sub-clause 4.14 could be separated into 2 sub-clauses a general section consisting of existing text and a new section explaining how to align with the horizon. It might not be allowed to do this in the design rules however ? if not the new text can be added in the same sub-clause. In the example below its split into 2 sub-clauses.

4.14	Reference coordinate system

4.14.1	General

Radiated requirements are stated in terms of electromagnetic characteristics (e.g. EIRP and EIS) at certain angles with respect to the base station. To be able to declare radiated characteristics part of radiated requirements a reference coordinate system is required. The reference coordinate system is should be associated to an identifiable physical feature on the base station enclosure. The location of the origin and the orientation of the reference coordinate system are for the base station manufacturer to declare.

The reference coordinate system is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () as showed in figure 4.14-1-1.

Figure 4.14-1-1: Reference coordinate system

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°, inclusive. Note that  is defined as positive along the down-tilt angle.

4.14.2	Coordinates with respect to horizon

OTA spatial emissions are defined with respect to the horizon. The general BS coordinate system is defined with respect to the BS enclosure. BS orientation and the horizon are related by the mechanical tilt as follows:

θH=-θ+MT

Where

θH – angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

 is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°

MT – angle representing the mechanical tilt defined between -90° and +90° . Note MT is positive towards the ground

4	Summary

In this paper we look at the require declarations for providing the information needed to test the EEIRP requirement. The following proposals are made and a draft CR is provided in the annex.

Proposal 1: EEIRP specific beam identifier need to be added to the declarations list.

Proposal 2: Add EEIRP specific beam identifier declaration as follows:

| D3a | EEIRP beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. For BS required to meet EEIRP requirements in sub-clause 6.9. the following beams shall also be declared 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ intended to be operated with the minimum mechanical tilt (D64) If the maximum steering range and the minimum mechanical tilt (D64) are not simultaneously supported, the manufacturer shall declare and test the following two instances: 2a) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ intended to be operated with the minimum mechanical tilt (D64) and the reduced electrical steering range 2b) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ intended to be operated with the maximum electrical steering range and increased mechanical tilt (D64) (Note 22) |
| --- | --- | --- |

And add note 22

NOTE 22: Applicable to band n104 only

Proposal 3: Modify the mechanical tilt declaration as follows:

| D.64 | The mechanical tilt | The declaration of mechanical tilt. Declared for every beam in (D3a) |
| --- | --- | --- |

Proposal 4: Modify declaration D8, D9, D10, D11, 12 to include EEIRP case beam identifiers D.3a

Observation 1: We should agree if down-tilt refers to mechanical tilt (if so merge the terms) or mechanical tilt and electrical pre-tilt (if so we should clarify this)

Proposal 5: Use sub-clause 4.14 to document the alignment of the coordinate systems

References

[1]	R4-2419820	Draft CR to TS 38.141-2 TA spatial emissions	Huawei

[2] R4-2419817	Draft CR for TS 38.141-2: Declarations	CATT

[3] R4-2419238	Further discussion on Expected EIRP test	Huawei

4	Annex – draft CR to 38.141-2 v18.7.0 (on top of R4-2419817)

<Start of the change>

4.6	Manufacturer's declarations

The following BS manufacturer's declarations listed in table 4.6-1, when applicable to the BS under test, are required to be provided by the manufacturer for radiated requirements testing for BS type 1-H, BS type 1-O and BS type 2-O.

For the BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3], clause 4.6.

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 3) | x | x | x |
| D3a | EEIRP beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. For BS required to meet EEIRP requirements in sub-clause 6.9. the following beams shall also be declared 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ intended to be operated with the minimum mechanical tilt (D64) If the maximum steering range and the minimum mechanical tilt (D64) are not simultaneously supported, the manufacturer shall declare and test the following two instances: 2a) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ intended to be operated with the minimum mechanical tilt (D64) and the reduced electrical steering range 2b) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ intended to be operated with the maximum electrical steering range and increased mechanical tilt (D64) (Note 21) | x | x | n/a |
| D.4 | Operating bands and frequency ranges | List of NR operating band(s) supported by the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Supported bands declared for every beam (D.3) and (D.3a). (Note 4) | c | x | x |
| D.5 | BS requirements set | Declaration of one of the NR base station requirement's set as defined for BS type 1-H, BS type 1-O, or BS type 2-O. | c | x | x |
| D.6 | BS class | Declared as Wide Area BS, Medium Range BS, or Local Area BS. | c | x | x |
| D.7 | BS channel band width and SCS support | BS supported SCS and channel bandwidth per supported SCS. Declared for each beam (D.3) and each operating band (D.4). | c | x | x |
| D.8 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). | x | x | x |
| D.9 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3) and (D.3a). | x | x | x |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax:The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3) and (D.3a). | x | x | x |
| D.11 | Rated beam EIRP | The rated EIRP level per carrier (Prated,c,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared for every beam (D.3) and (D.3a). (Note 12, 14, 18, 20) | x | x | x |
| D.12 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3) and (D.3a). | x | x | x |
| D.13 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D.4 – D.12) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. | x | x | x |
| D.14 | Parallel beams | List of beams which have been declared equivalent (D.13) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. | x | x | x |
| D.15 | Number of carriers at maximum TRP | The number of carriers per operating band the BS is capable of generating at maximum TRP declared for every beam (D.3). | n/a | x | x |
| D.16 | Operating bands with multi-band dependencies | List of operating bands which are generated using transceiver units supporting operation in multiple operating bands through common active RF components. Declared for each operating band for which multi-band transceiver is used. | c | x | n/a |
| D.17 | Maximum radiated Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band, declared for each supported operating band (D.4). (Note 15) | c | x | x |
| D.18 | Maximum Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D.16). | c | x | n/a |
| D.19 | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (D.52). | c | x | x |
| D.20 | CA-only operation | Declared of CA-only (with equal power spectral density among carriers) but not multiple carriers operation, declared per operating band (D.4) and per beam (D.3). | c | x | x |
| D.21 | Maximum number of supported carriers per operating band in multi-band operations | Maximum number of supported carriers per supported operating band declared to have multi-band dependencies (D.16). | c | x | n/a |
| D.22 | Contiguous or non-contiguous spectrum operation support | Ability of BS to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier in an operating band. | c | x | x |
| D.23 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a |
| D.24 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.23). (Note 5) | x | x | n/a |
| D.25 | OTA sensitivity supported BS channel bandwidth and SCS | The BS supported SCS and channel bandwidth per supported SCS by each OSDD. | x | x | n/a |
| D.26 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD. | x | x | n/a |
| D.27 | Minimum EIS for FR1 (EISminSENS) | The minimum EISminSENS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared per NR supported channel BW for the OSDD (D.30). The lowest EIS value for all the declared OSDD's is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. (Note 6) | x | x | n/a |
| D.28 | EIS REFSENS for FR2 (EISREFSENS_50M) | The EISREFSENS_50M level applicable in the OTA REFSENS RoAoA, (used as a basis for the derivation of the FR2 EISREFSENS for other channel bandwidths supported by BS). (Note 7) | n/a | n/a | x |
| D.29 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D.31) for each OSDD. | x | x | n/a |
| D.30 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD. (Note 8) | x | x | n/a |
| D.31 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D.30). (Note 9) | x | x | n/a |
| D.32 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D.33). | x | x | n/a |
| D.33 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. | x | x | n/a |
| D.34 | OTA coverage range | Declared as a single range of directions within which selected TX OTA requirements are intended to be met. (Note 10) | x | x | x |
| D.35 | OTA coverage range reference direction | The direction describing the reference direction of the OTA converge range (D.34). (Note 11) | x | x | x |
| D.36 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. | x | x | x |
| D.37 | The rated carrier OTA BS power, Prated,c,TRP | Prated,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band. (Note 12, 14, 18, 20) | n/a | x | x |
| D.38 | Rated transmitter TRP, Prated,t,TRP | Rated total radiated output power. Declared per supported operating band. (Note 12,14, 18, 20) | n/a | x | x |
| D.39 | CLTA placement for co-location test | The manufacturer shall declare the side of EUT where radiating elements are placed closest to the edge of EUT when applicable. The CLTA shall be placed at the EUT side where radiating elements are placed closest. | n/a | x | n/a |
| D.40 | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | c | x | x |
| D.41 | Additional operating band unwanted emissions | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.7.4 apply. (Note 16, Note 19) | c | x | x |
| D.42 | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another operating band are deployed. | c | x | x |
| D.43 | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another operating band. | c | x | n/a |
| D.44 | Single-band RIB or multi-band RIB | List of single-band RIB and/or multi-band RIB for the supported operating bands (D.4). | c | x | n/a |
| D.45 | Single or multiple carrier | BS capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band, per RIB. (Note 17) | c | x | x |
| D.46 | Maximum number of supported carriers per operating band | Maximum number of supported carriers. Declared per supported operating band, per RIB. (Note 15) | c | x | x |
| D.47 | Total maximum number of supported carriers | Maximum number of supported carriers for all supported operating bands. Declared per RIB. | c | x | x |
| D.48 | Other band combination multi-band restrictions | Declare any other limitation under simultaneous operation in the declared band combinations (D.16), which have any impact on the test configuration generation. | c | x | n/a |
| D.49 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a BS in a particular operating band. Declared per operating band (D.4). | c | n/a | n/a |
| D.50 | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band. Declared per operating band (D.4). | c | x | x |
| D.51 | Maximum supported power difference between carriers is different operating bands | Maximum supported power difference between any two carriers in any two different supported operating bands. Declared per operating bands combination (D.52). | c | x | n/a |
| D.52 | Operating band combination support | List of operating bands combinations supported by single-band RIB(s) and/or multi-band RIB(s) of the BS. | c | x | n/a |
| D.53 | OTA REFSENS RoAoA | Range of angles of arrival associated with the OTA REFSENS. | n/a | x | x |
| D.54 | OTA REFSENS receiver target reference direction | Reference direction inside the OTA REFSENS RoAoA (D.53). | n/a | x | x |
| D.55 | OTA REFSENS conformance test directions | The following four OTA REFSENS conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. | n/a | x | x |
| D.56 | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. | x | x | x |
| D.57 | Rated beam EIRP at lower end of the fractional bandwidth (Prated,c,FBWlow) | The rated EIRP level per carrier at lower frequency range of the fractional bandwidth (Prated,c,FBWlow), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared per beam for all supported frequency ranges (D.56). (Note 12, 13, 14, 15, 18, 20) | x | x | x |
| D.58 | Rated beam EIRP at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh) | The rated EIRP level per carrier at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared per beam for all supported frequency ranges in (D.56). (Note 12, 13, 14 ,15, 18, 20) | x | x | x |
| D.59 | Relation between supported maximum RF bandwidth, number of carriers and Rated maximum TRP | If the rated transmitter TRP and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated transmitter TRP; - The reduced total output power at the maximum number of supported carriers. | n/a | x | x |
| D.60 | Inter-band CA | Declaration of operating band(s) combinations supporting inter-band CA. Declared per operating band combination (D.52). | c | x | x |
| D.61 | Intra-band contiguous CA | Declaration of operating band(s) supporting intra-band contiguous CA. Declared per operating band with CA support. | c | x | x |
| D.62 | Intra-band non-contiguous CA | Declaration of operating band(s) supporting intra-band non-contiguous CA. Declared per operating band with CA support. | c | x | x |
| D.63 | Total maximum number of supported carriers in multi-band operation | Maximum number of supported carriers for all supported operating bands declared to have multi-band dependencies (D.16). | c | x | n/a |
| D.64 | The mechanical tilt | The declaration of mechanical tilt. Declared for every beam in (D3a). | x | x | n/a |
| D.100 | PUSCH mapping type | Declaration of the supported PUSCH mapping type for FR1 as specified in TS 38.211 [20], i.e., type A, type B or both. | c | x | n/a |
| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2, i.e., pos0, pos1, or both. | n/a | n/a | x |
| D.102 | PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [20], i.e., format 0, format 1, format 2, format 3, format 4. | c | x | x |
| <unchanged ommited> |  |  |  |  |  |
| D.127 | PUSCH with enhanced DM-RS type | Declaration of support enhanced DM-RS type as specified in TS 38.211 [17]. | x | x | n/a |
| D.128 | PRACH format for HAPS scenario | Declaration of supported PRACH format(s) for HAPS scenario, i.e., format 1. | x | x | n/a |
| NOTE 1: Manufacturer declarations applicable per BS requirement set were marked as "x". Manufacturer declarations not applicable per BS requirement set were marked as "n/a". NOTE 2: For BS type 1-H, the only radiated declarations are related to EIRP and EIS requirements. For BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3]. For declarations marked as 'c', related conducted declarations in TS 38.141-1 [3] apply. When separately declared, they shall still use the same declaration identifier. NOTE 3: Depending on the capability of the system some of these beams may be the same. For those same beams, testing is not repeated. NOTE 4: These operating bands are related to their respective single-band RIBs. NOTE 5: As each identified OSDD has a declared minimum EIS value (D.27), multiple operating band can only be declared if they have the same minimum EIS declaration. NOTE 6: If the BS type 1-H or BS type 1-O is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. NOTE 7: Although EISREFSENS_50M level is based on a reference measurement channel with BWChannel = 50 MHz, it does not imply that BS has to support 50 MHz channel bandwidth. NOTE 8: Not applicable for BS type 2-O. NOTE 9: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. NOTE 10: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error, TAE or EVM. NOTE 11: The OTA coverage reference direction may be the same as the Reference beam direction pair (D.8) but does not have to be. NOTE 12: If a BS type 2-O is capable of 64QAM DL operation but not capable of 256QAM DL operation, then up to two rated output power declarations may be made. One declaration is applicable when configured for 64QAM transmissions and the other declaration is applicable when not configured for 64QAM transmissions. NOTE 13: If D.57 and D.58 are declared for certain frequency range (D.56), there shall be no "Rated beam EIRP" declaration (D.11) for the operating band containing that particular frequency range. NOTE 14: If a BS is capable of 1024QAM DL operation then up to three rated output power declarations may be made. One declaration is applicable when configured for 1024QAM transmissions, a different declaration is applicable when configured 256QAM transmissions and the other declaration is applicable when configured neither for 256QAM nor 1024QAM transmissions. NOTE 15: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. NOTE 16: If BS is declared to support Band n20 (D.4), the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 17: In case of BS type 1-H, this declaration applies per TAB connector. NOTE 18: If a BS type 2-O is capable of 256QAM DL operation, then up to three rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions, a different declaration is applicable when configured for 64QAM transmissions and the other declaration is applicable when not configured neither for 256QAM nor 64QAM transmissions. NOTE 19: If BS is declared to support Band n24 (D.4), the manufacturer shall declare if the BS may operate in geographical areas where FCC regulations apply. Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 20: If a BS is capable of 256QAM DL operation but not capable of 1024QAM DL operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions, and the other declaration is applicable when not configured for 256QAM transmissions NOTE 21: The relationship between the spatial spurious emission requirements for D.3a, the OTA peak directions set maximum steering directions (D.10) and the mechanical tilt (D.64) is described in sub-clause 4.14.2. NOTE 22: Applicable to band n104 only |  |  |  |  |  |

<Next change>

4.14	Reference coordinate system

4.14.1	General

Radiated requirements are stated in terms of electromagnetic characteristics (e.g. EIRP and EIS) at certain angles with respect to the base station. To be able to declare radiated characteristics part of radiated requirements a reference coordinate system is required. The reference coordinate system is should be associated to an identifiable physical feature on the base station enclosure. The location of the origin and the orientation of the reference coordinate system are for the base station manufacturer to declare.

The reference coordinate system is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () as showed in figure 4.14.1-1.

Figure 4.14.1-1: Reference coordinate system

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°, inclusive. Note that  is defined as positive along the down-tilt angle.

4.14.2	Coordinates with respect to horizon

OTA spatial emissions are defined with respect to the horizon. The general BS coordinate system is defined with respect to the BS enclosure. BS orientation and the horizon are related by the mechanical tilt as follows:

θH=-θ+MT

Where

θH – angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

 is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°

MT – angle representing the mechanical tilt defined between -90° and +90° . Note MT is positive towards the ground

<End of changes>