3GPP TSG-RAN WG4 Meeting #114 	R4-2501855

Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	On modification of existing BS TX requirements for SBFD

Agenda Item:	7.23.2.2

Document for:	Approval

Introduction

During RAN4#113, BS RF requirements for SBFD have been discussed. Following conclusions for existing TX requirements [6], we would like to share our views regarding the open issues.

Discussion

Per WF in [6], the following agreement was reached.

Agreement:

- FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

From the discussion paper in last meeting [5], the following observations are made

Observation 1: For protection of SBFD UL, the limit -49 dBm/MHz for protection of BS UL can be reused.

Observation 2: available guard band for different operators’ spectrum cases are different, which depends on the spectrum allocation: number of operations in one band and bandwidth for each operator.

Observation 3: The ratio of transition guard/sub frequency band is higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

Observation 4: with larger frequency offset than ΔfOBUE, the required rejection for protection of SBFD UL is achievable for the analog filter.

Still based on RAN4 co-ex simulation methodology, we use the same simulation assumptions as in Rel-18 SBFD SI and focus on SBFD-SBFD deployment. The only difference is we traverse the ACIR in order to get a desired BS-BS ACIR level.

Following figure demonstrates how UL throughput loss @5%-tile, i.e. cell edge, could be improved with an increasing ACIR.

The grid shift is set to 10%. Around 38 dB improvement on ACIR (comparing to the existing 42.5dB ACIR determined by 45dB ACLR and 46dB ACS) can be enough to guarantee <3% UL throughput loss.

The grid shift is set to 20%. Around 30 dB improvement on ACIR can be enough to guarantee <3% UL throughput loss.

The grid shift is set to 50%. Around 17 dB improvement on ACIR can be enough to guarantee <3% UL throughput loss.

As discussed in [7], it is proposed to adopt 20% grid shift to be used for both TX and RX requirements definition for adjacent channel co-existence cases. Around 30 dB improvement are needed, which results a close protection level to -49 dBm/MHz.

Proposal 1: For protection of SBFD UL, it is proposed to define the limit -49 dBm/MHz as the additional emission mask requirements.

Proposal 2: For protection of SBFD UL, text proposal for the additional emission mask requirement is proposed as below

==== Text proposal for SBFD additional OBUE requirement for TS 38.104====

6.6.4.2.5.8	Additional emissions limits for SBFD-capable BS

In regions where SBFD-capable BSs are operating in adjacent channel. For protection of SBFD UL sub-band in adjacent channel, emissions shall not exceed the maximum levels specified in table 6.6.4.2.5.8-1. f is the separation between the SBFD DL edge frequency and the nominal -3 dB point of the measuring filter closest to the edge.

Table 6.6.4.2.5.8-1: additional OBUE requirement for protection of SBFD UL

| Declared frequency offset of measurement filter -3dB point, f | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| f | -49 dBm | 1MHz |

==== End of Text proposal ====

Conclusion

In this contribution we discussed on the BS TX requirement for SBFD. We have the following observations and proposals:

Observation 1: For protection of SBFD UL, the limit -49 dBm/MHz for protection of BS UL can be reused.

Observation 2: available guard band for different operators’ spectrum cases are different, which depends on the spectrum allocation: number of operations in one band and bandwidth for each operator.

Observation 3: The ratio of transition guard/sub frequency band is higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

Observation 4: with larger frequency offset than ΔfOBUE, the required rejection for protection of SBFD UL is achievable for the analog filter.

Proposal 1: For protection of SBFD UL, it is proposed to define the limit -49 dBm/MHz as the additional emission mask requirements.

Proposal 2: For protection of SBFD UL, text proposal for the additional emission mask requirement is proposed as below

==== Text proposal for SBFD additional OBUE requirement for TS 38.104====

6.6.4.2.5.8	Additional emissions limits for SBFD-capable BS

In regions where SBFD-capable BSs are operating in adjacent channel. For protection of SBFD UL sub-band in adjacent channel, emissions shall not exceed the maximum levels specified in table 6.6.4.2.5.8-1. f is the separation between the SBFD DL edge frequency and the nominal -3 dB point of the measuring filter closest to the edge.

Table 6.6.4.2.5.8-1: additional OBUE requirement for protection of SBFD UL

| Declared frequency offset of measurement filter -3dB point, f | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| f | -49 dBm | 1MHz |

==== End of Text proposal ====

- References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958	Way Forward for [111][313] NR_duplex_evo	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2419244, On modification of existing BS TX requirements for SBFD, Huawei, HiSilicon

- R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF, Huawei

- R4-2501856, On modification of existing RX RF requirements for SBFD