3GPP TSG-RAN WG4 Meeting#114                                R4-2501869

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.24.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement of Ambient IoT device

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

Discussion

- 2.1. RF requirement for Ambient IoT device

- In the following section, we would like to share some initial understandings on the RF requirements for A-IoT device.

- Table 1. The initial analysis for RF requirement of A-IoT device

| Tx part |  |
| --- | --- |
| Maximum output power/Output to input power gain within A-IoT carrier | For Device 1, from our understanding, it is highly dependent on maximum input power of CW signal and its backscattering gain according to its input CW signal e.g.-6dBi. |
| Output dynamics | Minimum output power This requirement might be still needed. e.g. with -36dBm as lowest input power and -6 backscattering gain for Device 1. ON-OFF power and time mask This might need more discussions since A-IoT device or cheap tag might don’t have capability to switch ON-OFF by itself at least for Device 1. |
| Transmit frequency error requirement | This could be further discussed based on the some practical measurement results for it. |
| EVM requirement | EVM for backscattering OOK signal is not needed since OOK signal is not mapped by the legacy constellation, instead it’s reflected in the envelope level as following. The power stability or power accuracy for OOK ON signal and OOK OFF signal is important and also power difference between OOK ON and OOK OFF is essential to ensure the tag OOK detection performance. In addition, as pointed out in [3], RFID uses ASK modulation with PIE data encoding. The ASK modulation mask was defined as shown in Figure 2.4-2 below. The pulsewidth (PW), Modulation Depth ((A–B)/A), RF Envelope Ripple (M), RF envelope Rise time (tr,10–90), and RF envelope fall time (tf,10–90) are the major requirements. Figure 2-4-2: Interrogator-to-Tag RF envelope For A-IoT device EVM requirement, we propose to define the EVM requirement similar as RFID with OOK signal considered. For BPSK based D2R design,the legacy EVM requirement need also to be revisited. |
| OBW | The legacy OBW requirement could be used as starting point. |
| ACLR | This depends on further coexistence study. Considering the regional RFID requirement at 900MHz, both ACLR and SEM requirement are specified. |
| SEM | This depends on the outcome of coexistence study and regulatory requirement as part of input. Considering the regional RFID requirement at 900MHz, both ACLR and SEM requirement are specified. |
| Transmitter spurious emission requirement | The legacy transmitter spurious emission requirement could be used as starting point. |
| Transmitter intermodulation requirements | No need to define the Tx intermodulation requirement. |
| Rx part |  |
| REFSENS | The REFSENS requirement might be not based on the throughput metric and it should be dependent on the miss detection ratio and false alarm detection ratio instead if without any HARQ-ACK feedback. e.g. -26dBm for RF energy harvesting, -36dBm for OOK detection. |
| Maximum input power | At least the following aspect needs to be considered: - Maximum input power for R2D signal reception e.g. with OOK signal detection with measurement metric as miss detection ration and false alarm detection ratio; |
| ACS and ACSC | First of all, this depends on further coexistence study. In addition, based on the illustrated diagrams for Device 1, the BB LPF is needed, then some ACS and ACSC requirement (which is similar as LP-WUS ASCS requirements) is also supposed to be essential to verify its RF performance. |
| Blocking requirement | During the SI phase, it was agreed not to define the blocking requirement potentially, however considering the regional RFID requirement at 900MHz, blocking requirement is still defined. At least from our understanding, the blocking requirement is still needed for Device 1. |
| OOBB | No need to define the Rx OOBB requirement. |
| RX IMD | No need to define the Rx IMD requirement. |
| Rx spurious emission | No need to define the spurious emission requirement. However this is defined for RFID device when device is not working (e.g. idle mode, empty loaded status) |
| Rx spurious response requirement | No need to define the spurious response requirement. |

Conclusions

In this contribution, we shared some further views on the RF requirement for Ambient IoT device and proposals are made as following:

- Proposal 1: please find the initial analysis for RF requirement of Ambient IoT device in Table 1.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Approved.

- R4-2414304, WF on Co-existence study for ambient IoT, Approved

- ISO/IEC 18046-3 Information technology – Radio frequency identification device performance test methods – Part 3: Test methods for tag performance