3GPP TSG-RAN WG4 Meeting#114                                R4-2501872

Athens, Greece, 17th – 18th Feb, 2025

Agenda item:	7.12.1.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for UL-MIMO for ATG UE in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. In last RAN4#110bis meeting, there were some initial discussions on ATG UE supporting UL MIMO, however there are lots of remaining issues. In this contribution, we would like to share further views on ATG UE supporting UL MIMO.

Issue 3-1-1: ULFPTx mode for UL-MIMO WF: Consider whether and how to introduce this requirement. - Option 1: ULFPTx mode 1/2/Full in the following table can be supported depending on UE PA implementation - Table 2: Example of ATG ULFPTx (R-MOP = x dBm) ATG ULFPTx mode; PA1(dBm); PA2(dBm). Mode2; x; (x-3)~(x-1). Mode-Full; x; x. Mode1; (x-3)~(x-1); (x-3)~(x-1). - Option 2: Current ULFPTx mode requirements for legacy UE in TS38.101-1 can be reused for ATG UE, and there is no other specification affect. - Option 3: No need to extend ULFPTx mode to ATG UE. Issue 3-1-2: whether preclude the single antenna port related requirement WF: - Option 1: Yes. - Option 2: No. Issue 3-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18 Agreement in RAN4#102 meeting: - If the legacy requirement for UL MIMO is defined at each antenna connector, - For ATG UE with antenna array, the requirement should be updated [at each TAB connector]. - If the legacy requirement for UL MIMO is defined per layer, FFS - Option 1: for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer. - Option 2: for ATG UE with capability antennaArrayType-r18, the requirement should be updated per layer without explicitly emphasize the TAB connector.. Agreement: - If the legacy requirement for UL MIMO is defined at each antenna connector, - For ATG UE with antenna array, the requirement should be updated at each TAB connector - If the legacy requirement for UL MIMO is defined per layer - for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer

Discussion

- 2.1. RF requirements for UL MIMO

During the previous RAN4 meetings, we have some initial discussion on ULFPTx mode, as shown in the following diagrams extracted from TR 38.837,

- Rel-16 ULFPTx mode 1 is targeted for single layer transmission with two antenna ports instead of two layers transmission.

- ULFPTx Mode 2 Mechanism-1 is targeted for SRS port virtualization and ULFPTx Mode 2 Mechanism-2 is targeted for single layer transmission with single antenna port.

- ULFPTx Mode 0 is targeted for single layer transmission with single antenna port.

From our understanding, ATG UE with UL MIMO is targeted for two layer transmission instead of single layer transmission to enhance the uplink capacity instead of uplink coverage improvement.

Fig. 5.3.3.1-1. Illustration of Rel-16 ULFPTx Mode 1

ULFPTx Mode 2 Mechanism-1 (SRS port virtualization):

Fig. 5.3.3.2-1. Illustration of Rel-16 ULFPTx Mode 2 (Mechanism-1, SRS port virtualization)

ULFPTx Mode 2 Mechanism-2 (TPMI indication):

Fig. 5.3.3.2-2. Illustration of Rel-16 ULFPTx Mode 2 (Mechanism-2, TPMI indication)

Fig. 5.3.3.3-1. Illustration of Rel-16 ULFPTx Mode 0

Proposal 1: ULFPTx Mode is not relevant to this feature.

From single antenna port related feature which is also called TxD in the specification or SCDD in the implementation, this is mainly designed for single port transmission with two physical transmission ports with some cyclic shifting between signals transmitted in the time domain to mitigate the fading response of the channel, however this is not useful for ATG UE operating almost in the AWGAN channel. In other words, the artificial channel response added by TxD or SCDD as shown in following figure might have negative impacts of ATG UE operation. In again, we don’t see its justification on the benefit or usefulness of this feature.

Figure 2.3.1. the frequency-domain spectrum of field data

Proposal 2: not to consider the single antenna port transmission TxD for ATG UE operation.

Conclusions

In this contribution, we shared some initial views on the RF requirements for ATG UE supporting CA and UL MIMO in Rel-19 and proposals are made as following:

Proposal 1: ULFPTx Mode is not relevant to this feature.

Proposal 2: not to consider the single antenna port transmission TxD for ATG UE operation.

References

- RP-240839, New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321919,Big CR for TS 38.101-1 for NR ATG, Agreed.

- R4-2403864, CR for 38101-1 to update ATG related signaling name, Agreed

- R4-2406594, WF on Rel-19 ATG UE requirements, Approved.

- R4-2410687,WF on Rel-19 ATG UE RF requirements, Approved.