3GPP TSG-RAN4 Meeting #114	R4-2501902

# Athens, GR, 17th – 21st February, 2025

Source:	LG Electronics

Title:	ATG UE RF requirements for UL-MIMO

Agenda item:	7.12.1.2

Document for:	Discussion

Introduction

In this paper, we provide our views on ATG UE RF requirements for UL-MIMO based on the approved WF[1].

Discussion

Based on the approved WF[1, 2, 3], we would like to discuss ULFPTx mode and single antenna port related requirement for ATG UL-MIMO.

ULFPTx mode for UL-MIMO

In the existing NR UL MIMO, ULFPTx mode is specified for PC2 and PC1.5 as follows.

|  | # of layer |  |
| --- | --- | --- |
|  | 2 Tx port | 4 Tx port |
| PC2 | 1 | 1 |
| PC1.5 | 1 | 1, 2 |

According to objective in WID, UL MIMO with 2TX for single CC is targeted so that both 1 layer and 2 layers can be supported for ATG UL MIMO.

In case of 1 layer, ULFPTx mode 1/2/Full can be also supported depending on UE PA implementation as seen in Table 2.1.

Table 2.1: Example of ATG ULFPTx (R-MOP = xdBm)

| ATG ULFPTx mode | PA1(dBm) | PA2(dBm) |
| --- | --- | --- |
| Mode2 | x | (x-3)~(x-1) |
| Mode-Full | x | x |
| Mode1 | (x-3)~(x-1) | (x-3)~(x-1) |

Proposal 1: For ATG UL-MIMO, consider ULFPTx mode and reuse the existing ULFPTx mode requirements.

Whether to preclude the single antenna port related requirement

The existing NR UL-MIMO requirements for single layer and single antenna-port (W=1) can be applied as follows.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.2J apply for at least one antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.2J apply for at least one TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the rated maximum output power applies for sum of two antenna connectors for the ATG UE not indicating the capability antennaArrayType-r18.

- the rated maximum output power applies for sum of two TAB connectors for the ATG UE indicating the capability antennaArrayType-r18.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity and the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16,

- the requirements in clause 6.2J apply for at least one antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.2J apply for at least one TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

For single layer  UL MIMO operation and two antenna-ports,

- the rated maximum output power shall be met to a UE that supports ULFPTx feature and is configured in its declared full power mode.

Proposal 2: For ATG UL-MIMO, consider the existing requirement related to single antenna port.

Conclusion

In this contribution, we provided our view on ULFPTx mode and single antenna port related requirement for ATG UL-MIMO.

Proposal 1: For ATG UL-MIMO, consider ULFPTx mode and reuse the existing ULFPTx mode requirements.

Proposal 2: For ATG UL-MIMO, consider the existing requirement related to single antenna port.

- References

[1] R4-2420338, “WF on NR ATG UE RF”, CMCC

[2] R4-2417121, “WF on UE RF for ATG enhancement”, CMCC

[3] R4-2414322, “WF on Rel-19 ATG UE RF requirements”, CMCC

[4] RP-241213, “Revised WID on Enhancements for Air-to-ground network for NR”, CMCC