3GPP TSG-RAN WG4 Meeting #114	R4-2501905

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.18.2

Source:	Ericsson

Title:	On MMSE-IRC receiver for interference mitigation with 8Rx

Document for:	Discussion

# 1	Introduction

Following the objectives of the WID, companies have reached several agreements below with respect to the test scope:

Test scope - RAN4 to introduce PDSCH demodulation requirements for both TDD and FDD - Not to cover PDSCH absolute physical layer throughput requirements for 8Rx - RAN4 to introduce CQI reporting requirements under ICI for 8Rx

To establish parameter assumptions for the test cases, it was encouraged to share initial evaluations during this meeting to assess the performance gain of the IRC receiver compared to the MMSE receiver. Following the last meeting’s discussion of down-selecting parameter assumption candidates, we shared our simulation results for PDSCH demodulation performance and CQI reporting based on the agreed evaluation cases in our companion simulation results paper. In this contribution, we present our perspectives along with observations obtained from the evaluations to facilitate better determination of the parameter assumptions.

# 2	Inter-cell interference

In this section, we discussed the scenario of inter-cell interference cancellation with 8Rx reception.

## 2.1	PDSCH requirements with inter-cell interference

As a result of last RAN4 #113 meeting’s discussion, following parameter assumptions are further agreed on top of the previous agreements:

Rank and MCS Agreement: - Use the following for evaluation. TBD on requirements: - Rank1: MCS17 - Rank2: MCS17 - Rank4: MCS13

From our simulation results, targeting rank 1 and rank 2 cases, we have observed ideal gain from IRC receiver over MMSE receiver, see [2]. Therefore, we propose to define PDSCH requirement with rank 1 + MCS17 and rank 2 + MCS17.

Proposal 1: Define PDSCH requirements with rank1 + MCS17 and rank2 +MCS17

As for the rank 4 case, we have observed the followings:

Observation 1:

- The SNR at 70% maximum throughput for IRC receiver is 9.21dB, which is not too low and not too high

- The performance gain is 5.26dB, which is good enough for differentiating the IRC receiver and MMSE receiver

In addition, we are of the opinion that rank 4 is a significant and typical scenario especially for 8Rx UEs comparing with rank 1 case. Based on this, we propose to define requirements for rank 1, rank 2 and rank 4. To consider reducing the test workload, if necessary, we prefer to give up on rank 1 case.

Proposal 2: Define PDSCH requirements with rank 4 + MCS13, in addition to rank 1 and rank 2 cases

## 2.2	CQI reporting

It has been agreed to include CQI reporting requirements for inter-cell interference scenario following legacy assumptions from Rel-17 that is to assume fading condition for the target cell and AWGN condition for the interference cell. It’s encouraged for companies to bring simulation results for both rank 1 and rank 2 (for the target cell) to evaluate the performance so as to facilitate the main open issue for the CQI reporting requirements: rank.

Figure 2.2-1 Throughput for interference model and white noise model

Figure 2.2-2 Gain comparison for rank 1&2

While comparing the performance with rank 1 and rank 2, it’s been observed that the gamma (ratio of the throughput of rank 2 obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source) is larger than that of rank 1 in most of the SINR points.

Observation 2: gamma value for rank 2 is larger than that of rank 1

Based on the above, we propose to consider rank 2 instead of rank 1 for the CQI reporting requirements.

Proposal 3: Consider rank 2 for CQI reporting requirement under ICI scenario

More CQI reporting results can be found in our companion paper [2].

# 3	Intra-cell inter-user interference

In this section, we discussed the scenario of intra-cell inter-user interference cancellation with 8Rx reception.

## 3.1	PDSCH requirements with intra-cell inter-user interference

As a result of last RAN4 #113 meeting’s discussion, following parameter assumptions are further agreed on top of the previous agreements:

Antenna configuration and MIMO correlation Agreement: - Rank 2+2 for 4T8R ULA Low - Rank 4+4 for 8T8R ULA Low Precoding method for the co-scheduled UE Agreement: - Rank 2+2: orthogonal - Rank 4+4 (if introduced): orthogonal DMRS Ports Agreement: - For 4+4, use different CDM group. Serving UE with ports {1000, 1001,1004, 1005} and co-scheduled UE with ports {1002, 1003, 1006, 1007} Other parameters Agreement: - For PDSCH demod requirements in intra-cell inter-user interference reuse the parameters from Rel-17 2RX/4RX for those not explicitly discussed/ captured.

Two left open issues are specified as follows:

Which MCS to be considered for the rank 2+2 case

According to our simulation results below, rank 2+2 with MCS17 has similar performance as rank 2+2 with MCS19. For the PDSCH requirements, we propose to consider MCS17 instead of MCS19.

Table 3.1-1 PDSCH simulation results for rank 2+2

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 1 | 2+2 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 9.23 | N/A | INF |
| 2 | 2+2 | 10 / 15 | MCS19 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 10.73 | N/A | INF |

Proposal 4: Consider MCS17 for PDSCH requirements with rank 2+2

Whether to consider the rank 4+4 case

We have evaluated the possibility of introducing requirements for rank 4+4 from simulation point of view.

Following are the expected simulation cases and results for rank 4+4 from our side.

Table 3.1-2 PDSCH simulation results (baseline)

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 3 | 4+4 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | 13.9 | N/A | INF |

Table 3.1-3 PDSCH simulation results (simplified)

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 3 | 4+4 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | N/A | N/A | N/A |

It can be observed that the baseline receiver works fine for canceling the interference even with higher rank, while the simplified receiver not.

Observation 3: For the case with rank 4+4 plus baseline receiver, the operating SNR for IRC receiver is suitable for requirements, and the gain over MMSE receiver is infinity so that it is quite easier to differentiate when testing

Observation 4: For the case with rank 4+4 plus simplified receiver, the maximum Tput can not be reached by both IRC and MMSE receivers

We confirm the value of evaluating the performance and having a test for rank 4+4 case even it is not as easier as rank 2+2 to be seen in the real network. However, we are open for not having such test/requirement given the fact that there will be no network signaling for IRC receiver with 8Rx reception, it is risky for the network to schedule rank 4+4 without the knowledge of the UE receiver assumption (baseline or simplified).

# 4	Applicability rule and release independent

Test applicability to skip 2/4Rx requirements

Test applicability to skip 2/4Rx requirements - Proposals - Option 1: Defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed. - Option 1a: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 1b: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

For UEs supporting both 8Rx and 2/4Rx, in Rel-18, we have already introduced test applicability rules to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed as shown below. We prefer to apply the same approach for the upcoming PDSCH requirements with 8Rx plus IRC.

Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list. UE supports 8Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3 (Note 2,3). UE supports both 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3 (Note 1). UE supports both 8Rx and 2Rx; PDSCH; All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2 (Note 2,4). PBCH; All tests in Clause 5.4.2.

Observation 5: Test applicability rule have been defined in Rel-18 to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed

Proposal 5: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Test applicability for FDD/TDD

Test applicability for FDD/TDD - Proposals - Option 1: For 8Rx inter-cell interference scenario, for demodulation performance tests, introduce similar applicability rules defined in section 5.1.1.10 for UE supporting both duplex mode TDD and FDD as legacy 2Rx/4Rx, i.e., if UE is supporting both TDD and FDD with 8Rx, UE will pass test case under HomNet scenario with FDD mode, and pass test case under HetNet scenario with TDD mode. - Option 2: Do not reduce UE effort to only comply to either 1 or 2 interference cells.

It’s acceptable for us to reuse the same test applicability rule as Rel-17 2/4Rx MMSE-IRC requirements.

#### 5.1.1.10 Applicability of requirements for PDSCH with inter cell interference Tests; Applicability notes. All tests in Clause 5.2.2.1.15, 5.2.3.1.15, 5.2.2.2.16 and 5.2.3.2.16; If UE supporting both duplex mode TDD and FDD with 2RX, only test 1-1 in clause 5.2.2.1.15 and test 1-2 in clause 5.2.2.2.16 will be applied. If UE supporting both duplex mode TDD and FDD with 4RX, only test 1-1 in clause 5.2.3.1.15 and test 1-2 in clause 5.2.3.2.16 will be applied..

Proposal 6: If necessary, reuse the same test applicability rule as 2/4Rx MMSE-IRC requirements, i.e., if UE supporting both TDD and FDD with 8Rx, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode.

Release independent

There are three options left for further discussion:

Release independent - Proposals - Option 1: Discuss release independence requirements at a later stage in the WI. - Option 1a: RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113. - Option 2: The new requirements for 8Rx should be release independent from Rel-18. - Option 2a: The requirement should be released independent from Rel-18 if no new signalling or UE capability introduced. - Option 3: The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

Since the 8Rx UE feature is introduced in Rel-18, we are of the opinion that the upcoming PDSCH and CQI requirements for both inter-cell interference and intra-cell interference scenarios should be release independent from Rel-18.

Proposal 7: The new requirements for 8Rx should be release independent from Rel-18.

# 5	Summary

In this contribution, we shared our views and results on left open issues for introducing PDSCH and CQI reporting requirement. Our proposals are summarized as follows:

Proposal 1: Define PDSCH requirements with rank1 + MCS17 and rank2 +MCS17

Observation 1:

- The SNR at 70% maximum throughput for IRC receiver is 9.21dB, which is not too low and not too high

- The performance gain is 5.26dB, which is good enough for differentiating the IRC receiver and MMSE receiver

Proposal 2: Define PDSCH requirements with rank 4 + MCS13, in addition to rank 1 and rank 2 cases

Observation 2: gamma value for rank 2 is larger than that of rank 1

Proposal 3: Consider rank 2 for CQI reporting requirement under ICI scenario

Proposal 4: Consider MCS17 for PDSCH requirements with rank 2+2 case

Observation 3: For the case with rank 4+4 plus baseline receiver, the operating SNR for IRC receiver is suitable for requirements, and the gain over MMSE receiver is infinity so that it is quite easier to differentiate when testing

Observation 4: For the case with rank 4+4 plus simplified receiver, the maximum Tput can not be reached by both IRC and MMSE receivers

Observation 5: Test applicability rule have been defined in Rel-18 to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed

Proposal 5: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 6: If necessary, reuse the same test applicability rule as 2/4Rx MMSE-IRC requirements, i.e., if UE supporting both TDD and FDD with 8Rx, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode.

Proposal 7: The new requirements for 8Rx should be release independent from Rel-18.

# References

[1]	RP-2419784: Way forward for [113][322] NR_demod_Ph5_Part2_UE, Nokia, RAN4 #113, Orlando, United States

[2]	R4-2501906: Simulation results for MMSE-IRC receiver for 8Rx reception, RAN4 #114, Athens, Greece