3GPP TSG-RAN WG4 Meeting #114	R4-2501934

Athens, Greece, 17-21 February, 2025

Source: 	Qualcomm Incorporated

Title: 	Study of SCM for demodulation performance requirements

Agenda item:	 7.16.2

Document for:	Agreement

- Introduction

In the previous RAN4 Meeting #113, we contributed to the SCM Study Item sharing considerations on the Study Item on the use spatial channel model for demodulation performance requirements.

According to the outcome of the discussion during the meeting and based on the approved Way Forward, we provide in this contribution further observations as part of the ongoing study.

# SCM Methodology Alignment

## Metrics for Comparison

With respect to the test cases to be simulated to study the Spatial Channel Modelling, RAN4 reached some tentative agreements in the previous meeting, which helped set up a baseline direction for the scenarios to focus on.

However, we have not concluded which metric will be considered for alignment or comparison in the context of FMCS PDSCH results, for both SU and MU MIMO (8RX/8L, 4RX/4L, 4RX/2L+2L). In the context of fixed modulation and coding schemes, it is standard practice for RAN4 to collect simulation results for the SNR that allows to achieve 70% throughput, and we believe that it is most reasonable to keep the same metric to evaluate and compare different channels.

Proposal 1: RAN4 to use SINR corresponding to 70% of the Maximum Throughput for FMCS PDSCH results for alignment and for comparison in the TS 38.753.

## Simulation Assumptions

In this section we address some parameters that we have identified as missing from the set in the last WF for the purposes of simulation alignment:

Proposal 2: RAN4 to use DMRS Port Index 7 to simulate 2+2 layers in MU-MIMO.

Proposal 3: RAN4 to consider only baseline receiver assumption for 8RX/8L simulation alignment for SCM. Do not consider 8RX simplified receiver.

Proposal 4: RAN4 to use 365ns as delay spread for the purpose of SCM channel alignment.

# CDL Based Methodology Alignment

## CDL Modelling: Number of Channel Taps

It has been discussed in the past that the maximum number of channel taps is 12 for channels used in RAN4, to facilitate test equipment implementation and reduce the variability in the results. To that end, we propose to define a set of CDL tables

Proposal 5: RAN4 to agree to truncate CDL channel delay profiles to maximum 12 delays, and scale the cluster powers accordingly to keep the original delay spread.

Proposal 6: RAN4 to agree to use the table included here below for the definition of TS 38.753 Option A3, based on CDL-C in 38.901 with taps truncation and scaling to DS=365ns.

## Scaling of angles

In the WF for RAN4#113 the following options are included for the topic of angle scaling in CDL models:

#### Angles of Departure and Angles of Arrival

Way Forward:

Option 1: Replace the mean angle   in the last position of departure and arrival angles generated equation by the desired angle as  .

Set mf,desired based on the LOS direction between the TX and RX

Option 2: Following 38.827 (i.e., mf,desired = mf,model).

We believe RAN4 should further discuss these options. In our understanding, the choice of option 2 has a very different interpretation with respect to option 1, and the proponents should illustrate

Proposal 7: RAN4 to better discuss angle scaling options and impact.

Proposal 8: For the purposes of simulation alignment and comparison in the scope of TS 38.753, RAN4 should not apply any angle scaling procedure. If simulation results are submitted with angle scaling, this should be disclosed along with the results for clarity.

## Simulation Results and CDL 78.753 (Option A2)

In our companion paper R4-25xyxyx, we have submitted a number of simulation results using the CDL-C tables as included in Table 7.7.1-3 in TR 38.901.

According to the CDL model above, we have used the following parameters:

- Delay Scaling: according to 38.901, using 365ns

- Angle Scaling: no angle scaling

- Tap reduction: no tap reduction

- Randomness reduction:

  - Fixed ray coupling according to Table 7.2-6 in TR 38.827

  - Fixed initial phases for 2x2 polarization matrices and initial phases according to Table 7.2-8 in TR 38.827

- SNR normalization and channel scaling: according to the procedure agreed with the WF and included for info in Annex 7 in this document

Observation 1: Simulation results for CDL 38.753 based on Table 7.7.1-3 in TR 38.901 (Option A2) are included in the companion contribution R4-2501998.

Proposal 9:Include the details for Option A2 above in the CDL section for TS 38.753.

# Discussion outside of the scope of the current SI

## Notes of the RAN4#113 Way Forward on SCM

One of the subtopic in the  WF in R4-2419782 welcomed contributions on the topic of new requirements introduction. According to the SID, there is no objective to define requirements in this SI, as such we propose the following:

Proposal 9: RAN4 to not discuss further requirements introduction in the scope of the current SI.

- Conclusions

Proposal 1: RAN4 to use SINR corresponding to 70% of the Maximum Throughput for FMCS PDSCH results for alignment and for comparison in the TS 38.753.

Proposal 2: RAN4 to use DMRS Port Index 7 to simulate 2+2 layers in MU-MIMO.

Proposal 3: RAN4 to consider only baseline receiver assumption for 8RX/8L simulation alignment for SCM. Do not consider 8RX simplified receiver.

Proposal 4: RAN4 to use 365ns as delay spread for the purpose of SCM channel alignment.

Proposal 5: RAN4 to agree to truncate CDL channel delay profiles to maximum 12 delays, and scale the cluster powers accordingly to keep the original delay spread.

Proposal 6: RAN4 to agree to use the table included here below for the definition of TS 38.753 Option A3, based on CDL-C in 38.901 with taps truncation and DS=300ns.

Proposal 7: RAN4 to better discuss angle scaling options and impact.

Proposal 8: For the purposes of simulation alignment and comparison in the scope of TS 38.753, RAN4 should not apply any angle scaling procedure. If simulation results are submitted with angle scaling, this should be disclosed along with the results for clarity.

Observation 1: Simulation results for CDL 38.753 based on Table 7.7.1-3 in TR 38.901 (Option A2) are included in the companion contribution R4-2501998.

Proposal 9: RAN4 to not discuss further requirements introduction in the scope of the current SI.

- References

[1] RP-241610, “Study on spatial channel model for demodulation performance requirements”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17th – 20th June, 2024;

[2] 3GPP TR 38.827 V16.8.0 (2022-09), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE); (Release 16)”

[3] R4-2419782, “Way Forward [113][320] NR_SCM”, 3GPP TSG-RAN WG4 Meeting #113, Orlando, Florida, USA, 18th – 22nd November, 2024

# ANNEX: CDL-C CDP Tables for Information

## CDL-C Cluster Delay Profile (CDP) after spatial filtering and delay scaling

Please find in this introduction some notes on the Cluster Delay Profiles (CDP) tables included in this section:

- SCM used to generate the tables: CDL-C Table 7.7.1-3 in TR 38.901

- AAV configuration: specified in the tables header

- Base Station AE radiation pattern: defined in TR 38.901 Table 7.3-1.

- Delay spread used for scaling: 365ns

- TX-RX port combination: due to the nature of the spatial filtering, different tx/rx port combinations will result in slightly different scaling of the various clusters. For the purposes of CDP derication, we have included only the results of TX port index 0 and RX port index 0

- SNR normalization factor: the normalization factor included in the tables represent the global value for all TX-RX ports, and it is not specific to the TXID and RXID.

| CDL-C 901 AAV_Option_3_4x4/8x8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TXID | RXID | CLID | CLUSTER POWER LIN | CLUSTER POWER DB | CLUSTER DELAY NS |
| 0 | 0 | 0 | 0.029632696 | -15.28228833 | 0 |
| 0 | 0 | 1 | 0.143114216 | -8.443172234 | 76.6138 |
| 0 | 0 | 2 | 0.105895165 | -9.751238678 | 80.9938 |
| 0 | 0 | 3 | 0.046860656 | -13.2919164 | 85.0089 |
| 0 | 0 | 4 | 0.039303972 | -14.05563556 | 79.4243 |
| 0 | 0 | 5 | 0.255623757 | -5.923987861 | 232.36 |
| 0 | 0 | 6 | 0.158629962 | -7.996147808 | 235.353 |
| 0 | 0 | 7 | 0.117686576 | -9.292730726 | 239.441 |
| 0 | 0 | 8 | 0.002730638 | -25.63735919 | 240.317 |
| 0 | 0 | 9 | 0.003407828 | -24.67522322 | 289.629 |
| 0 | 0 | 10 | 0.00053977 | -32.67791351 | 299.776 |
| 0 | 0 | 11 | 3.98E-05 | -43.99864454 | 340.765 |
| 0 | 0 | 12 | 0.010748066 | -19.68669686 | 448.404 |
| 0 | 0 | 13 | 0.002664106 | -25.74448433 | 477.531 |
| 0 | 0 | 14 | 0.00071287 | -31.46989746 | 792.199 |
| 0 | 0 | 15 | 1.94E-05 | -47.13207687 | 989.337 |
| 0 | 0 | 16 | 2.14E-05 | -46.69615718 | 1554.5 |
| 0 | 0 | 17 | 9.09E-05 | -40.41595943 | 1679.12 |
| 0 | 0 | 18 | 7.24E-06 | -51.39965944 | 2003.93 |
| 0 | 0 | 19 | 3.22E-05 | -44.91983115 | 2046.82 |
| 0 | 0 | 20 | 3.13E-05 | -45.04127155 | 2301.88 |
| 0 | 0 | 21 | 8.13E-06 | -50.90126549 | 2422.66 |
| 0 | 0 | 22 | 2.61E-06 | -55.83806642 | 2570.6 |
| 0 | 0 | 23 | 1.31E-06 | -58.82144345 | 3158.1 |
| SUM POWER |  |  | 0.917804547 |  |  |
| NORMALIZATION FACTOR LIN: 1.628603573073217 |  |  |  |  |  |

| CDL-C 901 AAV_Option_1X_4x4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TXID | RXID | CLID | CLUSTER POWER LIN | CLUSTER POWER DB | CLUSTER DELAY NS |
| 0 | 0 | 0 | 0.003504802 | -24.55336492 | 0 |
| 0 | 0 | 1 | 0.027765986 | -15.56486905 | 76.6138 |
| 0 | 0 | 2 | 0.01944923 | -17.11097596 | 80.9938 |
| 0 | 0 | 3 | 0.007698117 | -21.13615504 | 85.0089 |
| 0 | 0 | 4 | 0.002816586 | -25.50276996 | 79.4243 |
| 0 | 0 | 5 | 0.418421572 | -3.783859336 | 232.36 |
| 0 | 0 | 6 | 0.256665112 | -5.906331601 | 235.353 |
| 0 | 0 | 7 | 0.192070617 | -7.165390675 | 239.441 |
| 0 | 0 | 8 | 8.01E-06 | -50.96424166 | 240.317 |
| 0 | 0 | 9 | 0.000128712 | -38.9038112 | 289.629 |
| 0 | 0 | 10 | 2.85E-07 | -65.45681684 | 299.776 |
| 0 | 0 | 11 | 1.41E-08 | -78.49303031 | 340.765 |
| 0 | 0 | 12 | 0.000433536 | -33.6297459 | 448.404 |
| 0 | 0 | 13 | 3.53E-05 | -44.52299771 | 477.531 |
| 0 | 0 | 14 | 2.03E-06 | -56.91490746 | 792.199 |
| 0 | 0 | 15 | 8.02E-09 | -80.95707607 | 989.337 |
| 0 | 0 | 16 | 1.30E-08 | -78.86641118 | 1554.5 |
| 0 | 0 | 17 | 6.42E-09 | -81.92739454 | 1679.12 |
| 0 | 0 | 18 | 1.03E-08 | -79.88312023 | 2003.93 |
| 0 | 0 | 19 | 1.04E-08 | -79.82595928 | 2046.82 |
| 0 | 0 | 20 | 2.26E-09 | -86.45843276 | 2301.88 |
| 0 | 0 | 21 | 4.37E-08 | -73.59955654 | 2422.66 |
| 0 | 0 | 22 | 8.83E-09 | -80.54243227 | 2570.6 |
| 0 | 0 | 23 | 5.51E-09 | -82.59220231 | 3158.1 |
| SUM POWER |  |  | 0.929000014 |  |  |
| NORMALIZATION FACTOR LIN: 0.394802541404316 |  |  |  |  |  |

| CDL-C 901 AAV_Option_1X_8x8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TXID | RXID | CLID | CLUSTER POWER LIN | CLUSTER POWER DB | CLUSTER DELAY NS |
| 0 | 0 | 0 | 0.007053951 | -21.51567562 | 0 |
| 0 | 0 | 1 | 0.127450693 | -8.946577987 | 76.6138 |
| 0 | 0 | 2 | 0.092130576 | -10.35596216 | 80.9938 |
| 0 | 0 | 3 | 0.03783101 | -14.22152059 | 85.0089 |
| 0 | 0 | 4 | 0.013491301 | -18.69946177 | 79.4243 |
| 0 | 0 | 5 | 0.316374648 | -4.997983249 | 232.36 |
| 0 | 0 | 6 | 0.193711941 | -7.128436077 | 235.353 |
| 0 | 0 | 7 | 0.144863335 | -8.390415211 | 239.441 |
| 0 | 0 | 8 | 6.35E-06 | -51.97190927 | 240.317 |
| 0 | 0 | 9 | 0.000107512 | -39.68543954 | 289.629 |
| 0 | 0 | 10 | 2.21E-07 | -66.55142238 | 299.776 |
| 0 | 0 | 11 | 1.06E-08 | -79.74655038 | 340.765 |
| 0 | 0 | 12 | 0.000523084 | -32.81428468 | 448.404 |
| 0 | 0 | 13 | 3.07E-05 | -45.13245976 | 477.531 |
| 0 | 0 | 14 | 1.54E-06 | -58.13858683 | 792.199 |
| 0 | 0 | 15 | 6.26E-09 | -82.03374314 | 989.337 |
| 0 | 0 | 16 | 9.75E-09 | -80.11122149 | 1554.5 |
| 0 | 0 | 17 | 4.80E-09 | -83.18776327 | 1679.12 |
| 0 | 0 | 18 | 7.73E-09 | -81.11762593 | 2003.93 |
| 0 | 0 | 19 | 8.09E-09 | -80.91844646 | 2046.82 |
| 0 | 0 | 20 | 1.69E-09 | -87.71948656 | 2301.88 |
| 0 | 0 | 21 | 3.35E-08 | -74.75562062 | 2422.66 |
| 0 | 0 | 22 | 8.90E-09 | -80.50826127 | 2570.6 |
| 0 | 0 | 23 | 6.34E-09 | -81.97726201 | 3158.1 |
| SUM POWER |  |  | 0.933576928 |  |  |
| NORMALIZATION FACTOR LIN: 0.482719741578442 |  |  |  |  |  |

| CDL-C 901 AAV_Option_1Y_4x4/8x8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TXID | RXID | CLID | CLUSTER POWER LIN | CLUSTER POWER DB | CLUSTER DELAY NS |
| 0 | 0 | 0 | 0.030332848 | -15.1808681 | 0 |
| 0 | 0 | 1 | 0.151633187 | -8.192057364 | 76.6138 |
| 0 | 0 | 2 | 0.110397997 | -9.570388081 | 80.9938 |
| 0 | 0 | 3 | 0.045631458 | -13.40735655 | 85.0089 |
| 0 | 0 | 4 | 0.03718959 | -14.29578615 | 79.4243 |
| 0 | 0 | 5 | 0.262462413 | -5.809328827 | 232.36 |
| 0 | 0 | 6 | 0.160624796 | -7.941874105 | 235.353 |
| 0 | 0 | 7 | 0.120099722 | -9.20457999 | 239.441 |
| 0 | 0 | 8 | 3.90E-04 | -34.08482186 | 240.317 |
| 0 | 0 | 9 | 0.003486469 | -24.57614171 | 289.629 |
| 0 | 0 | 10 | 2.93E-05 | -45.32554588 | 299.776 |
| 0 | 0 | 11 | 3.08E-05 | -45.11239619 | 340.765 |
| 0 | 0 | 12 | 0.004208536 | -23.75868944 | 448.404 |
| 0 | 0 | 13 | 8.04E-04 | -30.95006522 | 477.531 |
| 0 | 0 | 14 | 7.46E-04 | -31.27133063 | 792.199 |
| 0 | 0 | 15 | 1.03E-06 | -59.8677202 | 989.337 |
| 0 | 0 | 16 | 1.21E-05 | -49.17687753 | 1554.5 |
| 0 | 0 | 17 | 7.99E-05 | -40.97633964 | 1679.12 |
| 0 | 0 | 18 | 6.40E-06 | -51.93754442 | 2003.93 |
| 0 | 0 | 19 | 1.34E-06 | -58.74491658 | 2046.82 |
| 0 | 0 | 20 | 2.77E-05 | -45.57057073 | 2301.88 |
| 0 | 0 | 21 | 5.54E-06 | -52.56151675 | 2422.66 |
| 0 | 0 | 22 | 1.11E-07 | -69.5427331 | 2570.6 |
| 0 | 0 | 23 | 5.53E-08 | -72.57040849 | 3158.1 |
| SUM POWER |  |  | 0.928201471 |  |  |
| NORMALIZATION FACTOR LIN: 0.620885350979501 |  |  |  |  |  |

| CDL-C 901 AAV_Option_2_4x4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TXID | RXID | CLID | CLUSTER POWER LIN | CLUSTER POWER DB | CLUSTER DELAY NS |
| 0 | 0 | 0 | 0.002864333 | -25.42976481 | 0 |
| 0 | 0 | 1 | 0.005887951 | -22.30035784 | 76.6138 |
| 0 | 0 | 2 | 0.005076203 | -22.94461011 | 80.9938 |
| 0 | 0 | 3 | 0.002159406 | -26.6566561 | 85.0089 |
| 0 | 0 | 4 | 0.000922556 | -30.35007274 | 79.4243 |
| 0 | 0 | 5 | 0.392999038 | -4.056085122 | 232.36 |
| 0 | 0 | 6 | 0.242044464 | -6.161048462 | 235.353 |
| 0 | 0 | 7 | 0.176164695 | -7.540811246 | 239.441 |
| 0 | 0 | 8 | 2.33E-04 | -36.33140369 | 240.317 |
| 0 | 0 | 9 | 3.27E-05 | -44.85761612 | 289.629 |
| 0 | 0 | 10 | 7.41E-05 | -41.29948097 | 299.776 |
| 0 | 0 | 11 | 7.86E-06 | -51.04315736 | 340.765 |
| 0 | 0 | 12 | 0.002994086 | -25.23735705 | 448.404 |
| 0 | 0 | 13 | 4.79E-05 | -43.20046912 | 477.531 |
| 0 | 0 | 14 | 5.51E-05 | -42.59079851 | 792.199 |
| 0 | 0 | 15 | 2.44E-06 | -56.13318403 | 989.337 |
| 0 | 0 | 16 | 5.00E-06 | -53.00750968 | 1554.5 |
| 0 | 0 | 17 | 1.37E-05 | -48.64126377 | 1679.12 |
| 0 | 0 | 18 | 1.16E-06 | -59.37088252 | 2003.93 |
| 0 | 0 | 19 | 4.45E-06 | -53.51573217 | 2046.82 |
| 0 | 0 | 20 | 4.70E-06 | -53.28025362 | 2301.88 |
| 0 | 0 | 21 | 1.22E-06 | -59.12839936 | 2422.66 |
| 0 | 0 | 22 | 2.91E-08 | -75.36109905 | 2570.6 |
| 0 | 0 | 23 | 2.66E-08 | -75.7480658 | 3158.1 |
| SUM POWER |  |  | 0.831595773 |  |  |
| NORMALIZATION FACTOR LIN: 0.921048575076897 |  |  |  |  |  |

| CDL-C 901 AAV_Option_2_8x8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TXID | RXID | CLID | CLUSTER POWER LIN | CLUSTER POWER DB | CLUSTER DELAY NS |
| 0 | 0 | 0 | 0.003521326 | -24.53293824 | 0 |
| 0 | 0 | 1 | 0.007238472 | -21.40353127 | 76.6138 |
| 0 | 0 | 2 | 0.006240532 | -22.04778355 | 80.9938 |
| 0 | 0 | 3 | 0.00265471 | -25.75982953 | 85.0089 |
| 0 | 0 | 4 | 0.001134163 | -29.45324617 | 79.4243 |
| 0 | 0 | 5 | 0.483141279 | -3.159258556 | 232.36 |
| 0 | 0 | 6 | 0.297562234 | -5.264221895 | 235.353 |
| 0 | 0 | 7 | 0.216571613 | -6.64398468 | 239.441 |
| 0 | 0 | 8 | 2.86E-04 | -35.43457712 | 240.317 |
| 0 | 0 | 9 | 4.02E-05 | -43.96078956 | 289.629 |
| 0 | 0 | 10 | 9.11E-05 | -40.40265441 | 299.776 |
| 0 | 0 | 11 | 9.67E-06 | -50.1463308 | 340.765 |
| 0 | 0 | 12 | 0.00368084 | -24.34053048 | 448.404 |
| 0 | 0 | 13 | 5.88E-05 | -42.30364256 | 477.531 |
| 0 | 0 | 14 | 6.77E-05 | -41.69397194 | 792.199 |
| 0 | 0 | 15 | 2.99E-06 | -55.23635747 | 989.337 |
| 0 | 0 | 16 | 6.15E-06 | -52.11068311 | 1554.5 |
| 0 | 0 | 17 | 1.68E-05 | -47.74443721 | 1679.12 |
| 0 | 0 | 18 | 1.42E-06 | -58.47405595 | 2003.93 |
| 0 | 0 | 19 | 5.47E-06 | -52.6189056 | 2046.82 |
| 0 | 0 | 20 | 5.78E-06 | -52.38342705 | 2301.88 |
| 0 | 0 | 21 | 1.50E-06 | -58.23157279 | 2422.66 |
| 0 | 0 | 22 | 3.58E-08 | -74.46427249 | 2570.6 |
| 0 | 0 | 23 | 3.27E-08 | -74.85123923 | 3158.1 |
| SUM POWER |  |  | 1.022339003 |  |  |
| NORMALIZATION FACTOR LIN: 1.021230703134317 |  |  |  |  |  |