3GPP TSG-RAN WG4 Meeting #114	R4-2501960

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	Testing issues for CSI compression two-sided models

Agenda item:	7.20.3

Document for:	Discussion

Introduction

In this contribution, we discuss the testing issues for CSI compression use case, which is a two-sided models use case.

The discussion on the test and reference encoder/decoder design continued during RAN4#113‎ [1] meeting and based on the agreements in RAN4#112bis ‎[2] and RAN4#111‎[3] meetings, three decoders (from Samsung, Huawei and CATT) were selected based on the SGCS performance. In this contribution, we share the performance results obtained by cross training our encoder using Nokia’s dataset and selected companies’ decoders (Option 3 Track 1).

Furthermore, following Option 3 Track 2 agreement from RAN4#113, we also provide results using mixed dataset, generated by Qualcomm using the mechanism agreed in email discussion, for training our encoder/decoder pair.

Finally, we also provide our view on Option 4 and suggested some further insights and proposed updates on the agreed procedure for Option 4a and 4b.

Discussion

Derivation of test decoder and encoder with Option 3

Following agreement was reached in RAN4#113 meeting for the next steps of Option 3:

Option 3 next step - decoder(s) selection Agreement: choose the decoders with Low, medium, high SGCS among the decoders submitted by companies Continue the feasibility study on the two tracks below: Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Option 3 Track 1 results:

Nokia’s encoder/decoder using Nokia’s dataset:

For our NOKIA model with joint training, utilizing the agreed-upon CNN-based encoder and decoder, we achieved an SGCS of 0.705 for 2-bit scalar quantization (using a sigmoid function and [1 3 5 7]/8 quantized values on the encoder side), and an SGCS of 0.755 without quantization (with no sigmoid function on the encoder side). Additionally, we experimented with freezing the NOKIA-trained decoder and training the encoder from scratch, resulting in SGCS outcomes very similar to the joint training scenario for both quantized and non-quantized cases.

The separate/sequential training provides similar SGCS results compared to the joint training case using similar datasets.

Nokia’s encoder + selected decoders using Nokia’s dataset:

As agreed during the previous RAN4 #113 meeting, three decoders from Samsung (SAM0), CATT (CAT0), and Huawei (HWS1) have been selected.

- CATT decoder:

  - The first one is CAT0 non-quantization training model (the previous version). Our system is: [Training Dataset from NOKIA + Own Encoder + frozen CAT0 decoder]. We then validate it with the testing dataset from NOKIA. The SGCS obtained is about 0.598.

  - The second one is CAT0_v3 with sigmoid function and 2-bits scalar quantization ([1 3 5 7]/8 as quantized values) in the encoder side. Our system is: [Training Dataset from NOKIA + Own Encoder + frozen CAT0_v3 decoder]. We then validate it with the testing dataset from NOKIA. The obtained SGCS is about 0.510.

- Samsung decoder:

  - SAM decoder is not trained under quantization. The system we use is: [Training Dataset from NOKIA + Own Encoder + frozen SAM decoder]. We then validate it with the testing dataset from NOKIA. The obtained SGCS is about 0.749 (compared to 0.755 using our own enc and dec without quantization).

- Huawei decoder:

  - Huawei’s decoder is trained with sigmoid function and [-3 -1 1 3]/8 quantized values on the encoder side. Our system is: [Training Dataset from NOKIA + Own Encoder + Sigmoid + Quantization + frozen HW decoder then reshape to 256, 13, 32, 2]. We then validate it with the testing dataset from NOKIA. The obtained SGCS is about 0.601.

The results for Option 3 Track 1 are summarized in Table 1.

Table 1. Summary of SGCG results of selected decoders. Enc training and testing are done using NOKIA dataset

| Selected decoder | Sigmoid/Quantization | Test SGCS |
| --- | --- | --- |
| CAT0r4_113mldc_v3 | Sigmoid/2bits, [1 3 5 7]/8 | 0.509 |
| CAT0r4_113mldc | No Sigmoid/NoQ | 0.598 |
| HWS1R4_113mldc | Sigmoid/2bits, [-3 -1 1 3]/8 | 0.601 |
| SAM0R4_mldc_v2 | No Sigmoid/NoQ | 0.749 |

There is a notable degradation in SGCS performance when utilizing Huawei's and CATT's frozen selected decoders on Nokia's dataset. However, better SGCS performance is observed when using Samsung's frozen selected decoder on Nokia’s dataset.

The observed SGCS performance degradation may be attributed to differences in datasets between the companies or potential unobserved variations in the selected encoder architecture. Additionally, it is hypothesized that Samsung's dataset and Nokia's dataset are stochastically similar, resulting in comparable model performance as noted above.

It is observed from Track 1 results that a uniform dataset should be used in the next evaluation steps of the study.

RAN4 to consider further alignment on the combination of activation function and quantization scheme to be used in the evaluation.

Option 3 Track 2 results:

Nokia's encoder-decoder model, designed in accordance with agreed architectural standards and incorporating a sigmoid function and 2-bit scalar quantization (quantized values: [1, 3, 5, 7]/8), was trained using a mixed dataset supplied by Qualcomm. This trained model was then evaluated using individual datasets from each company, selecting the last 70,000 samples from each dataset for testing. The procedure was repeated for the same Nokia model, adhering to the architectural standards but excluding quantization and the sigmoid function. The outcomes are presented in Table 2.

Table 2. Summary of SGCS performance of Nokia’s encoder-decoder pair trained on mixed dataset and tested on the individual company’s dataset

| Test Dataset | SGCS for Nokia’s model with quantization | SGCS for Nokia’s model with no quantization |
| --- | --- | --- |
| Mixed | 0.692 | 0.746 |
| Vivo | 0.701 | 0.767 |
| OPPO | 0.706 | 0.753 |
| MediaTek | 0.698 | 0.753 |
| Ericsson | 0.659 | 0.715 |
| CATT | 0.656 | 0.699 |
| Nokia | 0.698 | 0.748 |
| Qualcomm | 0.610 | 0.623 |

When comparing the SCGS performance data from Track 1, which involves selecting three decoders at RAN4#113 (as shown in Table 1), with that from Track 2, where a mixed dataset was created at RAN4#113 (as shown in Table 2), it is evident that Track 2 results are more consistent among different companies’ models than those of Track 1.

The model trained on a mixed dataset without quantization demonstrates superior SGCS performance compared to the model trained on the same dataset but with 2-bit scalar quantization ([1, 3, 5, 7]/8).

Since the performance is good while uniform dataset is used for the training of both encoder and decoder, RAN4 should pursue Track 2 over Track 1 for the feasibility study of test decoder Option 3.

Option 3 other results:

Nokia’s encoder/decoder using Mixed dataset:

In addition to the dataset from NOKIA, we also trained the selected models with the mixed dataset. Table 3 presents the joint training and separate/sequential training for Nokia’s model performance with the mixed dataset. We observed that the separate/sequential training provides a similar SGCS results compared to the joint training case, which is also observed with NOKIA’s dataset.

Table 3. Nokia model performance with own and mixed datasets

| Dataset | Sigmoid/Quantization | Joint Training |  |  | Separate/Sequential Training |
| --- | --- | --- | --- | --- | --- |
|  |  | Train | Validation | Test | Test |
| Nokia | Sigmoid/2 bits, [1 3 5 7]/8 | 0.719 | 0.705 | 0.705 | 0.705 |
|  | No Sigmoid/NoQ | 0.769 | 0.755 | 0.755 | 0.754 |
| Mixed | Sigmoid/2 bits, [1 3 5 7]/8 | 0.705 | 0.690 | 0.692 | 0.689 |
|  | No Sigmoid/NoQ | 0.760 | 0.746 | 0.746 | 0.745 |

The SGCS performance of the model trained on the mixed dataset is slightly lower than that of the model trained on a single dataset. This difference is likely due to the increased diversity from the various data sources in the mixed dataset.

Nokia’s encoder + selected decoders using Mixed dataset:

The same steps outlined in Option 3 Track 1 were executed with the three selected decoders and Nokia’s decoder (trained on Nokia's own eigenvector dataset) on the mixed dataset. Detailed descriptions and model SGCS performances are summarized below and presented in Table 4:

- CATT decoder: The CAT0_v3 with sigmoid function and 2-bits scalar quantization ([1 3 5 7]/8 as quantized values) in the encoder side. Our system is: [Training with the Mixed Dataset+ Own Encoder + Sigmoid + Quantization + frozen CAT0_v3 decoder]. We then validate it with the testing data from the mixed dataset. The obtained SGCS is about 0.491.

- SAM decoder is not trained under quantization. The system we use is: [Training with the Mixed Dataset + Own Encoder + frozen SAM decoder]. We then validate it with the testing data from the mixed dataset. The obtained SGCS is about 0.688.

- Huawei decoder is trained with sigmoid function and [-3 -1 1 3]/8 quantized values on the encoder side.  Our system is: [Training with the Mixed Dataset + Own Encoder + Sigmoid + Quantization + frozen HW decoder then reshape to 256, 13, 32, 2]. We then validate it with the testing data from the mixed dataset. The obtained SGCS is about 0.541.

- NOKIA decoder:

  - The first one is NOKIA non-quantization training model. Our system is: [Training with the Mixed Dataset + Own Encoder + frozen NOKIA decoder (trained based on NOKIA dataset)]. We then validate it with the testing data from the mixed dataset. The obtained SGCS is about 0.689.

  - The second one is NOKIA with sigmoid function and 2-bits scalar quantization ([1 3 5 7]/8 as quantized values) on the encoder side. Our system is: [Training with the Mixed Dataset + Own Encoder + Sigmoid + Quantization + frozen NOKIA decoder (trained based on NOKIA dataset)]. We then validate it with the testing data from the mixed dataset. The obtained SGCS is about 0.629.

Table 4. Summary of SGCS performance of selected decoders on Nokia’s dataset and the Mixed dataset. The SGCS values are based on the test dataset

| Selected Decoder | Sigmoid/Quantization | Train Enc with Ref Dec on NOKIA dataset | Train Enc with Ref Dec on the Mixed dataset |
| --- | --- | --- | --- |
| CAT0r4_113mldc_v3 | Sigmoid/2bits, [1 3 5 7]/8 | 0.509 | 0.491 |
| HWS1R4_113mldc | Sigmoid/2bits, [-3 -1 1 3]/8 | 0.601 | 0.541 |
| SAM0R4_mldc_v2 | No Sigmoid/NoQ | 0.749 | 0.688 |
| NOKIR4_113mldc_v3 | Sigmoid/2bits, [1 3 5 7]/8 | 0.704 | 0.629 |
| NOKIR4_113mldc_v3_NoQ | No Sigmoid/NoQ | 0.755 | 0.689 |
| QCOMR4_113mldc | Sigmoid/NoQ | 0.502 | 0.495 |

From results in Table 4, it is observed that if encoder and decoder are trained with different datasets, significant degradation in the performance can occur.

Table 5 summarizes SGCS results of Nokia’s frozen decoders (with and without quantization) using Nokia’s dataset and mixed dataset.

Table 5. Summary of SGCS results of Nokia’s frozen decoders

| Decoder | Sigmoid/Quantization | Enc training dataset | Frozen Dec training dataset | Testing dataset | Test SGCS |
| --- | --- | --- | --- | --- | --- |
| NOKIR4_113mldc_v3 | Sigmoid/2bits, [1 3 5 7]/8 | NOKIA | NOKIA | NOKIA | 0.704 |
|  |  | Mixed | NOKIA | Mixed | 0.629 |
|  |  | Mixed | Mixed | Mixed | 0.689 |
| NOKIR4_113mldc_v3_NoQ | No Sigmoid/NoQ | NOKIA | NOKIA | NOKIA | 0.755 |
|  |  | Mixed | NOKIA | Mixed | 0.689 |
|  |  | Mixed | Mixed | Mixed | 0.745 |

When encoder and decoder are trained during the sequential training on the same dataset, the test SGCS performance is comparable to the joint encoder/decoder training.

Derivation of test decoder and encoder with Option 4

Following agreement was reached in RAN4#113 meeting for the next steps of Option 4:

Option 4a(Dataset based) for 2-sided model Agreement: - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - Step 5: Label selected dataset(s) (mapping of encoder input to latent message) [using encoder] corresponding to the selected test decoder(s) from option 3 - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders(using different structures) Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset

Option 4b (encoder based) for 2-sided model Agreement: - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - Step 6: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

In this section, we will comment on the differences between the agreed feasibility procedures and our proposed procedures and propose clarification and additions to fully evaluate the feasibility of Options 4a and 4b.

For Option 4a, which is the dataset-based option, there are three aspects the procedures to discuss.  Step 4 in the agreed procedure corresponds to Step 3 in our proposal, where the common dataset used for training the test decoder is selected.  When this step is based on Option 3, track 1, the selected dataset must be common since the primary aspect of Option 4a is a common, standardized dataset used to train the test decoder.  This should be made clear in the procedure.  When this step is based on Option 3, track 2, the mixed dataset has now become a common dataset between companies.  However, the procedure should be clear that the mixed dataset is common as well.

Clarify in Step 4 of the Option 4a feasibility procedure that the selected dataset, whether based on track 1 or track 2 of Option 3, should be common for test decoder development.

In Step 5 of agreed procedure for Option 4a, the dataset is labeled to provide an interoperable mapping from the eigenvalues to the compressed CSI feedback.  The step indicates that this labeling is done “using encoder” but does not clearly identify the encoder to use.  The encoder to use is the encoder selected in the first three steps of the agreed procedure.  In particular, the step is performed for each of the encoders selected in Option 3.  Therefore, one labeled dataset is created for each of the selected encoders.  These datasets are different because the mapping from eigenvalues to the feedback labels may be different for each encoder.

Clarify in Step 5 of the Option 4a feasibility procedure that the encoder(s) used to label the selected dataset is the selected encoder(s) from Option 3.

Step 6 in the RAN4 agreed procedure corresponds to Steps 4-6 in our proposal, where the test decoder and own encoders are trained.  We separate the development of the test decoders and the own encoders in our procedure because the procedure is meant to mimic the operation of the test equipment vendor developing a test decoder and the operation of a UE-side vendor developing an encoder for inference.  The training of the test decoder will be based on the common dataset from the previous step.  However, for the training of their own encoders, companies can use the common dataset or their own dataset, where the labels in their own dataset would be based on the reference encoder/decoder selected earlier in the procedure for interoperability.

Clarify in Step 6 of the Option 4a feasibility procedure that the training of the “own decoder(s)” and “own encoder(s)” is separate since the actions of different vendors (test equipment vendors and UE-side vendors, respectively) are being mimicked in this step.

For Option 4b, which is the encoder-based option, we address several steps in the agreed procedure where improvement or clarification is needed.  Steps 1-3 of the agreed procedure for the feasibility study indicate that the results of Option 3 should be reused.  One of the outcomes of these steps in Option 3 is the selection of the test encoder/decoder pair.  Since the test encoder has already been selected (in fact, we choose to study three separate test encoders), no encoder needs to be selected in Step 4.  In fact, the purpose of tracks 1 and 2 of Option 3 is for companies to train their own encoders, which are different from the encoder which is paired with the test decoder.  Later in the procedure, companies will train their own encoders based on the selected encoder.

Clarify in Step 4 of the Option 4b feasibility procedure that the selected encoders are the ones paired with the test decoders selected in Option 3, not the own encoders trained in track 1 and track 2 of Option 3.

The first part of Step 5 of the agreed feasibility procedure for Option 4b is to train “own decoder(s)” which mimics the process carried out by the test equipment vendors in creating their own test decoders.  This corresponds to Step 5 in our proposed procedure for the Option 4b feasibility study.  What isn’t stated in the agreed procedure is the source of the dataset used to train these decoders.  Since there is no common training dataset assumed, Step 4 in our proposed procedure makes it clear that each company is free to use their own dataset, where the labels are created using the selected encoder. The ability of each test equipment vendor to use their own dataset for training their test decoder is one of the reasons to consider a reference encoder and is an important part of the feasibility study.

Clarify in Step 5 of the Option 4b feasibility procedure that the decoders are trained using a dataset which has been selected by the company mimicking the test equipment vendor where the dataset labels are obtained using the reference encoder.

After completing the first part of Step 5 of the agreed Option 4b procedure, a reference encoder has been selected and companies have trained their test decoders.  At this stage, companies now begin acting as UE-side vendors to train their individual encoders.  This step is described in our proposed procedure in Step 6, but it has not been included in the agreed procedure.  This step is essential to exploring the feasibility of Option 4b since UE-side vendors will train their encoders to be interoperable with the reference encoder but can have control over the dataset used for the training.  Since there is no common dataset in Option 4b, the dataset used to train the UE-side encoder would be different than the dataset used to train the test decoder.  The datasets could be similar in nature (e.g., based on similar distributions of channels) but would contain different samples.  This step must be performed before the final performance evaluation in the second part of Step 5.

Add a step to the Option 4b feasibility procedure after the first part of existing Step 5 for companies to train their own encoders, acting as UE-side vendors.

Once the reference encoder, test decoders, and UE-side own encoders have been trained and selected, the next step is to evaluate the performance of the UE-side own encoders using each of the test decoders.  This is the second part of Step 5 of the agreed procedure and Step 7 of our proposed procedure.  Ideally, an encoder tested with any test decoder should consistently pass the requirements test or consistently fail the requirements test.  The degree to which this occurs can be assessed by testing a UE-side encoder using each of the test decoders and considering the spread of the performance results for this encoder.  The performance spread should be small (details are FFS).  In order to provide a good comparison for these performance tests, a common dataset should be used to evaluate the performance with all UE-side encoders and all test decoders.  This dataset is created in Step 3 of our proposed procedure where the common dataset is only used for this performance evaluation – it is not meant to be used for model training.  We propose to modify the Option 4b procedure to include the creation and use of such a dataset.

Clarify the second part of Step 5 of the Option 4b feasibility procedure that the performance testing of the UE-side encoders is done with a common dataset used with all of the test decoders.  Include a step to agree on the common dataset used for this performance testing.

Conclusion

In this paper, we share our further views and simulation results on the two-sided CSI compression use case. Specifically, we cover the following aspects:

- Derivation of test decoder and encoder with Option 3

- Derivation of test decoder and encoder with Option 4

In the paper, the following Observations and Proposals were made:

Observation 1: The separate/sequential training provides similar SGCS results compared to the joint training case using similar datasets.

Observation 2: There is a notable degradation in SGCS performance when utilizing Huawei's and CATT's frozen selected decoders on Nokia's dataset. However, better SGCS performance is observed when using Samsung's frozen selected decoder on Nokia’s dataset.

Proposal 1: It is observed from Track 1 results that a uniform dataset should be used in the next evaluation steps of the study.

Proposal 2: RAN4 to consider further alignment on the combination of activation function and quantization scheme to be used in the evaluation.

Observation 3: When comparing the SCGS performance data from Track 1, which involves selecting three decoders at RAN4#113 (as shown in Table 1), with that from Track 2, where a mixed dataset was created at RAN4#113 (as shown in Table 2), it is evident that Track 2 results are more consistent among different companies’ models than those of Track 1.

Observation 4: The model trained on a mixed dataset without quantization demonstrates superior SGCS performance compared to the model trained on the same dataset but with 2-bit scalar quantization ([1, 3, 5, 7]/8).

Proposal 3: Since the performance is good while uniform dataset is used for the training of both encoder and decoder, RAN4 should pursue Track 2 over Track 1 for the feasibility study of test decoder Option 3.

Observation 5: The SGCS performance of the model trained on the mixed dataset is slightly lower than that of the model trained on a single dataset. This difference is likely due to the increased diversity from the various data sources in the mixed dataset.

Observation 6: From results in Table 4, it is observed that if encoder and decoder are trained with different datasets, significant degradation in the performance can occur.

Observation 7: When encoder and decoder are trained during the sequential training on the same dataset, the test SGCS performance is comparable to the joint encoder/decoder training.

Proposal 4: Clarify in Step 4 of the Option 4a feasibility procedure that the selected dataset, whether based on track 1 or track 2 of Option 3, should be common for test decoder development.

Proposal 5: Clarify in Step 5 of the Option 4a feasibility procedure that the encoder(s) used to label the selected dataset is the selected encoder(s) from Option 3.

Proposal 6: Clarify in Step 6 of the Option 4a feasibility procedure that the training of the “own decoder(s)” and “own encoder(s)” is separate since the actions of different vendors (test equipment vendors and UE-side vendors, respectively) are being mimicked in this step.

Proposal 7: Clarify in Step 4 of the Option 4b feasibility procedure that the selected encoders are the ones paired with the test decoders selected in Option 3, not the own encoders trained in track 1 and track 2 of Option 3.

Proposal 8: Clarify in Step 5 of the Option 4b feasibility procedure that the decoders are trained using a dataset which has been selected by the company mimicking the test equipment vendor where the dataset labels are obtained using the reference encoder.

Proposal 9: Add a step to the Option 4b feasibility procedure after the first part of existing Step 5 for companies to train their own encoders, acting as UE-side vendors.

Proposal 10: Clarify the second part of Step 5 of the Option 4b feasibility procedure that the performance testing of the UE-side encoders is done with a common dataset used with all of the test decoders.  Include a step to agree on the common dataset used for this performance testing.

- References

- R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm Incorporated, RAN4#113 meeting, 18th – 22nd November 2024, Orlando, USA.

- R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#112bis meeting, 14th – 18th October 2024, China.

- R4-2410570, WF on AI/ML, RAN4#111, Fukuoka city, Japan, May 20th – 24th, 2024.