3GPP TSG-RAN WG4 Meeting #114                                                              R4-2501966

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:	Study on testing issues for CSI compression two-sided models

Agenda item:			7.20.3

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two use cases (i.e., beam management and positioning accuracy enhancements) [2].

Furthermore, the study objectives with corresponding checkpoints in RAN#105 (Sep ‘24) particularly on the two sub-use cases of CSI feedback enhancements are provided [2]. From RAN4 perspective, it is tasked to further analyze the various testing options for two-sided models, i.e., particularly related to the use case of CSI compression among all (sub-)use cases discussed in Rel-18 study item.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

As per the discussions in RAN#105, the exploration of testability and interoperability challenges related to AI-driven CSI compression is set to continue within the remaining timeframe of Release 19, while the relevant objectives are included in a newly set-up study item [RP-243245]. In this contribution, we aim to further the discussion on testing issues for CSI compression two-sided models.

# 2. Discussion

## 2.1 Standardization for Test decoder Option 3

As per the discussions in RAN4#110bis, RAN4 is examining the feasibility of this approach in terms of standardization processes. As further discussed in RAN4#112, RAN4 agreed the way forward [8] on how to proceed the evaluation on test decoder Option 3 (test decoder specified in RAN4 spec.), with the detailed procedure for Option 3 next steps captured as:

Issue 4-1: Option 3 next steps Agreement: - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

Based on RAN4 offline discussion after RAN4#113, most companies have shared their decoder based on their own dataset, and few of them even shared their datasets. Next step will be Step 4, i.e., “Each company brings results for training of “own encoder” with selected decoder(s): performance alignment to be checked/discussed; using “own” data or data shared by other companies”.

#### 2.1.1 CNN-based evaluation for CSI compression

In the post-meeting email discussion after RAN4#111, the detailed CNN model parameters for both encoders and decoders are provided as follows. It should be noted that the CNN-based structures for encoder/decoder are given without strong technical justification and not yet favored by most companies, since the purpose of this evaluation is only for the feasibility study on standardization procedure.

Table 1. CNN model parameters for encoder.

| Model architecture parameters for encoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | FLOPs | 47M (vs. 1.6M for eTypeII) |
|  | Model size | 448Kbyte |
|  | Num of Resnet blocks | 6 |
|  | Layer after resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer before resnet blocks | 1 layer (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer and direct combination |
|  | CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
|  | Kernel/Channel size within a resnet block | (3,3,32) |
|  | Size of message (feature map) passed across layers | (13,32) |

Table 2. CNN model parameters for decoder.

| Model architecture parameters for decoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | FLOPs | 740M |
|  | Model size | 7.2Mbyte |
|  | Num of Resnet blocks | 6 |
|  | Layer before resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer after resnet blocks | 1 layer (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer and direct combination |
|  | CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
|  | Kernel/Channel size within a resnet block | (3,3,128) |
|  | Size of message (feature map) passed across layers | (13,32) |

Table 3. Other model related parameters:

| Activation function | ReLU |
| --- | --- |
| Latent message size | 32 float values |
| Payload size, bits | 64 per layer. |
| Quantization | 2 bits |

When we build up the CNN-based encoder and decoder, we examined the value of FLOPs given in Table 1 and 2, and found that both values of “FLOPs” given are actually for the number of Multiply-Accumulate (MAC) operations, which is defined as a common step that computes the product of two numbers and adds that product to an accumulator [9]. Strictly speaking, it is not the FLOPs (Floating Point Operations), which is used more commonly for neural network computation complexity, and usually 1 MACs can be interpreted as 2 FLOPs.

Proposal 1: Capture the CNN model parameters for CSI compression encoder/decoder into TR 38.843, with the correction of the metric of FLOPs:

- CNN complexity values (47M and 740M for encoder/decoder respectively) is mistakenly provided, which shall be corrected by 9.39*107 FLOPs and 1.48*109 FLOPs for encoder/decoder respectively.

#### 2.1.2 Encoder training based on a reference decoder

Based on the email discussions after RAN4#113, the encoder training is performed based on a pseudo standardized decoder:

- Reference decoder: We firstly select CATT decoder without quantization and Samsung decoder without quantization to perform encoder training. For other decoders, we may further update the results after the Tdoc submission deadline.

- Dataset: Based on companies’ contributed dataset listed in the following table, the encoders have been trained correspondingly.

- Training method: Based on corresponding datasets from companies, the encoder is further trained by strictly following the training hyperparameters alignments provided in R4-2414447. To firstly get alignment with other companies’ results more easily, there is no quantization considered in both encoder and decoder sides while the float-based latent messenger is considered only.

In the following table, the comparison of SGCS has been provided over 6 companies’ datasets. Under each companies’ dataset, the three rows of SGCS values are corresponding to the 3 types of training results:

(1) Row-1: SGCS based on joint encoder + decoder training, for which the SGCS results are provided in corresponding companies’ paper given in the last column;

(2) Row-2: SGCS based on newly trained encoder + Samsung decoder (ONNX format);

(3) Row-3: SGCS based on newly trained encoder + CATT decoder (ONNX format).

| Dataset | Encoder | Reference Decoder | SGCS of CNN without quantization | Reference paper (only for joint en/decoder training) |
| --- | --- | --- | --- | --- |
| Samsung | Joint en/decoder training by Samsung |  | 0.6872 | R4-2418095 |
|  | Retrained encoder | Samsung ONNX | 0.6910 (0.55%) | N/A |
|  | Retrained encoder | CATT ONNX | 0.3663 (-46.70%) | N/A |
| CATT | Joint en/decoder training by CATT |  | 0.7800 | R4-2415174 |
|  | Retrained encoder | Samsung ONNX | 0.6390 (-18.07%) | N/A |
|  | Retrained encoder | CATT ONNX | 0.7995 (2.51%) | N/A |
| Ericsson | Joint en/decoder training by Ericsson |  | 0.7584 | R4-2416314 |
|  | Retrained encoder | Samsung ONNX | 0.7241 (-4.52%) | N/A |
|  | Retrained encoder | CATT ONNX | 0.5698 (-24.86%) | N/A |
| Nokia | Joint en/decoder training by Nokia |  | 0.7550 | R4-2419178 |
|  | Retrained encoder | Samsung ONNX | 0.7477 (-0.97%) | N/A |
|  | Retrained encoder | CATT ONNX | 0.6004 (-20.47%) | N/A |
| Oppo | Joint en/decoder training by OPPO |  | 0.7710 | R4-2417618 |
|  | Retrained encoder | Samsung ONNX | 0.7570 (-1.81%) | N/A |
|  | Retrained encoder | CATT ONNX | 0.6173 (-19.94%) | N/A |
| vivo | Joint en/decoder training by vivo |  | 0.7840 | R4-2418748 |
|  | Retrained encoder | Samsung ONNX | 0.7763 (-0.98%) | N/A |
|  | Retrained encoder | CATT ONNX | 0.6206 (-20.85%) | N/A |

Based on the above table, the following observations can be provided:

Observation 1: The feature distributions of CATT dataset and Samsung dataset can be quite different. The ONNX decoders both show much worse SGCS in the other company’s dataset (-18.07% and -46.70%).

Observation 2: The feature distribution of Samsung dataset is in general aligned with other companies’ dataset, based on acceptable degradation by applying Samsung decoder over other datasets, with the generalization losses (compared to joint training) ranging from 0.55% to -4.52%.

For the method of reference decoder (Option 3 decoder) to be used for private training (i.e., performed by company-A in a dataset provided by company-A), based on the above results, we can have the following observations:

Proposal 2: The feasibility of private encoder training (performed by company-A in a dataset provided by company-A) for Option 3 reference decoder can be only confirmed if:

- The feature distribution of dataset to train reference decoder should be aligned with the dataset used for private training.

#### 2.1.3 Further alignment of Option 3 decoder with quantization

In the above practice, i.e., non-quantization-aware training, the overall model structure is CNN_ecoder  float-based latent message   CNN_decoder. Furthermore, the latent variables input into decoder have no amplitude limitation.

In the quantization-aware training, the overall model structure should be CNN_encoder  sigmoid  scalar quantization with codebooks [1/8,3/8,5/8,7/8]  scalar de-quantization with the same codebooks  inverse sigmoid  CNN_decoder. Due to sigmoid function, the CNN_decoder's input probably ranges in [-3, 3].

Observation 3: The two type decoders handle different value ranges, so the no quantization-aware decoder can't be directly used to train quantization-aware encoder.

Proposal 3: Companies shall share both decoder without quantization from no quantization-aware training and decoder with 2-bits scalar quantization from quantization-aware training.

In RAN4#113, the processing scope of the shared decoder has not been clarified. The question of whether the decoder should be obtained from non-quantization-aware training or quantization-aware training should be answered. If the latter one is chosen, the decoder should cover {scalar quantization with codebooks [1/8,3/8,5/8,7/8]  scalar de-quantization with the same codebooks  inverse sigmoid  CNN_decoder}, or only {scalar de-quantization  inverse sigmoid  CNN_decoder} or other options existed?

Observation 4: For option 3 decoder-based encoder training, different performance will be obtained for quantization-aware training and non-quantization-aware training.

During the encoder training after RAN4#113, it is found that different quantization codebooks were used by companies, which gives additional complexity for further alignment of Option 3 decoder with quantization. Therefore, we see the necessity of clarify the details in encoder and decoder to make alignment for both quantization-aware training and non-quantization-aware training, as below:

Proposal 4: For non-quantization-aware training, below detailed model structure characteristics shall be adopted for performance alignment:

- No sigmoid layer included in the encoder to be trained

- No inverse sigmoid layer included in the shared decoder

- Float32 latent values can be feed into shared decoder directly

Proposal 5: For non-quantization-aware training, below detailed model structure characteristics shall be adopted for performance alignment:

- Sigmoid layer after FC layer included in the encoder to be trained

- Inverse sigmoid layer before FC layer included in the shared decoder (to unload the decompressing burden of subsequent CNN layers and improve the accuracy)

- Fixed quantization codebooks [1/8,3/8,5/8,7/8] to match Sigmoid layer

- The shared decoder only contains inverse sigmoid layer + CNN layers but not de-quantization layer (because unexpected issues may occur in conversion to ONNX and inverse conversion to PyTorch when decoder contains de-quantization layer)

## 2.2 Discussion on Test decoder Option 4

For test decoder Option 4, the following sub-options are agreed in RAN4#112 [13]:

-	4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder)

-	4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

FFS whether training dataset (channel information) needs to be specified

-	4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

And the following WF is agreed based on the discussion in RAN4#112bis [14]:

Interested companies are invited to bring further proposals for the next steps for the feasibility study of Option 4(for any of the sub-options of Option 4) - Analysis of commonality with option 3(whether any steps or data generated for Option 3 can be reused) would be useful

Among the three sub-options presented, we initially observe a paradox with Option 4b. This option has been considered due to the challenges in developing a test decoder as per RAN4 requirements, yet it necessitates the process of documenting a reference encoder within the specifications. From this standpoint, the feasibility of Option 4b depends upon the confirmed feasibility of Option 3.

Observation 5: The feasibility of Option 4b depends upon the confirmed feasibility of Option 3.

To make the progress, the following agreement is achieved for both Option 4a and 4b in RAN4#113 [16]:

Issue 1-3: Option 4a(Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - Step 5: Label selected dataset(s) (mapping of encoder input to latent message) [using encoder] corresponding to the selected test decoder(s) from option 3 - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders(using different structures) Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset Issue 1-4: Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - Step 6: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

It should be noted that in RAN1 study, the following direction A, B and C are agreed in RAN1#118:

Agreement RAN1#118 Continue the study of following directions - on-device operation and UE side offline engineering - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) … - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues … - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues … Agreement RAN1#118bis For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Considering the above progress in RAN1, we see the similarity of Option 4a and 4b compared with Direction A 4-1 and 3a-1.

Observation 6: The similarity between Option 4 in RAN4 and Direction A in RAN1 can be noted.

Considering the parallel discussion, we see no point to repeat RAN1 discussion in RAN4, while considering Option 4b (encoder-based) strongly depends on the feasibility study in Option 3, we see no reason to continue the discussion on Option 4b in parallel.

Accordingly, we would like to provide the following proposal to clarify option 4:

Proposal 6: Before Option 3 feasibility is confirmed or disconfirmed, the discussion on Option 4 can be deprioritized, by considering the factors including

- to avoid parallel discussion with RAN1 Direction A down-selection.

- the study on the feasibility of Option 4b can be speed up if Option 3 is confirmed.

# 3. Conclusion

In this contribution, we provided our viewpoints on the on the interoperability and testability aspects for AI-CSI compression use case, accordingly the following observations and proposals are obtained:

Standardization for Test decoder Option 3

Proposal 1: Capture the CNN model parameters for CSI compression encoder/decoder into TR 38.843, with the correction of the metric of FLOPs:

- CNN complexity values (47M and 740M for encoder/decoder respectively) is mistakenly provided, which shall be corrected by 9.39*107 FLOPs and 1.48*109 FLOPs for encoder/decoder respectively.

Observation 1: The feature distributions of CATT dataset and Samsung dataset can be quite different. The ONNX decoders both show much worse SGCS in the other company’s dataset (-18.07% and -46.70%).

Observation 2: The feature distribution of Samsung dataset is in general aligned with other companies’ dataset, based on acceptable degradation by applying Samsung decoder over other datasets, with the generalization losses (compared to joint training) ranging from 0.55% to -4.52%.

Proposal 2: The feasibility of private encoder training (performed by company-A in a dataset provided by company-A) for Option 3 reference decoder can be only confirmed if:

- The feature distribution of dataset to train reference decoder should be aligned with the dataset used for private training.

Observation 3: The two type decoders handle different value ranges, so the no quantization-aware decoder can't be directly used to train quantization-aware encoder.

Proposal 3: Companies shall share both decoder without quantization from no quantization-aware training and decoder with 2-bits scalar quantization from quantization-aware training.

Observation 4: For option 3 decoder-based encoder training, different performance will be obtained for quantization-aware training and non-quantization-aware training.

Proposal 4: For non-quantization-aware training, below detailed model structure characteristics shall be adopted for performance alignment:

- No sigmoid layer included in the encoder to be trained

- No inverse sigmoid layer included in the shared decoder

- Float32 latent values can be feed into shared decoder directly

Proposal 5: For non-quantization-aware training, below detailed model structure characteristics shall be adopted for performance alignment:

- Sigmoid layer after FC layer included in the encoder to be trained

- Inverse sigmoid layer before FC layer included in the shared decoder (to unload the decompressing burden of subsequent CNN layers and improve the accuracy)

- Fixed quantization codebooks [1/8,3/8,5/8,7/8] to match Sigmoid layer

- The shared decoder only contains inverse sigmoid layer + CNN layers but not de-quantization layer (because unexpected issues may occur in conversion to ONNX and inverse conversion to PyTorch when decoder contains de-quantization layer)

Discussion on Test decoder Option 4

Observation 5: The feasibility of Option 4b depends upon the confirmed feasibility of Option 3.

Observation 6: The similarity between Option 4 in RAN4 and Direction A in RAN1 can be noted.

Proposal 6: Before Option 3 feasibility is confirmed or disconfirmed, the discussion on Option 4 can be deprioritized, by considering the factors including

- to avoid parallel discussion with RAN1 Direction A down-selection.

- the study on the feasibility of Option 4b can be speed up if Option 3 is confirmed.

# 4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402390, “Discussion on testability and interoperability issues for AI-CSI”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] R4-2405653, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#110bis, April 2024.

[6] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[7] R4-2409762, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#111, May 2024.

[8] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[9] https://en.wikipedia.org/wiki/Multiply%E2%80%93accumulate_operatio

[10] R4-2411636, “Further study on testability and interoperability issues for AI-CSI”, RAN#112, Aug. 2024.

[11] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.

[12] R4-2416495, “Further study on testability and interoperability issues for AI-CSI compression”, Samsung, RAN4#112bis, Oct. 2024.

[13] R4-2417207, “WF on data sharing for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[14] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[15] R4-2418095, “Further study on testability and interoperability issues for AI-CSI compression”, Samsung, RAN4#113, Nov. 2024.

[16] R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm, RAN4#113, Nov. 2024.

# 5. Appendix

Following the findings detailed in TR 38.843 [1], RAN4 has progressed its research through RAN4#110 and RAN4#110bis, with consensus and future directions to further the 2-sided model feasibility by executing the standardization procedure as documented in way forwards [4] [6]. As further discussed in RAN4#112, RAN4 agreed the way forward [8] on how to proceed the evaluation on test decoder Option 3 (test decoder specified in RAN4 spec.), with the detailed agreements captured as:

Issue 4-1: Option 3 next steps Agreement: - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused Issue 4-2: Simulation results and alignment Agreement: - perform a round of alignment until RAN4#112-Bis - refined simulation parameters can be found in R4-2414447 Issue 4-4: Options for Option 4 Agreement: Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

Based on another round of alignment and discussion in RAN4#112bis, the following agreements are provided [14]:

Issue 4-1: Simulation results discussion for Option 3 Agreement: Next step: Interested companies to share decoder and encoder ML models and dataset in next meeting (RAN4#113) - Model and dataset used to generate the data submitted to RAN4#112-bis Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format) Interested companies are invited to bring further proposals for the next steps for the feasibility study of Option 4(for any of the sub-options of Option 4) - Analysis of commonality with option 3(whether any steps or data generated for Option 3 can be reused) would be useful

Based on the approved WF in RAN4#113, the following agreements are provided [16]:

Issue 1-2: Option 3 next step - decoder(s) selection Agreement: choose the decoders with Low, medium, high SGCS among the decoders submitted by companies Continue the feasibility study on the two tracks below: Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Issue 1-3: Option 4a(Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - Step 5: Label selected dataset(s) (mapping of encoder input to latent message) [using encoder] corresponding to the selected test decoder(s) from option 3 - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders(using different structures) Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset Issue 1-4: Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - Step 6: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)