# 3GPP TSG-RAN WG4 Meeting # 114                                                         R4-2501988

# Athens, Greece, 17 – 21 February 2025

# Title:	NR Low-Band CA Switching

# Agenda Item:	7.8.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

At the last RAN plenary, a new Rel-19 WI was agreed on for NR Low-Band (LB) CA switching [1], where one of the RAN4 objectives is to specify the switching time-mask for carrier switching. This document captures our views on key contributors.

# Discussion

## Background

Figure 1 shows the proposed LB-LB CA solution from [2]. RAN4 is tasked to specify the UE requirements (e.g., time-mask) to allow switching between case 1 and case 2.

- Case 1: at TTI “N1,” the UE is configured for operation in a LB FDD band (e.g., band n5)

- Case 2: at TTI “N2,” the UE is configured for operation in a LB SDL band (e.g., band n29)

Figure 1: LB-LB CA switching proposal [2]

CA_n5-n29 and CA_n12-n29 are examples of band combinations for which this scheme may be useful.

This scheme may be considered as a DL load balancing scheme rather than a conventional DL CA scheme since, in the case of CA_n5-n29 for example, at any moment of time the DL data traffic is either received

- in the FDD DL band, where a maximum of 25 MHz DL CBW may be configured for band n5 or,

- in the SDL band, where a maximum of 10 MHz DL CBW may be configured for band n29.

## RF Front-end (RFFE) considerations for switching transients

This section identifies key RFFE contributors to case 1 – case 2 switching transients restricted to the case of FDD-SDL switching.

Transients associated with the FDD Power Amplifier (PA):

- For certain LB-LB FDD-SDL pairs of bands, limited isolation may require switch-off of the PA (e.g., CA_n12-n29).

- The case1 – case 2 – case 1 time-mask is therefore an ON-to-OFF-to-ON time-mask. Currently the transient period associated with the OFF-to-ON time-mask is 10 µs for FR1.

- At this stage, it is not clear if the last switching transition from case 2 to case 1 may require consideration of additional power transients (e.g., a change of RB allocation).

Transients associated with shared PLL / Local Oscillator (LO):

- The proposed solution allows balancing DL traffic between two DL LBs using a single antenna.

- It may be assumed that the FDD DL RF chain and the SDL DL RF chain may share a common PLL/LO.

- As background information, the following PLL transient periods were assumed when defining the UL TX switching requirements [3]:

  - 35 µs in cases when no PLL re-programming is required

  - 140 µs in cases when PLL re-programming is required

  - 210 µs in cases when PLL re-programming is required, and an RF chain reload is needed

- For LB-LB switching between an FDD band and an SDL band, we may assume as a starting point a minimum of 140 µs transient to account for PLL re-programming.

Antenna tuners:

- For the example case of CA_n5-2n9, the WID [1] mentions antenna design challenges due to high fractional bandwidth requirements. Antenna tuning might help optimize the performance.

  - Antenna tuners are usually implemented between the test antenna port connector and the physical smartphone antenna. Therefore, we assume that any transient related to the antenna tuner cannot be verified during conformance testing in most cases. This does not preclude RAN4 from considering any transient related to antenna tuning for field operation performance.

  - A closed loop antenna tuning scheme may require a longer time to settle than the PLL settling time.

  - During LB-LB FDD-SDL switching, RAN4 should assume that for certain band combinations TTI level switching may not allow re-configuration of the tuner between the FDD band and the SDL band.

  - We assume for cases with large frequency distance that the antenna is tuned to optimize the FDD UL band performance and may not be reconfigured for switching.

  - We therefore conclude that:

- The antenna performance for the DL FDD band and SDL band may not always be optimum.

- The transient related to antenna tuners may not need to be accounted for.

Observation: For LB-LB switching between an FDD band and an SDL band, RAN4 may consider as a starting point the following contributors to study the switching transient requirements:

- Power amplifier to account for the worst-case ON-to-OFF-to-ON that may be required to prevent high desense of the SDL band. FFS if additional transient may need to be considered to account for power transients due to RB allocation change upon the transition of SDL band  FDD band.

- PLL/LO settling time to account for the worst-case single antenna architecture where the PLL/LO shared may be shared between the FDD DL band RX chain and the SDL RX chain. As a starting point, RAN4 may assume a minimum of 140 µs for PLL re-programming,

Any transient related to antenna tuners may be ignored.

# Conclusion

This contribution summarizes our views on key RFFE contributors to study the time-mask/switching transient period requirements for the new WID on LB-LB switching.

Observation: For LB-LB switching between an FDD band and an SDL band, RAN4 may consider as a starting point the following contributors to study the switching transient requirements:

- Power amplifier to account for the worst-case ON-to-OFF-to-ON that may be required to prevent high desense of the SDL band. FFS if additional transient may need to be considered to account for power transients due to RB allocation change upon the transition of SDL band  FDD band.

- PLL/LO settling time to account for the worst-case single antenna architecture where the PLL/LO shared may be shared between the FDD DL band RX chain and the SDL RX chain. As a starting point, RAN4 may assume a minimum of 140 µs for PLL re-programming,

Any transient related to antenna tuners may be ignored.

# References

- RP-243317 New WID on low band carrier aggregation via switching, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024, Apple, TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE.

- RP-242532 Motivation for a new work item on low band carrier aggregation via switching in Rel-19, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024, AT&T, Anterix, Apple, Bell Mobility, Boost Mobile Network, BT plc, Ericsson, Google, HiSilicon, Huawei, Murata, Nokia, OPPO, Qorvo, Qualcomm, Samsung, Skyworks Solutions Inc., Sony, Southern Linc, TELUS, vivo, ZTE.

- R4-2315230 Remaining issues on Rel-18 Multi-carrier enhancements, 3GPP TSG-RAN WG4 Meeting #108bis, Xiamen, China, Huawei, Hisilicon.