3GPP TSG-RAN WG4 Meeting #114	R4-2502004

# Athens, Greece, 17 – 21 February 2025

Agenda item:		6.19.1

Source: 		Qualcomm Inc.

Title:	 	UE-to-UE coexistence and OOB blocking for B252

Document for:		Approval

## 1	Introduction

UE RF requirements for n252 (UL 2000-2020, DL 2180-2200) has been discussed for a few meetings, with main focus on UE-to-UE coexistence requirements towards bands n2 and n25, but final conclusion has not been reached yet. Another band-specific important aspect without conclusion is OOB blocking requirements and whether co-banding is considered similar to n256 requirements. As these requirements are relevant also for NTN IoT band 252, these requirements are discussed in this contributino.

## 2	Discussion

In RAN4#112-bis an agreement on UE-to-UE coexistence was reached

- Agreement:

  - Define a new more relaxed spurious emission requirement for TN-NTN UE to UE coexistence compared to -50dBm/MHz, that allows the NTN service to work effectively, across the full n252 band, whilst still protecting the TN UE at least in the majority of cases

    - Further discuss the actual value and methodology for the requirement (e.g. corresponding separation assumption)

    - Companies are requested to propose what emission level in dBm/MHz could be achieved for frequencies below 1995 MHz, considering the UE RF implementation, different uplink channel bandwidths and RB allocations

    - Further discuss how to capture the solutions in the specification

This was further discussed in RAN4#113, with the following captured the WF

- Tentative Agreement:

    - Agree to two different additional spurious emission requirements, each associated to an NS value and corresponding A-MPR, for protection of band b25/n25/b2/n2

      - FFS: -30 dBm/MHz for relaxed value.

        - FFS: This emission level applies and the associated A-MPR is allowed whether or not the NS value is signalled. Also, the protection level should a note equivalent to Note 15 in 38.101-1 which says “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.”

      - FFS: [-40] dBm/MHz for the stricter value based on current b23 specification

    - The requirement is applicable in every country where b25/n25/b2/n2 and n252 are applicable

    - Specify asymmetric channel bandwidths with 5, 10 and [15] MHz UL CHBW for band n252.

The work item is scheduled to conclude in March 25, which seems not possible given there is no conclusion on the UE-to-UE coexistence

Observation 1: It has already been agreed that UE-to-UE coexistence requirement is relaxed from -50 dBm/MHz

Observation 2: Work item may need to be extended due to slow progress for UE-to-UE coexistence.

As discussed in earlier meetings, UE-to-UE interference is a statistical phenomenon: UEs need to be near each other and also both UEs need to be scheduled at the same time. Given that the transmission bandwidth is so narrow for IoT NTN UEs, it is expected that no significant interference will reach the protected frequencies. If it can be agreed that requirement is not more stringent than -40 dBm/MHz then no A-MPR will be required. A-MPR has been specified for NS_24 for protection of B34 at 5 MHz frequency offset with -50 dBm/MHz limit. If the emission requirement ends up being -50 dBm/MHz, then A-MPR evaluations are needed.

To be able to conclude the work within Q2/25, a decision on the protection limit is needed. Given the previous agreements, it is suggested to focus the discussion on range between -30 dBm/MHz and -40 dBm/MHz.

Proposal 1: Conclude 2/25 protection limit by choosing from the candidate values of -30 dBm/MHz and -40 dBm/MHz aligned with decision for n252.

Proposal 2: No A-MPR is needed if requirement is not more stringent than -40 dBm/MHz

For out-of-band blocking different options were discussed for out-of-band blocking frequency ranges. 252 frequency range is overlapping with 256, and it is extremely likely that in practical UE designs the same filter is used to support both 252 and 256. However, when n256 was specified it was considered to share the same filter with 1/65. Realistically, a practical design will have only a single duplexer to support these bands. Therefore, 252 out-of-band blocking should be specified to accommodate co-banding with 256 and 65.

Observation 3: 252 can be co-banded with 65 and 256.

Proposal 3: Specify OOB boundaries for 252 as shown in Table 1

Table 1: Out of-band blocking for n252 and n256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 and n252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

## 3	Conclusion

In this contribution UE RF requirements for B252 were discussed. Following observations and proposals were made:

Observation 1: It has already been agreed that UE-to-UE coexistence requirement is relaxed from -50 dBm/MHz

Observation 2: Work item may need to be extended due to slow progress for UE-to-UE coexistence.

Proposal 1: Conclude n2/n25 protection limit by choosing from the candidate values of -30 dBm/MHz and -40 dBm/MHz.

Proposal 2: No A-MPR is needed if requirement is not more stringent than -40 dBm/MHz

Observation 3: 252 can be co-banded with 65 and 256.

Proposal 3: Specify OOB boundaries for 252 as shown in Table 1

Table 1: Out of-band blocking for n252 and n256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 and 252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

## 4	References