3GPP TSG-RAN WG4 Meeting #114	R4-2502010

# Athens, Greece, 17 – 21 February 2024

Agenda item:		7.10.2.2.1

Source: 		Qualcomm Inc.

Title: 		NR NTN high power UE Tx requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved. One of the objectives is high power UE operation for NTN. In this contribution UE RF requirements and feasibility of NTN HPUE are discussed.

## 2	Discussion

For UE RF architecture, NR TN HPUE conclusions need further consideration. For NR, PC2 is allowed to use dual Tx (tx diversity) implementation, but PC1.5 is specified to always apply dual Tx. For NTN UL-MIMO has not been considered, as UL-MIMO typically needs good SNR to give benefits. For NR NTN up to PC1.5 is specified currently for TN for TDD bands. The same or similar hardware could be used to support PC1.5 for NTN. Re-use from TN would mean that PC1.5 is based on dual-Tx architecture.

With PC2 being commercialized in handheld devices, also component designs have been evolved to tolerate higher powers. PC2 single Tx measurement results are commonplace in A-MPR evaluations in TN side. Also duplexers have evolved and more duplexers capable of handling PC2 power levels are coming to market.

Looking at commercially available components is not however the best way to the consider feasibility, as often the development of those components really gathers speed only after the standard allowing the use of higher powers becomes available.

Observation 1: Single Tx PC2 PAs and duplexers are feasible.

Observation 2: Typically, the component availability improves once the standard is in place allowing the use of such components – without a market there is limited need to make such components commercially available.

Proposal 1: PC1.5 is feasible for NR NTN for handheld devices. At least 2 Tx solution is feasible for FDD.

For non-handheld devices PC1.5 solution could use similar hardware as for PC1. PC1 for TN is specified for multiple FDD bands and assumes single Tx. TN PC1 solution could be scaled down to support NTN PC1.5 with single Tx, alternatively used to support NTN PC1.

Similar to a handheld device, 2Tx device can also be used in non-handheld devices. There are multiple reasons why also 2Tx PC1.5 needs to be considered. Firstly, similar RF front-end solution for both handheld and non-handheld allows using the same components for both, and therefore taking advantages of the economies of scale. Secondly, there are specific use cases where 2Tx solutions are preferable.

For example, in automotive domain solutions are often 2Tx – this may be even required to have a higher likelihood for working Tx if there is a need to place an emergency call after a crash. Similarly telematics systems may use 2Tx. This again allows taking advantage of components developed for handheld UE ecosystem.

Observation 3: 2Tx PC1.5 solution allows taking advantage of economies of scale of handheld ecosystem.

Observation 4: 2Tx PC1.5 solution is preferred for automotive industry use cases

Proposal 2: PC1.5 is feasible for NR NTN non-handheld devices. Requirements shall be defined at least for 2Tx solution.

Based on co-existence study results, PC3 ACLR could be sufficient to fulfil co-existence needs. However, this would diverge NTN and TN requirements from each other, as well as require to derive new MPR requirements. With 30 dBc ACLR requirement the PA would be performing worse as a starting point, and therefore also A-MPR values would be slightly increased compared to selecting PC2 ACLR of 31 dB. Therefore, it is suggested to adopt 31 dBc ACLR both for PC2 and PC1.5 requirements, with PC1.5 requirements being applied for both handheld and non-handheld designs.

Situation is somewhat different for PC1, as the difference between TN ACLR and simulation results is much larger. Also, TN PC1 usage has not become widespread and it is not clear if there are economies of scale to take advantage of. Therefore, it would be useful to relax the PC1 requirements to ease implementation and increase the likelihood that designs will appear.

Proposal 3: NR NTN UE ACLR is 31 dBc for PC2 and PC1.5, both for handheld and non-handheld UEs.

Proposal 4: NR NTN UE ACLR is 31 dBc for PC1 non-handheld UEs.

When ACLR is align with TN requirements, also MPR can be directly re-used.

Proposal 5: NR NTN PC2 and PC1.5 MPR are the same as specified for TN UE in TS 38.101-1

To conclude band specific Tx requirements, A-MPR evaluations are needed. Therefore, next step would need to be A-MPR evaluations against specified NS-values. For the evaluations it can be assumed that also SEM, EVM and in-band emission requirements are not changed.

Proposal 6: Aim for A-MPR evaluations for RAN4#114bis.

## 3	Conclusion

In this NTN HPUE operation was discussed. Following observations and proposals were made:

Observation 1: Single Tx PC2 PAs and duplexers are feasible.

Observation 2: Typically, the component availability improves once the standard is in place allowing the use of such components – without a market there is limited need to make such components commercially available.

Proposal 1: PC1.5 is feasible for NR NTN for handheld devices. At least 2 Tx solution is feasible for FDD.

Observation 3: 2Tx PC1.5 solution allows taking advantage of economies of scale of handheld ecosystem.

Observation 4: 2Tx PC1.5 solution is preferred for automotive industry use cases

Proposal 2: PC1.5 is feasible for NR NTN non-handheld devices. Requirements shall be defined at least for 2Tx solution.

Proposal 3: NR NTN UE ACLR is 31 dBc for PC2 and PC1.5, both for handheld and non-handheld UEs.

Proposal 4: NR NTN UE ACLR is 31 dBc for PC1 non-handheld UEs.

Proposal 5: NR NTN PC2 and PC1.5 MPR are the same as specified for TN UE in TS 38.101-1

Proposal 6: Aim for A-MPR evaluations for RAN4#114bis.

## 4	References