3GPP TSG-RAN WG4 Meeting #114	R4-2502013

# Athens, Greece, 17 – 21 February 2025

Agenda item:		7.29.2.1

Source: 		Qualcomm Inc.

Title:	 	NR NTN RedCap UE RF requirements

Document for:		Approval

## 1	Introduction

NR NTN RedCap UE RF requirements are not yet concluded for maximum output power and reference sensitivity. In this contribution these are analyzed and requirements are proposed.

## 2	Discussion

## Issue 1-1: PC3 output power for (e)RedCap

Agreement:

- FFS whether and how a potential new scheme to increase output power is addressed in specifications

  - Alt 1:Further confirm whether rel-18 power boost is already applicable for NR NTN including (e)RedCap

    - If confirmed, no further work is needed.

  - Alt 2: Power increase by network controlled optional new signalling (e.g capability)

  - Alt 3: Change in power tolerances

    - E.g. Upper tolerance exception to UE in specific regions via table note.

  - FFS whether solution is common for HD-FDD and FD-FDD

  - If no solution agreed, PC3 tolerance will remain unchanged to +/- 2 dB.

In RAN4#113 discussion on possible increase of upper tolerance of PC3 maximum output power was continued. The agreed WF provides multiple alternatives to consider.

Starting from alternative one, it is correct that rel-18 power boost schemes are already specified and applicable also for NR NTN, and with the envisioned specification updates to include (e)RedCap for NTN, they would be applicable also for RedCap UEs. In TS 38.101-1 powerboosting impacted two specification sections: MPR and configured maximum output power. UE which indicates support for power boosting without spectrum shaping 1 dB power boost is possible for inner allocations for DFT-s-OFDM pi/2 BPSK and QPSK, i.e. the waveforms that are most relevant also for NTN operation. For a subset of inner allocations also MPR is increase by the same amount as boost, meaning that boosting is optional for allocations which are most likely to have issues with emissions.

No restrictions to rule out RedCap UEs or NR NTN UEs are specified in any working group.

As TS 38.101-5 refers to TS 38.101-1 both for MPR and configured maximum output power, the boosting schemes are already applicable for NR NTN UEs. It is up to the UE to indicate support for the boosting capability.

Observation 1: Rel-18 power boosting specified in coverage enhancements work item is applicable also to NR NTN UEs in current specifications.

Alternative 2, i.e. a new capability to enable increased power seems unnecessary giving power boosting is already possible, and would not also provide the benefit of extending the upper tolerance, i.e. UE would still need to meet the +2 dB tolerance.

Alternative 3, i.e. change in power tolerance is possible, but specification changes would not be as simple as just increasing the maximum output power tolerance. This is because configured maximum output power sets the limits for the measured output power, Pumax. The upper limit for Pumax is based on power class without considering the tolerance. Instead, a separate Pcmax tolerance has been specified. Treatment of upper and lower tolerance are different, as lower tolerance considers also the lower tolerance of power class. Situation is the same in both LTE and NR specifications. Below an excerpt from TS 38.101/1 is shown.

If power boosting by tolerance change is considered, the required changes would include writing at least some part of the configured maximum output power section in TS 38.101-5. Downside of this is increased difficulty of maintaining configured maximum output power and increased likelihood that TN and NTN power control designs diverge.

Observation 2: Change in maximum output power tolerance would necessitate a change also in configured max output power, and as such including at least part of the configured maximum output power clause to TS 38.101-5

Finally, it was left FFS whether the applied solution is common for HD-FDD and FD-FDD. Original motivation for different tolerance was a difference in filter insertion loss. However, filters can have different loss also e.g. due to different process technology and quality of design. Power at antenna connector can also be different due to different routing inside the device or different output power of the PA. Therefore, there is no reason to limit the higher power only the HD-FDD operation.

Proposal 1: Maximum output power requirements shall be the same for HD-FDD and FD-FDD variants for RedCap.

Proposal 2: RAN4 to choose between relying on rel-18 power boosting scheme and keeping +/- 2 dB max output tolerance, or defining higher upper tolerance for both HD-FDD and FD-FDD and including necessary Pcmax definitions in TS 38.101-5 accordingly.

For reference sensitivity RAN4#113 left open a range from 0 to 0.2 dB improvement to reference sensitivity for 2Rx HD-FDD refsens for bands n255 and n254. This corresponds to choosing within 8.8 to 9 dB NF range for RedCap UEs. As of today, Rx requirements of TN TDD and TN RedCap UEs. Given that higher cost process technologies not increased power consumption are not expected for RedCap, most appropriate choice would be to align the NF assumption for NTN RedCap HD-FDD operating bands and TN TDD and RedCap Hd-FDD operating bands, and therefore specify requirements assuming no improvement to NF.

Proposal 3: Specify NR NTN 2Rx HD-FDD Refsens to align with TN TDD and TN 2Rx HD-FDD Refsens for all operating bands.

The final reference sensitivity values would look like in Table 1.

Table 1: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n254 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

## 3	Conclusion

In this contribution NTN (e)RedCap UE RF requirements were discussed. Following observations and proposals were made:

Observation 1: Rel-18 power boosting specified in coverage enhancements work item is applicable also to NR NTN UEs in current specifications.

Observation 2: Change in maximum output power tolerance would necessitate a change also in configured max output power, and as such including at least part of the configured maximum output power clause to TS 38.101-5

Proposal 1: Maximum output power requirements shall be the same for HD-FDD and FD-FDD variants for RedCap.

Proposal 2: RAN4 to choose between relying on rel-18 power boosting scheme and keeping +/- 2 dB max output tolerance, or defining higher upper tolerance for both HD-FDD and FD-FDD and including necessary Pcmax definitions in TS 38.101-5 accordingly.

Proposal 3: Specify NR NTN 2Rx HD-FDD Refsens to align with TN TDD and TN 2Rx HD-FDD Refsens for all operating bands.

With proposal 3 agreed the final reference sensitivity values would look like in Table 1.

Table 1: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n254 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

## 4	References