3GPP TSG-RAN WG4 Meeting #114	R4-2502018

# Athens, Greece, 17 – 21 February 2025

Agenda item:		6.5.3

Source: 		Qualcomm Inc.

Title: 		n26 PC2 A-MPR

Document for:		Approval

## 1	Introduction

n26 HPUE requirements were discussed in rel-18 but not completed. This contribution discussed the A-MPR for n26 PC2.

## 2	Discussion

Some A-MPR analysis was done for n26 PC2 in release 18. Some, but not all of it was captured in TR 38.896. The data captured does not represent conclusions in RAN4 but only covers simulation results from individual companies. There are four NS-values to consider: NS_12, NS_13, NS_14 and NS_15, as shown in Figure 1. Each of these requires their own A-MPR regions, and often there are different regions defined for different center frequency ranges too.

Figure 1: Summary of additional emission requirements

The additional emission requirements are specified with a measurement bandwidth of 6.25 kHz, and even the requirement definition itself states that emission measurement shall be sufficiently power averaged to ensure a standard deviation less than 0.5 dB. Looking at the simulation results in the TR, there is substantial noise in the simulation results, which could indicate that the measurement has not been sufficiently averaged.

The data in TR 38.896 seems to include simulation results only for QPSK modulation. It is known that emissions tend to be stronger with higher modulation order for DFT-s-OFDM due to increased PAPR. Therefore, it would be still useful to confirm the required A-MPR also with higher order modulations.

Finally, 3 MHz results are missing from TR 38.896 and A-MPR, as well as RSD, for 3 MHz needs further analysis.

Observation 1: At least 3 MHz A-MPR and RSD needs further analysis compared to TR 38.896 results

Observation 2: TR 38.896 includes simulation results only for QPSK, and higher modulation orders for DFT-s-OFDM may need higher A-MPR

Observation 3: Due to narrow measurement bandwidth, care is needed to ensure emissions are sufficiently averaged.

Observation 4: There seems to be no RAN4 agreement yet on A-MPR regions

In summary, A-MPR requirements for n26 are complex, and with clear baseline to compare against it is very time-consuming to compare all the results and get certainty on having sufficient A-MPR. To progress the work, it would be useful to aim to set a baseline for A-MPR regions to facilitate verifications from simulations against the agreed regions.

Proposal 1: Given the complexity of the n26 A-MPR specification, aim to establish a baseline for A-MPR regions during the meeting and to conclude the specification during Q2/2025.

## 3	Conclusion

In this contribution A-MPR for n26 PC2 was discussed. Following observations and proposal were made.

Observation 1: At least 3 MHz A-MPR and RSD needs further analysis compared to TR 38.896 results

Observation 2: TR 38.896 includes simulation results only for QPSK, and higher modulation orders for DFT-s-OFDM may need higher A-MPR

Observation 3: Due to narrow measurement bandwidth, care is needed to ensure emissions are sufficiently averaged.

Observation 4: There seems to be no RAN4 agreement yet on A-MPR regions

Proposal 1: Given the complexity of the n26 A-MPR specification, aim to establish a baseline for A-MPR regions during the meeting and to conclude the specification during Q2/2025.

## 4	References

[1]