3GPP TSG-RAN WG4 Meeting #114            	R4-2502032

Athens, GR, 17th-21st, February, 2025

Agenda item:	7.19.2

Source:	Samsung

Title:	View on CSI reporting requirements framework for CSI prediction

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised Rel-19 WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved as following objective related with CSI prediction part [1]

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model) - Functionality-based LCM leveraged from other use cases, when necessary and applicable - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for above two use cases for AI/ML LCM procedures and UE features [RAN4] - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4] - Finalize the testing framework and procedure for one-sided models and further analysis the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In the last RAN4#113 meeting, the testability and interoperability issue for CSI prediction was under discussed. The related agreed agreements are captured in the WF [2] as following

- Testability and interoperability issues for CSI prediction - Channel model for CSI prediction tests - For channel model for CSI prediction tests, TDL channel models will be used - FFS on whether CDL channel model is needed for generalization test - Reference throughput - Reference throughput is obtained based on random PMI with Rel-15 Type 1 single panel codebook - Testing Condition - Use static conditions as baseline - FFS on whether to have non-static

In this contribution, the view on the LCM procedures including performance monitoring, meanwhile, the testability and interoperability issue of CSI prediction was provided, including the CSI prediction test condition, and test configuration.

# 2	LCM related requirement

Performance Monitoring

Based on revised WID, one of objective for RAN4 is to specify the necessary core requirements for LCM procedures including performance monitoring.

In RAN1#114, the above agreement was agreed for performance monitoring for AI/ML based CSI prediction at the UE side.

Agreement in RAN1#114 For CSI prediction using UE-sided model, for performance monitoring, at least following aspects have been proposed by companies on performance monitoring for functionality-based LCM - Type 1 - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2 - UE reports the predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting) - Type 3 - UE calculates the performance metric(s) - UE report performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting) - Functionality selection/activation/deactivation/switching what is defined for other UE side use cases can be reused, if applicable - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report - Note: down selection is not precluded - Note: UE may make decision within the same functionality on model selection, activation, switching operation transparent to the NW

Moreover, in RAN1#117, the above was agreed to clarify the boundaries between the three Types and the corresponding specification impact.

Agreement in RAN1#117 For the boundary between type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured Observation For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843 - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism

For type 1 monitoring, UE may calculate the metric and report the monitoring output to the network. To facilitate UE reporting, the related CSI-RS resource shall be configured for measurement as predicted and monitoring, including the configuration f prediction and monitoring window.

For type 2 monitoring, gNB may compare the prediction CSI and ground truth CSI to determine the performance of the prediction. gNB may need to know the ground truth CSI based on UE report or measurement from Uplink.  To adjust the prediction window and other parameters, the network may also require ground truth CSI corresponding to multiple time instances within the prediction window of the predicted CSI. To report for target CSI may depend on UE’s capability may affect the CSI processing unit counting and its timeline. Therefore, the related CSI-RS resource shall be also configured, as well as the CSI processing timeline for predicted CSI reporting and ground truth CSI.

From RAN4 perspective, for type 1 and type 3 monitoring, the UE reported performance monitoring related information shall be test in RAN4, the testability of performance monitoring metric or output shall be further studied.

For type 2 monitoring, gNB need to calculate performance metric including at least intermediate KPI, such as SGCS, by comparing the current channel from the latest report with the predicted channel from the previous report

For CSI prediction accuracy, RAN4 has agreed to apply the relative throughput as requirement metric. While, for performance monitoring metric, although RAN1 agreed to use intermediate KPI, such as SGCS, the similar level for testing should be also be discussed to ensure consistency and accuracy of monitoring metrics.

Meanwhile, for type 2 monitoring, RAN4 shall further study whether to define requirement for both predicted CSI and reported ground truth CSI at the same time.

Based on revised WID, the normative work in RAN1 for CSI prediction will start from Q1 of 2025. Therefore, RAN4 impact for performance monitoring will start to discuss after the sufficient progress in RAN1.

Proposal 1:   Further study performance monitoring related requirement and testability according to the sufficient progress in RAN1.

# 3	CSI reporting requirement

## 3.1	Test Conditions

Channel model

Regarding the channel model, it was agreed that TDL will be used for CSI prediction testing in the last meeting. Since one of goal of CSI prediction is to track the channel variation and correlation in the time domain with considering the doppler impact, it should be enough to verify the functionality of AI/ML for CSI prediction. Therefore, TDL channel should be prioritized as baseline.  As for CDL channel, it was applied in RAN1 model performance evaluation. In general, it can provide more information about the spatial domain, it should be beneficial to check the model generalization with considering different spatial correlation level. Therefore, to ensure the model generalization, the CDL based channel model can be considered.

However, from RAN4 demodulation perspective, CDL-based channel models are not usually used. Typically, TDL based channel conditions are defined in the demodulation tests.

In Rel-19, the SI on spatial channel model (SCM) for demodulation under discussion. The WID of this SI was captured as following

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e., up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

Based on the WID, both CDL and TDL based on channel modeling was under studied. According to current progress, limited scenarios identified to be studied as following

- Scenario identified to be studied - SU-MIMO - FR1 - TDD - DL

And methods identified to be studied as following

- Methods identified to be studied - CDL-based methodologies - TR 38.827 based - TR 38.901 based - TDL based Methodologies - Introduce spatial component

Rel-19 study is expected to only focus on FR1 TDD DL SU-MIMO Uma scenario based on 38.827 CDL-C. Meanwhile, the testability and test repeatability for CDL channel model on demodulation and CSI testing should be further studied.

Observation 1:  Rel-19 SCM channel model study is expected to only focus on FR1 TDD DL SU-MIMO Uma scenario based on 38.827 CDL-C channel model. The testability and test repeatability for CDL channel model on demodulation and testing should be further studied.

Therefore, the performance impact on AI based CSI prediction with CDL channel model should be further studied. It was relied on the statable and testable CDL channel model for RAN4 demodulation requirements.

Meanwhile, only technical report was released for this SI in Rel-19, there is no specific CDL model introduced in the specification of Rel-19.

Based above observations, in our understanding, it is better that TDL based channel model should be prioritized for Rel-19 AI based CSI prediction testing. Further study the performance impact on AI CSI prediction with spatial channel model (e.g., CDL based channel model, or TDL based extended channel model) in Rel-20 pending on the progress of SCM channel model study in Rel-19 and specific CDL channel model in Rel-20.

Proposal 2:  Prioritize to use TDL channel model for AI based CSI prediction tests in Rel-19. Further study the performance impact on AI CSI prediction with spatial channel model (e.g., CDL based channel model, or TDL based extended channel model) in Rel-20 pending on the progress of SCM channel model study in Rel-19 and specific channel model in Rel-20.

Reference Throughput

In the last meeting, for CSI prediction testing, it was agreed that the reference throughout is obtained based on the random PMI with Rel-15 Type 1 single panel codebook, which is the same approach for legacy PMI reporting test for different codebook requirements.

In legacy PMI test, the PMI reporting requirement are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to UE reported PMI compared to the case when the transmitter is using random precoding, in terms of the ratio as following

Where the  is 90 % of the maximum throughput obtained at  using the precoders configured according to the UE reports.

Same as legacy PMI test, for AI/ML CSI prediction testing, the same SNR test point with 90% of maximum throughout can be considered as baseline, it should be sufficient to check the functionality and to ensure the proper performance with predicted CSI.

Regarding the codebook configured for AI-based CSI prediction, there is no discussion yet in RAN4#113 meeting. In RAN1 Rel-19 AI/ML CSI prediction study, Rel-18 eType II doppler codebook was applied for CSI report for both AI/ML and Non-AL/ML based CSI prediction.

Rel-18 eType II doppler codebook, its design targeting for high/medium UE velocities. The related CSI measurement configuration and reporting configuration has already enhanced to facilitate CSI prediction, including measurement window, reporting window. These parameters are agreed and more appropriate for non-AI based CSI feedback, it was not clear whether it can also be applied for AI based CSI prediction. Furthermore, in RAN4 Rel-18 eType II doppler codebook reporting requirement, it was challenging to find the proper test configuration that enables measurable PMI prediction gains over legacy baseline. Meanwhile, considering Rel-18 eType II doppler codebook is a UE optional feature with capability, we propose to use the Rel-15 Type I single panel codebook for AI-based CSI prediction to ensure the test feasibility

Regarding the non-AI/ML based CSI prediction, RAN1 had performed lots of evaluation for AI-based CSI prediction, compared with both benchm ark#1 of the nearest historical CSI and benchmark #2 of non-AI/ML based CSI prediction approach, where the non-AI/ML based CSI prediction (Benchmark #2) can include statistical CSI (e.g., based on Kalman filter, Wiener filter, and Auto-regression). For benchmark #2, it is up to UE implementation, including how to update the filter parameters. RAN1 has evaluated and identified gain compared to non-AI/ML based CSI prediction approach. From RAN4 minimum requirement perspective, since the purpose is to verify the proper UE behavior with AI-based CSI prediction, and the testing goal is to verify whether the minimum performance of AL/ML functionality/feature can be achieved, we propose to apply the benchmark#1 of the nearest historical CSI as non-AI baseline to ensure that UE behavior and performance for comparison as additional, considering it is expected that AI/ML based performance requirement should no be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists.

Based on the discussion in above, we propose the following baseline for AI-based CSI prediction testing

Proposal 3:  RAN4 can consider the following baseline for AI-based CSI prediction testing

- For SNR test point for AI-based CSI prediction, use 90% of maximum throughput as baseline

- For codebook configured for AI-based CSI prediction, use Rel-15 type I single panel codebook as baseline

- For non-AI/ML based CSI prediction, use benchmark#1 of the nearest historical CSI for PMI generation as baseline for comparison as additional

FFS on whether to have non-static scenario/configuration

Regarding the static and non-static scenario, the following was agreed in RAN4#110bis meeting

- Testing environment/framework - Both static and non-static scenarios/configurations could be needed for AI testing - RAN4 will further discuss how to use them case by case - FFS whether to use static scenario/configurations as baseline - Refine the definitions of static and non-static scenarios/configurations based on two bullets below - Static: channel model and SNR setting are fixed and do not change over the test, specific channel realizations may be dynamic - Non-static: non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other condition

From RAN4 PMI test requirement perspective, it was usually to define with the specific channel models and different SNR levels

For AI based CSI prediction, in terms of functionality test, we think it should be enough with static scenario/ configuration similar as legacy non-AI based CSI prediction. While considering the generalization test, to guarantee the AI model training on specific scenario/condition can still work well in other scenario/condition. Therefore, it should be also beneficial to consider the non-static scenario/configuration to check the generalization of the model.

Based on RAN1 evaluation, to verify the generalization performance over various configurations, the following aspects were considered as

- Various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h) - Various deployment scenario - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Various frequency granularity assumptions - Various antenna port numbers (e.g., 32 ports, 16 ports)

Regarding the non-static scenario/ configuration for CSI prediction, the key issue is how to introduce the non-static scenario/configuration during the demodulation test.  Meanwhile, also which non-static scenarios/configuration can be considered for CSI prediction tests. For some aspect, such as deployment scenario, and carrier frequency and antenna port numbers, these conditions will be not changed dynamically in the practice. The necessity of these aspect to change dynamically should be further checked. Therefore, the specific condition for non-static scenario should be further studied.

Another alternative solution for considering non-static conditions is to define requirements with multiple scenarios under different condition, where UE can train multiple models corresponding to specific conditions.

Based on RAN4#112 meeting, RAN4 agreed to further study whether and how many different test configurations/parameters are used for tests

Agreement in RAN4#112 Testing goals - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests - LCM would be tested

Therefore, to ensure the generalization aspect across different scenarios and conditions, RAN4 should further study the generalization scenario for CSI prediction use case.

Proposal 4:  Further study the which non-static scenarios/configuration can be introduced for CSI prediction. Another alternative solution for considering non-static conditions is to define requirements with multiple scenarios under different condition to ensure the generalization aspect across different scenarios and conditions. RAN4 should study the generalization scenario for CSI prediction use cases.

## 3.2	Test Configuration for CSI prediction

Test configuration for CSI-RS resource

In legacy PMI test, the periodic CSI-RS configuration was configured, with 5ms periodicity.  And for Rel-18 eType II doppler codebook test, only 1 time distance for prediction was considered, and K as 4 for both FDD and TDD, as following

Agreement in RAN4#110bis Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook

For RAN1 evaluation for AI CSI, both periodic and Aperiodic CSI-RS configuration are considered.

Agreement in RAN1#116 For the evaluation of the AI/ML based CSI prediction, consider the following CSI-RS configuration - Periodic: 5ms periodicity (baseline), 20 ms periodicity (encouraged) - Aperiodic: Optional, CSI-RS burst with K resources and time interval m slots (based on R18 MIMO eType -II ) - Note: Companies to report observation window (number/distance) and prediction window Agreement in RAN1#116bis For the AI/ML based CSI prediction, adopt the following assumption as a baseline for evaluation purpose - UE speed: 30km/h, 60km/h - Others can be additionally submitted, e.g., 10km/h, 120km/h - Observation Window (number/distance): 5/5ms, 10/5ms - Others can be additionally submitted, e.g., 4/5ms, 15/5ms - Prediction window (number/distance between prediction instance/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms, 4/5ms/5ms - Others can be additionally submitted, e.g., 2/5ms/5ms, 3/5ms/5ms, 1/5ms/10ms - For other assumption, reuse Rel-18 baseline Agreement in RAN1#116bis For the evaluation of the AI/ML based CSI prediction, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted - Note: The same selected parameter combination shall be applied for benchmarks - CSI report periodicity: 5ms, 20ms (encouraged) - Other can be additionally submitted, e.g., 10ms

Compared with RAN1 evaluation, it may be not suitable to use the Rel-18 TypeII-Doppler-r18 codebook test setup directly for AI based CSI prediction, since the Type II CSI is only available for aperiodic reporting.

Observation 2:  Compared with RAN1 evaluation, it may be not suitable to use the Rel-18 TypeII-Doppler-r18 codebook test setup directly for AI based CSI prediction, since the Rel-18 Type II CSI is only available for aperiodic reporting.

Moreover, RAN4 agreed to use the throughput achieved based on random PMI with Rel-15 Type I single panel codebook, meanwhile, as our preference, configure the Rel-15 type I single panel codebook as AI based CSI prediction for functionality verification, in our understanding, the test configuration of legacy PMI test with Rel-15 type I single panel codebook can be used as a starting point, the details can be referred in Appendix for both 16Tx and 32Tx

Meanwhile, regarding the assumption for CSI prediction, based on RAN1 and RAN4 evaluation, the following assumption can be considered as starting point

Table 1 CSI prediction parameters considering as starting point

| Parameters | Value |
| --- | --- |
| UE speed | 30km/h, 60km/h |
| Input/output CSI type | Raw channel matrix |
| Observation window | 5/5ms |
| Prediction window | 1/5ms/5ms |
| Performance metrics | Relative Throughput |
| Codebook configuration for CSI prediction | Rel-15 Type I single panel codebook |

Proposal 5:  RAN4 consider the following CSI prediction parameters as starting point,

| Parameters | Value |
| --- | --- |
| UE speed | 30km/h, 60km/h |
| Input/output CSI type | Raw channel matrix |
| Observation window | 5/5ms |
| Prediction window | 1/5ms/5ms |
| Performance metrics | Relative Throughput |
| Codebook configuration for CSI prediction | Rel-15 Type I single panel codebook |

The test configuration for PMI test can be referred in TS38.101-4 section 6.3.2.1.3 and section 6.3.2.21.4

On the other hand, RAN1#116bis agreed to further study whether there a need for specification enhancement about CSI-RS configuration for AI/ML based CSI prediction.

Agreement in RAN1#116bis For AI/ML based CSI prediction, at least for inference, legacy CSI-RS configuration can be a starting point. Further study on whether there is a need for specification enhancement Agreement in RAN1#116bis At least for inference, for UE-sided model-based CSI prediction, legacy feedback mechanism using codebook type set to “type II-Doppler-r18” is a starting point of discussion. Study the necessity and potential specification impacts including at least following aspects - CSI processing criteria and timeline

RAN1 may consider the new CSI measurement and reporting framework, as well as data collection procedure and priority.

Furthermore, based on revised WID, one of objective for CSI prediction is to study and specify the consistency of training/inference for model generalization performance.

RAN1 further agreed to study potential NW-side additional conditions that may impact on UE’s assumption for CSI prediction to ensure consistency between training and inference. In our understanding, it also impacts the configuration for AI based CSI prediction

Agreement in RAN1#118bis For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered Agreement in RAN1#118bis For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configuration are not precluded - Note: for other evaluation assumptions, reuse baseline of TR 38.843 - Note: report how to map antenna port to antenna elements - Note: report the backbone/structure of AI/ML model used

Also, for consistency between training and inference, as evaluated by companies, performance evaluation of UE-side CSI prediction with respect to data distribution mismatch that may arise from NW-side additional condition, considering TRP antenna settings with different TXRU mapping. Therefore, some NW-side additional conditions indication was considered and studied in RAN1.

Therefore, RAN4 should further discuss the test configuration and testing framework after the sufficient progress in RAN1 for defining AI based CSI prediction requirement.

Proposal 6:  Further discuss the test configuration and test framework after the sufficient progress in RAN1 for defining AI based CSI prediction requirement.

AI model

Regarding the AI model, different with two-sided model for CSI compression, where TE vendor does not need to train the AI model. Therefore, there is no interoperability issue. In the last meeting, some companies mentioned whether to consider the reference model for alignment, considering the potential variations in results caused by different AI models. Meanwhile, for different scenarios, the model sizes and structures will be different, e.g., different number of numbers of port, it is possible to train a single generic model based on the mixture of data sets with reasonable performance, but with certain degradation compared to condition-specific models. Therefore, RAN4 may need to discuss whether the reference model needed for alignment.

Proposal 6: RAN4 may need to discuss whether the reference model needed for alignment.

## 3.3	Initial Simulation Results

In this section, the initial simulation results are provided to check the AI model for CSI prediction for different UE velocity

3.3. 1	Intermediate KPI

For the intermediate KPI, both normalized mean-squared error (NMSE) of the predicted channel and squared generalized cosine similarity (SGCS) performance of the dominant channel eigenvector are taken into consideration.

In the performance evaluation, the raw channel matrices estimated based on the CSI-RS signals are feed into the AI model as input data. The CSI-RS interval is defined as 5ms. The AI model predicted the channel information in the next one CSI-RS slot, based on the historical 5 CSI-RS channel information, i.e., the observation window is 5/5ms and the prediction window is 1/5ms/5ms. The following table show the initial performance comparison of the AI/ML-based CSI prediction schemes under various UE moving speed scenarios.

Table 2 NMSE performance comparison over different UE speeds

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  |  | Non-AI Benchmark#1 |
| --- | --- | --- | --- | --- |
|  | 3 km/h | 30 km/h | 60 km/h |  |
| 3 km/h | 2.26×10-3 | 1.33 | 1.28 | 10.48 |
| 30 km/h | 1.22 | 2.36×10-3 | 1.41 | 10.68 |
| 60 km/h | 1.21 | 1.88 | 2.47×10-3 | 11.50 |

From the initial simulation result, it can be seen that the NMSE performance of the AI/ML-based scheme is significantly better than the nearest historical CSI scheme (benchmark#1). However, there’s a generalization problem for the AI/ML-based CSI prediction scheme, which needs further investigations.

Observation 3:  AI/ML based CSI prediction show better performance compared with nearest historical CSI scheme (benchmark#1), while generalization problem needs further study when defining requirement

# 4	Conclusion

In this contribution, the view on CSI reporting requirements framework was provided, including LCM related performance monitoring, and CSI reporting requirements

LCM related Performance Monitoring

Proposal 1:   Further study performance monitoring related requirement and testability according to the sufficient progress in RAN1.

CSI reporting requirement

Observation 1:  Rel-19 SCM channel model study is expected to only focus on FR1 TDD DL SU-MIMO Uma scenario based on 38.827 CDL-C channel model. The testability and test repeatability for CDL channel model on demodulation and testing should be further studied.

Proposal 2:  Prioritize to use TDL channel model for AI based CSI prediction tests in Rel-19. Further study the performance impact on AI CSI prediction with spatial channel model (e.g., CDL based channel model, or TDL based extended channel model) in Rel-20 pending on the progress of SCM channel model study in Rel-19 and specific channel model in Rel-20.

Proposal 3:  RAN4 can consider the following baseline for AI-based CSI prediction testing

- For SNR test point for AI-based CSI prediction, use 90% of maximum throughput as baseline

- For codebook configured for AI-based CSI prediction, use Rel-15 type I single panel codebook as baseline

- For non-AI/ML based CSI prediction, use benchmark#1 of the nearest historical CSI for PMI generation as baseline for comparison as additional

Proposal 4:  Further study the which non-static scenarios/configuration can be introduced for CSI prediction. Another alternative solution for considering non-static conditions is to define requirements with multiple scenarios under different condition to ensure the generalization aspect across different scenarios and conditions. RAN4 should study the generalization scenario for CSI prediction use cases.

Observation 2:  Compared with RAN1 evaluation, it may be not suitable to use the Rel-18 TypeII-Doppler-r18 codebook test setup directly for AI based CSI prediction, since the Type II CSI is only available for aperiodic reporting.

Proposal 5:  RAN4 consider the following CSI prediction parameters as starting point,

| Parameters | Value |
| --- | --- |
| UE speed | 30km/h, 60km/h |
| Input/output CSI type | Raw channel matrix |
| Observation window | 5/5ms |
| Prediction window | 1/5ms/5ms |
| Performance metrics | Relative Throughput |
| Codebook configuration for CSI prediction | Rel-15 Type I single panel codebook |

The test configuration for PMI test can be referred in TS38.101-4 section 6.3.2.1.3 and section 6.3.2.21.4

Proposal 6:  Further discuss the test configuration and test framework after the sufficient progress in RAN1 for defining AI based CSI prediction requirement.

Proposal 7: RAN4 may need to discuss whether the reference model needed for alignment.

Observation 3:  AI/ML based CSI prediction show better performance compared with nearest historical CSI scheme (benchmark#1), while generalization problem needs further study when defining requirement

# References

RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Sep, 2024.

R4-2420513, WF on AI/ML for NR air interface

# Appendix

Test configuration for multiple PMI with 16Tx Type I single panel

Table 6.3.2.1.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLC300-5 |
| Antenna configuration |  |  | High XP 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(9) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 12,(5) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Test configuration for single PMI with 32Tx Type I single panel

Table 6.3.2.1.4-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 32 x 2 (N1,N2) = (4,4) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(9) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,4) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |