3GPP TSG-RAN WG4 Meeting #114                                            R4-2502052

# Athens, Greece, 17th – 21st of February 2025

Title: 	Discussion on RRM requirements for NTN NB-IoT TDD

Agenda Item:	7.31.5 – RRM core requirements

Release:	Rel-19

Source: 	THALES, Iridium Satellite LLC

Document for:	Discussion

# For information

As described in the approved WID RP-242415 for New WID on introduction of IoT-NTN TDD mode (Iridium, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024), the study and work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band.

This work item includes the following objectives:

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2]

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

Current Work Plan from R4-2419663:

| RAN#106 (Dec. 2024) | RAN1#120 | 0.5 | - Based on the study outcome, discuss potential solutions - Identify potential specification impacts - Define the TDD pattern parameters |
| --- | --- | --- | --- |
|  | RAN2#129 | 0.5 | - Start discussion on TDD support for NB IoT NTN - Identify potential impacts on higher layers - Start the definition and configuration of signalling of the periodic pattern, and associated UE procedures. |
|  | RAN4#114 | 0.5+0.25 | - Decide on RF, discuss on RRM requirements that can be re-used from previously defined S/L band. Start preparing Draft CRs for the requirements that can be reused - Conclude on requirements stemming from introduction of TDD mode for NB-IoT NTN and potential impact of the periodic pattern: offset-subframes, DL-UL gap - Start discussion on demodulation performance and specific configuration related to the new TDD pattern |

Decisions from previous RAN4#113 IoT_NTN_TDD WF:

### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analysing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

Decisions from RAN plenary, December 2024, Madrid (WID update):

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline. No blind detection is assumed at the UE side. The value of N is fixed per band. Other parameters of the configuration of the periodic pattern (e.g. downlink to uplink guard period for TDD operation and the number of consecutive/contiguous DL and UL subframes) can be defined per cell.

- For this target band N=9 and minimum of D=8 and U=8 to be supported.

The RAN4#114 meeting agenda is:

7.31	Introduction of IoT-NTN TDD mode	[IoT_NTN_TDD]

7.31.1	General aspects	[IoT_NTN_TDD]

7.31.2	Band and system parameters (DL and UL channelization, channel bandwidth)	[IoT_NTN_TDD-Core]

7.31.3	SAN RF requirements	[IoT_NTN_TDD-Core]

7.31.4	UE RF requirements	[IoT_NTN_TDD-Core]

7.31.5	RRM core requirements	[IoT_NTN_TDD-Core]

7.31.6	Moderator summary and conclusions	[IoT_NTN_TDD-Core]

# General aspects for NTN NB-IoT TDD

Observation 1: First of all, RAN4 is kindly requested to take into account the WID update:

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline. No blind detection is assumed at the UE side. The value of N is fixed per band. Other parameters of the configuration of the periodic pattern (e.g. downlink to uplink guard period for TDD operation and the number of consecutive/contiguous DL and UL subframes) can be defined per cell.

- For this target band N=9 and minimum of D=8 and U=8 to be supported.

# RRM TDD NB-IoT specific requirements in TS 36.133

Agreement:

- For the following RRM requirements, modifications are expected due to the new periodic pattern(s):

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature.

Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc.

Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings.

Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured.

Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed.

Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

Proposal 1: Extension of the specific TDD NB-IOT NTN RRM requirements (i.e. for RLM, MSG3-based channel quality report, Cell re-selection, RRC Re-establishment, RRC Connection Redirection to Non-anchor Carrier, Cell measurements) should consider at least 90 ms (compared with 10 ms RRC legacy duration) since N=9.

Proposal 2: Extension of the specific TDD NB-IOT NTN RRM requirements (i.e. for RLM, MSG3-based channel quality report, Cell re-selection, RRC Re-establishment, RRC Connection Redirection to Non-anchor Carrier, Cell measurements) should consider Tx Duration/Rx Duration of at least 8 ms each 90 ms (since D/U=8ms and N=9).

Proposal 3: At least the following parameters should be revisited for TDD NB-IOT NTN RRM requirements:

- NPDCCH transmission parameters for out-of-sync and in-sync for Category NB1 UE TDD NTN (7.23A.2)

- Qout and Qin Evaluation Period in non-DRX for Category NB1 UE TDD NTN (7.23A.2.1)

- Qout and Qin Evaluation Period in DRX for Category NB1 UE TDD NTN (7.23A.2.2)

- Tsearch

- TUE_re-establish_delay_NB-IoT

- TRRC_procedure_delay

- Tperiod_DL_bitmap

- TDL-UL switch

# Conclusion

Observation 1: First of all, RAN4 is kindly requested to take into account the WID update:

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline. No blind detection is assumed at the UE side. The value of N is fixed per band. Other parameters of the configuration of the periodic pattern (e.g. downlink to uplink guard period for TDD operation and the number of consecutive/contiguous DL and UL subframes) can be defined per cell.

- For this target band N=9 and minimum of D=8 and U=8 to be supported.

Proposal 1: Extension of the specific TDD NB-IOT NTN RRM requirements (i.e. for RLM, MSG3-based channel quality report, Cell re-selection, RRC Re-establishment, RRC Connection Redirection to Non-anchor Carrier, Cell measurements) should consider at least 90 ms (compared with 10 ms RRC legacy duration) since N=9.

Proposal 2: Extension of the specific TDD NB-IOT NTN RRM requirements (i.e. for RLM, MSG3-based channel quality report, Cell re-selection, RRC Re-establishment, RRC Connection Redirection to Non-anchor Carrier, Cell measurements) should consider Tx Duration/Rx Duration of at least 8 ms each 90 ms (since D/U=8ms and N=9).

Proposal 3: At least the following parameters should be revisited for TDD NB-IOT NTN RRM requirements:

- NPDCCH transmission parameters for out-of-sync and in-sync for Category NB1 UE TDD NTN (7.23A.2)

- Qout and Qin Evaluation Period in non-DRX for Category NB1 UE TDD NTN (7.23A.2.1)

- Qout and Qin Evaluation Period in DRX for Category NB1 UE TDD NTN (7.23A.2.2)

- Tsearch

- TUE_re-establish_delay_NB-IoT

- TRRC_procedure_delay

- Tperiod_DL_bitmap

- TDL-UL switch

# Annex – previous meeting agreements

The WF covers the contributions submitted under the following AI:

- 7.28.1 General aspects and work plan	[IoT_NTN_TDD]

- 7.28.2 Study of impact due to periodic pattern	[IoT_NTN_TDD-Core]

### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analyzing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

### Issue 1-2: Impact on SAN RF requirements

Agreement:

- The assessment of the impact on SAN RF requirements (TS 36.108) shown in the table below will serve as a baseline for further discussion on the exact requirement definition. For the requirements to be impacted for [249] band, if any, NTN IoT 254 L-band Tx/Rx requirements in TS 36.108 will serve as a baseline. If identified, new requirements can be defined for SAN RF, e.g. transient period.

| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| --- | --- |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Add this specification to SAN starting from Rel-19 |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> FFS: Specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

### Issue 1-3: Impact on UE RF requirements

Agreement:

- The assessment of the impact on UE RF requirements (TS 36.102) shown in the table below will serve as a baseline for further discussion on the exact requirement definition.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for TDD band [249] | Note |
| --- | --- | --- | --- |
| 6.2B.1: UE maximum output power | Yes | Yes | Support UE power class 3 (23 dBm) Support UE power class 5 (20 dBm) |
| 6.2B.1: UE maximum output power tolerance | No |  |  |
| 6.2B.2: UE maximum output power reduction | Yes | Yes |  |
| 6.2B.3: UE additional maximum output power reduction | No | - | Note: Reuse which band A-MPR, B254 A-MPR is not defined yet and the band [249] UL frequency range is not fully aligned with B254. |
| 6.2.B.4: Configured transmit power | Yes | Yes |  |
| 6.3: Output power dynamics | Yes | Yes FFS | Note: For 36.102 6.3B.4, 36.102 refers to 36.101 6.3.5F, the testing period for aggerate power control may depend on different UL pattern. |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5B.4.2: General spurious emissions | Yes | Yes |  |
| 6.5B.4.3: Spurious emissions for UE co-existence | No |  | Band [249] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence. |
| 6.5B.4.4: Additional spurious emissions | No |  | Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of [B249]. |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | No | To be studied for IoT-NTN TDD mode. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6B.2: In-band blocking | No | Yes | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use IBB requirements from B255,254 and 253 |
| 7.6B.3: Out-of-band blocking | No |  | Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use OBB requirements from B255, 254 and 253 |
| 7.6B.4: Narrow band blocking | No | Yes |  |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes | Re-use spurious emission requirements from Table 7.9.1-1 of 36.101 |
| A.1 DL reference measurement channels |  |  | NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced. |
| A.2 OFDMA Channel Noise Generator (OCNG) |  |  | OCNG Patterns for TDD should be introduced. |
| A.3 Testing related to Satellite Access |  |  | No impact. |
| A.4 UL reference measurement channels |  |  | Reference measurement channels for TDD should be introduced. |

### Issue 2-1: Impact on RRM requirements

Agreement:

- From the RRM requirement point of view, no impact on the following requirements is expected due to the new periodic pattern(s):

  - UE uplink timing

  - PUR

Agreement:

- For the following RRM requirements, modifications are expected due to the new periodic pattern(s):

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature.

Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc.

Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings.

Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured.

Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed.

Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

Demodulation aspect:

No observation in this meeting.

# Annex for information – previous meeting discussions

### Issue 1-1: Operating Bands

<Band number>

- RAN4 to consider the following proposals for the requirement definition during the normative work item phase in future RAN4 meetings.

  - Assign “[249]” E-UTRA operating band number for the new TDD [1616-1626.5] MHz NTN NB-IoT frequency band introduction.

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| [249] | [1616] MHz | - | [1626.5] MHz | [1616] MHz | - | [1626.5] MHz | TDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

  - Use a channel raster of 100 kHz and a Step size of <1>

  - Both FDL_low and FUL_low equal 1616 MHz.

  - For the range of both NDL and NUL:

    - Option 1: Starts from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) or 2^18-2^14-1=245759 (in-between FDD NTN NOffs-DL and FDD NTN NOffs-UL)

    - Option 2: {198196 - 198300}