3GPP TSG RAN WG4 Meeting #114	R4-2502057

Athens, Greece, 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	5.23.1

Source: 	Ericsson

Title:	RRM requirements for L1/L2 based inter-cell mobility

Document for:	Discussion

# Introduction

RAN4 has agreed to support LTM without L1-RSRP support or without L1-RSRP reports. In this contribution, we address some of the key remaining open issues related to supporting LTM implementation.

# Discussion

Clarification PDCCH order to RACH delay

As shown in the copied 3GPP text below, 38.213 specifies the delay for determining PDCCH order to RACH by referencing certain components in 38.133. One such component is ∆RF/BB preparation.

TS 38.133 specifies that the component ∆RF/BB preparation=TBWPSwitchDelay if PRACH is outside UL BWP of PCell, but withing UL BWP of any active SCell. TBWPSwitchDelay refer to Table 8.6.2-1 in 38.133 for the BWP switch delays for different SCS and mentions the smaller SCS should be used to determine the switch time.

However, in case of LTM, the switch could be to transmit PRACH format 0/1 where SCS is 1.25kHz or 5 kHz. Since the BWP switch delay do not refer to 1.25KHz or 5KHz, it is not clear about which delay is applicable. The same issue is addressed in RAN1 by using the SCS configuration of µ=0, and we suggest RAN4 to clarify the same.

- If PRACH is outside UL BWP of PCell, but withing UL BWP of any active SCell, for PRACH transmission using 1.25kHz or 5 kHz SCS, UE determines BWP switch delay assuming µ=0.

Maximum time gap between TCI state activation and cell switch command:

Other open issue is what is the maximum time gap between completion of TCI activation and cell switch command to skip the T/F tracking. In last meeting following WF is agreed.

< Agreement>: Conclude the issue in RAN4#114

  - Option 1:

    - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

      - For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms.

        - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

      - For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 160ms.

  - Option 2:

    - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

      - For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms.

      - For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms with conditions

        - Supporters are encouraged to propose the conditions for discussion next meeting

      - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

We think the tentative time gap of 480ms agreed in last meeting can be quite helpful for the on the NW implementation in inter-CU scenarios. For the case of inter-CU LTM, when UE sends PRACH to target cell, we expect the following sequence of operations before the cell switch command is received at the UE.

- Target DU estimate the TA

- Target DU sends TA command to target CU.

- Target CU sends TA command to source CU

- Source CU sends TA command to source DU

- Source DU prepares cell switch command

- Source DU send cell switch command to the UE

- Potential retransmissions of the cell switch command due to weak channel conditions in the HO region

Since different CU can be involved, and the CU can belong to different vendors, we may not guarantee to perform all these operations within 160ms. We also understand the concern raised during last meeting about the L3 measurements may be impacted due to additional T/F tracking UE may need to perform on the activated TCI states. We are fine to have this statement on the L3 measurements.

Among the options which are open from the last meeting, difference between the options are intra-frequency LTM and inter-frequency LTM are treated differently regarding the time gap between the TCI state activation and cell switch command. We think both inter-frequency and intra-frequency are equally probable for LTM and the need for larger delay (due to deployment constraints) exists in both the cases. Since the longer delay is allowed for the L3 measurements during the 480ms, we think the impact to UE implementation is limited. Hence, we suggest agreeing on option 2.

- In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

- For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms.

- For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms with conditions

- During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

Early ASN1 decoding or fast RRC processing:

<Agreement>: Conclude the issue in RAN4#114

  - Option 1 (HW)

    - When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

      - The requirements are applicable to the case NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion, and the wording can be updated.

  - Option 2 (Nokia, ZTE, E///)

      - When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

  - Option 3 (Apple, HW)

      - The issue is closed

In previous meetings, RAN4 agreed that the UE would perform early ASN.1 decoding upon TCI state activation or PDCCH order RACH. After the UE performs early ASN.1 decoding, if there are further DL/UL synchronization requests from the NW belonging to different cells, the UE's behaviour regarding early ASN.1 decoding was discussed. One concern raised by UE vendors was that TCI state activation is free for the NW, whereas requiring the UE to perform early ASN.1 decoding after each TCI state activation would result in additional overhead for the UE. Therefore, one proposal was to avoid specifying the subsequent early ASN.1 decoding behaviour for the UE. The main intention behind this was to discourage the NW from scheduling early TCI state activation and then changing the TCI state activation later.

From the NW perspective, we cannot rule out the possibility that the NW may need to change the target cell after the TCI state is activated, possibly due to a new measurement report from the UE. When an event is met for multiple cells, if UE do not have sufficient grant to report all the cells at the same time, the order in which UE reports the measurements to the NW is up to the UE. If the UE sends the MR in an order not expected by the NW, NW may send TCI activation towards ‘wrong’ cell at first and when NW receive other measurement report, NW may need to trigger another TCI state activation towards the other cell of NW preference. We believe that the NW sending a TCI state activation command intentionally to multiple cells may not happen in real-time. However, in some cases as we described above, the NW may need to trigger DL/UL synchronization to a different cell. Hence, to allow NW flexibility, the ASN.1 decoding behaviour should be specified.

- When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

Clarification of UE capabilities 39-3-3 and 39-3-5:

In last meeting the following clarification was agreed on 39-3-6.

    - ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with LTM L1 measurements, i.e. SSB resources configured by CSI-MeasConfig are also counted.

Similar clarification was proposed for 39-3-3 and 39-3-5. We provide our understanding on the FGs 39-3-3, 39-3-5, 39-3-6.

| Index | FG | Components | Candidate values | Our understanding |
| --- | --- | --- | --- | --- |
| 39-3-3 | Number of total cells to be measured | The max number of total cells of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement | {1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21,22,23,24} Note: when same PCI is present in serving and candidates, one PCI is counted only once in total cells | UE may support 16 CC with CA (for which UE can support L1 measurement) and can further support 8 LTM candidate cells. Combined value is 16+8. Hence, the total cells include both candidate cells and all the serving cells (including the cells which were not configured as LTM candidate cells). |
| 39-3-5 | Number of SSB resources for L1-RSRP measurement per frequency layer | 1. The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps 2. The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps | Candidate value of Component 1: {1,2,3,4,5,6,7,8} Candidate value of Component 2: {1,2,3,4,5,6,7,8} | This FG is per frequency layer, and our understanding of this FG is that it is only applicable to LTM candidates. The main reason for this is, RAN4 did not define L1-RSRP per frequency layer till Rel-17 as the L1-RSRP is only supported for serving cell. One can argue that all the serving cells are intra-frequency cells and should be included. However, as we can see, the candidate values are lower than UE capability of serving cell L1-RSRP measurement (beamManagementSSB-CSI-RS). We also observed that there is an error in the candidate values, and it seems to have written only from FR1 perspective and may not work for FR2. Hence, we see a need for updating the candidate values and send LS to RAN2. |
| 39-3-6 | Number of total SSB resources to be measured | The max number of total SSB resources of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | Candidate values: {2,4,8,12,16,32,64} Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Since the UE may use existing capability of beamManagement to accommodate LTM, we agreed that UE should not report smaller than beamManagementSSB-CSI-RS. However there seems to be an error in the note. We refer to Component 2 of FG 2-24 of the MIMO, but the component 2 refer to maxNumberCSI-RS-Resource. |

- FG 39-3-5 is only applicable to LTM candidates and the candidate values should be updated to cover the FR2 candidate cells as well. Include the candidate values of 12, 16, 20, 24, 32, 40, 48, 64 to existing candidate values.

- Update the 39-3-6 FG candidate values description from Component 2 of 2-24 to component 1 of 2-24.

# Summary and Conclusion

In this contribution we have analysed RRM requirements for L1/L2 based inter-cell mobility and made following proposals.

- If PRACH is outside UL BWP of PCell, but withing UL BWP of any active SCell, for PRACH transmission using 1.25kHz or 5 kHz SCS, UE determines BWP switch delay assuming µ=0.

- In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

- For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms.

- For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms with conditions

- During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

- When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

- FG 39-3-5 is only applicable to LTM candidates and the candidate values should be updated to cover the FR2 candidate cells as well. Include the candidate values of 12, 16, 20, 24, 32, 40, 48, 64 to existing candidate values.

- Update the 39-3-6 FG candidate values description from Component 2 of 2-24 to component 1 of 2-24.

# References

- R4-2410302 WF on mobility enhancement’s part 1, Mediatek

- RP-221799 “WID on Mobility enhancements”, MediaTek