3GPP TSG RAN WG4 Meeting #114	R4-2502063

Athens, Greece, 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	7.22.3.2

Source: 	Ericsson

Title:	Discussion on other RRM requirements in MIMO Ph5

Document for:	Discussion

# Introduction

In this contribution we analyze the potential RAN4 RRM impacts for the CJT calibration reporting.

# Discussion

CJT enhancements

In RAN4#112bis meeting, RAN4 agreed to specify the core requirements based on the outcome of the simulation study and made the following agreement.

•	RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later.

•	Further discuss the simulation assumptions offline during this meeting.

Way forward:

•	Companies are encouraged to bring simulation assumption/results in the next meeting. Simulation assumption in Appendix A can be used as a starting point for information only.

In RAN4#113 meeting, following simulation assumptions [2] were agreed.

- Agree with the following table for delay/frequency offset reporting. FFS on phase offset.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 2nd priority: 2, 4, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |

In the following sections we present the simulation results based on the above simulation assumptions and make few observations and proposals concerning the core requirements definition.

### Simulation results for frequency offset

| Number of TRS resource sets in a measurement | 30kHz SCS, A_FO=100 ppb, 500 UEs; FO error (absolute) in ppb estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 5.854 | 1 | 8 | 26 | 2.849 | 1 | 4 | 12 |
| 2 TRS | 4.853 | 1 | 7 | 18 | 2.165 | 1 | 3 | 8 |
| 3 TRS | 3.794 | 1 | 5 | 16 | 1.916 | 1 | 3 | 7 |
| 5 TRS | 3.106 | 1 | 4 | 12 | 1.433 | 1 | 2 | 6 |
| 10 TRS | 2.234 | 1 | 3 | 9 | 1.030 | 1 | 2 | 4 |

| Number of TRS resource sets in a measurement | 15kHz SCS, A_FO=100 ppb, 500 UEs; FO error (absolute) in ppb estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 3.265 | 1 | 5 | 14 | 1.553 | 1 | 2 | 7 |
| 2 TRS | 2.396 | 1 | 4 | 9 | 1.107 | 1 | 2 | 5 |
| 3 TRS | 1.905 | 1 | 3 | 8 | 0.991 | 1 | 2 | 4 |
| 5 TRS | 1.547 | 1 | 2 | 6 | 0.794 | 1 | 2 | 3 |
| 10 TRS | 2.234 | 1 | 3 | 9 | 1.030 | 1 | 2 | 4 |

As we can see from the above simulation results, for frequency offset, for SNR of -3dB and 3dB, frequency offset averaged over multiple samples (e.g., averaged over 3 samples or 5 samples) gives better accuracy. Hence, we suggest defining the CJT requirements with multi-sample measurement. Further from the simulations results, 5 sample averaging do not give significant improvement over the 3-sample averaging.

- RAN4 to define the delay requirements of the CJTC frequency offset with 3 sample averaging.

### Simulation results for delay offset

| Number of TRS resource sets in a measurement | 30kHz SCS, A_DO=0.5CP, 500 UEs; DO error (absolute) in CP estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 0.012 | 1 | 4 | 10 | 0.012 | 1 | 3 | 9 |
| 2 TRS | 0.010 | 1 | 3 | 8 | 0.010 | 1 | 3 | 8 |
| 3 TRS | 0.009 | 1 | 3 | 7 | 0.008 | 1 | 3 | 7 |
| 5 TRS | 0.007 | 1 | 2 | 6 | 0.007 | 1 | 2 | 6 |
| 10 TRS | 0.005 | 1 | 2 | 4 | 0.005 | 1 | 2 | 4 |

| Number of TRS resource sets in a measurement | 15kHz SCS, A_DO=0.5CP, 500 UEs; DO error (absolute) in CP estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 0.009 | 1 | 3 | 7 | 0.008 | 1 | 2 | 6 |
| 2 TRS | 0.007 | 1 | 2 | 6 | 0.006 | 1 | 2 | 5 |
| 3 TRS | 0.006 | 1 | 2 | 6 | 0.005 | 1 | 2 | 5 |
| 5 TRS | 0.006 | 1 | 2 | 4 | 0.004 | 1 | 2 | 4 |
| 10 TRS | 0.004 | 1 | 2 | 3 | 0.003 | 1 | 1 | 3 |

As we can see from the above simulation results, for delay offset, for SNR of -3dB and 3dB, delay offset averaged over multiple samples (e.g., averaged over 3 samples or 5 samples) gives better accuracy. Hence, we suggest defining the delay offset reporting requirements with multi-sample measurement. Further from the simulations results, 5 sample averaging provide significant improvement over the 3-sample averaging and single sample measurement.

- RAN4 to define the delay requirements of the CJTC delay offset with 5 sample averaging

### Simulation assumptions and results for Phase offset

Table below summarizes the simulation parameters for reciprocity-based CJTC phase offset reporting:

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz TDD |
| SCS | 30kHz SCS |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| TRP antenna configuration | 1x2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | cjtc-p, periodic CSI-RS using 1 or 2 ports. CSI-RS is precoded by phase of channel estimate from latest SRS with a 1-1 map between SRS and CSI-RS port number |
| CSI-RS periodicity | 20 slots, with slot offset 2 and 3 |
| SNR | -3dB, 3dB |
| Initial UL/DL phase offset between TRPs | 0 degree, 60 degree, 90 degree |
| SRS configuration | Periodic SRS every 10 slots, 48 RB bandwidth |

Impact of number of reported bits on wideband UL/DL phase offset estimation error (50 and 95-percentile) is shown in the tables below:

| Initial phase offset (deg), number of CSI-RS ports | 180 UEs, 20 CSI reports per UE (400 slots); 50%ile of error (absolute) in deg over all reports |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile error | 50%ile in # levels (16) | 50%ile in # levels (32) | 50%ile in # levels (64) | 50%ile error | 50%ile in # levels (16) | 50%ile in # levels (32) | 50%ile in # levels (64) |
| 0 deg, 1 port | 7.595 | 1 | 1 | 2 | 5.255 | 1 | 1 | 1 |  |
| 60 deg, 1 port | 7.674 | 1 | 1 | 2 | 5.422 | 1 | 1 | 1 |  |
| 90 deg, 2 ports | 5.026 | 1 | 1 | 1 | 3.483 | 1 | 1 | 1 |  |

| Initial phase offset (deg), number of CSI-RS ports | 180 UEs, 20 CSI reports per UE (400 slots); 95%ile of error (absolute) in deg over all reports |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 95%ile error | 95%ile in # levels (16) | 95%ile in # levels (32) | 95%ile in # levels (64) | 95%ile error | 95%ile in # levels (16) | 95%ile in # levels (32) | 95%ile in # levels (64) |
| 0 deg, 1 port | 23.001 | 2 | 3 | 5 | 14.757 | 1 | 2 | 3 |  |
| 60 deg, 1 port | 22.372 | 1 | 2 | 4 | 14.977 | 1 | 2 | 3 |  |
| 90 deg, 2 ports | 15.261 | 1 | 2 | 3 | 10.229 | 1 | 1 | 2 |  |

As we can see from the above simulation results, the error measured at 50% point was 5 degrees at SNR of 3dB. If we use the 16 levels in reporting, UE will report it as 22.5 degrees. That means the quantization introduce additional 17.5 degrees error. This error makes the report almost useless for the CJT. As we analyzed in our previous contribution to RAN4, R4-2416335, phase offset has to be within 30 degrees and 10 degrees for some of the applications to work in the field. Even when UE can estimate the PO as low as 5 degrees, this kind of quantization error makes the report to be useless for CJTC. Hence, we suggest RAN4 sending LS to RAN1 to increase the number of levels to avoid the quantization error impact the quality of the CJT phase offset report.

- RAN4 to send LS to RAN1 to increase the number of levels for the phase offset reporting

Asymmetric DL sTRP/UL mTRP

In this section we further discuss the 2TA requirements and UL TCI state switching requirements for UL only TRP.

2TA for the asymmetric DL sTRP/UL mTRP

RAN1 agreements from RAN1#118bis meeting are copied below.

RAN1 Agreement

Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA

- The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA

  - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped.

- FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

In last meeting, Rel-18 2TA requirements were agreed to be considered as a starting point for defining the requirements for 2TA for asymmetric DL sTRP/UL mTRP. We copy some of the requirements introduced in Rel-18 in the table below.

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point. . For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAGas the uplink signal. For intra-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1. For inter-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, - if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG. Where NTA and NTA offset are commanded by the network independently for each TAG [TS 38.331]. NTA offset for first TAG and second TAG are as defined in n-TimingAdvanceOffset and n-TimingAdvanceOffset2 respectively, if configured, otherwise it is NTA offset defined in Table 7.1.2-2. In the requirements of clause 7.1.2, the SSB applies for both CD-SSB and NCD-SSB if the UE supports ncd-SSB-BWP-Wor-r18.

On a high level, RAN4 defined following requirements for 2TA in Rel-18.

- DL reference point for PUCCH/PUSCH/SRS/PRACH

- UL transmit timing requirements

- Gradual timing adjustment requirements

In short, the requirements defined in Rel-18 consider mDCI and SSB being transmitted from multiple TRPs. Since it is mDCI, either of the TRP can schedule the RACH and since SSB association to a TRP was not defined, we had to define somewhat complex requirements. However, in the asymmetric DL sTRP/UL mTRP scenario, though two TA are supported, based on the RAN1 agreement, only one downlink reference timing is supported and same is applied to both the TAGs. At the same time, for the DL reference timing of the UL only node, RAN1 is discussing whether to apply a DL offset on top of the DL reference timing acquired from the anchor TRP. We suggest that RAN4 hold off on the discussion regarding DL reference timing of the UL-only node until RAN1 makes further progress. We copied RAN1 part of the discussion in annex.

For the anchor TRP, since there is SSB transmission, DL reference timing of the anchor TRP can be defined as the first detected path (in time) of the downlink frame from the anchor TRP.

- For asymmetric DL sTRP/UL mTRP with two TAs, DL reference timing of the anchor TRP can be defined as the first detected path (in time) of the downlink frame from the anchor TRP.

- For asymmetric DL sTRP/UL mTRP with two TAs, DL reference timing of the UL only TRP is FFS.

UE UL transmit timing requirements:

Transmit timing requirements can follow the legacy principle. That means, for each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the reception of the downlink reference point. N_TA offset can be used as per the RAN1 definition.

- For each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the downlink reference point of the respective TAG.

Gradual timing adjustment requirements:

We think Rel-18 principles can be reused for the requirements definition. We copy the Rel-18 requirement below for reference.

For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

- For asymmetric DL sTRP/UL mTRP with two TAs, gradual timing adjustment applies for each TAG and shall be met for each TAG.

- For each TAG/TRP, when the transmission timing error between the UE and the reference timing exceeds ±Te then the UE shall adjust the timing such that timing error is within ±Te. The reference timing shall be  before the downlink reference timing of the respective TAG/TRP.

Active uplink TCI state switching delay for UL only TRP

Following WF was agreed in last meeting.

Way forward:

- Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

- Option 2: RAN4 to specify UL TCI state switching requirements.

- Option 3: FFS.

As per our understanding, UL TCI state switch involves following steps:

- Pathloss estimation

- Timing acquisition

- QCL acquisition

Pathloss estimation: As per our understanding, RAN1 is discussing how to configure the PL-RS for the UL only TRP. One of the proposals under discussion was anchor TRP DL-RS can be used for pathloss computation along with PL-offset indicated in the TCI state configuration.

Timing acquisition: As per our understanding, RAN1 is discussing the Dl reference timing aspect. One of the proposals was to use anchor TRP DL reference timing with some DL offset.

QCL acquisition: We think SRS can be configured as a QCL source for deriving spatial info relation. With SRS being configured as QCL RS, next question that arises is how the UE acquires the spatial info for the SRS. As per RAN1 specification, we think the spatial relation info for the SRS can be obtained using the SRS resource set configured with beamManagement. When NW configures SRS resource set with usage set as beamManagement, UE transmits the SRS resources in the SRS resource set in different spatial direction and network measures the received SRS to assess the quality of various SRS resources in the SRS resource set. Based on these measurements, the NW selects the SRS to be configured as the source RS for the UL-TCI state.

For SRS resources configured for beam management, only one SRS resource in each set can be transmitted at a time. Each SRS resource set can have multiple SRS resources, with the maximum number (K) depending on the UE capability. When the UE report the K as 4, NW can configure the SRS resource set with usage as beamManagement with 4 SRS resources.

UE behavior on which direction the SRS resource is transmitted and in how many directions it is transmitted is not specified in RAN1 or RAN4 requirements. To make sure sufficient spherical coverage can be achieved for SRS based BM, RAN4 may need to specify the TX beam sweeping factor. For example, as shown in figure 1, RX beam sweeping using SSB assumes beam sweeping factor of 8 and UE may be allowed to sweep different direction to ensure good spherical coverage. Similarly, as shown in figure2, SRS based TX beam sweeping may be needed to ensure good spherical coverage.

Figure 1: RX beam sweeping using SSB

Figure 2: SRS based BM with TX beam sweeping

- RAN4 to specify SRS based beam management requirements.

- RAN4 to specify UL TCI state switching requirements for UL only TRP.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

CJTC reporting

- RAN4 to define the delay requirements of the CJTC frequency offset with 3 sample averaging.

- RAN4 to define the delay requirements of the CJTC delay offset with 5 sample averaging.

- RAN4 to send LS to RAN1 to increase the number of levels for the CJTC phase offset reporting.

Asymmetric DL sTRP/UL mTRP:

- For asymmetric DL sTRP/UL mTRP with two TAs, DL reference timing of the anchor TRP can be defined as the first detected path (in time) of the downlink frame from the anchor TRP.

- For asymmetric DL sTRP/UL mTRP with two TAs, DL reference timing of the UL only TRP is FFS.

- For each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the downlink reference point of the respective TAG.

- For asymmetric DL sTRP/UL mTRP with two TAs, gradual timing adjustment applies for each TAG and shall be met for each TAG.

- For each TAG/TRP, when the transmission timing error between the UE and the reference timing exceeds ±Te then the UE shall adjust the timing such that timing error is within ±Te. The reference timing shall be  before the downlink reference timing of the respective TAG/TRP.

- RAN4 to specify SRS based beam management requirements.

- RAN4 to specify UL TCI state switching requirements for UL only TRP.

# References

[1] RP-234007, New WID: NR MIMO Phase 5
[2] R4-2420108, WF on RRM requirements for NR_MIMO_Ph5

[2] R4-2420209 Simulation assumptions for CJT calibration

# Annex

To enable two TAs with respect to two TRPs, UE need to obtain DL reference timing and UL TA command towards UL-only TRP. Next, we further discuss the aspects of DL reference timing and UL TA towards the UL-only TRP. The two-TA feature in Rel-18 assumes non-ideal synchronization (i.e., TRPs slot boundaries are not perfectly time aligned and there can be synchronization error of up to e.g., 3µs) between the TRPs. As per the WID, asymmetric and non-collocated deployment is supported for UL-only TRP. That means the synchronization assumptions in Rel-18 are applied for Rel-19 UL-only TRP also as operators may not deploy separate TRPs for using the UL-only TRP feature (e.g., operators may use same deployment for Rel-18 and Rel-19).

As UL-only TRP do not transmit any SSB, UE may have to rely on Anchor TRP SSB to derive the DL reference timing. When slot boundaries of Anchor TRP and UL-only TRP are perfectly aligned in time (i.e., synchronization error is 0µs), UE may use the DL reference timing of Anchor TRP to perform random access procedure and obtain TA command to handle the asymmetric propagation delay difference between Anchor TRP to UE and UL-only TRP to UE.

However, in the practical deployments (such asymmetric deployments considered in this WI) it may be sometimes impossible to maintain perfect synchronization between different TRPs even for intra-DU deployments, since most of the NW deployments use switched fronthaul technic between non-collocated TRPs. Figure 2 illustrates the synchronization scenario where the timing of UL-only TRP is ahead of Anchor TRP. When UL-only TRP is ahead of Anchor TRP slot timing, when UE acquires DL timing from Anchor TRP and transmits PUSCH using the same DL reference timing, it may result in inter-slot interference at UL-only TRP.

Figure 2: Synchronization scenario UL-only TRP ahead of Anchor TRP slot timing

As per TS 38.133, initial transmit timing for RACH transmission, UE needs to acquire frame boundary within ±Te error (Te is much smaller compared to possible synchronization error between TRP), and transmits RACH after acquiring DL synchronization, RACH preamble is always expected to receive after the start of the RACH slot at the NW.

To analyze the issue with practical synchronization error for UL-only TRP, we consider three possible synchronization scenarios as shown in Figure 3. When the UL slot boundaries of anchor TRP and UL- only TRP are not perfectly synchronized (scenario 2 and 3) as shown in Figure 3, using DL timing of anchor TRP to transmit RACH preamble to UL TRP may result in early arrival of RACH preamble or later arrival of the RACH preamble at the UL- only TRP.

Figure 3: Different synchronization scenarios at UL- only TRP and anchor TRP

When UL- only TRP is behind anchor TRP slot timing, when UE acquires DL timing from anchor TRP and transmit RACH using the same DL reference timing, it may result in early arrival of RACH preamble at UL- only TRP (i.e., before the start of RACH occasion or RACH slot at the reception window at the UL TRP as shown in Figure 4). This results in inter-slot interference at the UL- only TRP and UL TRP may not be able to determine the timing advance correctly for the UE.

Figure 4: RACH arrival at UL- only TRP and Anchor TRP with anchor TRP DL reference timing

When UL- only TRP is ahead of anchor TRP slot timing, when UE acquires DL timing from anchor TRP and transmit RACH using the same DL reference timing, it may result in late arrival of RACH preamble at UL- only TRP. Based on the below table showing guard period (GP) for different PRACH preamble formats, we think late arrival may not be an issue for the UL- only TRP deployment scenario where the UE is expected to be close to UL- only TRP and GP is large compared to synchronization error.

Table 2: Guard period for different PRACH preamble formats

| Preamble format | 0 | 1 | 2 | 3 | B1 | B2 | B3 | B4 | C0 | C2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GP in µs | 97 | 716 | 953 | 97 | 2.3 | 7 | 11.7 | 25.8 | 37 | 94 |
| Propagation delay (in km) that GP can cover | 29km | 215km | 285km | 29km | 0.7700m | 2.1km | 3.7km | 7.5km | 11km | 28km |

Based on the above discussion, we have the following proposal to consider practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in an asymmetric deployment, which for example could be handled by introducing a timing offset between the Anchor TRP and the UL-only node, which the network signals to the UE such that the UE can determining one reference timing for the Anchor TRP and a second reference timing for the UL-only node.

Study whether and how to address practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in asymmetric DL sTRP UL mTRP deployment.

- FFS: UE can be provided with a DL reference timing offset. DL reference timing for communication with UL-only TRP for the UE is obtained from the DL reference timing of the anchor TRP and DL reference timing offset.