3GPP TSG-RAN WG4 Meeting #114	R4-2502071

Athens, Greece, February 17 – 21, 2025

Source:	Nokia

Title:	RRM core requirements for AI/ML positioning accuracy enhancement

Agenda item:	7.19.4

Document for:	Discussion

Introduction

In RAN#102, the Release-18 study on artificial intelligence (AI)/machine learning (ML) for NR air interface (FS_NR_AIML_Air) [1] was completed [2] and the findings (with agreements and open issues) were documented in the outcome technical report 3GPP TR 38.843 V18.0.0 [3]. Furthermore, a new Release-19 work item on artificial intelligence (AI)/machine learning (ML) for NR air interface (NR_AIML_Air) [4] was also approved in RAN#102 to start the normative work for the general AI/ML framework for air interface and to enable the recommended use cases in the preceding study.

During the SI, as mentioned in the extract from 3GPP TR 38.843 V18.0.0 [3] below, following sub use cases for positioning accuracy enhancement are considered:

The following are selected as representative sub-use cases: - Direct AI/ML positioning: - AI/ML model output: UE location - e.g., fingerprinting based on channel observation as the input of AI/ML model - AI/ML assisted positioning: - AI/ML model output: new measurement and/or enhancement of existing measurement - e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement More specifically, the following Cases are considered for the study: - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning One-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

During SI, RAN4 studied different test metrics for performance requirements for AI/ML enabled positioning use case.  Here is an extract from 3GPP TR 38.843 V18.0.0 [3] covering these aspects:

Both direct AI/ML positioning and AI/ML assisted positioning are considered. For metrics for positioning requirements/tests, the candidate options include - Option 1: positioning accuracy: Ground truth vs. reported - only option available for direct positioning - Option 2: CIR/PDP, channel estimation accuracy - Option 3: ToA, RSTD and RSRP, and RSRPP - Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above The feasibility and testability of different options should be further justified in WI.

In RAN4#110, it has been agreed on the testability and interoperability issue for AIML positioning [5].

2.3 Testability and interoperability issues for positioning accuracy enhancement 2.3.1 Agreements in ad-hoc session (R4-240xxxx) Issue 3-2: Requirements for case 3a/3b RAN4 will not define positioning accuracy requirements for case 3a/3b Issue 3-6: Requirements for case 2a/2b RAN4 to come back to case 2a/2b based on progress in the other working groups

In RAN4#110bis, it has been agreed on the testability and interoperability issue for AIML positioning [6].

Issue 3-1: Requirements for case 1 Agreement: postpone discussion until reporting scheme(if defined) is clear. if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced

In RAN4#111, it has been agreed on the testability and interoperability issue for AIML positioning [7].

Issue 3-2: Requirements for case 2a Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups Issue 3-3: Requirements for case 2b Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups

In RAN4#112, it has been agreed on the testability and interoperability issue for AIML positioning [8].

Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS Issue 3-5: Reported metrics for case 2b Agreement: postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported. Issue 3-6: Reported metrics for case 3a/3b Agreement: postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

In RAN4#112bis, it has been agreed on the testability and interoperability issue for AIML positioning [9] while no agreements on RRM core requirement were achieved [11].

Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.

The issues on AI/ML based positioning require further discussion on selected sub use cases. In this paper, the performance requirements for UE-based positioning (i.e., case 1) and NG-RAN node assisted positioning (i.e., cases 3a and 3b) are discussed in the aspects of performance test metric, measurement accuracy requirements, testability, and LCM-related latency requirements.

Discussion

Requirements for Case1

This section discusses the requirements in the use cases with a UE-side model.

Measurement accuracy requirements

Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

In RAN1#117, it is confirmed that the path-based measurement in the context of AIML-based positioning is the existing specifications’ measurements including their measurement reporting. When the measurements in AIML positioning are the same as the measurements defined in the legacy positioning, the various target positioning requirement types for NR positioning in Rel-18 summarized in TR38.859 can be considered as the candidate measurements in AIML positioning. In particular, the functionality input may be one of the existing measurements such as PRS-RSRP, PRS-RSRPP, etc. Also, if any new measurements such as CIR, DP, and/or PDP are agreed in RAN1 as functionality input, there are impacts in RAN4 measurement accuracy requirements for these new measurements.

In our view, the input to the AI/ML functionality should have and satisfy any measurement accuracy requirements to ensure the AI/ML functionality performs as expected. Hence, RAN4 should define the requirements on the measurement types supported in the legacy positioning (e.g., RSTD, PRS-RSRP, and PRS-RSRPP). Moreover, RAN4 should study how to define the requirements on new measurement is supported (e.g., CIR, DP, and PDP).

For AIML positioning in Case 1, RAN4 to define the requirements on the measurements supported in legacy positioning (e.g., RSTD, PRS-RSRP, and PRS-RSRPP) and define the requirements on the new measurements (e.g., CIR, DP, and PDP) based on RAN1 progress.

Additionally, the accuracy requirements can be applicable to the inputs and outputs of AIML model. While the types of input and output are depending on the use cases, defining the requirements for the input or output will have an impact on the positioning accuracy. Therefore, RAN4 can discuss whether/when the measurement accuracy should be defined.

RAN4 to study the necessity of any requirements for the input of the AIML model in Case 1.

Requirements for consistency between training and inference

In NR positioning, DL-PRS configuration is provided in a hierarchy. There can be at most 4 positioning frequency layers (PFL) and each positioning frequency layer has at most 64 TRPs. Each TRP per positioning frequency layer can have 2 DL-PRS Resource sets, thus resulting in a total of 8 DL-PRS resource sets per TRP and each DL-PRS resource set can have up to 64 DL-PRS resources. Each DL-PRS resource corresponds to a beam. Having 2 different DL-PRS resource sets per positioning frequency layer per TRP allows gNB to configure one DL-PRS resource set of wide beams and another DL-PRS resource set of narrow beams for each positioning frequency layer.

Figure 1 DL-PRS configuration hierarchy considering PFL, DL-PRS resource sets and DL-PRS resources.

Existing measurement requirements for NR positioning in TS 38.133 for RSTD, RTOA, UE/gNB Rx-Tx time difference and for PRS-RSRPP / SRS-RSRPP are based on paths. However, accuracy requirements are only defined for first path RSTD, RTOA, UE Rx-Tx, gNB Rx-Tx and PRS-RSRPP / SRS-RSRPP, although additional paths can be reported but for them, no accuracy requirements exist.

Single PFL and dual PFL requirements are specified in TS 38.133 for RSTD, PRS-RSRP and UE Rx-Tx time difference measurements, but accuracy requirements are distinguished only for RSTD. For dual PFL, accuracy for RSTD is decreased from Y=32 Tc to Y=256 Tc, see clause 10.1.23.2 and furthermore the RF margin is increased as defined in Table 10.1.23.2-5a for FR1 and Table 10.1.23.2-6a for FR2. This accuracy degradation and additional uncertainty for dual PFL will result in more difficult (and longer) training and inherence procedures for AI/ML positioning. Thus, single PFL should be the starting point for AI/ML positioning.

RAN4 requirements (e.g., in Case 1) should be firstly defined with respect to single PFL.

When collecting data for training UE-side model cases, the measurements (such as channel samples in the form of CIR, PDP, DP) serve as the training input. Consequently, the data collection process that creates the dataset for training models is dependent on the positioning configuration used to obtain these measurements. When a trained model is used in the inference stage, if the current positioning configuration (e.g., PFL) is different from the positioning configuration used in the training stage, it is not effective to use such a trained model for inference.

Therefore, in both training and inference stages, it is essential to remain in the same positioning configuration (i.e., to maintain consistent configuration). To maintain the consistent positioning configuration in training and inference stages, the consistency needs to be checked to verify the same positioning configuration. In particular, the UE and network (e.g., LMF) must ensure the positioning configuration during inference phase matches with the training phase’s positioning configuration. Specifically, such a consistent positioning configuration can be achieved by validating the PF) across training and inference. Hence, the requirements on the measurements can be defined to ensure that same PFL is configured in training and inference.

The consistency between training and inference should be achieved by having a requirement on consistent configuration (e.g., same PFL) when UE-sided models are used.

Requirements on ground truth accuracy in data collection

In RAN1#116-bis meeting, training data collection of AI/ML based positioning has been discussed to identify the collect ed data samples as follows.

Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

In Part B, the data collection with ground-truth dataset is essential for training. In particular, the training data needs to include both the ground-truth UE location and measurements associated with the UE’s location. To enhance the accuracy of AIML positioning, it is important to have an accurate ground-truth information for positioning labelling. When collecting the ground-truth information from the UE side, a high accuracy positioning information needs to be acquired by the UE. If UE is unable to acquire a high accuracy ground-truth information, the network/testing equipment may assist UE to acquire a high accuracy ground-truth positioning label. For instance, UE may obtain positioning label from multiple sources e.g., non-NR and/or NR RAT-dependent positioning methods. Therefore, the degree of accuracy on ground-truth positioning label can be defined as a quality indicator. The quality indicator on the ground-truth accuracy can be subject to a new requirement.

RAN4 to study a new requirement on a quality indicator for the ground-truth accuracy on positioning label in data collection.

Positioning accuracy requirements

In Case 1 for direct AI/ML positioning, the output of the AI/ML functionality is UE position. As a metric for positioning requirements and tests, we propose the positioning accuracy metric as follows over the selected test positions:

- At each test position, the following steps will be followed:

  - Get the output from AI/ML functionality inference with the input of radio measurements,

  - Compute a positioning error, e.g., absolute error value between estimated position and true position, and

  - Confirm that the positioning error is within the acceptable range (i.e., positioning accuracy requirements).

For instance, positioning accuracy requirements are already specified for NR A-GNSS where the minimum performance requirements are defined in TS 37.571-1 Clause 13.3. Therefore, a similar approach should be envisaged in the context of AI/ML-enabled direct positioning.

Positioning accuracy should be considered as the performance metric in Case 1.

One of the main challenges in introducing Case 1 requirements on positioning accuracy is the availability of accurate enough UE position that could be used as a ground truth for AI/ML based position. As discussed above, when such ground truth is known to the UE or test equipment (TE) then the AI/ML-based positioning can be compared to it, i.e., positioning error can be evaluated. However, the AI/ML based Case 1 positioning functionality should not be left without any requirements.

Even when ground truth, i.e., accurately known position of the UE, for Case 1 AI/ML-based positioning is not available in the testing setup for absolute positioning accuracy calculation, the new functionality should not be left without any requirements or tests.

One approach to test UE-based positioning (Case 1) could be to compare the position inferred by the UE not to the ground truth but to the position provided with the same functionality in a different moment of time or conditions. For example, it should be insured that UE is not reporting very different locations after insignificant change in the radio conditions. In this case, the relative positioning accuracy metric can be considered instead of absolute accuracy relevant to the ground truth.

RAN4 to consider relative positioning accuracy metric for Case 1, i.e., requirement on the possible change in AI/ML-based position due to a change/state of the environment/configuration.

LCM related aspects: latency requirements for LCM actions

WID RP-234039 [4] also discusses RAN4-related aspects to be considered during WI phase for Beam Management and Positioning use cases. The WID covers core requirements aspects as follows.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

The WID also covers the aspects related to performance part.

- For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

Also, the agreement related LCM procedures in RAN4#111 are as follows:

Agreement: RAN 4 to consider defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning) and candidate are: - Identification - Selection - Activation - Deactivation - Switching - Fallback to non-AI operation - Performance monitoring - Others are not precluded

In Case 1, UE can be capable of performing different positioning techniques, e.g., capabilities of the different AIML models/functionalities and legacy positioning techniques while initially having different requirements for each model/functionality and technique/algorithm. If a performance monitoring process detects a performance degradation to a certain point, it is possible to switch the currently running model / functionality with another model / functionality. In the same case, UE can be also enabled to perform a fallback operation to a legacy/default positioning technique and algorithm. For instance, a fallback operation is needed when the measurements can be more suitable for a legacy/default positioning technique than the current model/functionality. It is also possible to utilize AIML feature to determine a measurement requirement feature. If UE’s measurement on the upcoming RS indicates a potential satisfactory of positioning measurements, the UE may be required to process positioning measurements. By avoiding unnecessary processing of positioning measurements with a detected performance degradation, UE can reduce the power consumption. Therefore, when the AI/ML functionality is degrading the system performance lower than any required level associated with the current model/functionality, the continuous operation of such a functionality with the detected performance degradation may have a catastrophic impact on the overall system performance.

Therefore, it is crucial to detect the performance degradation and stop this model/functionality, either by falling back to legacy method or by switching to another model/functionality, within a specified time. The specified time allowed to switch/disable the model/functionality should guarantee that the system performance remains within the acceptable levels.

If an LCM action is required and it is not taken in a timely manner, the performance degradation for AI/ML enabled Positioning use case may be degraded to undesirable level.

RAN4 to define the time latency limit on UE’s LCM actions indicated by network.

In RAN4#111 meeting, various candidates for were considered for defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning). Since RRM requirements may not be necessary for all listed candidates, RAN4 should prioritize the following candidates with a high impact on RRM requirements:

- Activation

- Deactivation

- Switching

- Fallback to non-AI operation, and

- Performance monitoring.

RAN4 to define RRM requirements for LCM, prioritizing Activation, Deactivation, Switching, Fallback to non-AI operation, and Performance monitoring.

Model performance monitoring

AIML positioning discussion in RAN1 has been made the agreements on model performance monitoring.

RAN1 #116-bis Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. RAN1 #119 Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

In Case 1, the model performance monitoring is expected to run on the UE, and a signal may be sent from the UE to LMF. In Option A-3, the Rel-18 assistance data is sent from LMF to the target UE for PRU measurement. PRU may be a reference UE strategically positioned at a fixed, known location to the LMF. For instance, PRU can be seen a device with a known location that assists in determining the location of a UE. Since the precise location of the PRU is assumed to be known, the PRU measurements may require a high precision. To enhance the positioning accuracy, the PRU measurement may need to have a separate accuracy requirement that is stricter than the requirement applied to a regular UE. Thus, RAN4 should study the necessity of the requirements for the PRU measurements in AIML positioning.

Until performance monitoring aspects are agreed in RAN1, RAN4 will wait to discuss the necessity of UE / PRU requirements for performance monitoring.

Requirements for Case 2a/2b

Requirements for Intermediate results in Case 2a

In Case 2a, LOS/NLOS indicator can be supported for reporting. The reported value of LoS/NLoS indicator is the input to the LMF. Therefore, the accuracy of LoS/NLoS indicator can affect the positioning performance. RAN4 should discuss any potential impact of the accuracy requirements of LoS/NLoS indicator.

RAN4 can discuss whether/how to define the requirements on LoS/NLoS.

Requirements for timing information in Case 2b

UE-assisted/LMF-based positioning with LMF-side model is assumed in Case 2b. RAN1 has an agreement on AI/ML based positioning in case 2b to define the supported types of time domain channel measurements, i.e., (a) timing information and (b) paired timing information and power information. Such measurements are supported for UE reporting to LMF. The positioning can use RSTD, RTOA, or UE/gNB Rx-Tx time difference. When the UE reports Rx-Tx time difference measurements with its determined reporting resolution to the LMF, the gNB shall have the corresponding reporting to the LMF. Also, depending on the measurement accuracy, LMF positioning accuracy can be different. RAN4 may discuss any requirement on time-domain channel measurements.

- RAN4 to discuss whether to reuse the legacy measurement period and accuracy requirements and to define any additional measurement requirement in Case 2b.

  - Requirements for Case 3a/3b

Report mapping aspects

- RAN4#112bis agreed to extend the applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.

- Clause 13 of TS 38.133 on Measurement Performance Requirements for NR gNB also specifies report mapping for following UL measurement types: UL SRS RSRP and AoA/ZoA. Whilst angular measurements were not discussed in RAN1 for AI/ML based positioning, it is noted that the UL SRS-RSRPP measurement type was introduced in TS 38.215 in Rel-17 to complement the UL SRS-RSRP measurement. Thus, it is proposed to extend also the applicability of existing report mappings for the UL SRS-RSRP measurement type to AI/ML based positioning.

RAN4 to extend the applicability of existing report mappings also for UL SRS-RSRP to AI/ML based positioning in Cases 3a/3b.

Dependency on positioning techniques and use cases

It has been considered to assume each positioning use cases separately. When an AI/ML model located on UE or network is used, DL- or UL-based positioning can be performed by using the assistance of LMF that utilizes either DL or UL positioning measurements. For instance, the scenario in Case 3a can be described as UE transmits an SRS and gNB performs the AI/ML-assisted measurements and sends LMF the measurement or any intermediate results so that the LMF can determine the UE’s coordinate. In most use cases, the requirements can be discussed and specified within the given scenario of each case.

If there is no limit on the applicable positioning technique on the LMF side, LMF may determine the UE coordinates based on a multi-RTT method. For multi-RTT support, the information on the UE Rx-TX and gNB Rx-Tx measurements needs to be acquired by LMF. If the scenario in Case 3a is separately considered, gNB can send a measurement report to LMF. However, since an SRS is transmitted only, gNB cannot perform the gNB Rx-Tx measurement. Also, for the support of multi-RTT, the LMF also needs to acquire a UE Rx-Tx measurement reported by the UE. To this end, LMF needs to perform the UE Rx-TX measurement by assuming the scenario in Case 2a with the additional assumption that UE also transmits an SRS. Note that, in Case 2a, only gNB transmits a PRS and UE measures and sends a report on UE Rx-Tx measurement. As aforementioned, if each case is separately considered for the requirements, it can be difficult to consider all potential positioning techniques that are applicable to the LMF.

If a positioning technique needs a support of both DL and UL based positioning measurements, such as multi-RTT technique, the UE and gNB have to send the obtained measurement reports that are coordinated to enable LMF to efficiently enhance the positioning accuracy. For instance, when a model is running on gNB for positioning, the positioning measurement reporting of UE and gNB can be coordinated by enabling UE to receive the assistance information with measurement reporting for positioning from LMF and also receive the information on positioning measurement report from gNB. On the other hand, when UE has a model, the UE can receive the assistance information needed for positioning measurement from LMF and share the information to gNB on the measurement status so that the positioning measurements of on UE and gNB sides are coordinated for multi-RTT.

Therefore, it can be discussed on the support of the various positioning techniques such as multi-RTT and any limitations on the requirements due to the assumed scenario in each use case.

RAN4 to establish a common understanding of the support for the various positioning techniques such as multi-RTT and discuss the RRM requirements of positioning techniques supported by other WGs.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to AI/ML based positioning. In the paper, the following observations and proposals were made:

Case 1

- For AIML positioning in Case 1, RAN4 to define the requirements on the measurements supported in legacy positioning (e.g., RSTD, PRS-RSRP, and PRS-RSRPP) and define the requirements on the new measurements (e.g., CIR, DP, and PDP) based on RAN1 progress.

- RAN4 to study the necessity of any requirements for the input of the AIML model in Case 1.

- RAN4 requirements (e.g., in Case 1) should be firstly defined with respect to single PFL.

- The consistency between training and inference should be achieved by having a requirement on consistent configuration (e.g., same PFL) when UE-sided models are used.

- RAN4 to study a new requirement on a quality indicator for the ground-truth accuracy on positioning label in data collection.

- Positioning accuracy should be considered as the performance metric in Case 1.

- Even when ground truth, i.e., accurately known position of the UE, for Case 1 AI/ML-based positioning is not available in the testing setup for absolute positioning accuracy calculation, the new functionality should not be left without any requirements or tests.

RAN4 to consider relative positioning accuracy metric for Case 1, i.e., requirement on the possible change in AI/ML-based position due to a change/state of the environment/configuration.

If an LCM action is required and it is not taken in a timely manner, the performance degradation for AI/ML enabled Positioning use case may be degraded to undesirable level.

RAN4 to define the time latency limit on UE’s LCM actions indicated by network.

RAN4 to define RRM requirements for LCM, prioritizing Activation, Deactivation, Switching, Fallback to non-AI operation, and Performance monitoring.

Until performance monitoring aspects are agreed in RAN1, RAN4 will wait to discuss the necessity of UE / PRU requirements for performance monitoring.

Case 2a/2b

RAN4 can discuss whether/how to define the requirements on LoS/NLoS.

- RAN4 to discuss whether to reuse the legacy measurement period and accuracy requirements and to define any additional measurement requirement in Case 2b.

Case 3a/3b

RAN4 to extend the applicability of existing report mappings also for UL SRS-RSRP to AI/ML based positioning in Cases 3a/3b.

RAN4 to establish a common understanding of the support for the various positioning techniques such as multi-RTT and discuss the RRM requirements of positioning techniques supported by other WGs.

- References

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#96, Budapest, Hungary, June 06 – June 09, 2022.

- RP-233132, Status report for SI on AI/ML for NR air interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15, 2023.

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15, 2023.

- R4-2406617, WF on AI/ML, Qualcomm, RAN4#110bis, Changsha, China, April 15 - 19, 2024.

- R4-2403712, WF on AI/ML, Qualcomm, RAN4#110, Athens, Greece, February 26 – March 1, 2024.

- R4-2410570, WF on NR ALML air interface, Qualcomm, RAN4#111, Fukuoka, Japan, May 20 – 24, 2024.

- R4-2404323, WF on NR ALML air interface, Qualcomm, RAN4#112, Maastricht, Netherlands, Aug. 19 – 23, 2024.

- R4-2417212, WF on NR ALML air interface, Qualcomm, RAN4#112bis, Hefei, China, Oct. 14 – 18, 2024.

- R1-2407797, Discussion on AI/ML-based positioning enhancement, ZTE, Hefei, China, Oct. 14 – 18, 2024.

- R4-2420336, WF on AI/ML air interface part 2, Ericsson, RAN4#113, Orlando, USA, Nov. 18 – 22, 2024.