3GPP TSG-RAN WG4 Meeting #114	R4-2502072

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	Testability and Interoperability Issues for AI/ML Mobility

Agenda item:	7.21.3

Document for:	Discussion

Introduction

In RAN4#114 meeting, following aspects are to be discussed during this meeting.

- General Aspects of AI/ML Mobility

- RRM Requirements of AI/ML Mobility

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution we provide our views and discuss different aspects of testability and interoperability issues in RAN4 for AI/ML Mobility.

Discussion

This document addresses the testability and interoperability issues related to AI/ML mobility SI in RAN4 based on the agreements and the recommended way forward of the issues from the RAN4#113 meeting.

Following topics are covered:

- Testing setup

- Prediction consistency in time domain

- Testing aspects of inter-carrier scenarios

- Testing of generalization aspects

Testing setup

Agreement from RAN4#113 meeting [1]:

- Agreement - RAN4 to further discuss: - Whether and how legacy testing setup for L3 measurements can be reused for FR1 and FR2

Considering measurement prediction use case within AI/ML mobility, the testing will involve either L1 or L3 measurements or both, either measured or predicted, on a preselected set of cells / beams (either intra or inter frequency). The considered trained ML model outputs the predicted measurements on a preselected time horizon. It is therefore necessary to map the measured values to the predicted value with the selected prediction horizon, accounting for the standardized filtering procedure when estimating the L3 measurements based on L1 or L3 measurements.

RAN4 should define test setups considering L1 or L3 measurements from serving and neighbouring cells with related configuration (inputs, prediction horizon).

RAN4 should consider the following additional aspects for AI/ML mobility use case testing compared to the legacy testing:

- L3 filtering

- channel model (CDL/ TDL etc.)

- number of AoAs may be different from BM use case due to higher number of cells and beams requirements in AI/ML mobility use case.

- maximum number of cells

- maximum number of Tx beams

RAN4 should consider the additional aspects for testing (compared to the legacy test) in AI/ML mobility such as L3 filtering, channel model, maximum number of cells, Tx beams, and the number of AoAs.

In the RAN4#112bis meeting, the following agreements are captured in the way forward [3].

Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Agreement: - Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed

In case of intra cell (i.e., scenarios 2 and 4) measurement and prediction considering sub-use case 2, the legacy test setup can be used as baseline. However, for the case where the measurement is performed over one cell and the predictions are required for multiple cells/clusters and for the inter-frequency inter-cell scenario (i.e., scenario 3), there is a need for the higher number of cells in the test setup. Hence, RAN4 should consider a further study on the maximum number of cells required for the test.

The legacy test setup may be considered as baseline for intra cell scenarios (i.e., scenarios 2 and 4 with sub-use case 2). RAN4 should consider higher number of cells for inter-frequency inter-cell scenario (i.e., scenario 3).

Channel conditions may vary frequently and randomly in real time environments. Test setup should be able to emulate the variations of the radio channel for evaluating the prediction accuracy of the models/functionalities. Hence, RAN4 should further study on identifying the suitable fading channel models such as CDL or TDL for a FR1 testing.

The test setup and the channel model, which are currently under study for AI/ML BM use case in the WI on AI/ML NR for air interface, should be considered as a baseline for the FR2 AIML mobility testing.

For FR1, RAN4 should emulate radio channel variations by considering fading channels such as CDL or TDL to test and evaluate the prediction accuracy of AIML models.

For FR2, the testing setup and channel model in the AI/ML BM use case which is under study should be reused as a baseline for measurement prediction use case testing.

To define the test cases for measurement prediction, RAN4 must consider the impact of L3 filtering and the number of beams utilized within test environments. Currently, a majority of legacy test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero). Additionally, to streamline test configurations, only a limited number of beams are typically simulated in the test setup.

Figure 1: 3GPP TS 38.300 Measurement Model

As illustrated in Figure 1, when L3 filtering is absent, the L1 and L3 beam measurement levels at point A1 align. Furthermore, for L3 cell-level RSRP predictions at Point C, the predicted values may correspond to the L1 and L3 beam-level predictions when a single beam is employed for prediction purposes. This approach may lead to inefficient or inaccurate test design and UE performance evaluation in predicting L1/L3/cell level measurement since the lack of filtering and limited beam emulation can result in an oversimplified model that fails to capture realistic multi-beam scenarios, thereby impacting the reliability of performance assessments.

- Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup.

- Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E.

RAN4 to consider L3 filtering and multi-beam scenarios while studying the test cases for AI/ML mobility uses cases.

Prediction consistency in time domain

Way forward from RAN4#113 meeting [2] is captured below.

Issue 3-1-2: Prediction consistency in time domain - Candidate Options: - Option 1: Model different time-varying characteristics per cell/site due to moving UE trajectories - Option 2: how to introduce controlled randomness and time-domain variation/correlation - Option 3: TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes - Option 4: Consider additional side conditions - Recommended WF - To discuss above listed candidate options.

For inter-cell inter-frequency scenario (i.e., scenario 3) in measurement prediction use case, network assistant information can be different between the BM use case of AI/ML NR for air interface and AI/ML mobility use case. In contrast to the BM use case, the AI/ML mobility use case may have the multiple target cells/sites in the same or different frequency layers.

The moving UE trajectory in the case of AI/ML mobility will cause the time-varying received power and SINR. RAN4 will need to study how to model the different time-varying or non-static characteristics per cell/site. It is also needed to study how to model the different variation per test iteration, Hence, Option 1 should be considered for further study.

Furthermore, the test setup should be able to emulate the randomness and variations of the radio channel for evaluating the prediction accuracy of the models or functionalities. Hence, Option 2 may also be considered for further study.

RAN4 should consider Options 1 and 2 to study how to model different non-static channel characteristics per cell/site due to moving UE trajectories and to emulate channel variations during the test.

Testing aspects of inter-carrier scenarios

Way forward from RAN4#113 meeting [2] is captured below.

Issue 3-2-4: Testing aspects of inter-carrier scenarios - Proposals and Observations: - Proposal 1 (OPPO): - For inter-carrier scenario (inter-frequency inter-cell prediction), how to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighbouring site should be considered. - For inter-carrier scenario (inter-frequency inter-cell prediction), how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship should be considered. - Stable time-domain RSRP signal for the tests might be needed, e.g., configure DL signals of BS emulator with stable power which do not change with time - For inter-carrier scenario (inter-frequency inter-cell prediction), to ensure the testability of measurements for inter-cell prediction, channel modelling for inter-cell prediction should be considered. - For FR1, considering the test environment is conducted testing, RAN4 should re-evaluate the selection of channel models. Option 1: UMA channel, Option 2: CDL channel. - Proposal 2 (Ericsson): - The aim of testing for inter-cell (inter-frequency) scenario should be to validate that the measurements reported by the UE: - meet the requirements for measured cell, and - meet the requirements for the target cell. - Proposal 3 (Nokia): - RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI. - Proposal 4 (CATT): - For inter-carrier scenario (inter-frequency inter-cell prediction), consider colocation scenario only and the characteristics of colocation can be reflected by a minor RSRP difference among carriers in FR1 test. - Recommended WF - Above listed proposals need to be further discussed.

In AI/ML Mobility, in order to train and infer, AI/ML functionalities need a dataset/environment that guarantees certain consistency between primary and secondary carrier components. This constraint is not present in the AI/ML BM use case and therefore is not considered in the scope of ongoing WI. Thus, in the scope of AI/ML mobility SI, RAN4 should consider a method to guarantee the consistency between different carriers.

RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI.

Testing of Generalization aspects

In the scope of AIML_NR_air WI, the concern regarding overfitting of UE models of a functionality to the test environment, was raised by multiple companies, and companies were encouraged to bring proposals to avoid overfitting issue.  In our view, testing generalization aspects of a functionality provide a rather realistic and feasible solution to the overfitting issue. A UE functionality, if tested in different generalization scenarios, would not be overfitted to a certain test environment, and, therefore, this is an important topic to be discussed. However, RAN2 has not progressed much on the generalization related discussion, and it may be too early to start the discussion on this topic in RAN4.

RAN4 should wait for further progress on generalization discussions in RAN2 before starting any discussion on generalization related topics in RAN4.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to testability and interoperability of AI/ML Mobility use cases. Specifically, we cover following aspects:

- Testing setup

- Prediction consistency in time domain

- Testing aspects of inter-carrier scenarios

- Testing of generalization aspects

In the paper, the following Observations and Proposals were made:

Proposal 1: RAN4 should define test setups considering L1 or L3 measurements from serving and neighbouring cells with related configuration (inputs, prediction horizon).

Observation 1: RAN4 should consider the additional aspects for testing (compared to the legacy test) in AI/ML mobility such as L3 filtering, channel model, maximum number of cells, Tx beams and the number of AoAs.

Proposal 2: The legacy test setup may be considered as baseline for intra cell scenarios (i.e. scenarios 2 and 4 with sub-use case 2). RAN4 should consider higher number of cells for inter-frequency inter-cell scenario (i.e. scenario 3).

- Proposal 3: For FR1, RAN4 should emulate radio channel variations by considering fading channels such as CDL or TDL to test and evaluate the prediction accuracy of AIML models.

Proposal 4: For FR2, the testing setup and channel model in AI/ML BM use case which is under study should be reused as a baseline for measurement prediction use case testing.

Observation 2: Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup.

Observation 3: Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E.

Proposal 5: RAN4 to consider L3 filtering and multi-beam scenarios while studying the test cases for AI/ML mobility uses cases.

Proposal 6: RAN4 should consider Options 1 and 2 to study how to model different non-static channel characteristics per cell/site due to moving UE trajectories and to emulate channel variations during the test.

Proposal 7: RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI.

Proposal 8: RAN4 should wait for further progress on generalization discussions in RAN2 before starting any discussion on generalization related topics in RAN4.

- References

- R4-2420095 - RAN4#113 WF on AI/ML Mobility, Nokia

- R4-2418278 - RAN4#113 Topic summary for FS_NR_AIML_Mob, Nokia

- R4-2416855 - RAN4#112bis WF on AI/ML Mobility, Nokia