3GPP TSG-RAN WG4 Meeting # 114 	R4-2502086

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item: 5.12.1

Source: MediaTek Inc.

Title: Discussion on remaining issues for MGE2

Document for: Discussion

# Discussion

In this contribution, we discuss the following issues [1]:

Issue 1-1-1: When the UE is configured with Concurrent gaps with NCSG, whether all deactivated SCells are measured within NCSG, regardless the gap association? - Further discuss in the next meeting. Issue 1-1-2: When the measurement of the measurement object is related to a deactivated SCell, and after this SCell is activated, and when UE is configured with the concurrent measurement gap and NCSG, what is the expected UE behaviour? - Further discuss in the next meeting

Issue 1-1-1:

The answer for the question is yes, i.e. when the UE is configured with concurrent gaps with NCSG, all deactivated SCells are measured within NCSG, regardless the gap association.

- when the UE is configured with concurrent gaps with NCSG, all deactivated SCells are measured within NCSG, regardless the gap association.

Issue 1-1-2:

Once the SCell is activated then the UE should follow the gap association. An example is that a SCell is activated and hence the UE follow the gap association, then the SCell is deactivated, and the UE is configured with concurrent gaps with NCSG, thus the UE measure the SCell with NCSG. After that, when that SCell is re-activated again then the UE should resume the first operation prior deactivation, i.e. UE measure within gap association.

- When the SCell is activated then the UE should follow the gap association to perform measurements for that SCell.

# Summary

In summary:

Proposal 1: when the UE is configured with concurrent gaps with NCSG, all deactivated SCells are measured within NCSG, regardless the gap association.

Proposal 2: When the SCell is activated then the UE should follow the gap association to perform measurements for that SCell.

# References

- R4-2420213, “WF on RRM requirements for NR_MG _enh2”, MediaTek Inc, Orlando, US, Dec 2024.