3GPP TSG-RAN4 Meeting #114	R4-2502091

# Athens, GR, 17th – 21st February, 2025

Source:	LG Electronics

Title:	HPUE for intra-band UL CA

Agenda item:	7.1.1.1.2

Document for:	Discussion

# Introduction

In this paper, we provide our views on MPR of HPUE for intra-band contiguous/non-contiguous UL CA.

# Discussion

In last RAN4 meeting, the followings was approved in WF[1]. This paper discusses the requirement of HPUE for intra-band UL CA.

### PC1.5 for intra-band C & NC UL CA 1.1 MPR values for intra-band contiguous ULCA (with TxD) - Class B would not be urgent. As common understanding, RAN4 will work on Class C in this Rel-19 WID - Companies are encouraged to provide the MPR values for both handheld UE and FWA in next meeting 1.2 PSD assumption to derive MPR/A-MPR requirements for intra-band NC ULCA (with dualPA) - Equal PSD as starting point - Companies are encouraged to check the MPR difference with 6dB PSD imbalance - Inform RAN5 about the PSD condition for MPR derivation in RAN4 - The MPR difference compared to PC2 is supposed to be less than 3dB for PC1.5 in order to have performance gain over PC2 - Companies to clarify the PA configuration assumptions for MPR evaluation for PC3, PC2 and PC1.5 intra-band NC UL CA, for example, (PC3+PC3) for PC3 1.3 MPR applicability for intra-band contiguous and non-contiguous ULCA - FFS for NC CA in next meeting - Stop the discussion for contiguous CA because equal PSD has already been agreed as assumption to derive MPR requirements 1.4 PCMAX,c and PCMAX for intra-band NCCA (with dual PA) - PCMAX,C is upper bounded by 26dBm - How to reflect it in spec is FFS - For PCMAX, FFS in next meeting - Option 1: No change of current spec - Option 2: PPowerClass,CA = min{29,26+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dBm assuming 6dB PSD difference , or, =26+10*log(1+min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2)) assuming equal PSD, can be used as a starting point. 1.5 Relative power tolerance - FFS in next meeting

### 2-band inter-band UL NR-CA/EN-DC with 2Tx and/or 3Tx 2.1 Configurations - To create and maintain a configuration table listing all configurations can be considered in Rel-19 in next meeting, for information purpose - FFS specific MSD calculation procedure details for the band combination including PC2 FDD as a constituent band (details in section 4.1) - No need to enter such combination into the WID 2.2 Re-organize 2Tx/3Tx Notes in clause 5 and clause 6 - Companies are encouraged to consider the necessity and possibility to re-organize the Notes in future meetings 2.3 Release independence of 3Tx - For FWA, Rel-17 - For handheld UE, FFS in next meeting ### Increasing UE transmission power 3.1 PCMAX tolerance - Modify PCMAX tolerance for inter-band NR-CA as following - Provide draft CR for TS 38.101-1 (a revision of R4-2417096 which was endorsed in RAN4#112bis) in Feb meeting to reflect above agreement ### General aspect 4.1 MSD rules - Moderator Note: It is covered by R4-2420380 (Skyworks, Murata, Nokia, Qualcomm) 4.2 Duty cycle solution for SAR compliance - Introduction of new duty-cycle IE is not considered - For ENDC (FDD+FDD), only reply on P-MPR, and there is no further spec changes needed - Following as starting point - For PC1.5 of intra-band contiguous and non-contiguous CA: for sub-clause 6.2A.1.1 and 6.2A.1.2 of 38.101-1, refer to R4-2419448 - For PC1.5 of inter-band CA: for sub-clause 6.2A.1.3 of 38.101-1, refer to R4-2419448 - For EN-DC (FDD+TDD) for sub-clause 6.2B.1.3 of 38.101-3 refer to R4-2418744 - For EN-DC(TDD+TDD) for sub-clause 6.2B.1.3 of 38.101-3 (similar as R4-2418744)

## MPR evaluation of PC1.5 UE for NR intra-band UL contiguous CA with 2Tx (2x26dBm+1LO)

For PC1.5 intra-band contiguous UL CA, RAN4 agreed to define MPR/A-MPR requirements based on equal PSD assumption considering architecture of 2x26dBm+1LO.

- Simulation assumption

  - Parameters

| Channel Bandwidth | per CC: 100MHz aggregated CBW: 200MHz = 100+100 MHz |
| --- | --- |
| Maximum output power for aggregated CBW | 29dBm |
| Numerology | 30kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | DFT-s-OFDM, CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 34dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |

  - Antenna isolation

- 10dB for handheld

- 20dB for FWA

  - Test scenarios for contiguous RB allocation: Table 2.1-1

Table 2.1-1: NR intra-band contiguous CA MPR test scenarios (Contiguous RB allocation)

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 100MHz + 100MHz | 1 | 52RB221 | 52RB0 | Inner | 30 |
|  | 2 | 52RB221 | 63RB0 | Inner | 30 |
|  | 3 | 52RB221 | 78RB0 | Inner | 30 |
|  | 4 | 52RB221 | 131RB0 | Inner | 30 |
|  | 5 | 52RB221 | 157RB0 | Inner | 30 |
|  | 6 | 52RB221 | 189RB0 | Outer | 30 |
|  | 7 | 52RB221 | 262RB0 | Outer | 30 |
|  | 8 | 63RB210 | 189RB0 | Outer | 30 |
|  | 9 | 131RB142 | 189RB0 | Outer | 30 |
|  | 10 | 210RB63 | 210RB0 | Outer | 30 |
|  | 11 | 262RB11 | 262RB0 | Outer | 30 |

Here, the aggregated CA bandwidth is CA bandwidth class C which is composed of 2 CCs (CC1 + CC2).

  - Test scenarios for non-contiguous RB allocation: Table 2.1-2

Table 2.1-2: NR intra-band contiguous CA MPR test scenarios (Non-Contiguous RB allocation)

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer1/Outer2 RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 100MHz + 100MHz | 1 | 52RB0 | 52RB220 | Outer2 | 30 |
|  | 2 | 52RB94 | 52RB115 | Outer2 | 30 |
|  | 3 | 52RB100 | 52RB115 | Outer1 | 30 |
|  | 4 | 52RB142 | 52RB74 | Outer1 | 30 |
|  | 5 | 52RB173 | 52RB37 | Outer1 | 30 |
|  | 6 | 52RB178 | 52RB37 | Inner | 30 |
|  | 7 | 52RB220 | 52RB5 | Inner | 30 |
|  | 8 | 131RB0 | 131RB142 | Outer2 | 30 |
|  | 9 | 131RB100 | 131RB42 | Outer2 | 30 |
|  | 10 | 131RB105 | 131RB42 | Outer1 | 30 |
|  | 11 | 131RB142 | 131RB5 | Outer1 | 30 |
|  | 12 | 158RB0 | 158RB116 | Outer2 | 30 |
|  | 13 | 158RB100 | 158RB16 | Outer2 | 30 |
|  | 14 | 158RB100 | 158RB10 | Outer1 | 30 |
|  | 15 | 158RB115 | 158RB5 | Outer1 | 30 |
|  | 16 | 262RB10 | 262RB10 | Outer2 | 30 |

Table 2.1-3, and Table 2.1-4 show the MPR simulation results for the NR intra-band contiguous CA scenarios of Contiguous RB allocations for handheld UE and FWA respectively.

Table 2.1-3: Handheld PC1.5 MPR simulation results for intra-band contiguous CA of Contiguous RB allocations with 2x26dBm+1LO (100MHz+100MHz)

|  | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s- | 'PI/2 BPSK' | 1.8 | 1.7 | 1.6 | 2.3 | 3.2 | 4.1 | 5.3 | 4.5 | 4.9 | 5.8 | 6.2 |
| OFDM | 'QPSK' | 1.4 | 1.4 | 1.4 | 1.9 | 2.9 | 4.1 | 4.8 | 4.1 | 4.1 | 4.4 | 4.8 |
|  | '16QAM' | 2.6 | 2.6 | 2.6 | 2.8 | 3.0 | 4.2 | 5.3 | 4.2 | 4.5 | 4.9 | 5.3 |
|  | '64QAM' | 3.6 | 3.6 | 3.6 | 3.9 | 3.9 | 4.2 | 5.3 | 4.2 | 4.5 | 4.9 | 5.3 |
|  | '256QAM' | 5.3 | 5.3 | 5.3 | 5.3 | 5.3 | 5.3 | 5.7 | 5.3 | 5.3 | 5.3 | 5.3 |
| CP-OFDM | 'QPSK' | 3.4 | 3.4 | 3.4 | 3.4 | 3.4 | 5.1 | 6.6 | 5.1 | 5.9 | 6.2 | 6.6 |
|  | '16QAM' | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 5.1 | 6.6 | 5.1 | 5.9 | 6.2 | 7.1 |
|  | '64QAM' | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 6.6 | 5.5 | 5.9 | 6.2 | 7.1 |
|  | '256QAM' | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |

Table 2.1-4: FWA PC1.5 MPR simulation results for intra-band contiguous CA of Contiguous RB allocations with 2x26dBm+1LO (100MHz+100MHz)

|  | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s- | 'PI/2 BPSK' | 1.4 | 1.3 | 1.1 | 1.5 | 2.5 | 3.1 | 4.1 | 3.4 | 4.2 | 4.6 | 5.0 |
| OFDM | 'QPSK' | 1.0 | 1.0 | 1.0 | 1.2 | 2.2 | 3.0 | 4.1 | 3.0 | 3.3 | 3.7 | 4.1 |
|  | '16QAM' | 1.8 | 1.8 | 1.8 | 1.8 | 2.3 | 3.1 | 4.2 | 3.1 | 3.4 | 3.8 | 4.1 |
|  | '64QAM' | 2.8 | 2.8 | 2.8 | 3.1 | 3.1 | 3.1 | 4.6 | 3.1 | 3.4 | 4.1 | 4.2 |
|  | '256QAM' | 4.2 | 4.6 | 4.6 | 4.6 | 4.6 | 4.6 | 4.6 | 4.6 | 4.5 | 4.5 | 4.6 |
| CP-OFDM | 'QPSK' | 2.7 | 2.6 | 2.6 | 2.6 | 2.6 | 4.1 | 5.5 | 4.1 | 4.8 | 5.1 | 5.9 |
|  | '16QAM' | 3.8 | 3.8 | 3.7 | 3.7 | 3.7 | 4.1 | 5.5 | 4.1 | 4.8 | 5.1 | 5.9 |
|  | '64QAM' | 4.8 | 4.8 | 4.8 | 4.7 | 4.8 | 4.8 | 5.5 | 4.8 | 4.8 | 5.1 | 5.9 |
|  | '256QAM' | 6.8 | 6.8 | 6.8 | 6.8 | 6.8 | 6.3 | 6.8 | 6.3 | 6.7 | 6.8 | 6.8 |

Table 2.1-5, and Table 2.1-6 show the MPR simulation results for the NR intra-band contiguous CA scenarios of Non-contiguous RB allocations for handheld UE and FWA respectively.

Table 2.1-5: Handheld PC1.5 MPR simulation results for intra-band contiguous CA of Non-contiguous RB allocations with 2x26dBm+1LO (100MHz+100MHz)

|  | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s- | 'PI/2 BPSK' | 13.7 | 14.2 | 9.8 | 5.8 | 5.8 | 5.7 | 2.4 | 2.4 | 1.8 | 7.9 | 5.7 | 5.4 | 4.0 | 7.5 | 5.4 | 5.4 | 5.0 | 6.2 |
| OFDM | 'QPSK' | 12.8 | 12.8 | 8.0 | 4.9 | 4.9 | 4.4 | 1.7 | 1.7 | 1.4 | 6.5 | 4.4 | 4.4 | 3.1 | 6.1 | 4.1 | 4.1 | 3.7 | 4.8 |
|  | '16QAM' | 12.8 | 13.3 | 8.4 | 5.3 | 5.3 | 4.8 | 2.2 | 2.2 | 2.4 | 7.0 | 4.9 | 4.9 | 3.2 | 6.6 | 4.5 | 4.5 | 4.2 | 5.3 |
|  | '64QAM' | 12.8 | 13.3 | 8.4 | 5.3 | 5.3 | 4.8 | 3.5 | 3.5 | 3.6 | 7.0 | 4.9 | 4.9 | 3.8 | 6.6 | 4.5 | 4.5 | 4.2 | 5.3 |
|  | '256QAM' | 12.8 | 13.3 | 8.4 | 5.3 | 5.3 | 4.8 | 4.8 | 4.8 | 5.3 | 7.0 | 5.3 | 5.3 | 5.3 | 6.6 | 5.3 | 5.3 | 5.3 | 5.3 |
| CP-OFDM | 'QPSK' | 13.1 | 13.6 | 10.3 | 6.7 | 6.7 | 6.6 | 3.1 | 3.1 | 3.4 | 8.4 | 6.2 | 6.2 | 3.9 | 7.9 | 5.9 | 5.9 | 5.5 | 6.6 |
|  | '16QAM' | 13.1 | 13.6 | 10.3 | 6.7 | 6.7 | 6.6 | 4.1 | 4.1 | 4.5 | 8.4 | 6.2 | 6.2 | 4.5 | 7.9 | 5.9 | 5.9 | 5.5 | 6.6 |
|  | '64QAM' | 13.2 | 13.6 | 10.3 | 6.7 | 6.7 | 6.6 | 5.4 | 5.4 | 5.5 | 8.4 | 6.2 | 6.2 | 5.5 | 7.9 | 5.9 | 5.9 | 5.5 | 7.1 |
|  | '256QAM' | 13.2 | 13.6 | 10.3 | 7.1 | 7.1 | 7.4 | 7.4 | 7.4 | 7.5 | 8.4 | 7.0 | 7.0 | 7.5 | 7.9 | 7.1 | 7.0 | 7.5 | 7.5 |

Table 2.1-6: FWA PC1.5 MPR simulation results for intra-band contiguous CA of Non-contiguous RB allocations with 2x26dBm+1LO (100MHz+100MHz)

|  | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s- | 'PI/2 BPSK' | 13.0 | 13.4 | 9.1 | 5.1 | 5.1 | 4.6 | 1.5 | 1.5 | 1.4 | 7.2 | 4.6 | 4.6 | 3.0 | 6.3 | 4.3 | 4.3 | 3.9 | 5.0 |
| OFDM | 'QPSK' | 12.1 | 12.1 | 7.3 | 4.1 | 4.1 | 3.7 | 1.0 | 1.0 | 1.0 | 5.8 | 3.3 | 3.3 | 2.4 | 5.4 | 3.3 | 3.3 | 3.0 | 3.7 |
|  | '16QAM' | 12.1 | 12.1 | 7.7 | 4.6 | 4.6 | 4.1 | 1.4 | 1.4 | 1.6 | 6.3 | 3.8 | 3.8 | 2.5 | 5.8 | 3.4 | 3.4 | 3.1 | 4.1 |
|  | '64QAM' | 12.1 | 12.5 | 7.7 | 4.6 | 4.6 | 4.1 | 2.5 | 2.5 | 2.8 | 6.3 | 3.8 | 3.8 | 2.8 | 5.8 | 3.8 | 3.4 | 3.1 | 4.2 |
|  | '256QAM' | 12.0 | 12.5 | 7.7 | 4.6 | 4.6 | 4.1 | 4.1 | 4.1 | 4.2 | 6.3 | 4.1 | 4.5 | 4.5 | 5.8 | 4.5 | 4.5 | 4.5 | 4.5 |
| CP-OFDM | 'QPSK' | 12.4 | 12.9 | 9.5 | 6.0 | 6.0 | 5.5 | 2.4 | 2.4 | 2.7 | 7.7 | 5.1 | 5.1 | 2.9 | 7.2 | 4.8 | 4.8 | 4.4 | 5.9 |
|  | '16QAM' | 12.5 | 12.9 | 9.6 | 6.0 | 6.0 | 5.5 | 3.4 | 3.4 | 3.8 | 7.7 | 5.1 | 5.1 | 3.4 | 7.2 | 4.8 | 4.8 | 4.4 | 5.9 |
|  | '64QAM' | 12.5 | 12.9 | 9.5 | 6.0 | 6.0 | 5.5 | 4.7 | 4.7 | 4.8 | 7.7 | 5.1 | 5.1 | 4.7 | 7.2 | 4.7 | 4.8 | 4.7 | 5.9 |
|  | '256QAM' | 12.4 | 12.9 | 9.5 | 6.4 | 6.4 | 6.7 | 6.3 | 6.7 | 6.8 | 7.7 | 6.3 | 6.3 | 6.7 | 7.2 | 6.3 | 6.3 | 6.8 | 6.8 |

Table 2.1-7 shows the maximum value of simulation results for intra-band contiguous CA of Contiguous RB allocations for handheld and FWA respectively.

Table 2.1-8 shows the maximum value of simulation results for intra-band contiguous CA of Non-contiguous RB allocations for handheld and FWA respectively.

Table 2.1-7 : PC1.5 MPR simulation results for intra-band contiguous CA of Contiguous RB allocations with 2x26dBm+1LO

| Modulation |  | MPR for bandwidth class C(dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | FWA |  |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 3.2 | 6.2 | 2.5 | 5.0 |
|  | QPSK | 2.9 | 4.8 | 2.2 | 4.1 |
|  | 16QAM | 3.0 | 5.3 | 2.3 | 4.2 |
|  | 64QAM | 3.9 | 5.3 | 3.1 | 4.6 |
|  | 256QAM | 5.3 | 5.7 | 4.6 | 4.6 |
| CP-OFDM | QPSK | 3.4 | 6.6 | 2.7 | 5.9 |
|  | 16QAM | 4.5 | 7.1 | 3.8 | 5.9 |
|  | 64QAM | 5.5 | 7.1 | 4.8 | 5.9 |
|  | 256QAM | 7.5 | 7.5 | 6.8 | 6.8 |

Table 2.1-8 : PC1.5 MPR simulation results for intra-band contiguous CA of Non-contiguous RB allocations with 2x26dBm+1LO

| Modulation |  | MPR for bandwidth class C(dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | FWA |  |  |
|  |  | inner | Outer1 | Outer2 | inner | Outer1 | Outer2 |
| DFT-s-OFDM | PI/2 BPSK | 2.4 | 5.8 | 14.2 | 1.5 | 5.1 | 13.4 |
|  | QPSK | 1.7 | 4.9 | 12.8 | 1.0 | 4.1 | 12.1 |
|  | 16QAM | 2.4 | 5.3 | 13.3 | 1.6 | 4.6 | 12.1 |
|  | 64QAM | 3.6 | 5.3 | 13.3 | 2.8 | 4.6 | 12.5 |
|  | 256QAM | 5.3 | 5.3 | 13.3 | 4.2 | 4.6 | 12.5 |
| CP-OFDM | QPSK | 3.4 | 6.7 | 13.6 | 2.7 | 6.0 | 12.9 |
|  | 16QAM | 4.5 | 6.7 | 13.6 | 3.8 | 6.0 | 12.9 |
|  | 64QAM | 5.5 | 6.7 | 13.6 | 4.8 | 6.0 | 12.9 |
|  | 256QAM | 7.5 | 7.5 | 13.6 | 6.8 | 6.8 | 12.9 |

In [2], the results of analysis are collected into the table below, where column MA,delta shows the additional MPR that is needed vs. the reference, which is here PC2 (23+23=26dBm) MPR values that are already defined in the 38.101-1. Total MPR would then be MA,total= MA + MA,delta..

|  | C1 | C2 | C1+C2 | MA,delta |
| --- | --- | --- | --- | --- |
| 23+23= 26dBm | 23 | 23 | 26.0 | 0 (Reference) |
| 23+26= 27.8dBm | 23 | 26 | 27.8 | 0.6 |
| 26+26= 29dBm | 26 | 26 | 29.0 | 1.0 |
| 26+31= 32.2dBm | 26 | 31 | 32.2 | 0.8 |
| 31+31= 34dBm | 31 | 31 | 34.0 | 0.5 |
| 20+20= 23dBm | 20 | 20 | 23.0 | 0.0 |
| 20+23= 24.8dBm | 20 | 23 | 24.8 | 0.0 |

For PC1.5 intra-band contiguous CA targeting to handheld, MPR are proposed as Table 2.1-9, and Table 2.1-10 considering the analysis (MA,delta.. = 1dB) with reference of PC2 MPR and implementation margin on top of the simulation results.

For PC1.5 intra-band contiguous CA targeting to FWA, MPR are proposed as Table 2.1-9, and Table 2.1-10 considering implementation margin on top of the simulation results.

Table 2.1-9 : PC1.5 MPR for intra-band Contiguous CA of Contiguous RB allocations with 2x26dBm+1LO

| Modulation |  | MPR for bandwidth class C(dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | FWA |  |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | ≤ 4.5 | ≤ 9.0 | ≤ 4.0 | ≤ 8.5 |
|  | QPSK | ≤ 4.5 | ≤ 9.0 | ≤ 4.0 | ≤ 8.5 |
|  | 16QAM | ≤ 4.5 | ≤ 9.0 | ≤ 4.0 | ≤ 8.5 |
|  | 64QAM | ≤ 7.0 | ≤ 9.0 | ≤ 6.5 | ≤ 8.5 |
|  | 256QAM | ≤ 9.0 | ≤ 9.5 | ≤ 8.0 | ≤ 8.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 9.5 | ≤ 4.5 | ≤ 9.0 |
|  | 16QAM | ≤ 5.0 | ≤ 9.5 | ≤ 4.5 | ≤ 9.0 |
|  | 64QAM | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 9.0 |
|  | 256QAM | ≤ 8.5 | ≤ 9.5 | ≤ 7.5 | ≤ 9.0 |

Table 2.1-10 : PC1.5 MPR for intra-band Contiguous CA of Non-contiguous RB allocations with 2x26dBm+1LO

| Modulation |  | MPR for bandwidth class C(dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | FWA |  |  |
|  |  | inner | Outer1 | Outer2 | inner | Outer1 | Outer2 |
| DFT-s-OFDM | PI/2 BPSK | ≤ 5.0 | ≤ 9.5 | ≤ 15.5 | ≤ 4.5 | ≤ 9.0 | ≤ 15.0 |
|  | QPSK | ≤ 5.0 | ≤ 9.5 | ≤ 15.5 | ≤ 4.5 | ≤ 9.0 | ≤ 15.0 |
|  | 16QAM | ≤ 5.0 | ≤ 9.5 | ≤ 15.5 | ≤ 4.5 | ≤ 9.0 | ≤ 15.0 |
|  | 64QAM | ≤ 6.0 | ≤ 9.5 | ≤ 15.5 | ≤ 5.5 | ≤ 9.0 | ≤ 15.0 |
|  | 256QAM | ≤ 8.5 | ≤ 9.5 | ≤ 15.5 | ≤ 7.5 | ≤ 9.0 | ≤ 15.0 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 10.0 | ≤ 16.5 | ≤ 4.5 | ≤ 9.5 | ≤ 16.0 |
|  | 16QAM | ≤ 5.5 | ≤ 10.0 | ≤ 16.5 | ≤ 4.5 | ≤ 9.5 | ≤ 16.0 |
|  | 64QAM | ≤ 7.0 | ≤ 10.0 | ≤ 16.5 | ≤ 6.0 | ≤ 9.5 | ≤ 16.0 |
|  | 256QAM | ≤ 9.0 | ≤ 10.0 | ≤ 16.5 | ≤ 8.0 | ≤ 9.5 | ≤ 16.0 |

#### Proposal 1: For PC1.5 intra-band contiguous UL CA, define separate MPR requirements for handheld and FWA

#### Proposal 2: For PC1.5 intra-band contiguous UL CA, consider MPR in Table 2.1-9 for contiguous RB allocation, MPR in Table 2.1-10 for non-contiguous RB allocation.

#### Proposal 3: For PC1.5 intra-band contiguous UL CA with targeting handheld, consider

MA,total= MA + MA,delta, where

- For PC1.5 the reference MPR (MA) is PC2 bandwidth class C Contiguous RB-allocation for UE indicating TxD supported.

- MA,delta = 1dB.

## MPR evaluation of PC1.5 UE for NR intra-band UL non-contiguous CA with dualPA architecture (2x26dBm+2LO)

For PC1.5 intra-band non-contiguous UL CA, RAN4 agreed to focus on dualPA architecture with 2 LOs and whether to define the requirements based on equal PSD or PSD imbalance with 6dB.

- Simulation assumption

  - Parameters

| Bandwidth | per CC: 20MHz |
| --- | --- |
| Aggregated CBW | 20MHz+G20MHz+20MHz |
| Gap | 20MHz |
| Maximum output power for aggregated CBW | 29dBm |
| Numerology | 15kHz |
| Modulation per CC | 256QAM |
| Waveform | DFT-s-OFDM, CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 34dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |

  - Antenna isolation

- 10dB for handheld

- 20dB for FWA

  - PSD between each CC

- EqualPSD

- Up to 6dB different PSD

- Assume PSD of lower Lcrb > PSD of higher Lcrb

  - Test scenarios :Table 2.2-1

Table 2.2-1: NR intra-band non-contiguous CA MPR test scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | B | Frequency of FIM3,low_block,low & FIM3,high_block,high | SCS1 (= SCS2) |
| --- | --- | --- | --- | --- | --- | --- |
| 20MHz + Gap20MHz+20MHz | 1 | 1RB0 | 1RB105 | B=0.36 | SEfreq_-30 | 15 |
|  | 2 | 1RB67 | 1RB38 | B=0.36 | SEMfreq_-13 | 15 |
|  | 3 | 1RB0 | 4RB102 | B=0.9 | SEfreq_-30 | 15 |
|  | 4 | 1RB67 | 4RB35 | B=0.9 | SEMfreq_-13 | 15 |
|  | 5 | 2RB0 | 8RB98 | B=1.8 | SEfreq_-30 | 15 |
|  | 6 | 2RB67 | 8RB31 | B=1.8 | SEMfreq_-13 | 15 |
|  | 7 | 5RB0 | 10RB96 | B=2.7 | SEfreq_-30 | 15 |
|  | 8 | 5RB67 | 10RB32 | B=2.7 | SEMfreq_-13 | 15 |
|  | 9 | 5RB0 | 15RB91 | B=3.6 | SEfreq_-30 | 15 |
|  | 10 | 5RB67 | 15RB27 | B=3.6 | SEMfreq_-13 | 15 |
|  | 11 | 5RB0 | 20RB86 | B=4.5 | SEfreq_-30 | 15 |
|  | 12 | 5RB64 | 20RB22 | B=4.5 | SEMfreq_-13 | 15 |
|  | 13 | 10RB0 | 30RB76 | B=7.2 | SEfreq_-30 | 15 |
|  | 14 | 10RB61 | 30RB14 | B=7.2 | SEMfreq_-13 | 15 |
|  | 15 | 20RB0 | 50RB56 | B=12.6 | SEfreq_-30 | 15 |
|  | 16 | 20RB61 | 50RB12 | B=12.6 | SEMfreq_-13 | 15 |
|  | 17 | 20RB0 | 60RB46 | B=14.4 | SEfreq_-30 | 15 |
|  | 18 | 20RB61 | 60RB12 | B=14.4 | SEMfreq_-13 | 15 |
|  | 19 | 20RB0 | 80RB26 | B=18.0 | SEfreq_-30 | 15 |
|  | 20 | 20RB61 | 80RB12 | B=18.0 | SEfreq_-30 | 15 |
|  | 21 | 30RB0 | 90RB16 | B=21.6 | SEfreq_-30 | 15 |
|  | 22 | 30RB61 | 90RB12 | B=21.6 | SEfreq_-30 | 15 |
|  | 23 | 40RB0 | 100RB6 | B=25.2 | SEfreq_-30 | 15 |
|  | 24 | 40RB61 | 100RB12 | B=25.2 | SEfreq_-30 | 15 |

Table 2.2-2 shows PSD difference of Scenarios in Table 2.2-1.

Table 2.2-2: PSD difference of Scenarios in Table 2.2-1

| Scenario# in Table 3-1 | LCRB1 (cc1) | LCRB2 (cc2) | PSD difference [dB] |  |
| --- | --- | --- | --- | --- |
|  |  |  | Equal PSD | Different PSD |
| 1, 2 | 1 | 1 | 0 | 0 |
| 3, 4 | 1 | 4 | 0 | 6 |
| 5, 6 | 2 | 8 | 0 | 6 |
| 7, 8 | 5 | 10 | 0 | 3 |
| 9, 10 | 5 | 15 | 0 | 4.77 |
| 11, 12 | 5 | 20 | 0 | 6 |
| 13, 14 | 10 | 30 | 0 | 4.77 |
| 15, 16 | 20 | 50 | 0 | 3.98 |
| 17, 18 | 20 | 60 | 0 | 4.77 |
| 19, 20 | 20 | 80 | 0 | 6 |
| 21, 22 | 30 | 90 | 0 | 4.77 |
| 23, 24 | 40 | 100 | 0 | 3.98 |

Table 2.2-3 shows the MPR simulation results for the NR intra-band non-contiguous CA for handheld UE and FWA with Equal PSD.

Table 2.2-4 shows the MPR simulation results for the NR intra-band non-contiguous CA for handheld UE and FWA with PSD of up to 6dB difference.

Table 2.2-3: Equal PSD based PC1.5 MPR simulation results for intra-band non-contiguous CA with 2x26dBm+2LO (20MHz+G20MHz+20MHz)

| Handheld | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM |  | 13.2 | 7.8 | 12.9 | 8.9 | 12.8 | 6.5 | 12.9 | 3.9 | 12.0 | 3.6 | 10.8 | 3.9 | 10.5 | 3.2 | 9.2 | 3.9 | 7.5 | 5.5 |
| CP-OFDM |  | 12.7 | 8.1 | 12.8 | 7.8 | 12.2 | 6.1 | 12.3 | 5.7 | 10.9 | 5.8 | 9.8 | 6.6 | 9.4 | 5.9 | 7.2 | 5.7 | 6.9 | 6.4 |
|  | #19 | #20 | #21 | #22 | #23 | #24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DFT-s-OFDM |  | 5.7 | 5.7 | 5.5 | 5.5 | 5.3 | 5.3 |  |  |  |  |  |  |  |  |  |  |  |  |
| CP-OFDM |  | 6.6 | 6.6 | 6.4 | 6.4 | 6.2 | 6.3 |  |  |  |  |  |  |  |  |  |  |  |  |
| FWA | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #18 |
| DFT-s-OFDM |  | 4.8 | 2.7 | 5.0 | 3.9 | 5.4 | 3.9 | 3.6 | 3.1 | 3.7 | 3.6 | 4.0 | 3.9 | 3.3 | 3.2 | 3.0 | 3.9 | 4.1 | 3.7 |
| CP-OFDM |  | 5.1 | 3.6 | 6.3 | 6.1 | 6.3 | 6.2 | 5.5 | 5.8 | 6.4 | 6.3 | 6.7 | 6.6 | 6.0 | 5.9 | 5.7 | 5.7 | 5.9 | 5.9 |
|  | #19 | #20 | #21 | #22 | #23 | #24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DFT-s-OFDM |  | 4.0 | 3.9 | 3.7 | 3.7 | 3.5 | 4.0 |  |  |  |  |  |  |  |  |  |  |  |  |
| CP-OFDM |  | 6.2 | 6.2 | 5.9 | 5.9 | 5.7 | 5.4 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.2-4: PSD of up to 6dB difference based PC1.5 MPR simulation results for intra-band non-contiguous CA with 2x26dBm+2LO (20MHz+G20MHz+20MHz)

| Handheld | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM |  | 13.3 | 7.8 | 12.8 | 5.4 | 13.1 | 4.0 | 12.6 | 3.6 | 11.2 | 3.1 | 9.8 | 3.1 | 9.2 | 2.7 | 6.2 | 2.7 | 5.2 | 4.7 |
| CP-OFDM |  | 12.7 | 8.1 | 12.2 | 6.7 | 12.5 | 5.8 | 12.0 | 5.4 | 10.1 | 4.5 | 9.2 | 4.5 | 8.6 | 4.5 | 7.5 | 4.6 | 7.0 | 6.5 |
|  | #19 | #20 | #21 | #22 | #23 | #24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DFT-s-OFDM |  | 5.1 | 5.1 | 4.6 | 4.6 | 4.6 | 4.6 |  |  |  |  |  |  |  |  |  |  |  |  |
| CP-OFDM |  | 6.9 | 6.4 | 6.5 | 6.4 | 6.4 | 6.0 |  |  |  |  |  |  |  |  |  |  |  |  |
| FWA | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #18 |
| DFT-s-OFDM |  | 4.8 | 2.7 | 4.8 | 2.2 | 3.8 | 1.8 | 3.8 | 1.8 | 3.3 | 1.9 | 2.8 | 1.9 | 3.3 | 1.5 | 2.8 | 2.4 | 2.8 | 2.8 |
| CP-OFDM |  | 5.1 | 4.1 | 5.2 | 4.0 | 5.1 | 4.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.7 | 4.6 | 4.2 | 4.2 | 4.6 | 4.6 | 4.2 | 4.6 |
|  | #19 | #20 | #21 | #22 | #23 | #24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DFT-s-OFDM |  | 2.8 | 2.7 | 2.8 | 2.8 | 2.3 | 2.8 |  |  |  |  |  |  |  |  |  |  |  |  |
| CP-OFDM |  | 4.6 | 4.6 | 4.2 | 4.1 | 4.1 | 4.2 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.2-5 shows the maximum value of simulation results for DFT-s-OFDM and CP-OFDM to meet -13dBm/MHz for handheld & FWA.

Table 2.2-6 shows the maximum value of simulation results for DFT-s-OFDM and CP-OFDM to meet -30dBm/MHz for handheld & FWA.

Table 2.2-5 : PC1.5 MPR simulation results for intra-band non-contiguous CA with 2x26dBm PA + 2LO (-13dBm/MHz)

| B | MPR (dB) for IM3 frequency (-13dBm/MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Handheld |  | FWA |  |
|  | EqaulPSD | Diff.PSD | EqaulPSD | Diff.PSD |
| 0 ≤ B <0.54 | 8.1 | 8.1 | 3.6 | 4.1 |
| 0.54 ≤ B < 1.08 | 8.9 | 6.7 | 6.1 | 4.0 |
| 1.08 ≤ B < 2.16 | 6.5 | 5.8 | 6.2 | 4.1 |
| 2.16 ≤ B < 3.24 | 5.7 | 5.4 | 5.8 | 4.5 |
| 3.24 ≤ B < 5.4 | 6.6 | 4.5 | 6.6 | 4.6 |
| 5.4 ≤ B ≤ 10.8 | 5.9 | 4.5 | 5.9 | 4.2 |
| 10.8 < B | 5.7 | 4.6 | 5.7 | 4.6 |

Table 2.2-6 : PC1.5 MPR simulation results for intra-band non-contiguous CA with 2x26dBm PA + 2LO (-30dBm/MHz)

| B | MPR (dB) for IM3 frequency (-30dBm/MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Handheld |  | FWA |  |
|  | EqaulPSD | Diff.PSD | EqaulPSD | Diff.PSD |
| 0 ≤ B < 1.44 | 13.2 | 13.3 | 6.3 | 5.2 |
| 1.44 ≤ B < 2.88 | 12.9 | 13.1 | 6.3 | 5.1 |
| 2.88 ≤ B < 5.76 | 12.0 | 11.2 | 6.7 | 4.7 |
| 5.76 ≤ B < 10.8 | 10.5 | 9.2 | 6.0 | 4.2 |
| 10.8 ≤ B < 23.04 | 9.2 | 7.5 | 6.2 | 4.6 |
| 23.04 ≤ B | 6.2 | 6.4 | 5.7 | 4.1 |

For PC1.5 intra-band non-contiguous CA, MPR can be proposed as Table 2.2-7, and Table 2.2-8 based on the simulation results when considering implementation margin.

Table 2.2-7 : PC1.5 MPR to meet -13dBm/MHz for intra-band non-contiguous CA with 2x26dBm+2LO(-13dBm/MHz)

| B | MPR (dB) for IM3 frequency (-13dBm/MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Handheld |  | FWA |  |
|  | EqaulPSD | Diff.PSD | EqaulPSD | Diff.PSD |
| 0 ≤ B <0.54 | ≤ 10.5 | ≤ 10.5 | ≤ 8.5 | ≤ 6.5 |
| 0.54 ≤ B < 1.08 | ≤ 10.0 | ≤ 9.5 | ≤ 8.0 | ≤ 6.5 |
| 1.08 ≤ B < 2.16 | ≤ 9.5 | ≤ 9.0 | ≤ 8.0 | ≤ 6.5 |
| 2.16 ≤ B < 3.24 | ≤ 8.5 | ≤ 8.0 | ≤ 7.5 | ≤ 6.5 |
| 3.24 ≤ B < 5.4 | ≤ 8.0 | ≤ 7.5 | ≤ 7.5 | ≤ 6.5 |
| 5.4 ≤ B ≤ 10.8 | ≤ 7.5 | ≤ 7.0 | ≤ 7.0 | ≤ 6.5 |
| 10.8 < B | ≤ 7.0 | ≤ 6.5 | ≤ 7.0 | ≤ 6.5 |

Table 2.2-8 : PC1.5 MPR to meet -30dBm/MHz for intra-band non-contiguous CA with 2x26dBm+2LO(-30dBm/MHz)

| B | MPR (dB) for IM3 frequency (-30dBm/MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Handheld |  | FWA |  |
|  | EqaulPSD | Diff.PSD | EqaulPSD | Diff.PSD |
| 0 ≤ B < 1.44 | ≤ 16.0 | ≤ 16.0 | ≤ 9.0 | ≤ 8.0 |
| 1.44 ≤ B < 2.88 | ≤ 15.5 | ≤ 14.5 | ≤ 9.0 | ≤ 8.0 |
| 2.88 ≤ B < 5.76 | ≤ 14.5 | ≤ 12.0 | ≤ 9.0 | ≤ 8.0 |
| 5.76 ≤ B < 10.8 | ≤ 12.5 | ≤ 10.5 | ≤ 9.0 | ≤ 8.0 |
| 10.8 ≤ B < 23.04 | ≤ 11.0 | ≤ 11.0 | ≤ 9.0 | ≤ 8.0 |
| 23.04 ≤ B | ≤ 10.0 | ≤ 10.0 | ≤ 9.0 | ≤ 8.0 |

MPR requirements needs to be specified as Table 2.2-9 and Table 2.2-10 based on EqualPSD because of showing larger MPR than NonEqualPSD.

Table 2.2-9 : PC1.5 MPR to meet -13dBm/MHz for intra-band non-contiguous CA with 2x26dBm+2LO(-13dBm/MHz)

| B | MPR (dB) for IM3 frequency (-13dBm/MHz) |  |
| --- | --- | --- |
|  | Handheld | FWA |
| 0 ≤ B <0.54 | ≤ 10.5 | ≤ 8.5 |
| 0.54 ≤ B < 1.08 | ≤ 10.0 | ≤ 8.0 |
| 1.08 ≤ B < 2.16 | ≤ 9.5 | ≤ 8.0 |
| 2.16 ≤ B < 3.24 | ≤ 8.5 | ≤ 7.5 |
| 3.24 ≤ B < 5.4 | ≤ 8.0 | ≤ 7.5 |
| 5.4 ≤ B ≤ 10.8 | ≤ 7.5 | ≤ 7.0 |
| 10.8 < B | ≤ 7.0 | ≤ 7.0 |

Table 2.2-10 : PC1.5 MPR to meet -30dBm/MHz for intra-band non-contiguous CA with 2x26dBm+2LO(-30dBm/MHz)

| B | MPR (dB) for IM3 frequency (-30dBm/MHz) |  |
| --- | --- | --- |
|  | Handheld | FWA |
| 0 ≤ B < 1.44 | ≤ 16.0 | ≤ 9.0 |
| 1.44 ≤ B < 2.88 | ≤ 15.5 | ≤ 9.0 |
| 2.88 ≤ B < 5.76 | ≤ 14.5 | ≤ 9.0 |
| 5.76 ≤ B < 10.8 | ≤ 12.5 | ≤ 9.0 |
| 10.8 ≤ B < 23.04 | ≤ 11.0 | ≤ 9.0 |
| 23.04 ≤ B | ≤ 10.0 | ≤ 9.0 |

#### Proposal 4: For PC1.5 intra-band non-contiguous UL CA, define separate MPR requirements for handheld and FWA.

#### Proposal 5: For PC1.5 intra-band non-contiguous UL CA, consider MPR in Table 2.2-9 and Table 2.2-10 to meet -13dBm/MHz and -30dBm/MHz respectively.

## PSD imbalance for NR intra-band non-contiguous CA with dualPA architecture

The condition of non-equal PSD (up to 6dB PSD imbalance) which is meaningful in real field needs to be specified.

- 6 dB ≤ { PCMAX,c1 – 10*log10(LCRB1*SCS1)} –  { PCMAX,c2 – 10*log10(LCRB2*SCS2)} ≤ 6 dB

Here, the upper bound of Pcmax,c is 26dBm per CC.

#### Proposal 6: For PC1.5 intra-band non-contiguous UL CA, consider up to 6dB PSD imbalance in configured transmitted power.

- 6 dB ≤ { PCMAX,c1 – 10*log10(LCRB1*SCS1)} –  { PCMAX,c2 – 10*log10(LCRB2*SCS2)} ≤ 6 dB

The upper bound of  PCMAX,CC1 and PCMAX,CC2 is 26dBm.

# Conclusion

In this contribution, we proposed our view on MPR of HPUE for intra-band UL CA. The proposals are as follows.

#### Proposal 1: For PC1.5 intra-band contiguous UL CA, define separate MPR requirements for handheld and FWA

#### Proposal 2: For PC1.5 intra-band contiguous UL CA, consider MPR in Table 2.1-9 for contiguous RB allocation, MPR in Table 2.1-10 for non-contiguous RB allocation.

Table 2.1-9 : PC1.5 MPR for intra-band Contiguous CA of Contiguous RB allocations with 2x26dBm+1LO

| Modulation |  | MPR for bandwidth class C(dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | FWA |  |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | ≤ 4.5 | ≤ 9.0 | ≤ 4.0 | ≤ 8.5 |
|  | QPSK | ≤ 4.5 | ≤ 9.0 | ≤ 4.0 | ≤ 8.5 |
|  | 16QAM | ≤ 4.5 | ≤ 9.0 | ≤ 4.0 | ≤ 8.5 |
|  | 64QAM | ≤ 7.0 | ≤ 9.0 | ≤ 6.5 | ≤ 8.5 |
|  | 256QAM | ≤ 9.0 | ≤ 9.5 | ≤ 8.0 | ≤ 8.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 9.5 | ≤ 4.5 | ≤ 9.0 |
|  | 16QAM | ≤ 5.0 | ≤ 9.5 | ≤ 4.5 | ≤ 9.0 |
|  | 64QAM | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 9.0 |
|  | 256QAM | ≤ 8.5 | ≤ 9.5 | ≤ 7.5 | ≤ 9.0 |

Table 2.1-10 : PC1.5 MPR for intra-band Contiguous CA of Non-contiguous RB allocations with 2x26dBm+1LO

| Modulation |  | MPR for bandwidth class C(dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | FWA |  |  |
|  |  | inner | Outer1 | Outer2 | inner | Outer1 | Outer2 |
| DFT-s-OFDM | PI/2 BPSK | ≤ 5.0 | ≤ 9.5 | ≤ 15.5 | ≤ 4.5 | ≤ 9.0 | ≤ 15.0 |
|  | QPSK | ≤ 5.0 | ≤ 9.5 | ≤ 15.5 | ≤ 4.5 | ≤ 9.0 | ≤ 15.0 |
|  | 16QAM | ≤ 5.0 | ≤ 9.5 | ≤ 15.5 | ≤ 4.5 | ≤ 9.0 | ≤ 15.0 |
|  | 64QAM | ≤ 7.0 | ≤ 9.5 | ≤ 15.5 | ≤ 6.5 | ≤ 9.0 | ≤ 15.0 |
|  | 256QAM | ≤ 8.5 | ≤ 9.5 | ≤ 15.5 | ≤ 7.5 | ≤ 9.0 | ≤ 15.0 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 10.0 | ≤ 16.5 | ≤ 4.5 | ≤ 9.5 | ≤ 16.0 |
|  | 16QAM | ≤ 5.5 | ≤ 10.0 | ≤ 16.5 | ≤ 4.5 | ≤ 9.5 | ≤ 16.0 |
|  | 64QAM | ≤ 7.0 | ≤ 10.0 | ≤ 16.5 | ≤ 6.0 | ≤ 9.5 | ≤ 16.0 |
|  | 256QAM | ≤ 9.0 | ≤ 10.0 | ≤ 16.5 | ≤ 8.0 | ≤ 9.5 | ≤ 16.0 |

#### Proposal 3: For PC1.5 intra-band contiguous UL CA with targeting handheld, consider

MA,total= MA + MA,delta, where

- For PC1.5 the reference MPR (MA) is PC2 bandwidth class C Contiguous RB-allocation for UE indicating TxD supported.

- MA,delta = 1dB.

#### Proposal 4: For PC1.5 intra-band non-contiguous UL CA, define separate MPR requirements for handheld and FWA.

#### Proposal 5: For PC1.5 intra-band non-contiguous UL CA, consider MPR in Table 2.2-9 and Table 2.2-10 to meet -13dBm/MHz and -30dBm/MHz respectively.

Table 2.2-9 : PC1.5 MPR to meet -13dBm/MHz for intra-band non-contiguous CA with 2x26dBm+2LO(-13dBm/MHz)

| B | MPR (dB) for IM3 frequency (-13dBm/MHz) |  |
| --- | --- | --- |
|  | Handheld | FWA |
| 0 ≤ B <0.54 | ≤ 10.5 | ≤ 8.5 |
| 0.54 ≤ B < 1.08 | ≤ 10.0 | ≤ 8.0 |
| 1.08 ≤ B < 2.16 | ≤ 9.5 | ≤ 8.0 |
| 2.16 ≤ B < 3.24 | ≤ 8.5 | ≤ 7.5 |
| 3.24 ≤ B < 5.4 | ≤ 8.0 | ≤ 7.5 |
| 5.4 ≤ B ≤ 10.8 | ≤ 7.5 | ≤ 7.0 |
| 10.8 < B | ≤ 7.0 | ≤ 7.0 |

Table 2.2-10 : PC1.5 MPR to meet -30dBm/MHz for intra-band non-contiguous CA with 2x26dBm+2LO(-30dBm/MHz)

| B | MPR (dB) for IM3 frequency (-30dBm/MHz) |  |
| --- | --- | --- |
|  | Handheld | FWA |
| 0 ≤ B < 1.44 | ≤ 16.0 | ≤ 9.0 |
| 1.44 ≤ B < 2.88 | ≤ 15.5 | ≤ 9.0 |
| 2.88 ≤ B < 5.76 | ≤ 14.5 | ≤ 9.0 |
| 5.76 ≤ B < 10.8 | ≤ 12.5 | ≤ 9.0 |
| 10.8 ≤ B < 23.04 | ≤ 11.0 | ≤ 9.0 |
| 23.04 ≤ B | ≤ 10.0 | ≤ 9.0 |

#### Proposal 6: For PC1.5 intra-band non-contiguous UL CA, consider up to 6dB PSD imbalance in configured transmitted power.

- 6 dB ≤ { PCMAX,c1 – 10*log10(LCRB1*SCS1)} –  { PCMAX,c2 – 10*log10(LCRB2*SCS2)} ≤ 6 dB

The upper bound of  PCMAX,C1 and PCMAX,C2 is 26dBm.

# References

[1] R4-2420329, “WF on HPUE RF requirements”, Samsung

[2] R4-2412024, “HPUE for intra-band UL CA”, LG Electronics