# 3GPP TSG-RAN WG4 Meeting # 114						R4-2502092

# Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			TP to TR38.774: BS RF requirement overview for LP-WUS

Agenda item:		7.26.3

Document for:		Approval

# Introduction

In this paper, we provide our view on BS RF requirement impact for LP-WUS.

# Discussion

In WF[1], the different aspects of the RF impact of the LP-WUS signal are listed and we share our view on these issues in below discussion.

#### Issue 1-1: Concept of LP-WUS power boosting

- Adhoc WF

  - “The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)” is considered as starting point for the definition.

    - FFS whether other cases can be additionally considered. E.g., single value declaration.

#### Issue 1-2: How to comprehend the de-boost on the NR transmission power

  - Consider the de-boosting behaviour together with other relevant issues

    - Companies are encouraged to think about the draft spec language to reflect corresponding proposals

#### Issue 1-3: Core requirement for LP-WUS power boosting

- WF

  - FFS the following options

    - Option 1: Leave the power boosting level to manufacturer declaration

    - Option 2: If the manufacturer declares power boosting, the minimum LP-WUS power boosting level should be X (>0) dB for Y MHz CBW

    - Other options are not precluded

In [4], the NR de-booting factor is derived in relation to the boosting factor x dB relative to average power and the definition is recapped as below.

Assume the boost power is x dB relative to average power of Pav in one RB, we have

PboostPav = 10x10 						(1)

Therefore, the remaining power left to transmit the other NR signal will be

Pcarrier-PWUS= Pav∙(NfullRB-10x10 ∙NRBWUS)

Futher assume the average of the NR signal power in one RB is Pdeboosting_NR, and we have

Pdeboosting_NR∙(NfullRB-NRBWUS)=Pcarrier-Pboost= Pav∙(NfullRB-10x10 ∙NRBWUS)

Therefore, the NR signal average power in linear is:

Pdeboosting_NR= PavNfullRB-10x10 ∙NRBWUSNfullRB-NRBWUS							(2)

As the boosting factor will be tested and the condition to test or how to design test model will decide how the boosting factor that could be declared by manufacture. The deboosting factor in equation (2) relates to the NR signal and there is a RE dynamic range in legacy requirement dedicating how low the NR signal can be de-boosted for a certain modulation.

Table 6.3.2.2-1: RE power control dynamic range

| Modulation scheme used | RE power control dynamic range (dB) |  |
| --- | --- | --- |
| on the RE | (down) | (up) |
| QPSK (PDCCH) | -6 | +4 |
| QPSK (PDSCH) | -6 | +3 |
| 16QAM (PDSCH) | -3 | +3 |
| 64QAM (PDSCH) | 0 | 0 |
| 256QAM (PDSCH) | 0 | 0 |
| 1024QAM (PDSCH) | 0 | 0 |
| NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station. |  |  |

It can be observed that for higher modulation (e.g 64QAM), there is no specified de-boosting, but for QPSK, -6 dB is allowed. In current test model of NR-FR1-TM1.2, the boosting is 3 dB, while for de-boosting it will be less than 3 dB as only 40% of PDSCH PRB are boosted (e.g if it would be 50% RB, the boosting is 3 dB and de-boosting will be exactly 3 dB to balance the same transmit carrier power). The purpose of the testing is to capture a worst case scenario where the full RB is transmitted from the base station and therefore, the rest of NR signal not boosted must be QPSK/16QAM otherwise no boosting will be allowed as 64QAM and higher than 64QAM not allowing de-boosting. When placing the boosting RB block, it is usually end up to be placed at the edge, for example, the location of the boosted RB is RBG#12, RBG#1, RBG#3 and RBG#5 for 10MHz and 15kHz SCS and the boosted RB location is illustrated in Figure 1.

The power boosting factor relates to the de-boosting factor allowed for the NR signal

The boosted RB is placed at the edge of transmission bandwidth to have the worst case to test

The full RB allocation is tested in the boosted RB test model in legacy NR.

From the above discussion, it can be observed that if the de-boosting factor is 3 dB following the legacy NR test model design, the boosted factor will be limited using the equation of (2) above. For the LP-WUS signal, we observe that the modulation is equivalent to the 64QAM signal due to the OOK-4 modulation with the additional DFT processing. This make the LP-WUS boosting itself difficult if comparing the 64QAM RE dynamic range. In summary, the boosting factor of the LP-WUS needs to consider both the signal quality requirement of the LP-WUS and the de-boosting factor allowed in the NR signal in test model design.

The power boosting factor relates also the signal quality requirement of LP-WUS.

Consider both the de-boosting factor of NR signal in test model design and the allowed signal quality requirement of LP-WUS.

From the above discussion, it can be observed that the de-boosting factor is only for conforming testing aspect. In the real network, when higher modulation transmitted from network together with the LP-WUS, there should be no de-boosting for NR signal. For issue 1-2, we have observation below:

Power de-boosting could be comprehended within the context of the conformance testing not the network operation.

For issue 1-1, our opinion is that this should consider the signal quality requirement of LP-WUS. From our analysis below, a 64QAM equivalent EVM requirement should be placed for LP-WUS OOK4 constellation. (e.g transmitted SNR of LP-WUS is at least 22 dB corresponding to 8% EVM requirement). In this case, 0 dB boosting is recommended considering the LP-WUS should be deployed with the installed based station. At network side, we think the LP-WUS will be deployed at a fixed position within a cell grid and not dynamically be changed. From the spectrum usage perspective, this can be more efficient when OOK1 and OOK4 could be configured in this LP-WUS RBs and for the same UE, either OOK1 or OOK4 will be received from network.

Not differentiating the OOK1 and OOK4 for power boosting discussion.

For issue 1-3, 0 dB power boosting of LP-WUS should be specified considering the LP-WUS signal quality requirement is equivalent to the 64 QAM EVM.

Single value for power boosting in issue 1-1.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR1-TM1.2

| Parameter | Value |
| --- | --- |
| Target percent of QPSK PDSCH PRBs boosted | x=40% |
| # of QPSK PDSCH RBGs which are boosted | NRBG=minxNRB-3P,12NRB-3+3modP-PP-NRB-3+3modP-PPmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NBWPsize=NRB-3 as the size of the bandwidth part and NBWPstart=3 |
| Locations of PDSCH RBGs which are boosted | NRB-3+3modPP-1 and if NRBG>1, 1, 3, …, 2NRBG-2+1 |
| Level of boosting (dB) | 3 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-3-PNRBG |
| Locations of PDSCH RBGs which are deboosted | Starting at RB#3 and excluding PRBs of RBGs which are boosted |
| Level of deboosting (dB) | 10log10NRB-3-103/10PNRBGNRB-3-PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 | QPSK |

Figure 1: The boosted RB location witin 52 RB grid (10MHz and 15kHz SCS)

Issue 1-6: General consideration on other requirements

- Adhoc WF

  - Discuss on whether transmit signal quality should be defined

    - FFS whether equivalent EVM requirement could be considered,

      - E.g. NR needs to pass 64QAM EVM test with LP-WUS embedded in the NR carrier

  - FFS whether no other new RF requirements than the dynamic range or potential transmit quality requirement identified for LP-WUS

As LP-WUS signal is new physical channel introduced in NR, our view is that the legacy RF requirement shall apply when LP-WUS signal is transmitted. As the OOK-m signal is synthesized with OFDM transmitter, such OOK waveform can only be identified at OOK WUR receiver with a channel bandwidth that equals to LP-WUS bandwidth. From transmitter perspective, the OFDM waveform is transmitted and therefore, RAN4 should further discuss what would be the difference when OOKm signal is generated comparing to the OFDM waveform and identify further RF impact if any.

In RAN1#118bis, the below working assumption is agreed:

Working Assumption

For overlaid OFDM sequences for LP-WUS in time or frequency domain, down-select between following two options for OOK-1 and OOK-4 M=1

- Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1.

  - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation

    - It doesn’t preclude additional operation for OOK-4 with M>1.

    - DFT size is 2^n

Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

This working assumption means the time sequence for overlaid OFDM symbol is ZC sequence. This applies both for OOK-1 and OOK-4. RAN1 also agrees on the processing for the OOK signal and this is illustrated in Figure 3.

Figure 3: OOK-4 signal processing and subcarrier mapping before IFFT

In our view, the RF impact analysis should be separated for OOK-1 and OOK-4 (m> 1). This is because we see the potential that no new RF requirement could be required for OOK-1 while more discussion will be needed for OOK-4 (m>1).

For OOK-1, as illustrated in Figure 4, the OOK bit 1 can be mapped to one OFDM symbol with the overlaid ZC sequence in this symbol and OOK bit 0 can be mapped to another OFDM symbol where no allocation within the LP-WUS RBs and therefore no energy would be detected by WUR. Following the RAN1 signal processing in Figure 3, the time domain ZC sequence can be generated in frequency domain and still it is ZC sequence illustrated in Figure 4.  For the frequence domain mapping of a ZC sequence before IFFT, this is the same with LTE PSS generation and there is no corresponding requirement or test for ZC sequence “modulation” in TS 36.104.  Our view is that there is no RF requirement needed for OOK-1 by following the LTE specification on this aspect. The ZC sequence generation is quoted below from TS. 36.211.

ZC sequence in frequency domain generation is specified for LTE PSS signal

No RF requirement defined for LTE PSS signal.

No new RF requirements is identified for OOK-1.

### 6.11.1	Primary synchronization signal (PSS)

#### 6.11.1.1	Sequence generation

The sequence  used for the primary synchronization signal is generated from a frequency-domain Zadoff-Chu sequence according to

Figure 4: OOK1 subcarrier mapping

Figure 5: ZC sequence in time and frequence domain.

- With OOK-4 (DFT-s or waveform fitting approaches) the gNB may need to support additional DFT (or another transform for LS approach) or pre-generate and store frequency domain symbols. This is illustrated in Figure 6.

Figure 6: OOK-4 subcarrier mapping

- After the DFT transform, the OOK-4 frequency domain pattern is illustrated in Figure 7. Such constellation pattern does not match any existing modulation schemes.  The measurement point for signal quality is defined after FFT according to Figure B.1-1 in TS 38.104. If this measurement point would be kept for signal quality measurement of OOK-4, RAN4 should discuss if a test with frequency domain data generated with a fix time domain OOK-4 pattern using procedure defined by RAN1 should be defined.

frequency domain testing data for OOK-4 be needed if the measurement point for signal quality would be kept.

According the current WID objective, the below objective still in line with the spirit of the study phase and keep the RF impact as min as possible.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline.

Figure 7: ZC sequence in frequency domain after DFT processing before IFFT

Figure B.1-1 [3]: Reference point for EVM measurement

- Considering WID objective and keeping the min specification impact for OOK-4, we suggest to testing the tolerance of the distortion of the frequency domain data of OOK-4 by remapping the frequency constellation of overlaid ZC sequence to one of existing modulation scheme. This will mimic the distortion of the signal by the noise from hardware. As one example, for OOK4 waveform, the higher modulation (e.g 64QAM) than QPSK may be used to map the frequency domain OOK4 symbols (left constellation in  Figure 6) to 64QAM constellation points (right constellation in  Figure 6) for CP-OFDM symbols as illustrated in Figure 6. Figure 7 and Figure 8 illustrates the initial LLS result to evaluate the impact of OOK and OFDM sequence detection due to the remapping, It can be observed that the performance loss of such mapping on WUS detection performance (OOK and OFDM) is around 0.2 dB when mapping to regular data structure generated from 64QAM and around 0.6 dB when mapped to 16QAM.

The performance loss for LP-WUS signal detection at WUR due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

From the distortion introduction by constellation re-mapping, it can be observed that the time domain signal quality of OOK-4 is quite robust and can be maintained without big degradation of SNR when the frequency domain OOK-4 data is distorted by the amount of EVM equivalent to the 64QAM. The frequency domain data of OOK-4 is degraded if it is distorted by the amount of the EVM equivalent to the 16QAM.

The amount of noise injected into OOK-4 signal in frequency domain after DFT can be tolerated at WUR if the noise level is equivalent to the minimum requirement of 64QM at base station.

Figure 8:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

Figure 9: Impact of mapping WUS constellation to existing QAM on OOK detection performance.

Figure 10: Impact of mapping WUS constellation to existing QAM on OFDM sequence detection performance.

The 64QAM modulation is mandatory tested in 38.141-1 and therefore, the LP-WUS signal quality test can be dependent on the PDSCH 64QAM and there is no need to introduce additional test. In the core specification, the LP-WUS signal quality requirement should be specified as this is new modulation scheme in Rel-19. The suggested wording in clause 6.5.2.2 as below:

The OOK4 signal quality test can be dependent on the 64QAM EVM test.

#### 6.5.2.2	Minimum Requirement for BS type 1-C and BS type 1-H

For BS type 1-C and 1-H, the EVM levels of each NR carrier for OOK4 modulation scheme on LP-WUS is specified in table 6.5.2.2-2.

Table 6.5.2.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for LP-WUS | Required EVM |
| --- | --- |
| OOK-4 | 8 % |

For the other requirement than the dynamic range and potential EVM requirement for OOK-4, we think no new requirement is identified for LP-WUS signal.

No other new RF requirement than the dynamic range or potential EVM requirement for OOK-4 is identified for LP-WUS.

TP starts here:

### 7.2.4	Transmitted signal quality

It is agreed that for overlaid OFDM sequences is designed for LP-WUS and the time sequence for overlaid OFDM symbol is ZC sequence. This applies both for OOK-1 and OOK-4. RAN1 also agrees on the processing for the OOK signal and this is illustrated in Figure 7.2.4-1. After the DFT transform, the OOK-4 frequency domain pattern is illustrated in Figure 7.2.4.2.

Figure 7.2.4-1: OOK-4 signal processing and subcarrier mapping before IFFT

Figure 7.2.4-2: ZC sequence in frequency domain after DFT processing before IFFT

- The tolerance of the distortion of the frequency domain data of OOK-4 is simulated by remapping the frequency constellation of overlaid ZC sequence to its nearest existing modulation constellation scheme. For example, for OOK4 waveform, the higher modulation (e.g 64QAM) than QPSK may be used to map the frequency domain OOK4 symbols (left constellation in  Figure 7.2.4-3) to 64QAM constellation points (right constellation in Figure 7.2.4-3) for CP-OFDM symbols as illustrated in Figure 7.2.4-3 . Figure 7.2.4-4 and Figure 7.2.4-5 illustrate the initial LLS result to evaluate the impact of OOK and OFDM sequence detection due to the remapping. It can be observed that the performance loss of such mapping on WUS detection performance (OOK and OFDM) is around 0.2 dB when mapping to regular data structure generated from 64QAM and around 0.6 dB when mapped to 16QAM.

- It is observed that the amount of noise injected into OOK-4 signal in frequency domain after DFT can be tolerated at WUR if the noise level is equivalent to the minimum requirement of 64QM at base station. When the noise level is equivalent to to minimum requirement of 16QAM, such distortion can not be tolerated at WUR.

Figure 7.2.4-3:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

Figure 7.2.4-4: Impact of mapping WUS constellation to existing QAM on OOK detection performance.

Figure 7.2.4-5: Impact of mapping WUS constellation to existing QAM on OFDM sequence detection performance.

# Conclusions

In this contribution, we present our view on the BS RF requirement with below observations and proposals:

Observation 1 The power boosting factor relates to the de-boosting factor allowed for the NR signal

Observation 2 The boosted RB is placed at the edge of transmission bandwidth to have the worst case to test

Observation 3 The full RB allocation is tested in the boosted RB test model in legacy NR.

Observation 4 The power boosting factor relates also the signal quality requirement of LP-WUS.

Proposal-1: Consider both the de-boosting factor of NR signal in test model design and the allowed signal quality requirement of LP-WUS.

Observation 5 Power de-boosting could be comprehended within the context of the conformance testing not the network operation.

Proposal-2: Not differentiating the OOK1 and OOK4 for power boosting discussion.

Proposal-3: For issue 1-3, 0 dB power boosting of LP-WUS should be specified considering the LP-WUS signal quality requirement is equivalent to the 64 QAM EVM.

Proposal-4: Single value for power boosting in issue 1-1.

Observation 6 ZC sequence in frequency domain generation is specified for LTE PSS signal

Observation 7 No RF requirement defined for LTE PSS signal.

Proposal-5: No new RF requirements is identified for OOK-1.

Observation 8 frequency domain testing data for OOK-4 be needed if the measurement point for signal quality would be kept.

Observation 9 WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline.

Observation 10 The performance loss for LP-WUS signal detection at WUR due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

Observation 11 The amount of noise injected into OOK-4 signal in frequency domain after DFT can be tolerated at WUR if the noise level is equivalent to the minimum requirement of 64QM at base station.

Proposal-6:The OOK4 signal quality test can be dependent on the 64QAM EVM test.

Proposal-7: No other new RF requirement than the dynamic range or potential EVM requirement for OOK-4 is identified for LP-WUS.

# References

- R4-2419802, WF for [113][308] NR_LPWUS, Huawei

- 38.141-1, Base Station (BS) conformance testing Part 1: Conducted conformance testing

- 38.104

- R4-2419482, BS RF requirement overview, Ericsson