# 3GPP TSG-RAN WG4 Meeting # 114						R4-2502097

# Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			TP for WUR RF requirement of REFSESN, ASC, ASCS

Agenda item:		7.26.2.2

Document for:		Approval

# Introduction

In this paper, we present our view on the WUR RF requirement of REFSENS, ASC, ASCS.

# Discussion

### REFSENS

In WF [1], it is agreed that OOK4 is used to derive the SNR and quoted as below:

Issue 2-1-1: Target FR1 SNR value for LP-WUS/WUR

Agreements:

  - Use worst case waveform of OOK-4 M=4 to derive SNR.

    - Some detailed aspects should be also confirmed, e.g., CRC bit (8), sequence length (8, 16), codepoint length of known part of the sequence)

For OOK WUR with OOK4 waveform, Table 1 lists the SNR points for different configuration according to Error! Reference source not found. and Error! Reference source not found..

Table 1: SNR target for OOK4 with different filter order

|  | OOK4 |
| --- | --- |
| Filter order 3 | - 2.5 dB |
| Filter order 5 | -1.5 dB |

For OFDM WUR, the 1% MSD is simulated, and the detection performance is illustrated in Figure 1. It can be observed that the SNR target is around -15 dB for AWGN channel with 4 OFDM symbols which is quite low compared to the OOK WUR SNR point.

Figure 1: OFDM sequence detection performance

Table 2: WUS resources configuration (4.32 MHz WUS bandwidth).

WUS payload (modulation)

1bit (OFDM, SSS-based time-correlation)

1 sequence of word payload

1/4 symbols for overlaid OFDM duration

The SNR for OFDM WUR detection is -3.2 dB for AWGN with 1 Zadoff Chu sequence and 1 symbol of OFDM symbol.

The SNR for OOK WUR detection depends OOK4 is -1.5 dB.

Issue 2-2-1: General views on REFSENS for LP-WUS

Agreements:

  - Discuss REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values.

Issue 2-2-3: companies input on SNR, NF and IM value for FR1 OOK-based LR

Agreements:

  - Use 2.5dB IM as a starting point, encourage companies to report the difference aspects due to OOK waveform receiver. The corresponding band should also be reported.

  - The total value of NF+IM can be considered.

  - On how to handle the NF values:

    - Option 1: For NF whether average value can be adopted.

    - Option 2: two sets NFs for different LP-WUR types with different power consumption target

    - Option 3: also consider coverage

  - Capture the table in WF as background information

As the SNR for OOK WUR and OFDM are different in general, RAN4 can try to reach consensus on each of them and if the SNRs are same in the end, the same SNR can be used for both OOK WUR and OFDM WUR.

RAN4 reach the consensus on SNR of OFDM WUR and OOK WUR before discussing whether the same or different SNR to be used in REFSENS.

For issue 2-2-1, it is not clear that how the UE should be tested if both OOK WUR and OFDM WUR can be supported for the same band. Assuming the REFSENS for OFDM WUR is lower than REFSENS of OOK WUR. For a overlaid signal design of LP-WUS signal, UE can decode either OFDM symbol or OOK symbol and waking up the MR afterwards. This will create a ambiguous situation on the target of the testing, is the OOK WUR or OFDM WUR that responding to the LP-WUS? To solve this ambiguity, a different UE ID should be configured for OFDM WUR and OOK WUR, i.e. the DUT UE ID is set for OOK WUR to test OOK WUR REFSENS but other UE ID set for OFDM WUR.

Ambiguity exists for testing REFSENS of two typs of WUR, e.g OOK WUR vs OFDM WUR if the UE support both for the same band.

Configure the DUT UE ID on either OOK part or OFDM part of the LP-WUS when testing only one type of WUR.

For issue 2-2-3, we think the IM should be kept as the same as legacy, which is 2.5 dB.

The issue of whether multiple NF should be defined, our opinion is that RAN4 should define a baseline REFSENS, then discuss how to define other based on further discussion. This may relate to coverage match investigation done by RAN1 and entry/exit condition of LP-WUS monitoring.  Though there is no limitation on which NF could be agreed in RAN4, the agreed NF would impact the LP-WUS signal design, for example, if a different NF would be agreed in Ran4 compared to the NF assumed in RAN1, RAN1 needs to be notified to design a corresponding SNR for LP-WUS. Our understanding is that the network resource used for LP-WUS could be higher if a larger NF would be agreed.

Worse NF may result in a higher resource allocation for LP-WUS.

RAN4 should deisgn a baseline REFSENS with a reasonable NF.

On the other hand, the UE power saving gain may be high at the cell edge as the receiver gain should be set highest when receiving a weak signal. If a WUR could be designed with a better NF, the coverage of LP-WUS for such WUR should be bigger compared to a WUR with baseline REFSENS performance. However, according to RAN1 and RAN2 agreement, the RSRP of the serving cell will be configured the same for all UE supporting WUR irrespective a WUR that have a better or worse NF.

Designing a WUR with better NF can benefit the power saving at cell edge.

The LP-WUS coverage is the same for all WUR irrespective the NF designated for WUR.

RAN4 can discuss to define different NF for WUR if the power saving gain can be realized by network

### FR2 REFSENS

In WF [1], there are some of the open issues

Issue 2-2-6: Baseline OFDM-based architecture for FR2 LP-WUR

Agreements:

  - FR2 LPWUR adopts zero-IF topology baseline assumption. FFS details

    - Multiple frequency conversion in RF domain is not precluded

  - For deriving a sensitivity requirement, the baseline OFDM-based FR2 LPWUR architecture is assumed to have the following features:

    - It comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized

    - FFS on whether and how to combine two Rx chains

    - Antenna gain is that of a single element

    - Coverage target should be considered

As the are two receiver branches proposed for the OFDM-based FR2 LPWUR, this is simulated in LLS with simulation assumption in Table 1 to see how to combine the signals from two receiver branch and impact on the SNR. We investigate two options to combine the data received from two branches:

- Option 1: select the data of antenna with maximum power per slot

- Option 2: combine data after envelop detector for each antenna

In LLS, the combination algorithm impact is mainly investigated. For example, there is no sync error modelled, no polarization mismatch etc. It can be observed that combination using option 1 can improve 2.4 dB@1% BLER compared with single antenna. And Option 2 improves 3.3 dB @1% BLER compared with single antenna.  As the diversity gain of 2.2 dB is assumed in legacy EIS requirement discussion, such gain is in par with the legacy MRC diversity gain. Therefore, there is a need to discuss whether the legacy MRC gain could be assumed in the WUR EIS requirement discussion. However, such gain may relate to different simulation parameter setting or combining method.

The combination gain could be in par with MRC combination for OOK 2Rx receiving.

Table 1: LLS simulation assumption

| - Waveform | - OOK-4 |
| --- | --- |
| - Bit length (pay load) | - 8 |
| - # OFDM symbol | - 4 |
| - Antenna configuration | - 1T1R, 2T2R |
| - Channel | - TDL-C, 300ns delay spread |

Figure 2: WUS BLER for different combination option

### ASCS

In WF [1], it is agreed to use 0 RB as guard RB between LP-WUS and NR signal. This means the SNR degradation caused by adjacent NR signal is neglectable. The NR signal is simulated with the same PSD with the wanted signal. The this means the ASCS selectivity is 0 dB. As discussed by one company, the wanted signal level can be set as REFSENS. The NR signal adjacent to the wanted signal power will be set according to the same PSD of LP-WUS signal and also the # of the RB that NR signal should be transmitted. This means there is scaling factor of the 10log (# RB of NR signal / # of RB of LP-WUS). Other testing parameter could be the channel bandwidth of the MR and location of the LP-WUS signal RB.  The testing parameter is listed in Table 2 below. The x dB can be set for a case with a small adjacent interfere and for big adjacent interferer case (maximum input power). This can be discuss further with the maximum input power requirement.

ASCS can be tested with both small adjacent interferer and big interferer case with selectivity of 0 dB.

Table 2: Testing parameter for ASCS

| Wanted signal | REFSENS + [ x dB] |
| --- | --- |
| NR signal | REFSENS + 10log (# RB of NR signal / # of RB of LP-WUS) + [x dB] |
| NR channel bandwidth | 5MHz |
| LP-WUS RB allocation | # RB adjacent to the RB of NR signal |

### ACS

The simulation assumption from the table below is used.

| Attributes | Assumptions |  |
| --- | --- | --- |
| Case name (waveform) | OOK-4 waveform |  |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |
| Channel structure | Total 8 bits |  |
| Chip rate | M=1 | M=2 |
| WUS duration | FFS |  |
| Coding | 1/2 rate Manchester coding |  |
| Time error | 0 |  |
| Residual Frequency error | 0/10/20 ppm |  |
| SCS | 30kHz |  |
| UE Channel BW | 20MHz (51 RB)-case 1 |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |
| Guard RB | - For ACS: 1/2/3/4 RB |  |
| Filter | - 3rd/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = 33dB NOTE: decide the interference level depending on SNR |  |
| Sampling rate | 7.68MHz |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS |  |
| Phase noise | Not modelled |  |
| Non-linearities | Not modelled |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |
| Phase noise | Phase noise profile. Phase noise (dBc/Hz). Offset; NF 9 dB; NF 12 dB; NF 15 dB. @5MHz; -99.4; -99.4; -99.4. @10MHz; -108.1; -105; -102.1. @15MHz; -120; -117; -114. |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |
| Performance metric | - 1% MDR/BLER - The following false alarm rate can be considered - 0.1% |  |

According to the WF[2], the phase noise can be considered in the LLS. In LLS simulation, the ACS cases are simulated with ACI =33/30with different shifted RB and filter order.

For OOK4, in Figure 3 it can be seen that filter order of 3 and 4 shifted RB cannot reach 1% BLER floor but increase the filter order to 5 and use 4 shifted RB can reach the 1% BLER for ACI = 33. In Figure 4, it can be observed that the shifted RB needs to be 2 only when ACI is relaxed to 30.

For OOK4, the ACI needs to be relaxed to 30 and 3 shifted RB will be needed for filter order of 3.

Figure 3:The BLER for ACI=33 with LPF order of 3 & 5, OOK4 and phase noise ON/OFF

Figure 4: The BLER for ACI=30 with LPF order of 3, shift RB number of 0/1/2 ,  OOK4 and phase noise ON/OFF

There are two test cases for ACS, case #1 and case #2.  For case #1, the ACS interferer is set relative to the wanted signal with the power difference of the selectivity value. For OOK4, ACI needs to be relaxed with 3 dB to fit the lower filter order of 3.

In LLS, the ACS interferer is set to ACS value relative to the WUR received signal. If the OOK1 is to be used, the ACS interference can be set relative to LP-WUS signal.  As the WUR has a higher REFSENS than REFSENS of MR, this means that the ACS interferer could be higher than ACS interfere of the MR.

Another approach is that ACS value is the attenuation between the ACS interference set by MR and the WUR REFSENS plus 14 dB. In this way, the ACS would be relaxed as the REFSENS of WUR is higher than REFSENS of MR.

The ACS interferer shall be set the same with MR and wanted signal is set as WUR REFSENS plus 14 dB for ACS test requirement.

# Conclusions

In this contribution, we present our view on WUR RF requirement for REFSENS, ASCS and ACS with below proposal and observations:

Observation 1 The SNR for OFDM WUR detection is -3.2 dB for AWGN with 1 Zadoff Chu sequence and 1 symbol of OFDM symbol.

Observation 2 The SNR for OOK WUR detection depends OOK4 is -1.5 dB.

Proposal-1: RAN4 reach the consensus on SNR of OFDM WUR and OOK WUR before discussing whether the same or different SNR to be used in REFSENS.

Observation 3 Ambiguity exists for testing REFSENS of two typs of WUR, e.g OOK WUR vs OFDM WUR if the UE support both for the same band.

Proposal-2: Configure the DUT UE ID on either OOK part or OFDM part of the LP-WUS when testing only one type of WUR.

Observation 4 Worse NF may result in a higher resource allocation for LP-WUS.

Proposal-3: RAN4 should deisgn a baseline REFSENS with a reasonable NF.

Observation 5 Designing a WUR with better NF can benefit the power saving at cell edge.

Observation 6 The LP-WUS coverage is the same for all WUR irrespective the NF designated for WUR.

Proposal-4: RAN4 can discuss to define different NF for WUR if the power saving gain can be realized by network

Observation 7 The combination gain could be in par with MRC combination for OOK 2Rx receiving.

Proposal-5: ASCS can be tested with both small adjacent interferer and big interferer case with selectivity of 0 dB.

Observation 8 For OOK4, the ACI needs to be relaxed to 30 and 3 shifted RB will be needed for filter order of 3.

Proposal-6: The ACS interferer shall be set the same with MR and wanted signal is set as WUR REFSENS plus 14 dB for ACS test requirement.

# References

- R4-2420528, WF on UE RF requirements for LP-WUS, vivo