# 3GPP TSG-RAN WG4 Meeting # 114						R4-2502104

# Athens, Greece,17th–21st February 2025

Source:		Ericsson, Sony

Title:			Other NTN UE RF impact

Agenda item:		7.29.2.2

Document for:		Approval

# Introduction

In this paper, we present our general overview on NTN UE RF impact on uplink capability objective.

# Discussion

For UL coverage enhancement objective, the RF requirement is relating to the OCC, which is mainly on inter-lost time domain OCC with PUSCH repetition type A.  The latest RAN1 agreement regarding the OCC is below:

​Agreement

RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows:

- Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs.

- Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences

- RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs.

- RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs.

Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact.

- RAN1 assumes no specification impact for CFO grouping

- RAN1 does not pursue closed-loop frequency adjustment commands.

- RAN1 assumes that RAN4 does not define new UE requirements for CFO.

From the RAN1 agreement, it can be observed that inter-slot OCC with length4 up to 4 UEs is agreed to be specified. There is note in RAN1 which does not assumes the new UE requirement for CFO. Our understanding is that RAN4 still need to discuss the RF impact for OCC4 as potential LS could be sent to RAN1 once consensus reached to confirm RAN1 assumption. Last meeting, RAN4 agree no RF impact on OCC2. However, there is no conclusion on the OCC4 yet. Below we continue to discuss the RF impact for OCC4.

Inter-slot OCC4 will be specified in RAN1 and RF impact to be discussed in RAN4.

LS should be sent to RAN1 when RAN4 reach consensus on RF impact evaluation on inter-slot OCC4 to confirm RAN1 assumption.

For inter-slot time domain OCC, as illustrated in Figure 1, the different OCC code will be used for different UE for the same frequency and time resources. At BS receiver, to decode one UE data, the combined signal with other UEs will multiply this UE’s own OCC code, at the same time, the other UE data with such OCC code would be cancelled out as the OCC codes between UEs are orthogonal. When decoding the intended UE data, the CFO of this UE can be compensated but the CFO of other UE cannot be compensated due to the different frequency offset from UEs, which shows up as interference and cause the impairment on the OCC orthogonality and therefore SNR degradation could occur.

Figure 1:Schematic view of OCC across slots

RAN1 has agreed the below impairment model:

Agreement

Adopt the table below for assumptions for modelling impairments for link level evaluation in NR NTN UL capacity and throughput enhancements

| Parameter | Value |
| --- | --- |
| TO | Reported by companies - With TO: Uniform selection from [-0.94us, 0.94us], where 0.94us=29Ts - Optional without TO |
| FO | Reported by companies - Uniform selection from [-0.1 ppm, +0.1 ppm], Variation of frequency error is negligible. - Optional: with lower maximum residual FO, to be reported by companies |
| Timing drift | Optional |
| Receiver algorithm | To be reported by companies, e.g. - MMSE |
| Channel estimation | - Real channel estimation |

From the above impairment model, it can be observed that the frequency offset is only relevant to the UE RF discussion. In [2], the FO impact on the LLS performance is observed and it is illustrated in Figure 2. For 4 UE inter slot OCC case, even if in higher repetition number, example, 16 repetitions, big CFO has negative impact on OCC performance and one of the mitigations could be the pairing the UE with selective FO. This also implies that UE should maintain a fixed CFO during the OCC transmission otherwise there is no guarantee on the OCC performance.

The CFO impairment has impact on the OCC performance in inter-slot OCC4 scheme.

UE should maintain the same CFO during the OCC transmission so the OCC performance gain could be achieved.

(a) OCC across 2 slots with/without CFO constant in 8 repetitions

(b) OCC across slots without impairments in 16 repetitions

(c) OCC across slots with TO and FO impairments in 16 repetitions

(d) FO spread impact on 4 UE OCC across slots in 16 repetitions

Figure 2: Link performance of VoIP on PUSCH with OCC across slots.

In Figure 2 LLS, the CFO offset is modelled with certain spread, meaning the CFO for each time slot can be any value within a range between [ -X, X] ppm. The 0.1ppm is the RF requirement measured at the UE antenna and expectation for OCC4 from the LLS results is that the frequency error spread should be confined within a certain limit less than +/- 0.1ppm. For example, UE#1 and UE#2 with FO spread of  +/- 100Hz, where the maximum FO difference between UE#1 and UE#2 will be 200Hz. It is possible the FO spread is low so the CFO grouping for the OCC4 would be possible. We explain it below.

The LO of uplink transmission can be controlled by the AFC (Automatic frequency control) function block as illustrated in Figure 3. For OFDM system, the frequency offset can be estimated together with the time offset from OFDM synchronization function block in base band. The residue frequency error estimated can be used as a control signal for the LO frequency tuning and frequency synchronization to the network. Figure 4 from [1] illustrates the residue frequency offset performance with simulation assumption listed in  Table 1. It can be observed that the 50-pencentile residue frequency error is zero and this means the long term average residue frequency offset is zero and UE can lock/synchronize its radio frequency to the network. Another observation is that the residue frequency offset is random variable with a certain variance. Such variance can be lowered through a low pass filter illustrated in Figure 3. With the initial frequency offset confined with [-0.1ppm, 0.1ppm] and a LPF applied, the filtered residue frequency offset could be below the 0.1ppm, when measured within multiple consecutive time slots. This is illustrated in Figure 5 with a standard normal distribution variable with variance of 1 and LPF with a cutoff frequency of 10*2pi radians/second. It can be observed that the standard deviation is reduced after the LPF.

The variance of frequency error performance from UE can be smaller than 0.1ppm measured within a multiple time slots duration.

The variance of residue frequency error can be reduced using a LPF.

Figure 3: AFC clock control

Table 1:Link-level Evaluation Assumption [1]

| Carrier Frequency | 4GHz |
| --- | --- |
| Default subcarrier spacing | 15kHz, 30kHz |
| Channel Model | TDL-C, 100ns |
| Target received baseband SNR | -6dB |
| False alarm rate | Less than 1% false alarm rate |
| NR-PSS/SSS detection | One shot detection |
| Frequency Offset | Randomly chosen from {-x ppm, +x ppm} where, x is 5 as mandatory and 10 as optional |

(a)  Residual time offset					(b) Residual frequency offset

Figure 4:Residual time and frequency error after synchronization [1]

Figure 5: output after LPF with Gaussian random variable input

Another factor to impact the variance of the residue frequency error estimation is the SNR of the received OFDM signal. The variance of the residue frequency error can be illustrated with the formular in [3] and is reprinted below where φ is the phase difference between the two training symbols and L is the length of the training symbol.

varφπ=1π2∙L∙SNR

It can be observed that such variance is inversely proportional to SNR. It can be concluded from the above discussion that both SNR of received signal and the LPF impact the variance of the frequency error performance. With a high SNR, the variance of the frequency error could be smaller than 0.1ppm specified for frequency error requirement. This can be seen from the test report on the NB-IoT[2] when tested. For the UE applying LPF before slewing the oscillator frequency and received a high SNR of the signal, the variance of frequency error will be limited within a range less than 0.1ppm and network can group these UE to apply OCC4 scheme. One thing needs to pay attention is that the UEs should not make the doppler frequency pre-compensation within some time window during which the network measures and selects the UE (CFO grouping) in terms of the variance of the frequency offset. This is because that pre-compensation of the doppler will make a frequency offset difference exceed the a predefined limit between two UEs within a CFO group. For example, UE#1 with variance X ppm and lock-in frequency f1 and UE#2 with variance Y ppm and lock-in frequency f2. Ideally, f1=f2 and network will group UEs when X-Y within a predefined range. UE can apply doppler frequency compensation once it sees a need to reduce the frequency error caused by doppler frequency due to satellite moving trajectory. This will add a frequency offset delta on top of the X-Y variance condition and this will derail the CFO grouping condition at network and degrade the OCC performance.

The variance of frequency error could be smaller than 0.1ppm when SNR of the received OFDM signal is high.

Allowing the doppler frequency pre-compensation could degrade the OCC performance due to potential change of the CFO group condition.

Disabling the doppler frequency pre-compensation within a certain time window where the network evaluates the CFO of the UEs and schedule the OCC schemes.

Figure 6: Frequency stability test [2]

Figure 7: CFO grouping and OCC scheduling.

As illustrated in Figure 7, network can evaluate CFO of UE and group the UEs with certain CFO variance within certain time window and at the same time, UE may not make the doppler pre-compensation to invalid the CFO grouping condition in case network schedules the OCC scheme for such UE.

From the above analyse, it can be seen that OCC scheme can be scheduled to the UEs where the variance of the frequency offset is low and no abrupt frequency adjustment during a certain time window assumed within a certain time window at network side. Defining new CFO requirement for UE for OCC scheme is not helpful as variance of the CFO is dictated by received SNR instead of the implementation (LPF helps, but can only reduce a certain amount variance). As such, it is recommended to send a LS to RAN1 with below text:

RAN4 discussed the RF impact of OCC schemes and reach consensus below:

- For OCC2, there is no RF impact.

- For OCC4, there is no RF impact on UE (i.e no new CFO requirement to be defined) and it is preferred to disabling the doppler pre-compensation during a certain time window where CFO is evaluated and OCC scheme is scheduled at network side.

Send LS to RAN1 with text above.

# Conclusions

In this contribution, we present our general overview for NTN UE RF impact for UL capacity objective with below observations and proposals:

Observation 1:Inter-slot OCC4 will be specified in RAN1 and RF impact to be discussed in RAN4.

Proposal-1: LS should be sent to RAN1 when RAN4 reach consensus on RF impact evaluation on inter-slot OCC4 to confirm RAN1 assumption.

Observation 2:The CFO impairment has impact on the OCC performance in inter-slot OCC4 scheme.

Observation 3:UE should maintain the same CFO during the OCC transmission so the OCC performance gain could be achieved.

Observation 4:The variance of frequency error performance from UE can be smaller than 0.1ppm measured within a multiple time slots duration.

Observation 5: The variance of residue frequency error can be reduced using a LPF.

Observation 6:The variance of frequency error could be smaller than 0.1ppm when SNR of the received OFDM signal is high.

Observation 7:Allowing the doppler frequency pre-compensation could degrade the OCC performance due to potential change of the CFO group condition.

Proposal-2:Disabling the doppler frequency pre-compensation within a certain time window where the network evaluates the CFO of the UEs and schedule the OCC schemes.

Proposal-3:Send LS to RAN1 with text above.

# References

- R1-1706007, NR synchronization signal design and evaluation, Ericsson

- https://fcc.report/FCC-ID/XMR201910BG95M3/5495630.pdf

- T. M. Schmidl and D. C. Cox, “Robust Frequency and Timing Synchronization for OFDM”, IEEE Trans. Commun., Vol. 45, No. 12, pp. 1613-1621,  Dec., 1997.