3GPP TSG-RAN WG4 Meeting # 114						R4-2502107

Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			[NTN NB-IoT] On NB-IoT in-band operation for NR NTN

Agenda item:		5.33.1

Document for:		Approval

# Introduction

In this paper, we present our thoughts on NB-IoT in-band operation for NR NTN.

# Discussion

In Rel-18, the stand-alone is introduced in NTN and there is request from satellite operators that the guard band and inband operation of NR should be added. However, there is some issues to be discussed before the inband/guard band mode to be added in the specification.

Rel-15 Mixed mode feature

This is a feature introduced in Rel-15 and stated in 36.300:

and quoted below:

## 5.5a   Multi-carrier operation for NB-IoT

For NB-IoT, multi-carrier operation is supported.

The UE in RRC_CONNECTED can be configured, via UE-specific RRC signaling, to a non-anchor carrier, for all unicast transmissions. The UE in RRC_IDLE, based on broadcast/multicast signaling, can use a non-anchor carrier for SC-PTM reception. The UE in RRC_IDLE can, based on broadcast signaling, use a non-anchor carrier for paging reception. The UE in RRC_IDLE or RRC_CONNECTED, based on broadcast signaling, can use a non-anchor carrier for random access. If the non-anchor carrier is not configured for the UE, all transmissions occur on the anchor carrier. The valid anchor and non-anchor carrier combinations are provided in Table 5.5a-1 for FDD and Table 5.5a-2 for TDD.

Table 5.5a-1: Anchor and non-anchor carrier deployment combinations in FDD

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band | Valid (Note 1) | Valid (Note 1) | Valid (Note 2, Note 3) |
|  | Guard-band | Valid (Note 1) | Valid (Note 1) | Valid (Note 2, Note 3) |
|  | Standalone | Valid (Note 2, Note 3) | Valid (Note 2, Note 3) | Valid (Note 2) |

Table 5.5a-2: Anchor and non-anchor carrier deployment combinations in TDD

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band | Valid (Note 1) | Valid (Note 1) | Invalid |
|  | Guard-band | Valid (Note 1) | Valid (Note 1) | Invalid |
|  | Standalone | Invalid | Invalid | Valid (Note 2) |

NOTE 1:  Both carriers associated with the same LTE cell.

NOTE 2:            Total frequency span to not exceed 20MHz and both anchor and non-anchor carriers synchronized

NOTE 3:	Not applicable to SC-PTM reception.

UE has a capability to support this mixed mode operation defined in 36.306:

#### 4.3.4.115      mixedOperationMode-r15

This field defines whether the UE supports multi-carrier operation where the anchor carrier is in standalone mode while the non-anchor carrier is in inband or guardand mode, and vice versa, for unicast, paging, and random access for FDD as specified in TS 36.300 [30]. This field is only applicable for UEs of any ue-Category-NB.

From legacy signalling perspective for mixed mode configuration, “operationModeInfo” is used in MIB-NB for anchor carrier, and “DL-CarrierConfigCommon-NB” is used in SIB-NB for RA and Paging on non-anchor carrier, and “DL-CarrierConfigDedicated-NB” is used in UE level signaling such as RRC Connection Setup in case UE is allocated to non-anchor carrier. However, in our understanding, there is no indication of the GB or SA mode in either “DL-CarrierConfigCommon-NB” or “DL-CarrierConfigDedicated-NB” for a non-anchor carrier. The summary of the operation mode indication in legacy 36.331 is summarized in Table 1 below.

Table 1: signaling indication on the operation mode

|  | In-band | Guard-band | Standalone |
| --- | --- | --- | --- |
| Anchor carrier | “operationModeInfo” is used in MIB-NB Value: Inband-samePCI or Inband-differentPCI | “operationModeInfo” is used in MIB-NB Value: Guardband-r13 | “operationModeInfo” is used in MIB-NB Value: standalone |
| Non-anchor carrier | “DL-CarrierConfigCommon-NB” is used in SIB-NB for RA and Paging Value: inbandCarrierInfo and samePCI-Indicator-r14 (Cond anchor-guardband-or-standalone) Provides the configuration of a non-anchor inband carrier. “DL-CarrierConfigDedicated-NB” is used in UE level signaling such as RRC Connection Setup Value: inbandCarrierInfo Provides the configuration of the anchor/ non-anchor inband carrier. If operationModeInfo is set to standalone in the MIB-NB, E-UTRAN only configures this field if the UE supports mixed operation mode. | For the same IE “DL-CarrierConfigCommon-NB” or “DL-CarrierConfigDedicated-NB” If inbandCarrierInfo not present, not defined, to be discussed if RAN4 can interpret it as “the operation mode can be the same with anchor carrier” | For the same IE “DL-CarrierConfigCommon-NB” or “DL-CarrierConfigDedicated-NB” If inbandCarrierInfo not present, not defined, to be discussed if RAN4 can interpret it as “the operation mode can be the same with anchor carrier” |

Table 2 : Possible mixed mode combination with IE indication in Table 1

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band | Yes | Yes | Yes, but only if UE support mixed mode for dedicated RRC |
|  | Guard-band | No, there is no dedicated IE for Guard-band for non-anchor | Yes inbandCarrierInfo not present, the operation mode can be the same with anchor carrier | No, there is no dedicated IE for Guard-band for non-anchor |
|  | Standalone | No, there is no dedicated IE for Guard-band for non-anchor | No, there is no dedicated IE for Guard-band for non-anchor | Yes, if inbandCarrierInfo not present, the operation mode can be the same with anchor carrier, |

Based on the Table 1 on the understanding of the operation mode indication from network, the possible mixed mode operation is listed in Table 2. It can be observed that with current signalling specification, there are a few combination cannot be supported as there is no dedicated signaling as listed in Table 1.

The current 36.331 cannot support all mixed mode combination as stated in 36.300.

For the current signaling, it seems RAN4 assumes the same operation mode for anchor and non-anchor carrier as the operationModeInfo IE for anchor is used to derive the operation mode for non-anchor. Such understanding should be checked with RAN2 and RAN1. According to our understanding, if inbandCarrierInfo is not present, there is no dedicated signalling to notify the UE which operation mode it is, so either GB or SA. But from product deployment, we understand that the same operation mode for anchor and non-anchor can be assumed if inbandCarrierInfo is not present. If RAN4 assumes the same operation mode for anchor and non-anchor using the operationModeInfo in anchor carrier without considering the inbandCarrierInfo, this will overwrite the case when InbandCarrierInfo is present in either DL-CarrierConfigDedicated-NB or DL-CarrierConfigCommon-NB. For example, if inband-samePCI is indicated in anchor and inband-differentPCI is indicated in inbandCarrierInfo in non-anchor. This creating a error case in configuration.

The same operation mode for anchor and non-anchor assumption in RAN4 will overwrite the signaling when inbandCarrierInfo is present and possibly creating an error case in configuration.

As the F_DL formular is different for non-anchor carrier in GB and SA mode, an indication between the stand-alone and GB/IB would be needed, therefore, mixed mode feature cannot be supported for NTN NB-IoT if the non-achor carrier is not in-band operation.

Mixed mode feature cannot be supported by NTN NB-IoT as the GB/SA operation is not indicated by network.

From the above discussion, there is a need to patch the signalling to complete the mixed mode support feature stated in 36.300. From this perspective, It is better not to support the mixed mode feature due to the above signalling limitation in NTN. Because there will be NBC issue for NTN NB-IoT as NTN NB-IoT UE supporting mix mode feature will not know whether it is GB or SA mode for non-anchor when anchor carrier is configured with IB mode but non-anchor is not configured with IB mode as illustrated in Table 2. A LS to RAN1/RAN2 should be considered if RAN4 reach consensus on it.

Mixed mode feature shall not be supported in NTN NB-IoT UE to avoid the NBC issue.

LS to RAN1/RAN2 if above proposal would be agreed with below LS text

- “ RAN4 started to introduce the inband operation for NTN IoT in NR NTN and make below assumption if NB-IoT UE does not support mixed mode feature stated in clause 5.5a in TS 36.300

- Non-achor carrier operation mode can be assumed the same with anchor carrier indicated in operationModeInfo when inbandCarrierInfo is not present in either DL-CarrierConfigDedicated-NB or DL-CarrierConfigCommon-NB.

- For a NB-IoT NTN UE that supports mixed mode feature stated in clause 5.5a in TS 36.300:

- According to the capability description in 4.3.4.115 in TS 36.306, a UE cannot presently derive the non-anchor operation mode (GB and SA).

- RAN4 want to ask if above assumption is correct according to RAN1/RAN2 specification and if not, RAN4 want to ask if there is a way to derive the non-anchor operation mode of GB and SA when inbandCarrierInfo is not present.”

As the GB/IB mode can be mixed before Rel-15 in TN without the mixed mode feature in Rel-15, we think it will be good to keep such in NTN also. With such understanding, the possible mixed mode operation will be listed below in Table 3.

Table 3 : Possible mixed mode combination with IE indication in Table 1

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band | Yes | Yes | NO (this needs mixed mode feature support which can be excluded for NTN) |
|  | Guard-band | NO | YES | NO (this needs mixed mode feature support which can be excluded for NTN) |
|  | Standalone | NO | NO | YES |

RAN4 align the understanding of specification using Table 3 for possible deployment mode.

Another issue is the understanding whether the inband-samePCI or inband-differentPCI can be deployed by NTN network considering some of the NPDSCH capability loss as for inband operation, there are LTE CRS in legacy LTE carrier.  Our view is that at least for inband-differentPCI, there is no need to transmit the LTE CRS as there is no LTE UE in NR NTN deployment.  However, the RE for LTE CRS transmission should be reserved and not used by NB-IoT allocation. This is for NB-IoT rate matching purpose.  There are some benefit on the inband-differentPCI indication, for example, the NRS will be transmitted in less subframes for inband deployment compared with guardband or SA deployment,  in 36.211, it states:

- On an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating inband-SamePCI or inband-DifferentPCI.

- -	If frame structure type 1 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #4 and in subframes #9 not containing NSSS, and in subframes #3 which contain SystemInformationBlockType1-NB when additionalTransmissionSIB1 is configured as TRUE

Comparing to the guardband or stand-alone:

- On an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating guardband or standalone.

- -	If frame structure type 1 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4 and in subframes #9 not containing NSSS.

There is benefit on spectrum utilization if inband-differentPCI could be indicated.

# Conclusions

In this contribution, we present our overview on the NB-IoT inband operation for NR NTN with below observation and proposals:

Observation 1 The current 36.331 cannot support all mixed mode combination as stated in 36.300.

Observation 2 The same operation mode for anchor and non-anchor assumption in RAN4 will overwrite the signaling when inbandCarrierInfo is present and possibly creating an error case in configuration.

Observation 3 Mixed mode feature cannot be supported by NTN NB-IoT as the GB/SA operation is not indicated by network.

Proposal-1: Mixed mode feature shall not be supported in NTN NB-IoT UE to avoid the NBC issue.

Proposal-2: LS to RAN1/RAN2 if above proposal would be agreed with below LS text

Proposal-3: RAN4 align the understanding of specification using Table 3 for possible deployment mode.

Observation 4 There is benefit on spectrum utilization if inband-differentPCI could be indicated.