# 3GPP TSG-RAN WG4 Meeting #114	R4-2502111

# Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 		Discussion on the updated ITU-R SM.329 recommendation (v13; 09/2024) for UE RF specifications

Agenda Item:	5.32.1

Document for:	Discussion

# Introduction

New version of the ITU-R Recommendation SM.329 on unwanted emissions in the spurious domain has been published 09/2024 as version 13 [1], replacing previous v12 (dated 09/2012). In this contribution, we provide analysis of the introduced modifications.

# Discussion

ITU-R Recommendation SM.329 in references in many RAN4 specifications, including UE RF and BS RF. Therefore, below we provide brief analysis of modifications introduced in v13 of SM.329.

Among many modifications introduced in v13 of SM.329, the following can be highlighted:

- Category Z equipment definition correction (clause 4.6),

- Corrections of Category Z limits for class A and class B equipment (field strength and EIRP) (clause 4.6),

- Updates to the field strength measurement methodology (Annex 1.2),

- Updates of threshold levels of interference for the RAS and space services using passive sensors, replacing table with reference to another ITU-R recommendation (Annex 3).

In the attached document, more details can be found on delta among v12 and v13 versions of SM.329 (v13 corrections as Track Changes).

Observation 1: There were no modifications on unwanted emission limits impacting RAN4 specifications.

However, it was observed that many RAN4 specifications use versioned references to SM.329 (e.g., SM.329-10, SM.329-12). Those are called Specific references, as opposed to Non-specific references (for description of those terms refer to TR 21.801).

As reference shall point to the latest version of the ITU recommendation which is “in force” (as opposed to previous “Suspended” versions), there is need to reassure that latest version of the ITU recommendation is used in RAN4 specifications (i.e., v13 of the SM.329).

Observation 2: RAN4 specifications shall not refer to Suspended versions of SM.329, i.e., SM.329-10, nor SM.329-12.

Observation 3: RAN4 specifications shall use either Specific reference (i.e. SM.329-13), or to use Non-specific reference (i.e. SM.329). Non-specific reference approach would be future-proof.

The following Rel-18/19 specifications were identified as requiring related SM.329 reference corrections to remove references to Suspended versions of SM.329:

UE RF:

- 36.101 (Rel-18/-19)

- 36.102 (Rel-18)

- 38.101-1 (Rel-18/-19)

- 38.101-2 (Rel-18)

- 38.101-5 (Rel-18)

# Conclusions

Based on the discussion above, set of CRs to Rel-18/-19 UE RF specifications was provided in [3-9].

For related discussion in BDAT room, discussion paper in [10] was submitted.

# References

[1] 	Recommendation ITU-R SM.329, Unwanted emissions in the spurious domain, v13, 09/2024, https://www.itu.int/dms_pubrec/itu-r/rec/sm/R-REC-SM.329-13-202409-I!!PDF-E.pdf

[2]		Recommendation ITU-R SM.329, Unwanted emissions in the spurious domain, v12, 09/2012, https://www.itu.int/dms_pubrec/itu-r/rec/sm/R-REC-SM.329-12-201209-S!!PDF-E.pdf

[3]		R4-2502112	Correction of reference to Suspended version of ITU-R SM.329 Recommendation, Rel-18	36.101

[4]		R4-2502113	Correction of reference to Suspended version of ITU-R SM.329 Recommendation, Rel-19	36.101

[5]		R4-2502114	Correction of reference to Suspended version of ITU-R SM.329 Recommendation, Rel-18	36.102

[6]		R4-2502115	Correction of reference to Suspended version of ITU-R SM.329 Recommendation, Rel-18	38.101-1

[7]		R4-2502116	Correction of reference to Suspended version of ITU-R SM.329 Recommendation, Rel-19	38.101-1

[8]		R4-2502117	Correction of reference to Suspended version of ITU-R SM.329 Recommendation, Rel-18	38.101-2

[9]		R4-2502118	Correction of reference to Suspended version of ITU-R SM.329 Recommendation, Rel-18	38.101-5

[10]	R4-2502119	Discussion on the updated ITU-R SM.329 recommendation (v13; 09/2024) for BS RF specifications