# 3GPP TSG-RAN WG4 Meeting #114	R4-2502128

# Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 		TP to TR 38.749: BS RF requirements section correction (clause 9)

Agenda Item:	6.22.3

Document for:	Approval

# Introduction

During RAN4#113 (Orlando) meeting, TP on BS RF requirements for TR 38.749 was Endorsed in [1]. Based on its review after the meeting, further corrections are proposed.

In this contribution, we provide TP with updates to the BS RF section of the TR 38.749.

# Conclusions

Proposal 1: Approve the attached TP to TR 38.749.

# References

[1] 	R4-2420355	TP for TR38.749 BS requirements(Clause 9), RAN4#113

# TP to TR 38.749

On top of the already Endorsed content of [1], below we provide TP with corrections highlighted.

------------------------------ Modified section ------------------------------

# 9	BS RF requirements

The Earth Exploration Satellite Service (EESS) protection requirements in the frequency range 23.6-24.0 GHz (as per WRC-19 Radio Regulations Resolution 750 [2] and EU Decision 2020/590 [4]) were specified via additional OTA OBUE requirements and additional OTA spurious emission requirements in the BS specifications.

The only modification in legacy specifications was that ITU-R applicability date (i.e. 1 September 2027) was considered in those specifications. Within this WI, the BS additional OTA OBUE limits and additional OTA transmitter spurious emissions requirements for protection of EESS were defined as:

-The emission level of -3dBm/200 MHz applies to BS brought into use on, or before 1 September 2027 in countries not adopting EU Decision 2020/590 [4], while

- The emission level of -9dBm/200 MHz applies to BS brought into use after 1 September 2027, or to BS in countries adopting EU Decision 2020/590 [4].

------------------------------ End of modified section -------------------------