# 3GPP TSG-RAN WG4 Meeting #114	R4-2502135

# Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 		Analysis of BS-to-BS coupling values for the TX IMD requirement optimisation

Agenda Item:	7.13.4.2

Document for:	Discussion

# Introduction

The following WF was Approved during RAN4#113 meeting (Orlando):

## Sub-topic 1-1: CLTA definition Way forward: - It is confirmed that other (than CLTA) test antennas are already allowed by existing network-node test specifications to be used for the OTA co-location requirements testing, but technical description for using another antenna is not defined. - Further investigate horn antennas as alternative (to CLTA) test antenna for high frequency bands. Provide inputs on the following aspects (not limited to): - Horn antenna performance evaluation and comparison with CLTA. - Test setup configuration for horn antennas, e.g. test antenna placement, etc. - Frequency range applicable for horn antennas. - Investigate whether there are feasible solutions to reassure that alternative (to CLTA) test methods provide the same EUT testing results (e.g. how to avoid situation, where one test method would pass, and the other would fail). Note: Analysis on other antennas than horn antennas are not precluded.

## Sub-topic 1-2: BS-to-BS isolation analysis Way forward: - Provide inputs on the proposed BS-to-BS coupling derivation methodology, considering valid co-location deployment scenarios: - Impact of beam steering for coupling analyses, - Investigation on the set of impacted specifications (core, test, etc.), - Investigation on impact for specific requirements (spurious emission, receiver blocking, [TDD OFF power] and TX IMD) - Investigation on the set of impacted OTA co-location test requirements, - Considered deployments to be modelled (horizontal antennas stacking, vertical stacking, etc.), - Other inputs not precluded. - Aim at band-agnostic solution for BS-to-BS isolation analysis, prioritizing FR1 bands (higher bands beyond FR1 as second priority).

## Sub-topic 1-3: TX IMD co-location Way forward: - Decision on potential limitation of TX IMD co-location requirement applicability is postponed till conclusion on BS-to-BS isolation analysis is reached (Sub-topic 1-2). - Applicable frequency limit for the potential removal of TX IMD co-location requirement to be derived based on the BS-to-BS isolation analysis outcomes (Sub-topic 1-2).

In this contribution, we provide summary of the BS coupling values, in order to proceed towards conclusion, and to allow further discussion on the TX IMD requirement.

# Discussion

According to the WF in [1], it was agreed to first conclude on BS-to-BS isolation analysis, before taking any decision on potential limitation of TX IMD co-location requirement applicability (i.e. to limit TX IMD co-location requirement up to certain frequency boundary.

For sake of progress, below we have collected BS-to-BS isolation information shared so far:

1. Huawei [3]: Coupling loss measurement results (3.5 GHz AAS):

| CH24 | CH8 | CH28 | CH12 | CH31 | CH15 |
| --- | --- | --- | --- | --- | --- |
| -60.74 | -63.91 | -53.94 | -54.28 | -42.18 | -45.99 |
| -63.14 | -61 | -54.02 | -55.58 | -49.12 | -47.97 |
| CH0 | CH16 | CH4 | CH20 | CH7 | CH23 |
| -57.81 | -61.95 | -55.79 | -58.07 | -51.06 | -60.3 |
| -54.3 | -59.61 | -52.43 | -52.98 | -44.03 | -47.51 |

2. ZTE [2]: Measurement results:

| Antenna type | 2.6G-BS1 | 2.6G-BS2 | 3.5G-BS1 | 3.5G-BS2 | 4.9G-BS1 |
| --- | --- | --- | --- | --- | --- |
| Isolation in vertical domain; 10 cm (dB) | <-54 | <-57 | <-52 | <-53 | <-56 |
| Isolation in horizontal domain; 10 cm (dB) | <-31 | <-40 | <-34 | <-35 | <-46 |

Simulation results: Based on the initial simulation results for FR1 high bands, coupling loss is expected to be larger than 50dBc regardless from array to array or array to antenna port.

Based on the above, the following minimum coupling loss information values were extracted:

2.6 GHz: 31 dB

3.5 GHz: 34 (horizontal coupling case as worst case) - 42 dB

4.9 GHz: 46 dB

Now comparing those values against the 30 dB coupling limits from legacy conducted requirements, one could draw a conclusion that the TX IMD radiated test may not be needed from 2.6 GHz upwards. However, having in mind that the ultimate goal of this exercise is to reduce requirements applicability and to reduce necessary testing effort, it is advised to apply additional safety offset. Results for 3.5 GHz provide quite wide span, so it may be hard to conclude on it easily. Therefore, it is alternatively suggested to focus on 4.9 GHz: If we would proceed with the TX IMD requirement applicability up to 5 GHz (rounding up 4.9 GHz band), we would have convincing offset (on top of legacy 30 dB conducted coupling assumption) of around 15 dB, which is considered to be large enough to say that above 5 GHz frequency, there is no need for TX IMD radiated requirement.

Proposal 1: Remove TX IMD CLTA test for frequency bands above 4.9 GHz.

# Conclusions

Based on the discussion above discussion, the following proposals were formulated:

Proposal 1: Remove TX IMD CLTA test for frequency bands above 4.9 GHz.

The above conclusion can be updated based on further simulation / measurement data from other companies, if any.

# References

[1] 	R4-2419797	Way Forward for [113][303] NR_BS_RF_Part2_CLTA, RAN4#113

[2]		R4-2419208	Further discussion on OTA test enhancement, RAN4#113, ZTE

[3]		R4-2416279	TX IMD test for high frequency band in FR1, RAN4#112-bis, Huawei